3GPP TSG RAN WG1 #108-e									    R1-2201532

e-Meeting, February 21st – March 8th, 2022

Agenda Item:	8.11.2

Source:	InterDigital, Inc.

Title:	On gNB-designated resources for inter-UE coordination and sensing in SL DRX

Document for:	Discussion and Decision

# Introduction

R17 SL enhancement WID was approved ([1]) at RAN#86 and includes a study for Mode 2 resource allocation enhancement for enhanced reliability and reduced latency. Inter-UE coordination as described below [1] is proposed as a candidate technology for discussion and evaluation in RAN1.

2. Resource allocation enhancement: - Study the feasibility and benefit of solution(s) on the enhancement(s) in mode 2 for enhanced reliability and reduced latency in consideration of both PRR and PIR defined in TR37.885 (by RAN#91), and specify the identified solution(s) if deemed feasible and beneficial [RAN1, RAN2] - Inter-UE coordination with the following. - A set of resources is determined at UE-A. This set is sent to UE-B in mode 2, and UE-B takes this into account in the resource selection for its own transmission. - Note: The solution should be able to operate in-coverage, partial coverage, and out-of-coverage and to address consecutive packet loss in all coverage scenarios. - Note: RAN2 work will start after RAN#89.

At RAN1 #106-e meeting, the following agreement ([2]) was reached regarding the type of resources provided by UE-A in inter-UE coordination Scheme 1.

Agreement For scheme 1, the following inter-UE coordination information signaling from UE-A is supported. FFS details including condition(s)/scenario(s) under which each information is enabled to be sent by UE-A and used by UE-B. - Set of resources preferred for UE-B’s transmission - Set of resources non-preferred for UE-B’s transmission

Sensing in SL DRX was also discussed at RAN1 #106-e meeting in response to a LS from RAN2 ([3]) and the following agreement is reached and responded to RAN2 in a reply LS ([4]).

Agreement A UE can perform SL reception of PSCCH and RSRP measurement for sensing during its SL DRX inactive time. - FFS: When such reception and measurement is performed, whether it is subject to specification, or is up to UE implementation - FFS: Other details

Partial sensing in SL DRX was further discussed at RAN1 #107bis-e meeting and the agreement below was reached regarding UE behavior when the UE is configured with performing partial sensing during SL DRX. Whether a UE performs SL reception of PSCCH and RSRP measurement for partial sensing on slots in SL DRX inactive time can be enabled or disabled by (pre)configuration per resource pool.

Agreement Whether UE performs SL reception of PSCCH and RSRP measurement for partial sensing on slots in SL DRX inactive time is enabled/disabled by (pre-)configuration per resource pool when partial sensing is configured in the UE by a higher layer. - When it is enabled, - When UE performs periodic-based partial sensing for a given Preserve, UE monitors only the default periodic sensing occasion. - When UE performs contiguous partial sensing, UE monitors a minimum of M slots for CPS. - Note, when it is disabled, the UE is not required to perform SL reception of PSCCH and RSRP measurement in SL DRX inactive time. - Note: no further optimization on the resource (re)selection procedure with regard to SL DRX operation is specified in Rel.17. - FFS the case when full sensing is configured in the UE by a higher layer

In this contribution, we present the benefit and feasibility of an additional application for Inter-UE coordination Scheme 1 using resources designated by gNB. Also, we continue analyzing both UE partial and full sensing behavior in SL DRX and its implication to specification.

# Discussion

Inter-UE coordination Scheme 1 based on gNB designated resources

The basic framework of inter-UE coordination is based on the following description in WID [1].

A set of resources is determined at UE-A. This set is sent to UE-B in mode 2, and UE-B takes this into account in the resource selection for its own transmission

The statement of “A set of resources is determined at UE-A” implies that UE-A can be in Mode 2 and the determination of the resources for inter-UE coordination is based on sensing or conflict-detection mechanisms performed autonomously by UE-A. However, while UE-B is explicitly specified as in Mode 2 as described in “This set is sent to UE-B in mode 2” in WID [1], the resource allocation mode of UE-A is not clearly expressed as in Mode 2. It is thus worthwhile to explore further the scenarios and associated benefit of inter-UE coordination with UE-A in Mode 1, specifically for Scheme 1 with a preferred resource set.

A scenario with UE-A in Mode 1 and UE-B in Mode 2 should be considered with regard to partial coverage, which is one of the targeted scenarios by the inter-UE coordination as stated below in WID [1].

“Note: The solution should be able to operate in-coverage, partial coverage, and out-of-coverage and to address consecutive packet loss in all coverage scenarios.”

In a partial coverage, UE-A can be an in-coverage (IC) UE and UE-B an out-of-coverage (OOC) UE. UE-A receives a set of resources from gNB in Mode 1 and provides the gNB-designated resources to UE-B. UE-B’s transmissions using gNB-designated resources will provide better reliability compared to transmissions using Mode 2 resource allocation baseline. It is also conceivable that UE-A performs certain level of determination to modify the gNB-designated resource set, e.g., by removing resources in conflict with its own transmissions (half-duplex issue) before sending the finalized resource set to UE-B.

As the difference between UE-A in Mode 1 or Mode 2 is mainly the way to determine the resources, the on-going RAN1 design of framework and procedure for inter-UE coordination scheme 1, especially related to the container and signalling, can be applicable to UE-A either in Mode 1 or Mode 2. The incremental design and normative work of inter-UE coordination Scheme 1 to include UE-A in Mode 1 can be reasonable and therefore should be considered.

- Proposal 1: Support inter-UE coordination Scheme 1 using a preferred resource set designated by gNB.

The gNB designated resource set can be desired for a unicast transmission between an IC UE-A and OOC UE-B or a groupcast transmission with both IC and OCC member UEs. It is also beneficial for a platoon with all member UEs in coverage. The platoon head receives a set of SL resources or a resource pool in Mode 1 from gNB specifically allocated to the platoon and provide these resources further to all member UEs. As a result, reliable resources become available for the platoon member UEs without signalling overhead (e.g., SL BSR) normally required to acquire these resources in Mode 1 by individual member UE.

Proposal 2: Consider gNB-designated resources at least for partial coverage and platoon scenarios.

Sensing in SL DRX

It was agreed ([2]) that a UE can perform SL reception of PSCCH and RSRP measurement for sensing during inactive time in SL DRX. When sensing and DRX are configured independently, a sensing window can potentially overlap with one or multiple inactive periods. The UE sensing behavior during the inactive time can thus impact both power saving and the PRR performance.

Agreement A UE can perform SL reception of PSCCH and RSRP measurement for sensing during its SL DRX inactive time. - FFS: When such reception and measurement is performed, whether it is subject to specification, or is up to UE implementation - FFS: Other details

Whether this behavior should be specified or up to UE implementation is FFS as indicated in the RAN1 #106e agreement above. In our view, a conceivable consequence of UE implementation is system (PRR) performance degradation due to transmissions using resources with very limited sensing or without sensing. These transmissions can be from UEs who are configured with long DRX inactive duration and determine to perform little or no sensing during the inactive periods to maximize power saving.

Therefore, we think it is important to regulate the UE sensing behavior during inactive periods in specifications to establish a baseline operation followed by all UEs and ensure a minimum system performance. For example, a minimum number of sensing slots can be specified in connection with SL DRX.

Proposal 3: Support specification of SL reception of PSCCH and RSRP measurement for sensing during SL DRX inactive time.

Also, a more granular trade-off between power saving and system performance can be achieved by specifying sensing levels linked to QoS requirements so that high QoS TBs are transmitted with more sensing and less power saving. For example, random selection can be applied when the priority of a TB for transmission is lower than a (pre)configured threshold. The number of sensing slots or the length of sensing window can be associated with QoS requirements to achieve a better balance between power saving and transmission quality.

Proposal 4: A UE in SL DRX performs random resource selection for TBs with priority lower than a (pre)configured threshold.

The UE partial sensing behavior has been agreed as indicated in the agreement below from RAN1 #107bis-e meeting. Further discussion is required to evaluate the UE full sensing behavior during SL DRX. When triggered in slot (n) for resource selection using full sensing, a UE performs available candidate resource determination based on the SL reception of PSCCH and RSRP measurement performed in each slot of a sensing window prior to slot (n). The sensing window is (pre)configured in a resource pool and the window size can be either 100 ms or 1100 ms.

Agreement Whether UE performs SL reception of PSCCH and RSRP measurement for partial sensing on slots in SL DRX inactive time is enabled/disabled by (pre-)configuration per resource pool when partial sensing is configured in the UE by a higher layer. - When it is enabled, - When UE performs periodic-based partial sensing for a given Preserve, UE monitors only the default periodic sensing occasion. - When UE performs contiguous partial sensing, UE monitors a minimum of M slots for CPS. - Note, when it is disabled, the UE is not required to perform SL reception of PSCCH and RSRP measurement in SL DRX inactive time. - Note: no further optimization on the resource (re)selection procedure with regard to SL DRX operation is specified in Rel.17. - FFS the case when full sensing is configured in the UE by a higher layer

With SL DRX, the sensing window may overlap with one or more inactive time periods depending on the SL DRX configuration. To perform SL reception of PSCCH and RSRP measurement in each slot as done in full sensing will significantly reduce the power saving benefit intended by SL DRX. A trade-off is thus desired between the transmission reliability provided by full sensing and power saving achieved by SL DRX. In our view, the agreed UE partial sensing behavior in SL DRX addresses the very trade-off and can be considered as a simple and effective solution. For example, a UE in SL DRX may expect only partial sensing to be configured by a higher layer.

Proposal 5: When a UE is configured with SL DRX, the UE only performs partial sensing.

# Conclusion

In this contribution, we discussed an application of inter-UE coordination using a resource set designated by gNB and the UE sensing behavior during SL DRX inactive periods. We propose the following:

- Proposal 1: Support inter-UE coordination Scheme 1 using a preferred resource set designated by gNB.

Proposal 2: Consider gNB-designated resources at least for partial coverage and platoon scenarios.

- Proposal 3: Support specification of SL reception of PSCCH and RSRP measurement for sensing during SL DRX inactive time.

- Proposal 4: A UE in SL DRX performs random resource selection for TBs with priority lower than a (pre)configured threshold.

Proposal 5: When a UE is configured with SL DRX, the UE only performs partial sensing.

# References

- RP-202846, “WID revision: NR sidelink enhancement”, LG Electronics

- RAN1 #106e Chairman’s Notes, August 2021

- R2-2010961, “LS to RAN1 on SL DRX design”, RAN2

- R1-2108580, “Reply LS on SL DRX design”, RAN1

- RAN1 #107bis-e Chairman’s Notes, January 2022