3GPP TSG RAN WG1 #108-e		  R1- 2201534

e-Meeting, February 21st – March 3rd, 2022

Agenda Item:     8.1.1

Source:	Spreadtrum Communications

Title:	Remaining issues on multi-beam enhancements

Document for:	Discussion and decision

Introduction

In this contribution, we discuss and give our views on some of the remaining issues on multi-beam enhancements.

Discussion

Unified TCI framework

During RAN1#107-e meeting, a working assumption on the coexistence between Rel-17 TCI and Rel-15/Rel-16 TCI/SpatialRelationInfo was achieved.

Working Assumption The UE is not expected to be configured with Rel-15/Rel-16 TCI/SpatialRelationInfo if the UE is configured with Rel-17 TCI in any CC in a band - The CC list for Rel-16 multi-CC beam indication should not contain any CC configured with Rel-17 TCI

Based on the discussion, the concern is about whether to apply the configuration restriction for each band or cross all bands, i.e., whether to keep the words ‘in a band’ or not. In our views, the configuration restriction should be applied for each band. The first reason is that Rel-17 TCI only apply to intra-band CA. Also, if the restriction is applied across all bands, UE cannot be configured with Rel-17 TCI in a band and another feature in another band that only support Rel-15/16 TCI/SpatialRelationInfo, such as M-TRP, positioning, etc.

Besides, an offline discussion was organized before this meeting. Based on the offline discussion, the following proposal was brought up as below.

Offline proposal 1.1: Confirm the following working assumption as an agreement with the following refinement (highlighted in red):

The UE is not expected to be configured with Rel-15/Rel-16 TCI/SpatialRelationInfo/PUCCH-SpatialRelationInfo (except spatialRelationInfoPos) if the UE is configured with Rel-17 TCI in any CC in a band

- The CC list for Rel-16 multi-CC beam indication should not contain any CC configured with Rel-17 TCI assuming different CC lists are used for Rel-16 and Rel-17 (to be confirmed by RAN2)

Based on our analysis, we would like to support it.

Proposal 1: Support the following offline proposal as below:

Confirm the following working assumption as an agreement with the following refinement (highlighted in red):

The UE is not expected to be configured with Rel-15/Rel-16 TCI/SpatialRelationInfo/PUCCH-SpatialRelationInfo (except spatialRelationInfoPos) if the UE is configured with Rel-17 TCI in any CC in a band

- The CC list for Rel-16 multi-CC beam indication should not contain any CC configured with Rel-17 TCI assuming different CC lists are used for Rel-16 and Rel-17  (to be confirmed by RAN2)

Regarding the Rel-17 TCI state indication for CORESET, the following agreement can be found below.

Agreement For Rel-17 unified TCI framework, on applying the indicated Rel-17 TCI state to PDCCH reception and the respective PDSCH reception: - For discussion purposes, define as follows: - ‘CORESET A’: A CORESET other than CORESET#0 associated with only UE-dedicated reception on PDCCH in a CC, comprising CORESETs in association with: - [USS and/or CSS Type 3] - ‘CORESET B’: A CORESET other than CORESET#0 associated with only non-UE-dedicated reception on PDCCH in a CC, comprising CORESETs in association with: - [CSS or CSS other than Type 3] - ‘CORESET C’: A CORESET other than CORESET#0 associated with both UE-dedicated and non-UE-dedicated reception on PDCCH in a CC - CORESET#0 - For Rel-17 TCI state indication, support per CORESET determination as follows: - For any PDCCH reception on a ‘CORESET A’ and the respective PDSCH reception, UE always applies the indicated Rel-17 TCI state. - For any PDCCH reception on a ‘CORESET B’ and the respective PDSCH reception, whether or not UE to apply the indicated Rel-17 TCI state associated with the serving cell is determined per CORESET by RRC - FFS: For intra-cell BM, whether CORESET C is supported or not - If CORESET C is supported, the TCI state of CORESET C - FFS: For inter-cell BM, whether CORESET C is supported or not - If CORESET C is supported, the TCI state of CORESET C - FFS: The TCI state of CORESET 0

If a CORESET is associated with both USS and CSS, in our views, the indicated Rel-17 TCI state should be applied to both USS and CSS. Otherwise, gNB should not configure CSS and USS to be associated with the same CORESET.

Regarding CORESET#0, in our views, since Rel-15 NR already support MAC CE based TCI state update for CORESET#0, which means CORESET#0 can be treated the same as the other CORESETs, i.e., transmitted by UE-specific beams. Therefore, in Rel-17, if CORESET#0 is associated with USS, the indicated Rel-17 TCI state should be applied.

Regarding Type 3 CSS, we slightly prefer to handle it like other CSS Types. The reason is that gNB can configure the Type 3 CSS to apply the indicated Rel-17 TCI state by RRC, if needed.

Therefore, we have the following proposal.

Proposal 2: For Rel-17 TCI state indication for CORESET,

- CORESET#0 is handled the same way like ‘CORESET A’

- If USS and CSS are associated with the same CORESET, both USS and CSS should apply the indicated Rel-17 TCI state

- Type 3 CSS is handled the same way like other CSS Types

Dynamic TCI state update signaling medium

Based on the following agreements, RAN1 has agreed to support DCI format 1_1/1_2 without DL assignment for common beam indication. Also, during the last meeting, the differentiation of UL/DL TCI state is achieved by MAC CE signaling design.

Agreement For beam indication with Rel-17 unified TCI, support DCI format 1_1/1_2 without DL assignment: - Use ACK/NACK mechanism analogous to that for SPS PDSCH release with both type-1 and type-2 HARQ-ACK codebook: - Upon a successful reception of the beam indication DCI, the UE reports an ACK - Note that upon a failed reception of the beam indication DCI, a NACK can be reported. - For type-1 HARQ-ACK codebook, a location for the ACK information in the HARQ-ACK codebook is determined based on a virtual PDSCH indicated by the TDRA field in the beam indication DCI, based on the time domain allocation list configured for PDSCH - For type-2 HARQ-ACK codebook, a location for the ACK information in the HARQ-ACK codebook is determined according to the same rule for SPS release - The ACK is reported in a PUCCH k slots after the end of the PDCCH reception where k is indicated by the PDSCH-to-HARQ_feedback timing indicator field in the DCI format, or provided dl-DataToUL-ACK or dl-DataToUL-ACK-ForDCI-Format1-2-r16 if the PDSCH-to-HARQ_feedback timing indicator field is not present in the DCI - When used for beam indication: - CS-RNTI is used to scramble the CRC for the DCI - The values of the following DCI fields are set as follows: - RV = all ‘1’s - MCS = all ‘1’s - NDI = 0 - Set to all ‘0’s for FDRA Type 0, or all ‘1’s for FDRA Type 1, or all ‘0’s for dynamicSwitch (same as in Table 10.2-4 of TS38.213) - FFS: Whether HPN is also used - Use the existing TCI field (always present) to signal the following: 1) Joint DL/UL TCI state, 2) DL-only TCI state (for separate DL/UL TCI), 3) UL-only TCI state (for separate DL/UL TCI) - FFS: Whether both DL TCI and UL TCI states can be signaled in one instance of beam indication DCI - FFS: Relation with joint vs separate TCI (DL and/or UL) switching, including M/N>1 if supported - In addition, use the following DCI fields as the fields are being used in Rel-16: - Identifier for DCI formats - Carrier indicator - Bandwidth part indicator - TDRA - Downlink assignment index (if configured) - TPC command for scheduled PUCCH - PUCCH resource indicator - PDSCH-to-HARQ_feedback timing indicator (if present) - The remaining unused DCI fields and codepoints are reserved in R17 - Support UE to report whether or not to support TCI update by DCI format 1_1/1_2. - For a UE supporting TCI update by DCI format 1_1/1_2, it must support TCI update by using DCI 1_1/1_2 with DL assignment, and support of the above feature for TCI update by DCI format 1_1/1_2 without DL assignment is UE optional - FFS: How to handle the case when there is only UL data - FFS: The case for UE being indicated with separate UL TCI in DCI format 1_1/1_2 with DL assignment - FFS: When more than one TCI codepoints are activated by MAC CE, the activated TCI state(s) for the lowest codepoint is/are applied - Support of this feature is UE optional - The “lowest codepoint” function can be configured on or off. - FFS: Interaction with the DCI based beam update if needed, whether/how to support the case with M or N > 1 if supported - Note: This agreement on DCI beam indication design is not to be used to be against the support of the cases of M/N>1. The support of M/N>1 will be separately discussed and not dependent on the decision here. Agreement For M=N=1, on Rel-17 unified TCI, for separate DL/UL TCI, one instance of beam indication using DCI formats 1_1/1_2 (with and without DL assignment) can be used as follows: - One TCI field codepoint represents a pair of DL TCI state and UL TCI state. If the DCI indicates such a TCI field codepoint, the UE applies the corresponding DL TCI state and UL TCI state. - One TCI field codepoint represents only a DL TCI state. If the DCI indicates such a TCI field codepoint, the UE applies the corresponding DL TCI state, and keeps the current UL TCI state. - One TCI field codepoint represents only an UL TCI state. If the DCI indicates such a TCI field codepoint, the UE applies the corresponding UL TCI state, and keeps the current DL TCI state. FFS: the cases of M or N>1

There’s an FFS on the HPN field. Based on our understanding, at least for M/N=1, the currently agreed DCI field design can already support the intended functionality. Similar as for R15 SPS release, HPN field can be fixed as all ‘0’s, and further extension can be discussed later.

Proposal 3: At least for M/N=1, HPN field is fixed as all ‘0’s.

MP-UE assumption to facilitate fast UL panel selection

During RAN1#107e meeting, the following consensus on MP-UE have been achieved.

Working Assumption Support the UE reporting a list of UE capability value sets - Each UE capability value set comprises the max supported number of SRS ports - For any two different value sets, at least one capability value needs to be different - FFS: If in addition also identical value sets are allowed. - Whether the UE capability value set can be common across all BWPs/CCs in same band or BC can be discussed in UE feature session Conclusion On Rel.17 enhancements to facilitate UE -initiated panel activation and selection via UE reporting a list of UE capability value sets, other than the max supported number of SRS ports (note: currently pending endorsement in proposal 4.A), there is no consensus on supporting another UE capability type Agreement On Rel.17 enhancements to facilitate UE -initiated panel activation and selection via UE reporting a list of UE capability value sets, the correspondence between each reported CSI-RS and/or SSB resource index and one of the UE capability value sets in the reported list is determined by the UE (analogous to Rel-15/16) and is informed to NW in a beam reporting instance. - The Rel-15/16 beam reporting framework is used, i.e. the index of corresponding UE capability value set is reported along with the pair of SSBRI/CRI and L1-RSRP/SINR (up to 4 pairs, with 7-bit absolute and 4-bit differential) in the beam reporting UCI and down select (maintenance) between the following two options: - Option 1: UE can report one index for all the reported CRIs/SSBRIs in one beam reporting - Option 2: UE can report one index for each reported CRI/SSBRI in one beam reporting. - FFS: whether/how to take DL-only panel into account in the report - FFS: Time-domain behaviour, e.g. the support periodic, semi-persistent, and aperiodic reporting - FFS: Semi-persistent and/or aperiodic reporting is triggered only when periodic reporting is configured - (Working assumptions): Support acknowledgement mechanism of the reported correspondence from NW to UE, which doesn't preclude reusing/reinterpreting existing signaling/procedure - FFS (maintenance): the application time for the reported correspondence (if any), the exact acknowledgement mechanism and whether spec impact is needed, e.g. based on TCI state update, BFR response like mechanism, including the application time for the reported correspondence, if any - No new DCI format and no new RNTI are introduced for this function.

Regarding the UE capability value set, since there’s no consensus on supporting another UE capability type, the UE capability value set will only contain the max supported number of SRS ports. Therefore, UE capability value set should be revised to UE capability value.

For the FFS on whether to allow reporting multiple identical capability value sets, it depends on how to report the association between CRI/SSBRI and the capability value set. Based on the agreement, UE should report the index of corresponding UE capability value set, rather than the value from the set. Therefore, it’s not necessary to restrict that at least one capability value for two different value sets needs to be different.

Proposal 4: Confirm the Working assumption with the following update.

Support the UE reporting a list of UE capability values sets

- Each UE capability value set comprises represents the max supported number of SRS ports

- The reported UE capability values within the list can be the same or different For any two different values sets, at least one capability value needs to be different

  - FFS: If in addition also identical value sets are allowed.

- Whether the UE capability values set can be common across all BWPs/CCs in same band or BC can be discussed in UE feature session

According to the agreement, there are two options on the reporting of the index of corresponding UE capability value set. From UE implementation point of view, if UE measured the beams with more than one panels, it’s highly possible that the first K strongest beams are measured by different panels. Besides, in order to achieve forward compatibility for simultaneous multi-panel UL transmission, it’s better to report candidate beams from different panels in one reporting instance.

Proposal 5: Support Option 2: UE can report one index for each reported CRI/SSBRI in one beam reporting.

Conclusion

In this contribution, we gave our views on the potential issues on multi-beam enhancements. The following proposals are achieved:

Proposal 1: Support the following offline proposal as below:

Confirm the following working assumption as an agreement with the following refinement (highlighted in red):

The UE is not expected to be configured with Rel-15/Rel-16 TCI/SpatialRelationInfo/PUCCH-SpatialRelationInfo (except spatialRelationInfoPos) if the UE is configured with Rel-17 TCI in any CC in a band

- The CC list for Rel-16 multi-CC beam indication should not contain any CC configured with Rel-17 TCI assuming different CC lists are used for Rel-16 and Rel-17  (to be confirmed by RAN2)

Proposal 2: For Rel-17 TCI state indication for CORESET,

- CORESET#0 is handled the same way like ‘CORESET A’

- If USS and CSS are associated with the same CORESET, both USS and CSS should apply the indicated Rel-17 TCI state

- Type 3 CSS is handled the same way like other CSS Types

Proposal 3: At least for M/N=1, HPN field is fixed as all ‘0’s.

Proposal 4: Confirm the Working assumption with the following update.

Support the UE reporting a list of UE capability values sets

- Each UE capability value set comprises represents the max supported number of SRS ports

- The reported UE capability values within the list can be the same or different For any two different values sets, at least one capability value needs to be different

  - FFS: If in addition also identical value sets are allowed.

- Whether the UE capability values set can be common across all BWPs/CCs in same band or BC can be discussed in UE feature session

Proposal 5: Support Option 2: UE can report one index for each reported CRI/SSBRI in one beam reporting.