3GPP TSG RAN WG1 #108-e		R1-2201540

e-Meeting, February 21st – March 3rd, 2022

Agenda Item:     8.1.4

Source:	Spreadtrum Communications

Title:	Discussion on CSI enhancements for M-TRP and FR1 FDD reciprocity

Document for:	Discussion and decision

Introduction

In this paper, we will present our opinions on CSI enhancement for multi-TRP transmission, and provide some text proposals.

Discussion

CSI enhancement for M-TRP transmission

In RAN1#103e meeting [1], we have achieved one agreement to state that CMRs corresponding to different TRPs respectively have the same number of ports among CMRs. But we notice that it has not captured into specification. So we suggest to have the following proposal.

Agreement For CSI measurement associated to a reporting setting CSI-ReportConfig for NCJT, [at least for multi-DCI based and single-DCI based schemes (scheme 1a)], NZP CSI-RS resources for channel measurement are associated to different TRPs/TCI states at resource level - CMRs corresponding to different TRPs respectively shall be configured within the same resource set (i.e. scheme 1-2) and have the same number of ports among CMRs. - At least ‘typeI-SinglePanel’ codebook is supported - FFS: Other codebook types - Note that RAN1 shall strive to finalize NCJT CSI enhancement with single reporting setting firstly. - The support of larger than 32 ports across two CMRs is optional for a UE supporting Rel. 17 mTRP CSI

Proposal 1: Suggest to adopt the following text proposal in 38.214.

------------------------------------------Start of Text Proposal#1 for TS 38.214--------------------------------------

- 5.2.1.4.2	Report Quantity Configurations

-----------------------------Unchanged part omitted--------------------------

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI', or 'cri-RI-LI-PMI-CQI' and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is configured with Ks≥2 resources, two Resource Groups with K1≥1 resources in Group 1, K2≥1 resources in Group 2, K1+K2=Ks, and N Resource Pairs:

-	each resource can contain, subject to UE capability, at most 32 CSI-RS ports.

-	each of the N Resource Pairs is associated to a CRI value.

-    CMR resources for one Resource Pair have the same number of ports.

------------------------------------------End of Text Proposal#1 for TS 38.214--------------------------------------

In RAN1#106b-e [2], we have achieved one agreement to state that CMRs corresponding to different TRPs respectively to be restricted within X continuous slot(s) without DL/UL switch between two CMRs. But we notice that the part ‘without DL/UL switch between two CMRs’ has been lost in the specification. So we suggest to have the following proposal.

Agreement For CSI measurement associated with a CSI-ReportingConfig for NCJT, support two CMRs within the same CMR pair configured for NCJT measurement hypothesis to be restricted within X continuous slot(s) without DL/UL switch between two CMRs - X=1, 2 - whereas X=1 implying the same slot and X=2 implying two adjacent slots - FFS other restrictions for FR2 - FFS whether UE capability is needed for X=2

Proposal 2: Suggest to adopt the following text proposal in 38.214.

------------------------------------------Start of Text Proposal#2 for TS 38.214--------------------------------------

- 5.2.2.3.1	NZP CSI-RS

-----------------------------Unchanged part omitted--------------------------

For a CSI-RS Resource Set for channel measurement configured with two Resource Groups and N Resource Pairs, the slot offsets of the two resources in a Resource Pair are configured within X∈{1,[2]} slots, without DL/UL switching in between the two resources, where X=1 implies that the two resources are configured in the same slot without DL/UL switching, and X=2 implies that the two resources are configured within two adjacent slots without DL/UL switching.

------------------------------------------End of Text Proposal#2 for TS 38.214--------------------------------------

Conclusion

In this contribution, we discussed some issues on CSI enhancement for M-TRP, and provide some text proposals.

Proposal 1: Suggest to adopt the following text proposal in 38.214.

------------------------------------------Start of Text Proposal#1 for TS 38.214--------------------------------------

- 5.2.1.4.2	Report Quantity Configurations

-----------------------------Unchanged part omitted--------------------------

If the UE is configured with a CSI-ReportConfig with the higher layer parameter reportQuantity set to 'cri-RI-PMI-CQI', or 'cri-RI-LI-PMI-CQI' and the corresponding NZP-CSI-RS-ResourceSet for channel measurement is configured with Ks≥2 resources, two Resource Groups with K1≥1 resources in Group 1, K2≥1 resources in Group 2, K1+K2=Ks, and N Resource Pairs:

-	each resource can contain, subject to UE capability, at most 32 CSI-RS ports.

-	each of the N Resource Pairs is associated to a CRI value.

-    CMR resources for one Resource Pair have the same number of ports.

------------------------------------------End of Text Proposal#1 for TS 38.214--------------------------------------

Proposal 2: Suggest to adopt the following text proposal in 38.214.

------------------------------------------Start of Text Proposal#2 for TS 38.214--------------------------------------

- 5.2.2.3.1	NZP CSI-RS

-----------------------------Unchanged part omitted--------------------------

For a CSI-RS Resource Set for channel measurement configured with two Resource Groups and N Resource Pairs, the slot offsets of the two resources in a Resource Pair are configured within X∈{1,[2]} slots, without DL/UL switching in between the two resources, where X=1 implies that the two resources are configured in the same slot without DL/UL switching, and X=2 implies that the two resources are configured within two adjacent slots without DL/UL switching.

------------------------------------------End of Text Proposal#2 for TS 38.214--------------------------------------

Reference

- Draft_Minutes_report_RAN1#103-e.

- Draft_Minutes_report_RAN1#106bis-e.

- 3GPP TS 38.214: "NR; Physical layer procedures for data", h00