3GPP TSG RAN WG1 #108-e                                                       R1-2201549

e-Meeting, February 21st – March 3rd, 2022

Agenda Item:	8.6.1.1

Source:	Spreadtrum Communications

Title:	Discussion on aspects related to reduced maximum UE bandwidth

Document for:	Discussion and decision

# Introduction

This contribution discusses the aspects related to the reduced maximum UE bandwidth. Hereafter, the terminology “initial BWP” means the BWP with index 0 (i.e. BWP#0) including the BWP#0 configured by dedicated-RRC for unicast purpose (i.e. BWP#0 configuration option 2).

In this contribution, “during initial access” means the stage before Msg4 (including Msg4), and “after initial access” means the stage after Msg4.

# RF retuning

## RF retuning for FH and “fast” BWP switch

Some companies proposed to support RF-based Frequency Hopping (FH), in a bandwidth wider than the RedCap UE bandwidth for RedCap UE. As another form of frequency hopping, some companies also proposed to support BWP switch based FH (i.e. “fast” BWP switch) in a bandwidth wider than the RedCap UE bandwidth for RedCap UE.

### Out of the scope of RedCap WID

The motivation is to exploit the frequency diversity gain for coverage improvement. However, coverage improvement dedicated for RedCap UE should be discussed in the coverage recovery subtopic. As shown below for RAN1#103e agreement, RF-based FH or “fast” BWP switch has been captured as a candidate scheme for the coverage recovery subtopic.

Agreements: Capture the following to the TR 38.875 - Coverage recovery for PUSCH was studied from several aspects, including cross-slot or cross-repetition channel estimation, lower DM-RS density in time domain, enhancements on PUSCH repetition Type A and/or Type B, frequency hopping or BWP switching across a larger system bandwidth - Some techniques, such as cross-slot or cross-repetition channel estimation, lower DM-RS density in time domain, enhancements on PUSCH repetition Type A and/or Type B have been studied also in the Rel-17 coverage enhancement SI - Potential specification impacts of frequency hopping or BWP switching across a larger system bandwidth include: - Frequency domain hopping offsets/positions - Faster switching/RF retuning time. - Note this aspect requires RAN4 involvement, where the corresponding study in RAN4 is not performed yet. - Transmission/reception interruption during RF retuning time Agreements: Capture the following to the TR 38.875 - Coverage recovery for PDSCH was studied from several aspects, including the use of the lower-MCS table, larger aggregation factor for PDSCH reception, cross-slot or cross-repetition channel estimation, increasing the granularity of PRB bundling, frequency hopping or BWP switching across a larger system bandwidth. - Some techniques, such as the lower-MCS table and larger aggregation factor for PDSCH reception are existing techniques with optional UE capability signaling - If cross-slot or cross-repetition channel estimation for PDSCH is supported, potential specification impacts include: - Time-domain precoder cycling and DM-RS configuration - If hopping or BWP switching across a larger system bandwidth is supported, potential specification impacts include - PDSCH hopping configuration - Faster switching/RF retuning time - Note this aspect requires RAN4 involvement, where the corresponding study in RAN4 is not performed yet. - Transmission/reception interruption during RF retuning time - Potential specification impacts of increasing the granularity of PRB bundling include - Related signaling design

On the other hand, at least for some UL channels (PUSCH/PUCCH/Msg3), the coverage enhancement is being discussed in the CE topic. The outcome of CE topic can be applied by RedCap UE, which can have more promising coverage gain than RF-based FH or “fast” BWP switch.

Therefore, whether to support RF-based FH or “fast” BWP switch in a bandwidth wider than the RedCap UE bandwidth for RedCap UE is actually out of the scope of the RedCap WID objectives.

Observation 1: Whether to support RF-based FH or “fast” BWP switch in a wider bandwidth than the RedCap UE bandwidth is out of the scope of the RedCap WID objectives.

Moreover, if the coverage recovery is approved as one of the RedCap WID objectives, other schemes, e.g. repetition, could be more efficient than RF-based FH or “fast” BWP switch. In other words, the priority of the schemes of the coverage recovery should be discussed before specifying RF-based FH or “fast” BWP switch.

Observation 2: If the coverage recovery subtopic is open to discussion, whether RF-based FH or “fast” BWP switch in a wider bandwidth than the RedCap UE bandwidth is prioritized than other schemes, e.g. repetition, should be further discussed.

### Technical concerns

We have the following technical concerns on “fast” BWP switch and RF retuning in a wider bandwidth.

- Co-scheduling RedCap UE and non-RedCap UE is problematic, because “fast” BWP switch and RF retuning need the symbol-level gap.

- The time gap for RF retuning will cancel the frequency diversity gain considering the time-domain resource overhead.

- Power consumption at RedCap UE is high, because power is consumed in each retuning, e.g. PLL adjustment, filter adjustment.

In addition, we have the following technical concerns dedicatedly on RF retuning in a wider bandwidth.

- Cost/complexity consumption at RedCap UE is high, because new UE behavior is needed, e.g. the timely PRB-level centre frequency retuning.

- The phase pre-compensation based on upconversion in 38.211 should be adjusted timely at UE side.

- The signalling for transmitter to inform receiver about the location of Direct Current (DC) is meaningless, e.g. UplinkTxDirectCurrentList and txDirectCurrentLocation, and the algorithm at receiver to mitigate the DC interference is also meaningless.

- NR UE does not operate in a BWP wider than the bandwidth it supports. If this law is broken, there could be spec modifications everywhere. It should be noted that frequency hopping in current spec just means the baseband based frequency hopping within a BWP.

Moreover, some companies observed the gain of frequency hopping in a wider bandwidth is marginal.

- The frequency diversity gain across 100MHz bandwidth (for FR1) may be marginal compared with that across 20MHz bandwidth, even when the number of the allocated PRBs is small.

- The frequency selective (gNB always allocating the best subband) gain may be too hard to be achieved, since UE cannot report the CQI of subband in fine time and frequency granularity.

Some companies have concern on UL resource fragmentation if the separate initial UL BWP is configured for RedCap UE, which is a typical implementation without RF retuning. However, if the shared initial UL BWP is configured and RF retuning is applied for RedCap UE, the time gap only for RedCap UE will still cause UL resource fragmentation.

In some companies’ simulation, the frequency diversity gain across 400MHz (for FR2) is obvious compared with that across 100MHz, if small data bandwidth is assumed. In FR2, the delay spread of channel is small, and the coherence bandwidth is wide, so frequency hopping across 100MHz may have much lower frequency diversity gain than frequency hopping across 400MHz. However, in our view, gNB can configure 4 BWPs (including BWP#0) scattered in 400MHz carrier and trigger BWP switch timely to exploit the frequency diversity gain. On other hand, coverage can be improved by features of CE topic. We think the features in CE topic is more friendly for UE implementation.

### Time gap for RF retuning

In [1], transition time for UE RF bandwidth adaptation has been provided from RAN4 perspective. The transition time is about 20 microseconds if the center frequency before and after the bandwidth adaptation is the same, and the transition time is about 50~200 microseconds if the center frequency before and after the bandwidth adaptation is different.

In our view, roughly, the time gap for RF retuning could be 1~3 symbols for 15kHz SCS and 2~6 symbols for 30kHz SCS. If we send LS to RAN4 for delay of RF retuning, based on [1], it is highly possible that RAN4 would reply the symbol-level delay is feasible for RF retuning. However, from RAN1 perspective, we still cannot see the necessity to introduce RF retuning (or new BWP switch) delay.

Furthermore, the cost of the RedCap UE will increase, if the RedCap UE has stringent pipeline.

Moreover, the majority companies shown concerns on RF retuning for misalignment of centre frequency between DL and UL, it can be read that it is majority view not supporting RF retuning within DL or UL.

Therefore, we think it is not necessary to send LS to RAN4 for RF retuning.

Proposal 1: It is not necessary to send LS to RAN4 for RF retuning.

## RF retuning for SSB processing

Whether the SSB is present in the active DL BWP is a discussion point. We have some analysis on RedCap UE behaviors and spec impact in the section.

### After initial access

After initial access before gNB receives the UE capability, gNB may not know the RedCap UE’s capability. For simplicity, here “after initial access” can be understood as the stage after gNB knows the RedCap UE’s capability.

#### AGC/sync/measurement

It is related to capabilities like FG 6-1 and FG 6-1a. We will discuss them under capability FG 6-1 and FG 6-1a respectively.

#### Capability FG 6-1

FG 6-1 is listed as follows.

FG 6-1 1) 1 UE-specific RRC configured DL BWP per carrier 2) 1 UE-specific RRC configured UL BWP per carrier 3) RRC reconfiguration of any parameters related to BWP 4) BW of a UE-specific RRC configured BWP includes BW of CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP includes SSB for SCell if there is SSB on SCell

Because SSB is contained in the active DL BWP, RedCap UE (capability FG 6-1 is mandatory) use SSB for AGC/sync/measurement.

#### Capability FG 6-1a

FG 6-1a are listed as follows.

FG 6-1a BW of UE-specific RRC configured BWP may not include BW of the CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP may not include SSB for SCell

A non-RedCap UE capable of FG 6-1a can implement FG 6-1a by two ways.

- Way-1: The non-RedCap UE can retune RF to process SSB/CORESET#0 outside the UE-specific BWP.

- Way-2: The non-RedCap UE can open a wide RF bandwidth to include the UE-specific BWP and SSB/CORESET#0, and the non-RedCap UE can receive data and SSB simultaneously even when they have different numerologies.

For Way-1, it implies the RedCap UE should also retune RF. For Way-2, it is not applicable for the RedCap UE.

Observation 3: In context of RedCap, FG 6-1a means the RedCap UE should retune RF to process SSB/CORESET#0 outside the UE-specific BWP.

If the RedCap UE should retune RF for SSB-based AGC/sync/measurement, it will cause the additional operations/complexities. In addition, the RF retuning will cause the time gap, e.g. measurement gap and scheduling gap. Moreover, FG 6-1a has FG 6-2, 6-3 or 6-4 as the pre-requisite (see Appendix A.2), which implies that FG 6-1a has the highest complexity among FGs of BWP operations.

#### CORESET#0 for FG 6-x

FG 6-1/6-1a/6-2/6-3/6-4 [2] all mention CORESET#0 to address CORESET of CSS (e.g. Type1-PDCCH CSS) in the UE-specific DL BWP, but it is common understanding the active DL BWP for RedCap UE may not contain CORESET#0. There could be two methods to solve this issue.

- Method-1: Remove “CORESET#0” in FG 6-1/6-1a/6-2/6-3/6-4. It means RedCap UE should monitor CSS PDCCH, e.g. SIB1/SIBx/RAR/paging, in the shared initial DL BWP which contains CORESET#0. The corresponding RF retuning is infrequent which can be acceptable.

- Method-2: Add a note in FG 6-1/6-1a/6-2/6-3/6-4, which is “For RedCap UE, CORESET#0 here means CORESET#0 or CORESET of CSS”. It means RedCap UE can monitor CSS PDCCH in the active DL BWP and RF retuning is not necessary.

Proposal 2: Remove “CORESET#0” or add a note in FG 6-1/6-1a/6-2/6-3/6-4. The note is “For RedCap UE, CORESET#0 here means CORESET#0 or CORESET of CSS”.

#### Capability “no gap”

The following specification in 38.133 shows an example of the definition of measurement gap for SSB based intra-frequency measurement.

A measurement is defined as a SSB based intra-frequency measurement provided the centre frequency of the SSB of the serving cell indicated for measurement and the centre frequency of the SSB of the neighbour cell are the same, and the subcarrier spacing of the two SSBs are also the same. The UE shall be able to identify new intra-frequency cells and perform SS-RSRP, SS-RSRQ, and SS-SINR measurements of identified intra-frequency cells if carrier frequency information is provided by PCell or the PSCell, even if no explicit neighbour list with physical layer cell identities is provided. The UE can perform intra-frequency SSB based measurements without measurement gaps if - the UE indicates ‘no-gap’ via intraFreq-needForGap for intra-frequency measurement, or - the SSB is completely contained in the active BWP of the UE, or - the active downlink BWP is initial BWP[3]. For intra-frequency SSB based measurements without measurement gaps, UE may cause scheduling restriction as specified in clause 9.2.5.3.

For the first condition (capability “no-gap”), it may mean that the non-RedCap UE should implement Way-2 as mentioned above. However, it is not applicable for the RedCap UE.

For the second condition (SSB in the active BWP), it is fine.

For the third condition (initial BWP), we can consider it for FR1 and FR2 respectively.

- For FR1

  - For non-RedCap UEs, the “initial BWP” can be the MIB-configured initial DL BWP or RRC configured initial DL BWP. It should be noted that RRC configured initial DL BWP contains CORESET#0 as the following 38.331 Text, which also contains SSB for FR1 due to multiplexing pattern 1.

The network configures the locationAndBandwidth so that the initial downlink BWP contains the entire CORESET#0 of this serving cell in the frequency domain.

  - For RedCap UEs, the “initial BWP” should mean the MIB-configured initial DL BWP, since the separate (RRC-configured) initial DL BWP may not contain SSB.

- For FR2

  - For non-RedCap UEs, although CORESET#0 does not contain SSB for multiplexing pattern 2/3, it is fine, since non-RedCap can open RF to cover both SSB and CORESET#0 in FR2.

  - For RedCap UEs, it is not true. However, RAN1 had conclusion in RAN1#104-e not to consider this issue. Hence, it is left to the implementation of RedCap UEs for FR2. The RF retuning may be implemented for FR2 for RedCap UEs, and complexity, power and latency efficiencies are not addressed for FR2 in common understanding shown as follows.

Conclusion: RAN1 does not consider acquisition time improvements for FR2 RedCap UEs with SSB and CORESET#0 multiplexing patterns 2 and 3 as part of this WI.

Therefore, the third condidition is not practical for RedCap UEs.

Observation 4: There is only one condition for “without measurement gap” in 38.133 for RedCap UEs, i.e. “the SSB is completely contained in the active BWP of the UE”.

Due to the abovementioned new RedCap UE behaviors for SSB processing, we suggest revisiting the conditions for “without measurement gap”.  If it is a problem, RAN1 can send LS to RAN2/4 for discussion.

Proposal 3: Conditions for “without measurement gap” in 38.133 should be revisited for RedCap UEs.

### During initial access

#### AGC/sync/measurement

RedCap UEs should perform AGC/sync/measurement with SSB during initial access. If there is no SSB contained in the initial DL BWP, RedCap UE should retune RF to perform AGC/sync/measurement and gaps for RF retuning should be maintained at RedCap UEs.

Observation 5: Gaps due to RF retuning for AGC/sync/measurement should be considered for discussion of the separate initial DL BWP during initial access.

### SSB/CORESET#0 multiplexing pattern 2/3

For FR2, CORESET#0 does not contain SSB for SSB/CORESET#0 multiplexing pattern 2/3. It is not an issue for non-RedCap UE, since non-RedCap UE can open RF containing both SSB and CORESET#0. However, RedCap UE cannot open RF containing both SSB and CORESET#0. In our view, RedCap UE needs to retune RF, but gNB can avoid selecting the entries that exceeds 100MHz BW containing both SSB and CORESET#0 for SSB/CORESET#0 multiplexing pattern 2/3 in FR2.

Observation 6: gNB can avoid selecting the entries that exceeds 100MHz BW containing both SSB and CORESET#0 for SSB/CORESET#0 multiplexing pattern 2/3 in FR2.

## RF retuning between DL/UL

For the current TDD system, the initial DL BWP should share the same center frequency with the initial UL BWP to avoid the RF retuning between DL and UL (TS 38.213).

For unpaired spectrum operation, a UE does not expect to receive a configuration where the center frequency for a DL BWP is different than the center frequency for an UL BWP when the BWP-Id of the DL BWP is same as the BWP-Id of the UL BWP.

### Center frequencies alignment

For the non-initial DL/UL BWP, it is common understanding that the center frequencies of DL BWP and UL BWP are the same for RedCap UEs. For the SIB1-configured initial DL/UL BWP, it is also common understanding that the center frequencies of DL BWP and UL BWP are the same for RedCap UEs.

Therefore, it is still majority view that the center frequencies of initial DL BWP and initial UL BWP are the same for RedCap UEs in most cases. In RAN1#106bis-e, it was agreed that the center frequencies of initial DL BWP and initial UL BWP are the same during random access when the initial DL BWP includes CD-SSB and the entire CORESET#0.

Agreement For FR1, - For TDD, center frequencies are assumed to be the same for the initial DL (FFS: if it does not include CD-SSB and the entire CORESET#0) and UL BWPs used during random access for RedCap UEs. - FFS: For Option 1 and Option 2, whether the case that the center frequencies are different is also supported, and whether RedCap UE can expect CD-SSB and CORESET#0 in this case - For TDD, center frequencies are assumed to be the same for non-initial DL and UL BWPs with the same BWP id for a RedCap UE.

### Exceptional case

However, for the MIB-configured initial DL BWP (i.e. CORESET#0) and the SIB1-configured initial UL BWP during random access, it is not the common understanding that the center frequencies are the same or can be different. It has been discussed in R15 starting from RAN1#95. In RAN1#98, there was a conclusion that the misalignment is allowed.

Conclusion: - For unpaired spectrum, the center frequencies of CORESET#0 and the initial DL/UL BWP configured by SIB1 can be the same or different. - This does not change the following RAN1 agreement Agreements in RAN1#94: - For PCell, the initial DL BWP can be configured in SIB1 to be the same as or different with the initial DL BWP as initially defined by CORESET#0 - The initial DL BWP configured in SIB1 includes the bandwidth of CORESET#0 - If the initial DL BWP configured by SIB1 is different with the initial DL BWP as initially defined by CORESET#0, the configuration of the initial DL BWP configured by SIB1 is applicable after the initial access

Behind the conclusion, it is common understanding that the non-RedCap UE can handle the case of the misalignment, e.g. by opening the wider RF bandwidth or performing one RF retuning after Msg4. For RedCap UE, it can perform one RF retuning in a certain timing, e.g. after Msg4 (with the shared initial DL/UL BWP) or during random access (when the separate initial UL BWP is used). We think this conclusion can be kept for RedCap UEs.

### Extending cases for center frequencies alignment

Along the progress of discussion on the separate initial DL BWP, the separate initial DL BWP in companies mind may not include CD-SSB and the entire CORESET#0 and can be used by RedCap UEs during random access. We think the above agreement can be extended to the case when the separate initial DL BWP does not include CD-SSB and the entire CORESET#0.

Proposal 4: For TDD, center frequencies are the same for the initial DL and UL BWP during random access for RedCap UEs, no matter whether or not it includes CD-SSB and the entire CORESET#0 or not.

## Comparison among RF retuning levels

From above discussion, we have the following comparison among RF retuning levels.

Table 1: Comparison among RF retuning levels

|  | RF retuning for FH and “fast” BWP switch | RF retuning for SSB processing | RF retuning between DL/UL | No RF retuning (SSB in the DL BWP, alignment b/w DL/UL) |
| --- | --- | --- | --- | --- |
| UE complexity | High | Medium | Medium | No additional complexity |
| Time gap | Symbol level | Slot level | Slot level | No time gap |

# Initial UL BWP

## Solve RO issue by separate initial UL BWP

In RAN1#106-e [3], it was firmed that the issue of RO outsider the RedCap UE bandwidth can be solved by the separate initial UL BWP.

Agreement Confirm the following working assumption from RAN1#105-e regarding RACH occasions. - For enabling/supporting that the RACH occasion (RO) associated with the best SSB falls within the RedCap UE bandwidth, support separate initial UL BWP for RedCap UEs (which is not expected to exceed the maximum RedCap UE bandwidth), and this separate initial UL BWP for RedCap includes ROs for RedCap UEs. - Note: these ROs can be dedicated for RedCap UEs or shared with non-RedCap UEs.

## Solve broadcast PUSCH/PUCCH issue by separate initial UL BWP

Since the separate initial UL BWP was working assumption, the issue of broadcast PUSCH/PUCCH outsider the RedCap UE bandwidth can be solved by the separate initial UL BWP naturally. This became a working assumption in RAN1#105-e [4].

Working assumption: - For enabling/supporting that PUCCH (for Msg4/[MsgB] HARQ feedback) and/or PUSCH (for Msg3/[MsgA]) transmissions fall within the RedCap UE bandwidth during initial access, support separate initial UL BWP for RedCap UEs (which is not expected to exceed the maximum RedCap UE bandwidth). - FFS: whether/how the specification also supports separate PUCCH/Msg3/[MsgA] PUSCH configuration/indication or a different interpretation of the same configuration/indication for RedCap (e.g., disabled frequency hopping or different frequency hopping)

In RAN1#106-e [3], it was agreed that in case a separate initial UL BWP is configured, network and enable/disable intra-slot PUCCH frequency hopping.

Agreement - In case a separate initial UL BWP is configured for RedCap UEs, it is supported that the network can enable/disable intra-slot PUCCH frequency hopping within the separate initial UL BWP in the PUCCH resource for HARQ feedback for Msg4/MsgB for RedCap UEs. - Working assumption: The frequency hopping is enabled/disabled at least via SIB.

With disabling intra-slot PUCCH frequency hopping, if the separate initial UL BWP is configured close to the edge of a carrier, the PUSCH fragmentation issue can be largely avoided.

In RAN1#106bis-e [5], the working assumption for enabling/disabling PUCCH FH via SIB is confirmed.

Agreement: Confirm the working assumption: - In case a separate initial UL BWP is configured for RedCap UEs, it is supported that the network can enable/disable intra-slot PUCCH frequency hopping within the separate initial UL BWP in the PUCCH resource for HARQ feedback for Msg4/MsgB for RedCap UEs. - The frequency hopping is enabled/disabled at least via SIB.

## The separate initial UL BWP

In RAN1#104bis-e [6], the higher level of options for initial UL BWP was proposed as follows.

Agreement: - During initial access, for the scenario where the initial UL BWP for non-RedCap UEs is configured to be wider than the RedCap UE bandwidth, down select among the following options in RAN1#105-e - Option 1: The scenario is allowed, and a RedCap UE can use the same UL BWP. - Option 2: The scenario is allowed, but a separate initial UL BWP no wider than the RedCap UE maximum bandwidth is configured/defined for RedCap UEs. - Option 3: The scenario is not allowed, and a RedCap UE is not expected to operate in an initial UL BWP wider than the RedCap UE maximum bandwidth. Agreement: - After initial access, for the scenario where the initial UL BWP for non-RedCap UEs is configured to be wider than the RedCap UE bandwidth, down select among the following options in RAN1#105-e: - Option 1: The scenario is allowed, and a RedCap UE can use the same UL BWP. - Option 2: The scenario is allowed, but a separate initial UL BWP no wider than the RedCap UE maximum bandwidth is configured/defined for RedCap UEs. - Option 3: The scenario is not allowed, and a RedCap UE is not expected to operate in an initial UL BWP wider than the RedCap UE maximum bandwidth.

In RAN1#105-e [4], it was agreed that the scenario where the initial UL BWP for non-RedCap UEs is configured to be wider than the RedCap UE bandwidth is allowed.

Agreements: - Both during and after initial access, the scenario where the initial UL BWP for non-RedCap UEs is configured to be wider than the maximum RedCap UE bandwidth is allowed.

In RAN1#105-e [4], the separate initial UL BWP became a working assumption.

- Working assumption: Both during and after initial access, for the scenario where the initial UL BWP for non-RedCap UEs is configured to be wider than the RedCap UE bandwidth, a separate initial UL BWP no wider than the RedCap UE maximum bandwidth is configured/defined for RedCap UEs. - FFS: whether/how to avoid or minimize PUSCH resource fragmentation due to PUCCH transmission for the above case - Support the case when the centre frequency is assumed to be the same for the initial DL and UL BWPs in TDD. - FFS whether or not to additionally support the case when the centre frequency is different; if so, how to minimize centre frequency retuning Working assumption: Both during and after initial access, even for the scenario where the initial UL BWP for non-RedCap UEs is not configured to be wider than the RedCap UE bandwidth, a separate initial UL BWP can optionally be configured/defined for RedCap UEs. - RO sharing between RedCap and non-RedCap is not precluded.

We think the separate initial UL BWP has the following benefits.

- For the scenario where the initial UL BWP for non-RedCap UEs is configured to be wider than the RedCap UE bandwidth, the separate initial UL BWP can avoid UE retuning RF to operate in a BWP wider than the RedCap UE bandwidth.

- Issues of RO and broadcast PUSCH/PUCCH outside the RedCap UE bandwidth can be well solved by the separate initial UL BWP.

- The separate initial UL BWP can be placed close to the edge of the carrier, and PUSCH fragmentation issue can be alleviated.

- It automatically support early indication in Msg1.

In RAN1#106bis-e [5], it was agreed that network can configure a separate initial UL BWP for RedCap UEs in SIB.

Agreement - For a cell that allows a RedCap UE to access, network can configure a separate initial UL BWP for RedCap UEs in SIB - It can be used both during and after initial access. - It is no wider than the maximum RedCap UE bandwidth. - It is always configured if the initial UL BWP for non-RedCap UEs is wider than the maximum RedCap UE bandwidth - This applies to both TDD and FDD (including FD FDD and HD FDD) cases - FFS whether part of the configuration is implicitly signaled

# Initial DL BWP

## No wider than the RedCap UE bandwidth

In RAN1#106-e [3], it was agreed that during initial access, initial DL BWP for non-RedCap UE is not expected to exceed the RedCap UE bandwidth.

Agreement Replace the RAN1#104bis-e working assumption with the following agreement: - During initial access, the bandwidth of the initial DL BWP for RedCap UEs is not expected to exceed the maximum RedCap UE bandwidth. - RedCap UEs and non-RedCap UEs can share the same MIB-configured initial DL BWP (including the bandwidth and location). - This does not preclude a SIB-configured initial DL BWP for non-RedCap UEs only with a wider bandwidth than the maximum RedCap UE bandwidth. - This does not preclude separate or additional bandwidth and location for initial DL BWP for RedCap UEs (FFS).

In RAN1#106-e [3], it was confirmed that after initial access, a RedCap UE is not expected to operate with an initial DL BWP wider than the RedCap UE bandwidth.

Agreement Confirm the following working assumptions from RAN1#105-e: - After initial access (i.e., after RRC Setup, RRC Resume, or RRC Reestablishment), for BWP#0 configuration option 1 (as in 38.331, Appendix B2), a RedCap UE is not expected to operate with an initial DL BWP wider than the maximum RedCap UE bandwidth. - After initial access (i.e., after RRC Setup, RRC Resume, or RRC Reestablishment), for BWP#0 configuration option 2 (as in 38.331, Appendix B2), a RedCap UE is not expected to operate with an initial DL BWP wider than the maximum RedCap UE bandwidth.

## The separate initial DL BWP

We think the separate initial DL BWP can be configured to RedCap UE, for the following reasons:

- Alignment of center frequency between DL and UL BWP can be easily realized, if the separate initial DL BWP and the separate initial UL BWP are both configured.

- The separate search space set can be configured to RedCap UE in the separate initial DL BWP, and the corresponding sparse PDCCH monitoring occasions can save the power for RedCap UE.

- The RAR CORESET can be configured to RedCap UE in the separate initial DL BWP, and the corresponding large AL can improve the coverage for RedCap UE.

- The separate PDSCH parameter can be configured/scheduled to RedCap UE in the separate initial DL BWP, and the corresponding small TB scaling can improve the coverage for RedCap UE.

- The separate initial DL BWP can offload the Msg2/Msg4 at the initial DL BWP for non-RedCap UE, especially for IoT application in which the massive connections should be assumed.

Therefore, we support the separate initial DL BWP for RedCap UE.

In RAN1#105-e [4], it became a working assumption that a separate initial DL BWP can be optionally configured/defined to RedCap UE at least for TDD at least after initial access.

Working assumption: At least for TDD, an initial DL BWP for RedCap UEs (which is not expected to exceed the maximum RedCap UE bandwidth) can be optionally configured/defined separately from the initial DL BWP for non-RedCap UEs at least after initial access - FFS the details of the configuration/definition - The configuration for a separately configured initial DL BWP for RedCap UEs is signaled in SIB. - whether to support that separate initial DL BWP for RedCap UEs can include a configuration of CORESET and CSS(s) - whether part of the configuration can be defined instead of signaled - If a separate initial DL BWP for RedCap UEs is configured/defined, this separate initial DL BWP for RedCap UEs can be used at least after initial access (i.e., at least after RRC Setup, RRC Resume, or RRC Reestablishment). - FFS during the initial access - FFS: whether a separately configured initial DL BWP for RedCap UEs needs to contain the entire CORESET #0, and, if not, the Redcap UE behaviour for CORESET #0 monitoring - FFS: supported bandwidths in the separate initial DL BWP - FFS: whether additional SSB is transmitted in the separately configured initial DL BWP for RedCap UEs - FFS: FDD case

In RAN1#106bis-e [5], it became a working assumption that network can configure a separate initial DL BWP for RedCap UEs in SIB.

Working Assumption - For a cell that allows a RedCap UE to access, network can configure a separate initial DL BWP for RedCap UEs in SIB. - Working assumption: It can be used during initial access - It can be used after initial access. - It is no wider than the maximum RedCap UE bandwidth. - FFS: It is always configured if the initial DL BWP for non-RedCap UEs is wider than the maximum RedCap UE bandwidth. - This applies to both TDD and FDD (including FD FDD and HD FDD) cases. - Working assumption: It applies at least after initial access for FR1 when MIB configured CORESET#0 is included

In RAN1#107-e [7], the main working assumption was confirmed, and the sub-working assumptions was almost agreed, for the case when the separate initial DL BWP includes CD-SSB and the entire CORESET#0.

Agreement - For both FR1 and FR2, for a cell that allows a RedCap UE to access, network can configure a separate initial DL BWP for RedCap UEs in SIB. At least the case when the separate initial DL BWP includes CD-SSB and the entire CORESET#0 is supported - It can be used in idle/inactive mode (including paging) and during and after initial access, when applicable - It is no wider than the maximum RedCap UE bandwidth. - This applies to both TDD and FDD (including FD FDD and HD FDD) cases.

With RAN2/4’s conclusion, the separate initial DL BWP is only used for random access during initial access. Furthermore, combined with RAN1’s agreement, during random access RedCap UE does not expect the NCD-SSB in the separate initial DL BWP. Therefore, it is the fact that during initial access there is no NCD-SSB in the separate initial DL BWP, and network vendors’ concern is resolved. Consequently, the separate initial DL BWP can be supported in the case when the separate initial DL BWP does not include CD-SSB and the entire CORESET#0.

Proposal 5: For both FR1 and FR2, for a cell that allows a RedCap UE to access, network can configure a separate initial DL BWP for RedCap UEs in SIB, no matter whether or not the separate initial DL BWP include CD-SSB and the entire CORESET#0.

The main blocking issue for the separate initial DL BWP is about whether the NCD-SSB is present or not in the separate initial DL BWP or not. In the discussion in RAN1#106bis-e [5], RAN1 listed the Option 1 and Option 2 regarding the presence of the NCD-SSB, and the companies cannot get consensus.

- For FR1, following options: - Option 1: - For a separate initial DL BWP (if it does not include CD-SSB and the entire CORESET#0), - RedCap UE does NOT expect it to contain SSB/CORESET#0/SIB. - For an RRC-configured active DL BWP (if it does not include CD-SSB and the entire CORESET#0), - RedCap UE does NOT expect it to contain SSB/CORESET#0/SIB. - Option 2: - For a separate initial DL BWP (if it does not include CD-SSB and the entire CORESET#0), - If it is configured for random access while not for paging in idle/inactive mode, RedCap UE does NOT expect it to contain SSB/CORESET#0/SIB. - FFS: For BWP#0 configuration option 1, whether the UE can expect SSB transmission in the separate initial DL BWP when it is used in connected mode. - If it is configured for paging, RedCap UE expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB. - For an RRC-configured active DL BWP in connected mode (if it does not include CD-SSB and the entire CORESET#0), - RedCap UE expects it to contain NCD-SSB for serving cell [FFS: or CSI-RS or measurement gap configuration] but not CORESET#0/SIB. - Note: if a separate initial/RRC configured DL BWP is configured to contain the entire CORESET#0, CD-SSB is expected by RedCap UE. - Note: The network may choose to configure SSB or MIB-configured CORESET#0 or SIB1 to be within the respective DL BWP. - FFS: For Option 1 and Option 2, whether RedCap UE can/cannot expect SSB under certain other conditions, e.g., for SSB monitoring periodicity (i.e., SMTC configuration) and DRX cycle - FFS: Whether additional mechanism for SI update or how SI update notifications and/or SI updates are signaled to RedCap UEs - FFS: FR2 case

## The NCD-SSB

Regarding the presence of the NCD-SSB, the network vendors mainly have the following concerns on the NCD-SSB.

- The potential large overhead of network resource due to the NCD-SSB;

- The feasibility (e.g. spec impact) of introducing the NCD-SSB.

For the overhead, thanks for the narrow bandwidth and the low duty cycle of SSB, in our view, the additional overhead due to the NCD-SSB is about 1%.

In RAN1#106bis-e [5], RAN1 decided to send an LS (R1-2110600) to RAN2/4 to ask about the feasibility and spec impact of NCD-SSB. Then, RAN2 replied the LS in R2-2111545, and RAN4 replied the LS in R4-2120327.  RAN2 thinks NCD-SSB is feasible basically but has some spec impact, e.g. RLM, BFD, and RAN4 thinks NCD-SSB is feasible basically but the possible spec impact needs the further study. Based on the reply from RAN2/4, RAN1 decided to start from Option 2 in RAN1#107-e [7]. The following agreement is for FR1.

Agreement - For FR1, - For a separate initial DL BWP (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective, - If it is configured for random access while not for paging in idle/inactive mode, RedCap UE does NOT expect it to contain SSB/CORESET#0/SIB. - Note: RAN1 assumes REDCAP UE performing Random access in the separate DL BWP does not need to monitor paging in a BWP containing CORESET#0 - Working assumption: If it is configured for paging, RedCap UE expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB from RAN1 perspective - For an RRC-configured active DL BWP in connected mode (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective, - A RedCap UE supporting mandatory FG 6-1 (but not optional FG 6-1a) expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB - A RedCap UE can indicate the following as optional capability: - Not need NCD-SSB: A RedCap UE can in addition optionally support relevant operation (except for standalone use for RRM measurement) based on for CSI-RS (working assumption) and/or FG 6-1a by reporting optional capabilities. - Note: if a separate initial/RRC configured DL BWP is configured to contain the entire CORESET#0, CD-SSB is expected by RedCap UE. - Note: The network may choose to configure SSB or MIB-configured CORESET#0 or SIB1 to be within the respective DL BWP. - Note: If a separate SIB-configured initial DL BWP for RedCap UEs contains the entire CORESET#0, the RedCap UE shall use the bandwidth and location of the CORESET#0 in DL during initial access. - Note: NCD-SSB periodicity is not required to be configured the same as that of CD-SSB - Note: Periodicity of NCD-SSB shall be not less than periodicity of CD-SSB

The following agreement is for FR2.

Agreement - For FR2, - For a separate initial DL BWP (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective, - If it is configured for random access while not for paging in idle/inactive mode, RedCap UE does NOT expect it to contain SSB/CORESET#0/SIB. - Note: RAN1 assumes REDCAP UE performing Random access in the separate DL BWP does not need to monitor paging in a BWP containing CORESET#0 - Working assumption: If it is configured for paging, RedCap UE expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB from RAN1 perspective - For an RRC-configured active DL BWP in connected mode (if it does not include CD-SSB and the entire CORESET#0) from RAN1 perspective, - A RedCap UE supporting mandatory FG 6-1 (but not optional FG 6-1a) expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB - A RedCap UE can indicate the following as optional capability: - Not need NCD-SSB: A RedCap UE can in addition optionally support relevant operation based on for CSI-RS (working assumption) and/or FG 6-1a by reporting optional capabilities. - Note: For SSB and CORESET#0 multiplexing pattern 1, if a separate initial/RRC configured DL BWP is configured to contain the entire CORESET#0, CD-SSB is expected by RedCap UE. - Note: The network may choose to configure SSB or MIB-configured CORESET#0 or SIB1 to be within the respective DL BWP. - Note: If a separate SIB-configured initial DL BWP for RedCap UEs contains the entire CORESET#0, the RedCap UE shall use the bandwidth and location of the CORESET#0 in DL during initial access. - Note: NCD-SSB periodicity is not required to be configured the same as that of CD-SSB - Note: Periodicity of NCD-SSB shall be not less than periodicity of CD-SSB

Furthermore, in RAN1#107-e, RAN1 sent an LS (R12112902) to RAN2/4 to include the above two agreements to show RAN1 views on NCD-SSB.

After RAN1#107-e, RAN#94-e plenary also discussed some contents about the LS, and had the following agreements in RP-213689.

Scheme 1 (i.e. UE in IDLE and INACTIVE monitors paging in an initial BWP associated with CD-SSB) is adopted for further work in Rel-17. Scheme 2 (i.e. UE in IDLE and INACTIVE monitors paging in an initial BWP associated with NCD-SSB) is not considered further in Rel-17. RAN2 should work on the assumption that the cell reselection measurements and cell ranking are performed based on measurements on the CD-SSB. This applies for intra- and inter-frequency measurements, and for IDLE and INACTIVE states.

In current RAN1 meeting, RAN2 replies the LS in R2-2201759, and RAN4 replies in R4-2200898. RAN2 officially confirmed that RedCap UE expects CD-SSB for paging, cell (re-)selection and measurement. Therefore, the working assumption that “If it is configured for paging, RedCap UE expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB from RAN1 perspective” should be removed.

Proposal 6: The working assumption that “If it is configured for paging, RedCap UE expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB from RAN1 perspective” should be removed.

## The continue-to-use CORESET#0 for the initial DL BWP

We list the possible combinations of initial DL/UL BWP in the following table.

Table 2: The possible combinations of initial DL/UL BWP

|  | The shared initial UL BWP | The separate initial UL BWP |
| --- | --- | --- |
| The shared initial DL BWP | Supported, but the bandwidth of the shared initial DL/UL BWP should be no wider than the max RedCap UE bandwidth initialDownlinkBWP in DownlinkConfigCommonSIB [or CORESET#0] initialUplinkBWP in UplinkConfigCommonSIB | Maybe supported, but the separate initial UL BWP may cause the PUSCH fragmentation issue (assuming the initial UL BWP for non-RedCap UE is wider) initialDownlinkBWP in DownlinkConfigCommonSIB [or CORESET#0] initialUplinkBWP in UplinkConfigCommonRedCapSIB |
| The separate initial DL BWP | It is corner case (no use case) and spec should not address it | Supported initialDownlinkBWP in DownlinkConfigCommonRedCapSIB initialUplinkBWP in UplinkConfigCommonRedCapSIB |

When the separate initial DL BWP is not configured, RedCap UE will use the location/bandwidth/SCS of CORESET#0 for the initial DL BWP by default during initial access. It has no “exceeding bandwidth” issue for the RedCap UE. However, how about after initial access?

In RAN1#107-e discussion, majority companies think the RedCap UE can continue to use the location/bandwidth/SCS of CORESET#0 for the initial DL BWP. In other words, the higher layer parameter LocationAndBandwidth in initialDownlinkBWP in DownlinkConfigCommonSIB is not applicable for the RedCap UE, [if it indicates a BWP wider than the max RedCap UE bandwidth and initialDownlinkBWP in DownlinkConfigCommonRedCapSIB is not configured].

initialDownlinkBWP The initial downlink BWP configuration for a PCell. The network configures the locationAndBandwidth so that the initial downlink BWP contains the entire CORESET#0 of this serving cell in the frequency domain. The UE applies the locationAndBandwidth upon reception of this field (e.g. to determine the frequency position of signals described in relation to this locationAndBandwidth) but it keeps CORESET#0 until after reception of RRCSetup/RRCResume/RRCReestablishment.

The Pros and Cons of the continue-to-use CORESET#0 for the initial DL BWP are listed as follows.

Table 3: The Pros and Cons of the continue-to-use CORESET#0 for the initial DL BWP

|  | Pros | Cons |
| --- | --- | --- |
| The continue-to-use CORESET#0 for the initial DL BWP | - Saving the signaling overhead for initialDownlinkBWP in DownlinkConfigCommonRedCapSIB | - Limitation: the bandwidth of the shared initial UL BWP should be no wider than the max RedCap UE bandwidth - Potentially the separate initial UL BWP may cause the PUSCH fragmentation issue - Potentially RedCap UE and non-RedCap UE have different initial DL BWPs and they are overlapping, which cause the complexity of the broadcast PDSCH scheduling at gNB - Potentially spec impact is large, e.g. conditions for continue-to-use CORESET#0 (it seems the controversial part in RAN1#107-e discussion) |

Honestly, we think the continue-to-use CORESET#0 should be carefully studied, since Pros are not so convinced. Anyway, if majority companies support it, we can accept it.

Proposal 7: It should be carefully studied that whether to support that the RedCap UE can continue using the location/bandwidth/SCS of CORESET#0 for the initial DL BWP.

# Non-initial DL/UL BWP

In RAN1#104bis-e [6], the working assumption was achieved for non-initial DL/UL BWP.

Working assumption: A RedCap UE cannot be configured with a non-initial (DL or UL) BWP (i.e., a BWP with a non-zero index) wider than the maximum bandwidth of the RedCap UE. - At least for FR1, FG 6-1 ("Basic BWP operation with restriction" as described in TR 38.822) is used as a starting point for the RedCap UE type capability.

After initial access, it is very natural that gNB configures the non-initial DL/UL BWP in UE-specific way. There is no need to put any restriction on this.

In RAN1#105-e [4], the above working assumption was agreed, and it was agreed to support FG 6-1a as a optional UE capability for RedCap UE.

Agreement:Take the following as an agreement, revised from the RAN1#104bis-e working assumption: - A RedCap UE cannot be configured with a non-initial (DL or UL) BWP (i.e., a BWP with a non-zero index) wider than the maximum bandwidth of the RedCap UE. - At least for FR1, FG 6-1 (“Basic BWP operation with restriction” as described in TR 38.822) is used as a starting point for the mandatory RedCap UE type capability. - This does not preclude support of FG 6-1a (“BWP operation without restriction on BW of BWP(s)” as described in TR 38.822) as a UE capability for RedCap UEs.

# Conclusion

As the conclusion, we have the following observations.

RF retuning for FH and “fast” BWP switch

Observation 1: Whether to support RF-based FH or “fast” BWP switch in a wider bandwidth than the RedCap UE bandwidth is out of the scope of the RedCap WID objectives.

Observation 2: If the coverage recovery subtopic is open to discussion, whether RF-based FH or “fast” BWP switch in a wider bandwidth than the RedCap UE bandwidth is prioritized than other schemes, e.g. repetition, should be further discussed.

RF retuning for SSB processing

Observation 3: In context of RedCap, FG 6-1a means the RedCap UE should retune RF to process SSB/CORESET#0 outside the UE-specific BWP.

Observation 4: There is only one condition for “without measurement gap” in 38.133 for RedCap UEs, i.e. “the SSB is completely contained in the active BWP of the UE”.

Observation 5: Gaps due to RF retuning for AGC/sync/measurement should be considered for discussion of the separate initial DL BWP during initial access.

Observation 6: gNB can avoid selecting the entries that exceeds 100MHz BW containing both SSB and CORESET#0 for SSB/CORESET#0 multiplexing pattern 2/3 in FR2.

As the conclusion, we have the following proposals:

RF retuning for FH and “fast” BWP switch

Proposal 1: It is not necessary to send LS to RAN4 for RF retuning.

RF retuning for SSB processing

Proposal 2: Remove “CORESET#0” or add a note in FG 6-1/6-1a/6-2/6-3/6-4. The note is “For RedCap UE, CORESET#0 here means CORESET#0 or CORESET of CSS”.

Proposal 3: Conditions for “without measurement gap” in 38.133 should be revisited for RedCap UEs.

RF retuning between DL/UL

Proposal 4: For TDD, center frequencies are the same for the initial DL and UL BWP during random access for RedCap UEs, no matter whether or not it includes CD-SSB and the entire CORESET#0 or not.

Initial DL BWP

Proposal 5: For both FR1 and FR2, for a cell that allows a RedCap UE to access, network can configure a separate initial DL BWP for RedCap UEs in SIB, no matter whether or not the separate initial DL BWP include CD-SSB and the entire CORESET#0.

Proposal 6: The working assumption that “If it is configured for paging, RedCap UE expects it to contain NCD-SSB for serving cell but not CORESET#0/SIB from RAN1 perspective” should be removed.

Proposal 7: It should be carefully studied that whether to support that the RedCap UE can continue using the location/bandwidth/SCS of CORESET#0 for the initial DL BWP.

# Reference

- R4-1702029, “[Draft] Reply LS on UE RF Bandwidth Adaptation in NR”, RAN4#82, MediaTek, Feb. 13th - 17th, 2017.

- TR 38.822 V16.0.0, “User Equipment (UE) feature list”, Jun. 2021.

- 3GPP RAN1#106-e Chairman’s Notes, Aug. 16th - 27th, 2021.

- 3GPP RAN1#105-e Chairman’s Notes, May 10th - 27th, 2021.

- 3GPP RAN1#106bis-e Chairman’s Notes, Oct. 11th - 19th, 2021.

- 3GPP RAN1#104bis-e Chairman’s Notes, Apr. 12th - 20th, 2021.

- 3GPP RAN1#107-e Chairman’s Notes, Nov. 11th - 19th, 2021.

# Appendix

## A.1 Agreements for initial DL/UL BWP in RAN1#104

Agreements: - Sharing of the same SSB and CORESET#0 between RedCap and non-RedCap UEs is supported when the bandwidth is no wider than the RedCap UE bandwidth - The initial DL BWP (derived based on MIB/SIB) for RedCap UEs can be the same as the initial DL BWP for non-RedCap UEs at least when the initial DL BWP is no wider than the RedCap UE bandwidth. - FFS: after initial access, whether a RedCap UE is allowed to operate with an initial DL BWP wider than the maximum RedCap UE bandwidth - Discuss further whether or not it is also applicable during initial access - The initial UL BWP (derived based on SIB) for RedCap UEs can be the same as the initial UL BWP for non-RedCap UEs at least when the initial UL BWP is no wider than the RedCap UE bandwidth. - FFS: during and after initial access, whether a RedCap UE is allowed to operate with an initial UL BWP wider than the maximum RedCap UE bandwidth - FFS whether or not to further introduce the following (e.g., for offloading purpose, for differentiation of RedCap vs. non RedCap UEs, for different BWP#0 configuration options, etc.) - Whether an additional CORESET can be configured for scheduling of RACH (msg2 & msg4)/Paging/SI messages for RedCap UEs - Whether the SIB-configured initial DL BWP for RedCap UEs can also be configured to be different from the SIB-configured initial DL BWP for non-RedCap UEs. - Whether the SIB-configured initial UL BWP for RedCap UEs can also be configured to be different from the SIB-configured initial UL BWP for non-RedCap UEs.

## A.2 FG 6-1/6-1a/6-2/6-3/6-4 in TR 38.822 [2]

| Features | Index | Feature group | Components | Prerequisite feature groups | Field name in TS 38.331 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 6. CA/DC, BWP, SUL | 6-1 | Basic BWP operation with restriction | 1) 1 UE-specific RRC configured DL BWP per carrier 2) 1 UE-specific RRC configured UL BWP per carrier 3) RRC reconfiguration of any parameters related to BWP 4) BW of a UE-specific RRC configured BWP includes BW of CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP includes SSB for SCell if there is SSB on SCell |  | n/a | This feature should be mandatory without capability signalling for at least BWPs which is the same as the set of specified channel BW UE-specific RRC configured DL/UL BWP can have the same or different numerology from the initial active DL/UL BWP | Mandatory without capability signalling |
|  | 6-1a | BWP operation without restriction on BW of BWP(s) | BW of UE-specific RRC configured BWP may not include BW of the CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP may not include SSB for SCell | 6-1, 6-2, 6-3, or 6-4 | bwp-WithoutRestriction | 6-1a is applicable to 6-1, 6-2, 6-3, or 6-4. | Optional with capability signalling |
|  | 6-2 | Type A BWP adaptation with same numerology | 1) Up to 2 UE-specific RRC configured DL BWPs per carrier 2) Up to 2 UE-specific RRC configured UL BWPs per carrier 3) Active BWP switching by DCI and timer 4) Same numerology for all the UE-specific RRC configured BWPs per carrier 5) BW of a UE-specific RRC configured BWP includes BW of the CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP includes SSB for SCell if there is SSB on SCell | 6-1 | upto2 in bwp-SameNumerology |  | Optional with capability signalling |
|  | 6-3 | Type B BWP adaptation with same numerology | 1) Up to 4 UE-specific RRC configured DL BWPs per carrier 2) Up to 4 UE-specific RRC configured UL BWPs per carrier 3) Active BWP switching by DCI and timer 4) Same numerology for all the UE-specific RRC configured BWPs per carrier 5) BW of a UE-specific RRC configured BWP includes BW of the CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP includes SSB for SCell if there is SSB on SCell | 6-1 | upto4 in bwp-SameNumerology |  | Optional with capability signalling |
|  | 6-4 | BWP adaptation with different numerologies | 1) Up to 4 UE-specific RRC configured DL BWPs per carrier 2) Up to 4 UE-specific RRC configured UL BWPs per carrier 3) Active BWP switching by DCI and timer 4) More than one numerologies for the UE-specific RRC configured BWPs per carrier 5) Same numerology between DL and UL per cell except for SUL at a given time 6) BW of a UE-specific RRC configured BWP includes BW of the CORESET#0 (if CORESET#0 is present) and SSB for PCell/PSCell (if configured) and BW of the UE-specific RRC configured BWP includes SSB for SCell if there is SSB on SCell | 6-1 | upto4 in bwp-DiffNumerology |  | Optional with capability signalling |