3GPP TSG RAN WG1 #108-e                                                   R1-2201550

e-Meeting, February 21st – March 3rd, 2022

Agenda Item:	8.6.1.2

Source:	Spreadtrum Communications

Title:	Discussion on other aspects for RedCap

Document for:	Discussion and decision

Introduction

According to the Rel.17 RedCap WID [1], HD-FDD type A should be specified with the minimum specification impact.

- Specify support for the following UE complexity reduction features [RAN1, RAN2, RAN4]: - Duplex operation: - HD-FDD type A with the minimum specification impact (Note that FD-FDD and TDD are also supported.)

In the last RAN1#107 meeting, collision issues for HD-FDD operation were discussed, and the following agreements were achieved [2].

Agreement - For Case 5 of dynamically scheduled UL transmission vs. SSB, support Option 2 at least for dynamically scheduled UL transmission other than Msg3 (re)transmission and PUCCH for Msg4 - Option 2: Reuse the existing collision handling principles of Rel-15/16 for NR TDD that SSB is prioritized over dynamically scheduled UL transmission Agreement - For MsgA PUSCH occasion overlapping with dynamic or semi-static DL reception, leave it to UE implementation to prioritize the DL reception or MsgA PUSCH transmission Agreement - For the case of the “back-to-back” non-overlapping UL/DL without sufficient gap between cell specific configured DL and cell-specific configured UL, e.g., SSB or PDCCH in CSS vs. valid RO, it is up to UE implementation to ensure that the switching time is satisfied Agreement  The “back-to-back” non-overlapping UL/DL without sufficient gap between cell-specific configured DL and dedicated configured UL may happen, i.e., allowed for HD-FDD UEs - E.g., SSB vs. CG PUSCH, PUCCH or SRS - Configured UL transmission is cancelled (as in the overlapping case) - The “back-to-back” non-overlapping UL/DL without sufficient gap between dedicated configured DL and cell-specific configured UL may happen, i.e., allowed for HD-FDD UEs - E.g., PDCCH in USS, SPS PDSCH, CSI-RS or DL PRS vs. valid RO - Leave it to UE implementation to cancel either DL reception or UL transmission to ensure sufficient switching time Agreement  No additional UE behavior for DL/UL collision handling is specified in Rel-17 if SFI monitoring is supported for HD-FDD RedCap UEs.

In this contribution, we discuss the remaining issues for HD-FDD operation.

Discussion

Remaining issues of collision case 5, 8 and 9 were discussed in the last meeting and those issues were resolved except a part of case 5. In this chapter, we discuss the remaining part of case 5.

- Case 5: Configured SSB vs. dynamically scheduled UL transmission

For case 5, option 2 was selected for the case of dynamically scheduled UL transmission other than Msg3 (re)transmission and PUCCH for Msg4 vs. SSB, i.e., when such a collison occurs, reuse the existing collision handling principles of Rel-15/16 for NR TDD that SSB is prioritized over dynamically scheduled UL transmission. While for the case of Msg3 (re)transmission and PUCCH for Msg4 vs. SSB, the solutions are need further confirmed and are listed below:

- Option 1: Dynamically scheduled UL transmission is prioritized over SSB.

- Option 2: Reuse the existing collision handling principles of Rel-15/16 for NR TDD that SSB is prioritized over dynamically scheduled UL transmission

Option 1 may beneficial to the initial access procedure, as UE typically would not perform SSB-based measurement during initial access and there could be some latency impact on initial access if SSB is prioritized over Msg3 (re)transmission and PUCCH for Msg4. However, the RedCap UEs are usually not latency-sensitive, the gain of option 1 is quite limited to RedCap UE.

In addition, option 2 was agreed for the cases of dynamically scheduled UL transmission other than Msg3 and PUCCH for Msg4, if option 1 is adopted here, different solutions were introduced for the same collision case in different phase (during the initial access and after initial access), it makes the UE behaviour unnecessarily complicated. In this regard, a unified solution is preferred.

Based on above, we prefer option 2 here.

Proposal 1: For Case 5 of SSB overlaps with Msg3 (re)transmission or PUCCH for Msg4, reuse the existing collision handling principles of Rel-15/16 for NR TDD that SSB is prioritized over dynamically scheduled UL transmission

Conclusion

Based on the analyses and discussions, we have the following proposals:

Proposal 1: For Case 5 of SSB overlaps with Msg3 (re)transmission or PUCCH for Msg4, reuse the existing collision handling principles of Rel-15/16 for NR TDD that SSB is prioritized over dynamically scheduled UL transmission

Reference

- RP-211574, Revised WID on support of reduced capability NR devices.

- Chairman’s Notes, RAN1#107, e-Meeting. November 11th – 19th, 2021.