3GPP TSG RAN WG1 Meeting #108-e                            	      R1- 2201559

E-Meeting, February 21st – March 3rd, 2022

Agenda Item:	8.15.1

Source:	Spreadtrum Communications

Title:	Discussion on enhancements to time and frequency synchronization for IOT NTN

Document for:	Discussion and decision

Introduction

The WID on NB-IoT and eMTC over Non-Terrestrial Networks (NTNs) is approved in [1]

Specify the following time and frequency synchronization enhancements, using NR_NTN_solutions WI agreements as baseline, according to Section 8 in TR 36.763: - UE pre-compensation including ephemeris format (orbital / Position -Velocity) - UE pre-compensation for UL synchronization in RRC_IDLE and RRC_CONNECTED states based at least on its GNSS-acquired position and the serving satellite ephemeris - Timing advance formula (granularity of the timing advance may be different) - Combination of Open (i.e. UE autonomous TA estimation, and common TA estimation) and Closed TA (i.e., received TA commands) control loops in RRC_CONNECTED state Agreements on the above are up to the decision in NR_NTN_Solutions WI and will be used for IoT NTN with minimum changes, if any. Specify the following time and frequency synchronization enhancements that are not covered by NR_NTN_Solutions WI agreements, according to Section 8 in TR 36.763: - Long PUSCH and PRACH Transmission enhancements: segmented UE pre-compensations, new UL gaps and/or implementation solutions, time units and duration of segments. - Validity timer for UL synchronization: satellite ephemeris, and potentially other aspects - DL synchronization enhancements: A single solution will be selected between: new channel raster, (part of) ARFCN-indication-in-MIB. - GNSS Measurements: Validity of a GNSS position fix and details of acquiring a GNSS position fix, duration of validity, in RRC CONNECTED mode for sporadic short transmission Specify the following IoT NTN specific timing relationships enhancements according to Section 8 in TR 36.763: - Timing relationships for NB-IoT / eMTC: as listed in Section 6.6.3 in TR 36.763 - UL scheduling for FDD-HD: Use of UE-specific TA and/or K_offset to avoid UL-DL collisions in FDD-HD - Signalling aspects in UE-specific TA maintenance and reporting, techniques to reduce the signalling load and determination of the UE-specific TA.

In this contribution, the remaining issues of the time and frequency synchronization enhancements to NB-IoT/eMTC over NTN are discussed.

Validity timer for UL synchronization

In RAN#107e, the following agreements on validity timer for UL synchronization had been made for NR NTN [1].

Agreement The serving satellite ephemeris and common TA related parameters are signalled in the same SIB message and have the same epoch time. Agreement A single validity duration for both serving satellite ephemeris and common TA related parameters is broadcast on the SIB. Agreement Validity timer for UL synchronization should be started/restarted with configured timer validity duration at the epoch time of the assistance information.

For NR NTN, it was agreed that the reference point for epoch time of the serving satellite ephemeris and Common TA parameters is the uplink time synchronization reference point. In our view, the definition of reference point for epoch time in NR NTN can be reused in IOT NTN.

Proposal 1: In IOT NTN, the reference point for epoch time of the serving satellite ephemeris and Common TA parameters is the uplink time synchronization reference point.

Long UL transmission on PUSCH and PRACH

In RAN1#107e, the following agreement on long UL transmission on PUSCH and PRACH has been made [1].

Agreement For UL Segmented transmission during RRC_CONNECTED: - If a segment duration is configured, the UE is expected to adjust the value for pre-compensation for a segment. - FFS: UL transmission segment duration for NPDCCH ordered NPRACH/NPUSCH for NB-IoT and PDCCH ordered PRACH/PUSCH/PUCCH for eMTC is configurable by dedicated RRC Signalling - For UE pre-compensation per segment, further discuss how the following options apply from one segment to the next segment, and potential down-selection among the options: - Option 1: Skip / drop / insert samples - Option 2: puncture OFDM symbols - Option 3: Blanking subframes/slots where UE skip a slot or a subframe - FFS whether this can be left to UE implementation or if specification impact is needed

In this section, we share our views on details of method(s) to drop / insert samples, blanking subframes / slots for UL Segmented transmission.

Long UL transmission on PUSCH

In RAN1#107e, it was agreed that UE pre-compensation per segment of NPUSCH for NB-IoT and PUSCH/PUCCH for eMTC is applied from one segment to the next segment by drop / Insert samples / Puncture OFDM symbols or blank subframes / slots where UE skip a slot or a subframe.

Agreement UE pre-compensation per segment of NPUSCH for NB-IoT and PUSCH/PUCCH for eMTC is applied from one segment to the next segment by using one or more of the following methods if supported by UE implementation - UE may drop / Insert samples / Puncture OFDM symbols - UE may blank subframes / slots where UE skip a slot or a subframe The total transmission time is not changed UE autonomously Drop / insert samples / Puncture OFDM symbols or Blank subframes / slots where UE drops a subframe / slot The method used for the UE pre-compensation is known to the eNB by a single UE capability - UE Blank subframes / slots where UE skip a slot or a subframe (slot is based on Sub Carrier Spacing) FFS Details of method(s) to drop / insert samples, blanking subframes / slots (slot is based on Sub Carrier Spacing)

When the UE's TA pre-compensation value changes from small to large, two adjacent segments will overlap. In this case, UE pre-compensation per segment of NPUSCH for NB-IoT and PUSCH/PUCCH for eMTC is applied from one segment to the next segment by drop samples or blank subframes / slots where UE skip a slot or a subframe and the total transmission time is not changed. When the UE's TA pre-compensation value changes from large to small, two adjacent segments will not overlap. In this case, UE pre-compensation per segment of NPUSCH for NB-IoT and PUSCH/PUCCH for eMTC is applied from one segment to the next segment by insert samples and the total transmission time is not changed.

Long UL transmission on PRACH

In RAN1#107e, the following agreements on long UL transmission on PRACH had been made [1].

Agreement For NB-IoT, UE pre-compensation per segment of NPRACH is applied from one segment to the next segment by using one or more of the following methods if supported by UE implementation - UE may drop / Insert samples - UE may blank subframe / repetition unit where UE drops a subframe / repetition unit The total transmission time is not changed FFS Details of method(s) to drop / insert samples / blank subframe / repetition unit FFS Specification impact Agreement For eMTC, UE pre-compensation per segment of PRACH is applied from one segment to the next segment by drop / insert samples in Guard Period of PRACH preamble. - The total transmission time is not changed - FFS Details of method(s) to drop / insert samples

For NB-IoT, UE pre-compensation per segment of NPRACH is applied from one segment to the next segment by drop / insert samples or blank subframe / repetition unit where UE drops a subframe / repetition unit. The UE may apply UE pre-compensation based on determined delay drift over the service link and feeder link and apply UE pre-compensation accordingly. When the UE's TA pre-compensation value changes from small to large, two adjacent segments will overlap. In this case, UE pre-compensation per segment of NPRACH is applied from one segment to the next segment by drop samples or blank subframe / repetition unit where UE drops a subframe / repetition unit and the total transmission time is not changed. When the UE's TA pre-compensation value changes from large to small, two adjacent segments will not overlap. In this case, UE pre-compensation per segment of NPRACH is applied from one segment to the next segment by insert samples and the total transmission time is not changed. In our view, there is no specification impact.

Proposal 2: There is no specification impact for the method(s) to drop / insert samples / blank subframe / repetition unit for UE pre-compensation per segment of NPRACH.

For eMTC, UE pre-compensation per segment of PRACH is applied from one segment to the next segment by drop / insert samples in Guard Period of PRACH preamble. When the UE's TA pre-compensation value changes from small to large, two adjacent segments will overlap. In this case, UE pre-compensation per segment of NPRACH is applied from one segment to the next segment by drop samples in Guard Period of PRACH preamble and the total transmission time is not changed. When the UE's TA pre-compensation value changes from large to small, two adjacent segments will not overlap. In this case, UE pre-compensation per segment of NPRACH is applied from one segment to the next segment by insert samples in Guard Period of PRACH preamble and the total transmission time is not changed.

Conclusion

As summary, we have the following proposals.

Proposal 1: In IOT NTN, the reference point for epoch time of the serving satellite ephemeris and Common TA parameters is the uplink time synchronization reference point.

Proposal 2: There is no specification impact for the method(s) to drop / insert samples / blank subframe / repetition unit for UE pre-compensation per segment of NPRACH.

Reference

3GPP RAN1#107-e Chairman’s Notes, November 11th – 19th, 2021.