3GPP TSG RAN WG1 #108-e		    R1- 2201561

e-Meeting, February 21st – March 3rd, 2022

Agenda Item:  8.16.8

Source:	Spreadtrum Communications

Title:	UE features for NR coverage enhancement

Document for:	Discussion and decision

Introduction

In this contribution, we gives our considerations on the UE features for NR coverage enhancement, based on the UE feature list was approved during RAN1 #107b-e meeting [1].

Discussion

30-1 to 30-3a

- Separate FGs and Type

  - Separate DG and CG-PUSCH FGs for FG 30-1 and FG 30-1a, FG 30-2 and FG 30-2a, FG 30-3, and FG 30-3a.

    - FG 30-1 and FG 30-1a should be separate, just to keep consistent with DG-PUSCH repetition and CG-PUSCH repetition with separate UE features in Rel-15. FGs 30-2a and 30-2, FGs 30-3a, and 30-3 are in the same situation, those two features should be separately reported.

  - Type: Per band.

    - First of all, the uplink coverage enhancements features can be applied to different bands. Clearly, different band may have different requirement and sub-group features.

- For compromise, for the proposal from the FL in RAN1 107b-e, we can live with the compromised proposal.

High priority compromised proposal 2-1’/2-2’/3-1’/3-5’: - Merge FGs 30-1 and 30-1a into an FG for DG, type 1 and 2 CG. Merged FG 30-1 has per band granularity - Merge FGs 30-2 and 30-2a into an FG for DG, type 1 and 2 CG. Merged FG 30-2 has per band granularity - FG 30-3 is not split into 2 separate FGs: 1st one for DG, 2nd one for CG. FG 30-3 has per band granularity - FG 30-3a is not separated to multiple FGs. FG 30-3a has per band granularity

- Prerequisite

  - FG 30-1: FG 5-17

  - FG 30-1a: FG 5-16.

    - Do not include 30-1, CG-PUSCH and DG-PUSCH are two separate FG. DG-PUSCH is not the prerequisite FG for CG-PUSCH.

  - FG 30-2: FG 5-17

  - FG 30-2a: FG 5-14 or 5-16.

    - Do not need 30-2, CG-PUSCH and DG-PUSCH are two separate FG. DG-PUSCH is not the prerequisite FG for CG-PUSCH.

30-4 to 30-4h: DM-RS bundling

- Type: FGs 30-4 and 30-4x are per band. Since DM-RS bundling cannot be implemented for some band combination.

- Prerequisite

  - FG 30-4: N/A

  - FG 30-4a: FG 30-4 and 5-17 or 5-16.

    - Do not need FG 30-1 or 30-2. DM-RS bundling can be supported without larger PUSCH repetition number, or available slot.

  - FG 30-4b: FG 30-4 and 11-5.

    - Do not need 30-1. Because FG 30-1 is for PUSCH repetition type A, it is not related with PUSCH repetition type B.

  - FG 30-4c: confirm FG 30-4 and 30-3

  - FG 30-4d: confirm FG 30-4 and 4-23

  - FG 30-4e: confirm 30-4a or 30-4b, or 30-4c

  - FG 30-4f: confirm 30-4d

Conclusions

In this contribution, we give our considerations for UE features of UL coverage enhancement.

References

R1-2200780	Updated RAN1 UE features list for Rel-17 NR after RAN1 #107bis-e	Moderators (AT&T, NTT DOCOMO, INC.)

R1-2200254	Summary on UE features for NR coverage enhancement	Moderator (NTT DOCOMO, INC.)