3GPP TSG RAN WG1 #108-e				                            R1-2201569

e-Meeting, February 21th – March 3rd, 2022

Agenda item:	8.1.2.2

Source:	LG Electronics

Title:	Enhancements on Multi-TRP inter-cell operation

Document for:	Discussion and Decision

Introduction

In this contribution, we discuss remaining issues for inter-cell multi-TRP operations, assuming multi-DCI based multi-PDSCH reception.

Discussion

In the last meeting, it was briefly discussed whether PDSCH/PDCCH from one MTRP cell is rate matched around the other MTRP cell SSB. Given that beam and precoding for PDSCH/PDCCH from MTRP non-serving cell are optimized, the interference from the PDSCH/PDCCH to serving cell SSB is much stronger than the interference from conventional non-serving cell which does not participate in MTRP operation. Therefore, non-serving cell PDSCH/PDCCH should be rate matched around serving cell SSB. Also, since UE needs to derive QCL parameters from MTRP non-serving cell SSB, serving cell PDSCH/PDCCH, which causes strong interference to non-serving cell SSB, should be rate matched around the non-serving cell SSB.

Proposal #1: PDSCH/PDCCH from serving cell should be rate matched around non-serving cell SSB and PDSCH /PDCCH from non-serving cell should be rate matched around serving cell SSB.

In addition, a few FFS point remains in the last meeting. Specifically, companies have different view on whether halfFrameIndex is included in SSB time domain position for non-serving cell. The need of halfFrameIndex for QCL measurement is unclear to us. Also, similar with Type0/0A/1, it makes sense to support Type 2 CSS only from serving cell given that inter-cell MTRP operation is working after RRC connected.

Proposal #2: halfFrameIndex for non-serving cell SSB is not needed for inter-cell MTRP operation.

Proposal #3: UE is not required to monitor a Type 2 CSS in a CORESET when the active TCI state is associated with a PCI different from serving cell PCI.

Conclusion

In this contribution, we discuss remaining issues for inter-cell multi-TRP operations, assuming multi-DCI based multi-PDSCH reception and our proposal is as follow:

Proposal #1: PDSCH/PDCCH from serving cell should be rate matched around non-serving cell SSB and PDSCH /PDCCH from non-serving cell should be rate matched around serving cell SSB.

Proposal #2: halfFrameIndex for non-serving cell SSB is not needed for inter-cell MTRP operation.

Proposal #3: UE is not required to monitor a Type 2 CSS in a CORESET when the active TCI state is associated with a PCI different from serving cell PCI.