3GPP TSG RAN WG1 #108-e				                            R1-2201573

e-Meeting, February 21th – March 3rd, 2022

Agenda item:	8.1.4

Source:	LG Electronics

Title:	CSI enhancements for Rel-17

Document for:	Discussion and Decision

Introduction

After RAN1 #107-e meeting [1], the first version of Rel-17 specification has been released. In this contribution, we provide our view on the remaining issue for the maintenance of Rel-17 CSI enhancement.

Discussion

Enhancements on multi-TRP transmission

Agreement @106b-e For CSI measurement associated with a CSI-ReportingConfig for NCJT, support two CMRs within the same CMR pair configured for NCJT measurement hypothesis to be restricted within X continuous slot(s) without DL/UL switch between two CMRs - X=1, 2 - whereas X=1 implying the same slot and X=2 implying two adjacent slots - FFS other restrictions for FR2 FFS whether UE capability is needed for X=2 TS38.214-h00 For a CSI-RS Resource Set for channel measurement configured with two Resource Groups and N Resource Pairs, the slot offsets of the two resources in a Resource Pair are configured within X∈{1,[2]} slots, without DL/UL switching in between the two resources, where X=1 implies that the two resources are configured in the same slot, and X=2 implies that the two resources are configured within two adjacent slots.

Regarding the above agreement, a FFS point should be clarified for the maintenance. In our perspective, a new UE capability for X=2 should be introduced. This is because the value of X can impact the UE implementation. For example, for two CMRs for NCJT CSI calculation, when X=2, it is necessary to occupy the memory for a longer time than when X=1. Furthermore, the value of X also can impact CSI accuracy and latency as discussed during the last meeting. So, it is preferred to report UE’s capability about the value of X.

Proposal #1: Support UE capability for X=2.

Conclusion

In this contribution, we discuss on the remaining issue for the maintenance of Rel-17 CSI enhancement and propose the following based on the discussion.

Proposal #1: Support UE capability for X=2.

Reference

[1] “3GPP RAN1 #107-e Chairman’s Notes”, e-Meeting, Nov. 11th – 19th, 2021