3GPP TSG RAN WG1 #108-e			        											             										 R1- 2201574

e-Meeting, February 21th – March 3rd, 2022

Agenda item:	8.16.1

Source:	LG Electronics

Title:	Discussion on Rel-17 UE feature for NR FeMIMO

Document for:	Discussion and Decision

Introduction

In RAN1#107bis-e, all agreements on UE feature for NR FeMIMO are captured in Section 6 of [1].

In this contribution, we share our views on remaining issues on the FeMIMO UE feature.

Discussion

  - Multi-beam Operation

23-1-1

- FG description: This FG is for ‘joint DL/UL TCI update’ and there should be another FG for ‘separate DL/UL TCI update’. It can also be considered to define separate FGs for inter-cell BM.

- Component 3, 4, 5(a), 6, 8(a), 11: These should be included in the basic FG 23-1-1 if basic FG is defined.

- Component 8(b), 8(c): These were agreed as optional in RAN1 so these should be in a separate FG. In addition, it was also agreed that UEs supporting TCI state indication by DCI without DL assignment must support TCI state indication by DCI with DL assignment.

- Component 2, 9, 12, 13: These belong to separate FG(s) for multi-CC BM. Reference BWP/CC based beam management can be a sub-FG for a general FG for multi-CC BM. In addition, component 12 and 13 can be deleted and merged with component 9 for simplicity.

- Component 7: It should be in a separate FG per previous RAN1 agreement

- Component 10, 14: We see no need to define these components.

Proposal 1: Adopt the following table for Rel-17 multi-beam 23-1-1.

| 23. NR_FeMIMO | 23-1-1 | Unified TCI with joint DL/UL TCI updatefor [intra- [and inter-cell] beam management | - Joint DL/UL TCI update with their components: (configuration mechanism, QCL rules, applicable source and target signals) - For PUCCH, PUSCH, and SRS, association between TCI state and UL PC settings except for PL RS - The maximum number of configured joint TCI states [per BWP per CC] [in a band] [in a band combination] - The maximum number of MAC-CE activated joint TCI states across all CCs [in a band] [in a band combination] a) The maximum number of MAC-CE activated joint TCI states per CC [in a band] [in a band combination] - The minimum beam application time between PUCCH of ACK and the first slot in Y symbols per SCS - TCI state indication [mode]: update and activation [in case of updates] a) MAC CE based TCI state indication [for one active TCI state] - [R17 mechanism reusing R15/16 signalling to indicate Support of indication/configuration of R17 TCI for individual DL channel/RS that cannot share the same unified TCI as UE-dedicated states for aperiodic CSI-RS, PDCCH, /PDSCH] , and SRS reusing the Rel-15/16 signaling/configuration design(s) Note: This has no impact on detail signaling design for SRS TCI indication |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-1-1a | TCI state indication by DCI | 1. Support of TCI state indication by DCI in addition to MAC CE based TCI state indication 2. Supported indication method a) MAC CE+DCI based TCI state indication using DCI formats 1_1/1_2 with DL assignment b) MAC CE+DCI based TCI state indication using DCI formats 1_1/1_2 without DL assignment Note: UEs supporting b) must also support a) |  |  |  |  |  |  |  |  |  |  |
| 23. NR_FeMIMO | 23-1-1b | Multi-CC simultaneous TCI state ID update | 2. Common multi-CC TCI state ID update and activation |  |  |  |  |  |  |  |  |  |  |
| 23. NR_FeMIMO | 23-1-1c | Multi-CC simultaneous TCI state ID update based on single TCI state pool in reference BWP/CC | 9. Reference BWP/CC configured with reference TCI state pool shared by a set of BWP/CC Note: agree component, final wording may change (e.g., when this is merged with other components/FGs) | 23-1-1b |  |  |  |  |  |  |  |  |  |
| 23. NR_FeMIMO | 23-1-1d | Beam misalignment for PL RS | 7. Beam misalignment between the DL source RS in the TCI state to provide spatial relation indication and the PL-RS |  |  |  |  |  |  |  |  |  |  |

23-1-2

- FG description: ‘for inter-cell BM [and mTRP]’ seems redundant, so suggest to delete it.

- Component 1, 2, 3: It should be included in the basic FG 23-1-2. For component 3, the number of additional PCIs for beam measurement can be typically common for FR1 and FR2. Moreover, suggest to remove the bracket on ‘RRC configured’ as agreed in RAN1#106bis-e

- Component 4-13: We see no strong need for these components. Some components such as component 4/5 may be addressed by existing 16-1g or a new component for Rel-17 analogous to 16-1g, e.g. 23-5-1a.

Proposal 2: Adopt the following table for Rel-17 multi-beam 23-1-2.

| 23. NR_FeMIMO | 23-1-2 | Inter-cell beam measurement and reporting | 1. Support of L1-RSRP measurement and reporting on SSB(s) with PCI(s) different from serving cell PCI - Support of up to K[=4] SSBRI-RSRP [pairs/beams] in one report [where at least one [pair/beam] associated with a PCI different from serving cell PCI can be reported] FFS: if K is a component candidate value - The maximum number of RRC configured PCI(s) different from serving cell PCI for L1-RSRP measurement | [2-24, 2-29] |  |  |  | [per band] |  |  |  | [2. Candidate value of {1,2,3,4}] [3. candidate values {1, 2, 4, 6}] [4. candidate values {1, 2, 4}] [5. candidate values: {2, 4, 8, FFS}] [6. candidate values: {4, 8, 16, 32, 64, FFS}] [7. candidate values {not support}] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

23-1-4

- Component 1: Regarding the candidate value, we support to include ‘0’ for max number of SRS ports per each value set to indicate that the UE has DL only panel(s).

Proposal 3: Adopt the following table for Rel-17 multi-beam 23-1-4.

| 23. NR_FeMIMO | 23-1-4 | MPUE support for UL | [depending on agreement] 1. Supported UE capability value [sets] and corresponding max number of SRS ports for each UE capability value [set] |  |  |  |  | FSPC per band |  |  |  | Candidate values: [Up to [4] value [sets] each with one value of {0, 1,2,4}] Note: the reported list contains only unique value [sets] This FG is a working assumption | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

  - Multi-TRP PDCCH

23-2-1

- Component 1: Remove [non-SFN scheme] and [including PDCCH repetition for Type 3 CSS]. SFN PDCCH has already defined as separate feature from PDCCH repetition and Type 3 CSS repetition is separately captured as FG 23-2-3.

23-2-1a

- We are fine with removing note and making this FG independent with reported value in component 2 in FG 23-2-1.

23-2-2

- Remove [with non-SFN TDM and/or] since this FG has no relation with SFN and TDM.

Proposal 4: Adopt the following table for Rel-17 M-TRP PDCCH.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-2-1 | PDCCH repetition | 1. Support of intra-slot PDCCH repetition based on two linked SS sets associated with corresponding CORESETs [with TDM and FDM (except FR2)] 2. Support of reporting one number as required number of BDs for the two PDCCH candidates FFS: 3. Support max number of overlaps when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate per scheduled component carrier |  |  |  |  |  |  |  |  | Component 2 candidate values: details 2 or 3 Component 4 candidate values: [{0,1,2,3}] [Note: UE supports PDCCH repetition for the following (basic) PDCCH monitoring capability: For type 1 CSS with dedicated RRC configuration, type 3 CSS, and UE-SS, the monitoring occasion is within the first 3 OFDM symbols of a slot] [Note: for component 3, if N PDCCH candidates are overlapped, the number of overlaps is counted as one.] | Optional with capability signalling |
| 23. NR_FeMIMO | 23-2-1a | Monitoring of individual candidates | Support of monitoring of individual candidates when one of the linked PDCCH candidates uses the same set of CCEs as an individual (unlinked) PDCCH candidate, and they both are associated with the same DCI size, scrambling, and CORESET | [23-2-1] |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-2-2 | Two QCL TypeD for CORESET monitoring in PDCCH repetition | Support of determining two QCL-TypeD for time-domain overlapping CORESETs in the same CC or for intra-band CA when UE is configured with PDCCH repetition FDM sheme | 23-2-1 |  |  |  | Per band |  |  | FR2 only |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-2-3 | PDCCH repetition for Type3 CSS | Support of PDCCH repetition for Type3 CSS |  |  |  |  |  |  |  |  |  | Optional with capability signalling |

  - Multi-TRP PUSCH/PUCCH

23-3-1

    - Remove [-CB] for feature group since it has already reflected in candidate component values.

    - Component 1: Remove [for CB] since it has already reflected in candidate component values. Remove square bracket for cyclic mapping since it was agreed in RAN 1 meeting.

    - Component 3: remove square bracket and support dynamic switching as a basic feature which makes it aligned with latest spec description.

    - Remove FFS and add Component 4 supporting two SRS resource sets whose usage is CB or non-CB.

23-3-2

    - Component 1: Remove square bracket for cyclic mapping since it was agreed in RAN 1 meeting.

    - Component 2: RAN 2 agreed to introduce a new IE for power control for mTRP FR 1 operation. So we can revise it as follows: Support of up to two PUCCH spatial relation per PUCCH resource for FR 2 and up to [two PC sets] per PUCCH resource for FR 1.

23-3-3

    - Remove square bracket for cyclic mapping since it was agreed in RAN 1 meeting.

Proposal 5: Adopt the following table for Rel-17 M-TRP PUSCH/PUCCH.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-3-1 | Multi-TRP PUSCH repetition (type A) | 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type A) - - sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions [2. The maximum number of PHR reporting across all CCs (including those related to M-TRP PUSCH repetition and the legacy Rel-15/16 PUSCH transmission)] 3. Support dynamic switching between multi-TRP PUSCH scheme and single-TRP PUSCH transmission 4. Support of two SRS resource sets for CB or non-CB PUSCH transmission |  |  |  |  |  |  |  |  | [Candidate component values: {CB, non-CB, both}] | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1a | Cyclic mapping | Support of cyclic mapping when the number of repetitions is larger than 2 for single DCI based M-TRP PUSCH repetition Type A |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1b | Second TPC field | Support of second TPC field for per TRP closed-loop power control for PUSCH with DCI formats 0_1 / 0_2 |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1c | Two PHR reporting | Support of PHR reporting related to M-TRP PUSCH repetition (calculate two PHRs (at least corresponding to the CC that applies m-TRP PUSCH repetitions), each associated with a first PUSCH occasion to each TRP, and report two PHRs.) |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1e | A-CSI report | Support of A-CSI report on two PUSCH repetitions |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1f | SP-CSI report | Support of SP-CSI report on two PUSCH repetitions |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1g | CG PUSCH transmission | Support of CG PUSCH transmission towards M-TRPs using a single CG configuration (Use same beam mapping principals as dynamic grant PUSCH repetition scheme.) |  |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-1-1 [-CB] | Multi-TRP PUSCH repetition (type B) | 1. Support of multi-TRP PUSCH repetition (based on PUSCH repetition type B) [for CB] FFS: Support PUSCH operations: CB based and NCB based and corresponding parameters including number of SRS resources |  |  |  |  |  |  |  |  | [Candidate component values: {CB, non-CB, both}] | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-2 | Multi-TRP PUCCH repetition | 1. Support of PUCCH repetition scheme 1 (inter-slot repetition) - sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions 2. Support of up to two PUCCH spatial relation per PUCCH resource for FR 2 and up to [two PC sets] per PUCCH resource for FR 1. | FFS |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-2b |  | Support of cyclic mapping for beam mapping/power control parameter set mapping for PUCCH repetitions scheme 1 and/or 3 when the number of repetitions is larger than 2 | FFS |  |  |  |  |  |  |  | FFS: candidate values | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-2c |  | Support of second TPC field for per TRP closed-loop power control for PUCCH with DCI formats 1_1 / 1_2 | FFS |  |  |  |  |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-3-3 | Multi-TRP PUCCH repetition-intra-slot | Support of PUCCH repetition scheme 3 (intra-slot repetition) - sequential mapping for repetitions larger than 2 - cyclic mapping for 2 repetitions | FFS |  |  |  |  |  |  |  |  | Optional with capability signalling |

  - Multi-TRP beam management

23-5-1

    - Component 2 and 3: It should be removed and can be covered by the separate row of 23-5-1a or existing 16-1g.

23-5-1a

    - Prefer to keep this row considering the number of increased beam measurement related resources in Rel-17, e.g., increased BFD-RSs from increased number of BFD-RS sets, inter-cell beam measurement. Similar discussion is ongoing for component 12, 13 of 23-1-2 with regard to inter-cell beam management, it seems to be natural to cover the component 12, 13 of 23-1-2 in this row, too.

23-5-2

    - Component 2: It can be redundant, since component 1 and 4 already imply that the UE supports M-TRP BFR with two BFD-RS sets.

    - Component 3: It is not needed if FG 23-5-2a is introduced.

23-5-2a

    - Component 1: Prefer to revise the component 1 as below, since we think there is no need to introduce value range. All UE must support at least one PUCCH-SR for BFRQ as Rel-16 BFR. All we need to define is to indicate whether to support two PUCCH-SR resources or not.

    - Component 2: Based on the below agreement, square bracket can be removed.

Agreement

Support to configure an association between a BFD-RS set on SpCell and a PUCCH-SR resource / SR configuration for per TRP BFR.

- FFS: Configure an association between a BFD-RS set on SCell and a PUCCH-SR resource / SR configuration for per TRP BFR

A UE capability signaling is introduced for indicating the support of this association. Above applies only for multi-DCI case.

Proposal 6: Adopt the following table for Rel-17 M-TRP beam management.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-5-1 | Group based L1-RSRP reporting enhancements | 1. Max number N of beam groups (M=2 beams per beam group) in a single L1-RSRP reporting instance based on measurement on two CMR resource sets |  |  |  |  |  |  |  |  | Component 1 candidate values: {1,2,3,4} | Optional with capability signalling |
| 23. NR_FeMIMO | 23-5-1a | Resources [for beam management , PL measurement, BFD, RLM, and new beam identification] | Note: Strive to align the final implementation of FG 23-5-1a with related R15/16 implementations If 23-5-1a ends up being identical to one or more of Rel.15/ 16 FGs this row will be deleted |  |  |  |  |  |  |  |  | Component 1 candidate values: FFS | Optional with capability signalling |
| 23. NR_FeMIMO | 23-5-2 | MTRP BFR enhancements | 1. Support of the maximum number of BFD-RS resources per set 4. Supported maximum number of BFD-RS resources across two BFD-RS sets per BWP |  |  |  |  |  |  |  |  | [Candidate values: {1, 2,…}] | Optional with capability signalling |
| 23. NR_FeMIMO | 23-5-2a | PUCCH-SR resources for MTRP BFRQ | 1. Support of two PUCCH-SR resources for MTRP BFRQ 2. Association between BFD-RS resource set on SpCell and a PUCCH SR resource |  |  |  |  |  |  |  |  |  | Optional with capability signalling |

  - Multi-TRP HST-SFN

23-6-1

    - Default QCL assumption for PDCCH is not clear to us because TCI state(s) for PDCCH can be explicitly configured by RRC/MAC-CE. So, the description for default QCL assumption should be removed.

    - The scheme combination for SFN scheme A PDCCH and single TRP PDSCH was agreed. So, a capability for this scheme combination should be supported. It can be supported by a separate FG as 23-6-1b.

23-6-1b

    - Default QCL assumption for PDSCH is defined as a separate FG. The description for default QCL assumption should be removed.

    - The capability for the scheme combination of SFN scheme A PDSCH and SFN scheme A PDCCH is defined 23-6-1. So, we do not need to consider this combination in this FG.

23-6-2

    - Suggest to remove the text related to default QCL assumption for PDCCH with the same reason as 23-6-1.

23-6-2b

    - Suggest to remove the text related to default QCL assumption for PDSCH with the same reason as 23-6-1b.

    - Suggest to revise the wording to be aligned with 23-6-1b.

Proposal 7: Adopt the following table for Rel-17 Multi-TRP HST-SFN.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-6-1 | SFN scheme A (scheme 1) for PDSCH and PDCCH | 1. Support of SFN scheme A for PDCCH scheduling SFN Scheme A PDSCH |  |  |  |  | [Per band or per FS or FSPC] |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-1c | SFN scheme A (scheme 1) for PDCCH only | 1. Support of SFN scheme A for PDCCH scheduling single TRP PDSCH |  |  |  |  | [Per band or per FS or FSPC] |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-1b | SFN scheme A (scheme 1) for PDSCH only | 1. Support of SFN scheme A for PDSCH only scheduled by single TRP PDCCH |  |  |  |  | [Per band or per FS or FSPC] |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-2 | SFN scheme B (TRP based pre-compensation) for PDSCH and PDCCH | 1. Support of SFN scheme B for PDCCH scheduling SFN Scheme B PDSCH | [23-6-1] |  |  |  | [Per band or per FS or FSPC] |  |  |  |  | Optional with capability signalling |
| 23. NR_FeMIMO | 23-6-2b | SFN scheme B (TRP based pre-compensation) for PDSCH only | 1. Support of SFN scheme B for PDSCH only scheduled by single TRP PDCCH | [23-6-1] |  |  |  | [Per band or per FS or per FSPC] |  |  |  |  | Optional with capability signalling |

  - SRS

23-8-1

    - Component 2: This component is not needed as legacy.

Proposal 8: Adopt the following table for Rel-17 SRS.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-8-1 | SRS triggering offset enhancement | 1. [Support of/The maximum number of configured available slots offsets for] determining aperiodic SRS location based on available slot | 2-52 |  |  |  | Per band |  |  |  | Candidate 1 component values: {1, 2, [3,] 4} | Optional with capability signalling |

  - CSI enhancement for Multi-TRP

23-7-1

    - Regarding candidate value set for component 2, the need of value 0 is not clear. And, value 2 is needed for both mode 1 with X=0 and mode 2, so it should be supported.

    - Regarding component 3, candidate value set is already clearly described, so we prefer to simplify the description of this component.

23-7-3

    - This FG is included in 23-7-1 as component 2, so this FG is not needed.

Other

    - Regarding the following agreement, a FFS point should be clarified for the maintenance. In our perspective, a new UE capability for X=2 should be introduced. This is because the value of X can impact the UE implementation. For example, for two CMRs for NCJT CSI calculation, when X=2, it is necessary to occupy the memory for a longer time than when X=1. Furthermore, the value of X also can impact CSI accuracy and latency. So, it is preferred to report UE’s capability about the value of X.

Agreement @106b-e

For CSI measurement associated with a CSI-ReportingConfig for NCJT, support two CMRs within the same CMR pair configured for NCJT measurement hypothesis to be restricted within X continuous slot(s) without DL/UL switch between two CMRs

- X=1, 2

  - whereas X=1 implying the same slot and X=2 implying two adjacent slots

- FFS other restrictions for FR2

FFS whether UE capability is needed for X=2

Proposal 9: Adopt the following table for Rel-17 CSI enhancement for Multi-TRP.

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 23. NR_FeMIMO | 23-7-1 | Basic Features of CSI Enhancement for Multi-TRP | - Support of NZP CSI-RS resource pairs used as CMR (channel measurement resource) pairs for NCJT measurement hypothesis: Support of N=1 - Maximum number of NZP CSI-RS resources in one CSI-RS resource set: Ks,max - CSI report mode - A list of [supported combinations, up to 16, across all CCs simultaneously, where each combination is] - [Maximum number of Tx ports in one NZP CSI-RS resource associated with a single-TRP measurement hypothesis] - Maximum number of Tx ports in one NZP CSI-RS resource associated with an NCJT measurement hypothesis - [Maximum total number of CMRs for single-TRP measurement] [per CC/across all CCs] - Maximum total number of CMRs for NCJT measurement [per CC/across all CCs] - [Maximum total number of Tx ports of NZP CSI-RS resources associated with single-TRP measurement hypotheses] [per CC/across all CCs] - Maximum total number of Tx ports of NZP CSI-RS resources associated with NCJT measurement hypotheses [per CC/across all CCs] - [Maximum total number of Tx ports of NZP CSI-RS resources associated with one NCJT measurement] - [A list of (Y1,Y2): UE can process Y1 NCJT CSI and Y2 sTRP CSI measurement hypothesis simultaneously in a CC] - [A list of (X1,X2): UE can process X1 NCJT CSI and X2 sTRP CSI measurement hypothesis simultaneously across all CCs] | FFS |  |  |  | [Per band and per BC] |  |  |  | Component 2 candidate value set: { 2, 3, 4, 5, 6, 7, 8} Component 3 candidate value set: { mode 1 with X=0, mode 2, both} Component 5 candidate values: - [{2, 4, 8, 12, 16, 24, 32}] - {2, 4, 8, 12, 16[, 24, 32]} - [{1,2,3,4 … 64}] - {2,3,4 … 64} - [{4,5,6, …, 256}] - {2,3,4, …, 256} [Component 6: The list can have maximum of 16 pairs. - Y1: {1 to 4} - Y2: {1 to 8}] [Component 7: The list can have maximum of 16 pairs. - X1: {1 to 16} - X2: {1 to 32}] Note: ‘NCJT’ is not used in RAN1 specifications and will be aligned with 38.214 | Optional with capability signalling |
| 23. NR_FeMIMO | 23-7-6 | Support of adjacent slots for a CMR pair | - Support of two CMRs within the same CMR pair configured for NCJT measurement hypothesis to be restricted within two continuous slots without DL/UL switch between two CMRs | 23-7-1 |  |  |  |  |  |  |  |  | Optional with capability signalling |

off adjacent slots for lots

References

[1] R1-2200765, Summary of UE features for further enhancements on NR-MIMO, RAN1#107b-e

[2] R1-2112976, Consolidated higher layers parameter list for Rel-17 NR, RAN1#107-e