3GPP TSG RAN WG1 #108-e		R1-2201580

e-Meeting, February 21st – March 3rd, 2022

Agenda Item: 	8.3.3

Source: 	Sony

Title: 	Remaining issues on intra-UE multiplexing & prioritisation

Document for: 	Discussion / decision

# Introduction

We made the following agreements & working assumptions in RAN1#107bis-e:

Agreement

For resolving collision of PUCCHs and PUSCHs of different priorities in step 2.2, LP PUSCH(s) overlapping with HP PUCCH which carries positive SR are dropped.

Working Assumption

For resolving collision of PUCCHs and PUSCHs of different priorities in step 2.2, LP PUSCH(s) overlapping with HP PUCCH which carries positive SR are dropped before UCI multiplexing.

- Step 1.2 behavior is not affected by the above

Agreement

For resolving collision of PUCCHs of different priorities without repetition within a time unit, down-select from the following options:

  - Option 1:

    - The reference PUCCH resource is determined as in Rel-15, i.e. based on the starting symbol and duration

    - In step 2.1-2, select up to one PUCCH resource overlapping with the reference PUCCH resource according to Rel-15 pseudo code

  - Option 2:

    - The reference PUCCH resource is determined as in Rel-15, i.e. based on the starting symbol and duration

    - In step 2.1-2, select all the PUCCH resources overlapping with the reference PUCCH resource according to Rel-15 pseudo code

  - Option 3:

    - The reference PUCCH resource is determined by prioritizing HP PUCCH over LP PUCCH on top of Rel-15 rules

    - In step 2.1-2, select all the PUCCH resources overlapping with the reference PUCCH resource according to Rel-15 pseudo code

  - Option 4:

    - The reference PUCCH resource is determined by prioritizing LP PUCCH carrying HARQ-ACK on top of Rel-15 rules

    - In step 2.1-2, If a LP PUCCH carrying HARQ-ACK overlaps with multiple HP PUCCHs and one of the HP PUCCH includes HARQ-ACK, only select the HP PUCCH including HARQ-ACK in step 2.1-2; otherwise, select all the PUCCH resources overlapping with the reference PUCCH resource according to Rel-15 pseudo code

FFS: Details on time units for all options

Working Assumption

For resolving collision of PUCCHs of different priorities without repetition within a time unit, the time unit of HP HARQ-ACK is used. For a LP PUCCH overlapping with multiple time units, down-select from:

  - Alt. 1: the LP PUCCH is associated with the first time unit with overlapping HP PUCCH(s)

  - Alt. 2: the LP PUCCH is associated with the first time unit with overlapping HP PUCCH with HARQ-ACK if any. Otherwise, the LP PUCCH is associated with the first time unit with overlapping HP PUCCH(s).

  - Alt. 3: the LP PUCCH is associated with the last time unit with overlapping HP PUCCH(s)

In this contribution, we discuss some remaining issues on intra-UE multiplexing.

# Discussions

## LP PUSCH & HP SR

It was agreed that an LP PUSCH is dropped if it overlaps with an HP PUCCH carrying SR.  For the scenario where an LP PUSCH overlaps with an HP PUCCH carrying HARQ-ACK and an HP PUCCH carrying SR as shown in Figure 1:

- If UCI multiplexing is performed first before considering the dropping of LP-PUSCH, then the HP HARQ-ACK would multiplex into the LP PUSCH which would overlap with the HP SR.   Since we agreed that LP PUSCH is dropped if it overlaps with HP SR, then the multiplexed HP HARQ-ACK in the LP PUSCH will also be dropped.  This outcome is not desirable as an HP HARQ-ACK is dropped.

- If the LP PUSCH dropping is performed first before considering UCI multiplexing, then this will result in no overlapping and the HP HARQ-ACK and HP SR can be transmitted.  This is the preferred outcome.

Figure 1: LP PUSCH overlapping HP HARQ-ACK & HP SR

Proposal 1: Confirm the following Working Assumption:

For resolving collision of PUCCHs and PUSCHs of different priorities in step 2.2, LP PUSCH(s) overlapping with HP PUCCH which carries positive SR are dropped before UCI multiplexing.

- Step 1.2 behavior is not affected by the above

## Reference PUCCH

In Rel-15, when more than two PUCCHs collide within a time unit, i.e. a slot, the multiplexing process is performed by firstly determining a reference PUCCH and other PUCCH colliding with the reference PUCCH are resolved such that a resultant PUCCH is produced.  If there are still remaining colliding PUCCHs, then another reference PUCCH is determined, and the process is repeated until a single resultant PUCCH remains.  The reference PUCCH is the PUCCH with the earliest starting symbol and if there is more than one PUCCH with the same earliest starting symbol, the PUCCH with the longest duration is selected.

For multiplexing of PUCCH with different L1 priorities, for the determination of the reference PUCCH, we agreed to down select from the following options:

  - Option 1:

    - The reference PUCCH resource is determined as in Rel-15, i.e. based on the starting symbol and duration

    - In step 2.1-2, select up to one PUCCH resource overlapping with the reference PUCCH resource according to Rel-15 pseudo code

  - Option 2:

    - The reference PUCCH resource is determined as in Rel-15, i.e. based on the starting symbol and duration

    - In step 2.1-2, select all the PUCCH resources overlapping with the reference PUCCH resource according to Rel-15 pseudo code

  - Option 3:

    - The reference PUCCH resource is determined by prioritizing HP PUCCH over LP PUCCH on top of Rel-15 rules

    - In step 2.1-2, select all the PUCCH resources overlapping with the reference PUCCH resource according to Rel-15 pseudo code

  - Option 4:

    - The reference PUCCH resource is determined by prioritizing LP PUCCH carrying HARQ-ACK on top of Rel-15 rules

    - In step 2.1-2, If an LP PUCCH carrying HARQ-ACK overlaps with multiple HP PUCCHs and one of the HP PUCCH includes HARQ-ACK, only select the HP PUCCH including HARQ-ACK in step 2.1-2; otherwise, select all the PUCCH resources overlapping with the reference PUCCH resource according to Rel-15 pseudo code

Option 3 and Option 4 deviate from Rel-15 where the rationale is to reduce the potential dropping of LP PUCCH in some corner cases, which does not justify the additional complexity/processes required for this optimization.  Option 1 and Option 2 select the reference PUCCH as per the Rel-15 procedure but it isn’t clear what the benefit is of Option 1 being limited to only overlapping PUCCH, which again deviates from the Rel-15 procedure.  Option 2 reuses the Rel-15 procedure and hence is our preference.

Proposal 2: For resolving collision of PUCCHs of different priorities without repetition within a time unit:

  - Option 2:

    - The reference PUCCH resource is determined as in Rel-15, i.e. based on the starting symbol and duration

    - In step 2.1-2, select all the PUCCH resources overlapping with the reference PUCCH resource according to Rel-15 pseudo code

## Multiplexing Time Unit

In Rel-15, the PUCCH time unit for UCI multiplexing is a slot.  In Rel-17 a PUCCH can have a time unit of slot and sub-slot and hence PUCCH of different time units can collide, such that a slot based LP PUCCH can overlap multiple HP PUCCHs carrying HARQ-ACKs in different sub-slots.  The time unit used to process such collisions needs to be clarified.  In RAN1#107bis-e, we made a working assumption that the time unit used is based on the HP HARQ-ACK and also considered three alternatives on which time unit the LP PUCCH is associated with.  We firstly would like to confirm this working assumption.

Proposal 3: Confirm the following working assumption:

For resolving collision of PUCCHs of different priorities without repetition within a time unit, the time unit of HP HARQ-ACK is used. For an LP PUCCH overlapping with multiple time units, down-select from:

  - Alt. 1: the LP PUCCH is associated with the first time unit with overlapping HP PUCCH(s)

  - Alt. 2: the LP PUCCH is associated with the first time unit with overlapping HP PUCCH with HARQ-ACK if any. Otherwise, the LP PUCCH is associated with the first time unit with overlapping HP PUCCH(s).

  - Alt. 3: the LP PUCCH is associated with the last time unit with overlapping HP PUCCH(s)

On the time unit which the LP PUCCH is associated with, Alt. 1 is straightforward and the least complex among the 3 alternatives and we have a preference for Alt. 1.

Proposal 4: For an LP PUCCH overlapping with multiple time units, the LP PUCCH is associated with the first time unit with overlapping HP PUCCH(s) (Alt. 1)

# Conclusion

In this contribution, we discuss some remaining issues on intra-UE multiplexing and we propose the following:

Proposal 1: Confirm the following Working Assumption:

For resolving collision of PUCCHs and PUSCHs of different priorities in step 2.2, LP PUSCH(s) overlapping with HP PUCCH which carries positive SR are dropped before UCI multiplexing.

- Step 1.2 behavior is not affected by the above

Proposal 2: For resolving collision of PUCCHs of different priorities without repetition within a time unit:

  - Option 2:

    - The reference PUCCH resource is determined as in Rel-15, i.e. based on the starting symbol and duration

    - In step 2.1-2, select all the PUCCH resources overlapping with the reference PUCCH resource according to Rel-15 pseudo code

Proposal 3: Confirm the following working assumption:

For resolving collision of PUCCHs of different priorities without repetition within a time unit, the time unit of HP HARQ-ACK is used. For an LP PUCCH overlapping with multiple time units, down-select from:

  - Alt. 1: the LP PUCCH is associated with the first time unit with overlapping HP PUCCH(s)

  - Alt. 2: the LP PUCCH is associated with the first time unit with overlapping HP PUCCH with HARQ-ACK if any. Otherwise, the LP PUCCH is associated with the first time unit with overlapping HP PUCCH(s).

  - Alt. 3: the LP PUCCH is associated with the last time unit with overlapping HP PUCCH(s)

Proposal 4: For an LP PUCCH overlapping with multiple time units, the LP PUCCH is associated with the first time unit with overlapping HP PUCCH(s) (Alt. 1)

# References

[1] R1-2200771, “Summary #6 of email thread [107bis-e-R17-IIoT-URLLC-03],” Moderator (CATT), RAN1#107bis-e