3GPP TSG RAN WG1 #108-e								R1-2201581

e-Meeting, February 21st – March 3rd, 2022

Agenda Item 	:	8.4.2

Source 	:	Sony

Title 	:	Discussion on ambiguity of common TA calculation

Document for 	:	Discussion and decision

- Introduction

In RAN1#107-e, the following agreements were made on UL time synchronization [1]:

Agreement:

Using indicated Higher-layer Common TA parameters, if configured, the UE can determine the one-way propagation time ( Delaycommon) used for NTA, common  calculation as follows:

Delaycommont= DCommon tepoch+ DCommonDrift× t-tepoch+DCommonDriftVariation× t-tepoch2

Where:

- DCommon = TACommon 2, DCommonDrift=TACommonDrift2 and DCommonDriftVariation=TACommonDriftVariation2

- TACommon, TACommonDrift and TACommonDriftVariation are Common TA parameter defined in RAN1 Meeting #106-bis-e

- Delaycommont is the distance between the satellite and the uplink time synchronization reference point divided by the speed of light. DL and UL are frame aligned at the reference point with an offset given by NTA,offset.

- NTA, common is derived by the UE based on Delaycommont to pre-compensate the two-way transmission delay between the uplink time reference point and the satellite.

Agreement:

Confirm the Working assumption on granularity and bits allocation for Common TA parameters: Value range, granularity and bits allocation of Higher-layer parameters TACommon, TACommonDrift, TACommonDriftVariation are as follows:

| Parameter name | Value range | Granularity | Bits allocation |
| --- | --- | --- | --- |
|  | 0 ...66485757 (i.e: 0… 270.73 ms) |  | 26 bits |
| TACommonDrift | - 261935… + 261935 (i.e: --53.33 … +-53.33 ) |  | 19 bits |
| TACommonDriftVariation | 0…29470 (0…0.60 ) |  | 15 bits |
| - Value ranges are given in unit of corresponding granularity |  |  |  |

Agreement

- When explicitly provided through SIB, Epoch time of assistance information (i.e. Serving satellite ephemeris and Common TA parameters) is the starting time of a DL sub-frame, indicated by a SFN and a sub-frame number signaled together with the assistance information.

- Otherwise, when indicated in SIB (other than SIB1), epoch time of assistance information (i.e. Serving satellite ephemeris and Common TA parameters) is implicitly known as the end of the SI window during which the SI message is transmitted.

- When provided through dedicated signaling, epoch time of assistance information (i.e. Serving satellite ephemeris and Common TA parameters) is the starting time of a DL sub-frame, indicated by a SFN and a sub-frame number.

Based on the above agreements, the UE can calculate its NTA, common from the Delaycommont, and the equation for Delaycommont was agreed as above. However, the current specification doesn’t define the equation for Delaycommont.

In this contribution we will discuss the issue and propose solutions to clarify this issue.

- Discussion

The current version of TS38.211, section 4.3.1 includes the following:

Uplink frame number  for transmission from the UE shall start TTA=NTA+NTA,offset+NTA,adjcommon+NTA,adjUETc before the start of the corresponding downlink frame at the UE where

- NTA and NTA,offset are given by clause 4.2 of [5, TS 38.213], except for msgA transmission on PUSCH where NTA=0 shall be used;

-	NTA,adjcommon is derived from the higher-layer parameters TACommon, TACommonDrift, and TACommonDriftVariation if configured, otherwise NTA,adjcommon=0;

-	NTA,adjUE is computed by the UE based on satellite-ephemeris-related higher-layers parameters if configured, otherwise NTA,adjUE=0.

The current specification only mentions that “NTA,adjcommon is derived from the higher-layer parameters TACommon, TACommonDrift, and TACommonDriftVariation”. This means that how to calculate the common transmission delay between the uplink time reference point and the satellite is up to UE implementation. However, if different UEs use different algorithms to calculate the transmission delay, misalignment of received timing at the gNB could occur. This misalignment could increase interference of received signals transmitted from different UEs. Therefore, we propose to capture the equation to calculate the transmission delay Delaycommont and epoch time definition which are agreed in RAN1 107-e.

Proposal 1: The agreed equation of Delaycommont and epoch time tepoch definition in RAN1 107-e should be captured in specification.

If transmission delay Delaycommont and epoch time tepochare defined in specification, UE can calculate NTA,adjcommon from the transmission delay. Therefore, following text proposal can be considered for the TS38.211 specification (the highlights are added by the authors of this Tdoc). If the issue is agreed in NR NTN, IoT-NTN should also capture the agreement in the LTE specifications and we provide relevant contribution in the IoT-NTN agenda item [2].

Proposal 2: Following the text proposal can be considered for TS38.211 specification:

>>>>>>>>>>>>>>>>>>>>>>>>>>>> unchanged text omitted >>>>>>>>>>>>>>>>>>>>>>>>>>>>

Uplink frame number  for transmission from the UE shall start TTA=NTA+NTA,offset+NTA,adjcommon+NTA,adjUETc before the start of the corresponding downlink frame at the UE where

- NTA and NTA,offset are given by clause 4.2 of [5, TS 38.213], except for msgA transmission on PUSCH where NTA=0 shall be used;

-	NTA,adjcommon is derived from two times the one-way propagation time Delaycommont which is calculated from TAInfo-r17 if configured. If TAInfo-r17 is not configured, NTA,adjcommon=0;

- the Delaycommon used for NTA,adjcommon is calculated as follows:

Delaycommont= DCommon tepoch+ DCommonDrift× t-tepoch+DCommonDriftVariation× t-tepoch2

Where:

- DCommon =taCommon-r172, DCommonDrift=taCommonDrift-r172 and DCommonDriftVariation=taCommonDriftVariant-r172

- tepoch is derived as follows:

  - EpochTime-r17 when configured through [SIB] or [dedicated signaling].

  - otherwise, when indicated in [SIB (other than SIB1)], epoch time of assistance information is implicitly known as the end of the SI window during which the SI message is transmitted.

-	NTA,adjUE is computed by the UE based on satellite-ephemeris-related higher-layers parameters if configured, otherwise NTA,adjUE=0.

>>>>>>>>>>>>>>>>>>>>>>>>>>>> unchanged text omitted >>>>>>>>>>>>>>>>>>>>>>>>>>>>

# 3	Conclusions

In this contribution, we discussed issue of the common TA misalignment. We proposed as follows:

Proposal 1: The agreed equation of Delaycommont and epoch time tepoch definition in RAN1 107-e should be captured in specification.

Proposal 2: Following the text proposal can be considered for TS38.211 specification:

>>>>>>>>>>>>>>>>>>>>>>>>>>>> unchanged text omitted >>>>>>>>>>>>>>>>>>>>>>>>>>>>

Uplink frame number  for transmission from the UE shall start TTA=NTA+NTA,offset+NTA,adjcommon+NTA,adjUETc before the start of the corresponding downlink frame at the UE where

- NTA and NTA,offset are given by clause 4.2 of [5, TS 38.213], except for msgA transmission on PUSCH where NTA=0 shall be used;

-	NTA,adjcommon is derived from two times one-way propagation time Delaycommont which is calculated from TAInfo-r17 if configured. If TAInfo-r17 is not configured,  NTA,adjcommon=0;

- the Delaycommon used for NTA,adjcommon is calculated as follows:

Delaycommont= DCommon tepoch+ DCommonDrift× t-tepoch+DCommonDriftVariation× t-tepoch2

Where:

- DCommon =taCommon-r172, DCommonDrift=taCommonDrift-r172 and DCommonDriftVariation=taCommonDriftVariant-r172

- tepoch is derived as follows:

  - EpochTime-r17 when configured through [SIB] or [dedicated signaling].

  - otherwise, when indicated in [SIB (other than SIB1)], epoch time of assistance information is implicitly known as the end of the SI window during which the SI message is transmitted.

-	NTA,adjUE is computed by the UE based on satellite-ephemeris-related higher-layers parameters if configured, otherwise NTA,adjUE=0.

>>>>>>>>>>>>>>>>>>>>>>>>>>>> unchanged text omitted >>>>>>>>>>>>>>>>>>>>>>>>>>>>

References

[1]	Chairman's Notes RAN1 #107-e, November 2021.

[2]	R1-2202408, Sony, “Maintenance of IoT-NTN time and frequency synchronisation,” RAN1 #108-e, February-March 2022.