3GPP TSG RAN WG1 Meeting #108-e				R1-2201599

e-Meeting, February 21st – March 3rd, 2022

Source 	: CAICT

Title 	: UE feedback enhancements for HARQ-ACK

Agenda Item	: 8.3.1

Document for	: Discussion / Decision

- Introduction

In the previous RAN1 meeting, the remaining issues about PUCCH carrier switching for HARQ feedback were discussed. In addition to the agreements in the appendix, the following items were achieved:

Conclusion

There is no consensus for introducing further specification support for the following:

- PUCCH cell switching between cells with shared spectrum channel access (in any mode)

- PUCCH cell switching between a cell with licensed spectrum and a cell with shared spectrum channel access (in any mode)

There is no consensus to support MAC CE activation indicating a set of values of pucch-SpatialRelationInfoId applicable to the alternative PUCCH sSCell for PUCCH cell switching in Rel-17.

There is no consensus to support joint configuration of PUCCH cell switching based on dynamic indication and SPS HARQ-ACK deferral in Rel-17.

Agreement:

For PUCCH cell switching based on semi-static time domain pattern, the Type 1 HARQ-ACK codebook construction is based on the k1 set(s) of the PCell / SPCell / PUCCH SCell.

Agreement

For dynamic PUCCH cell switching, the Type 1 HARQ-ACK codebook construction is based on the k1 set(s) of the dynamically indicated PUCCH cell.

There are still some remaining issues to be resolved. In this contribution, we will discuss these aspects.

- SPS operation with dynamic PUCCH cell switching

For SPS operation with PUCCH cell switching, it was agreed the dynamic target carrier indication in a DCI also applies to:

- HARQ-ACK corresponding to the first SPS PDSCH activated by Activation DCI based on the indication in the activation DCI

- HARQ-ACK corresponding to the SPS Release DCI based on the indication in the release DCI

Following the agreement, further clarification about the PUCCH cell for SPS HARQ-ACK without associated DCI is needed. Several alternatives were discussed earlier in the previous meeting, and the following alternative proposal was finally raised [1].

- Alternative Proposal 5 (from HW): For dynamic PUCCH cell switching, the HARQ-ACK of SPS PDSCH without associated DCI is to be transmitted on PCell / SPCell / PUCCH-SCell independently of the dynamically indicated PUCCH cell in the SPS activation DCI. Only the HARQ-ACK corresponding to the first SPS PDSCH activated by Activation DCI uses the PUCCH cell based on the indication in the activation DCI (based on the earlier agreement).

- The UE does not expected to be dynamically indicated for PUCCH transmission on the PUCCH sSCell in the SPS activation DCI in a slot overlapping with a PCell / SPCell / PUCCH-SCell slot where SPS HARQ-ACK subject to another SPS config would be transmitted.

To our understanding, this proposal keeps the previous agreements and provides more flexibility than the PUCCH cell is fixed to PCell / SPCell / PUCCH-SCell. The sub-bullet dispels the worry that the first activated SPS HARQ-ACK on SCell is possible to be overlapped with other SPS HARQ-ACK on PCell / SPCell / PUCCH-SCell which may troubles HARQ-ACK codebook construction and PUCCH resource determination.

Proposal 1: For dynamic PUCCH cell switching, the HARQ-ACK of SPS PDSCH without associated DCI is to be transmitted on PCell / SPCell / PUCCH-SCell independently of the dynamically indicated PUCCH cell in the SPS activation DCI. Only the HARQ-ACK corresponding to the first SPS PDSCH activated by Activation DCI uses the PUCCH cell based on the indication in the activation DCI (based on the earlier agreement).

- The UE does not expected to be dynamically indicated for PUCCH transmission on the PUCCH sSCell in the SPS activation DCI in a slot overlapping with a PCell / SPCell / PUCCH-SCell slot where SPS HARQ-ACK subject to another SPS config would be transmitted.

For SPS HARQ-ACK timing determination, a unified value for the first SPS PDSCH and the subsequent SPS PDSCH is preferred since the corresponding transmission relates to one kind of service. Since most of the SPS HARQ-ACK transmission is on PCell, the PDSCH-to-HARQ feedback timing indicator field in the DCI could be used to determine the value of k1 from the PCell’s K1 set. When the PUCCH cell for HARQ-ACK of the first SPS PDSCH is indicated to be sSCell, a reference slot in the PCell is determined with the PDSCH-to-HARQ feedback timing indication. Then the PUCCH slot in the sSCell that overlaps with the reference slot could be determined. If the reference slot in the PCell is longer than the SCell, the first slot in the SCell overlapping with the reference slot is used. An example is shown in the Fig.1. The “reference slot”, 2nd SPS HARQ-ACK slot, 3rd SPS HARQ-ACK slot is determined with k1 indication within PCell’s K1 set {2 3 4 5}. The 1st SPS HARQ-ACK transmission is indicated on the SCell. Then the 1st SPS HARQ-ACK is transmitted in the slot which corresponds to the “reference slot” on the sSCell.

Fig. 1 An example of SPS HARQ-ACK timing determination with dynamic PUCCH cell switching

Proposal 2: When dynamic PUCCH cell switching is configured, if the DCI is for SPS PDSCH activation, the PDSCH-to-HARQ_feedback timing indicator field value maps to the Pcell’s K1 set.

- PUCCH repetition and semi-static PUCCH cell switching

If PUCCH repetition and semi-static PUCCH cell switching is simultaneously supported, it is possible the PUCCH cell is switched during repetition period of time. Whether a PUCCH repetition transmission on a different target PUCCH cell from the PUCCH cell of the first PUCCH repetition is supported was discussed in the previous meetings. If it is supported, further decisions/clarifications needed were discussed. For example, the PUCCH resource determination in the sSCell should be determined. If the repetition moves to a shorter PUCCH slot on sScell, PUCCH should be transmitted on one of or each of the sScell slot is also to be clarified. In addition, the way of UCI combination on different PUCCH cells or possibly with different PUCCH formats should be further determined. On the other hand, PUCCH repetition on a single cell needs marginal specification impacts.                      For simplicity, it is proposed a PUCCH repetition transmission on a different target PUCCH cell from the PUCCH cell of the first PUCCH repetition is not supported.

From the reliability performance point of view, it is preferred the repetition is postponed as in Rel.16 until the number of repetition transmission is reached. For the performance of latency, the situation is just as the Rel.16 was. PUCCH repetition postponement due to PUCCH cell switching does not take place for each PUCCH repetition. It depends on the slot of the first PUCCH, and semi-static time domain pattern. The PUCCH repetitions which does not includes the PUCCH cell switching time could enjoy the benefits of PUCCH repetition and PUCCH cell switching. So it is not needed to exclude PUCCH repetition and PUCCH carrier switching are simultaneously configured.

Proposal 3: For semi-static PUCCH cell switching, a PUCCH repetition transmission on a different target PUCCH cell from the PUCCH cell of the first PUCCH repetition is not supported.

- A PUCCH slot mapped to different PUCCH cell is considered as invalid for PUCCH repetition and is not counted towards the total number of PUCCH repetitions / the repetition is post-poned as in Rel-16.

- Conclusion

In this contribution, we discussed the remaining aspects about PUCCH carrier switching for HARQ feedback in Rel.17 URLLC. The following proposals are reached:

Proposal 1: For dynamic PUCCH cell switching, the HARQ-ACK of SPS PDSCH without associated DCI is to be transmitted on PCell / SPCell / PUCCH-SCell independently of the dynamically indicated PUCCH cell in the SPS activation DCI. Only the HARQ-ACK corresponding to the first SPS PDSCH activated by Activation DCI uses the PUCCH cell based on the indication in the activation DCI (based on the earlier agreement).

- The UE does not expected to be dynamically indicated for PUCCH transmission on the PUCCH sSCell in the SPS activation DCI in a slot overlapping with a PCell / SPCell / PUCCH-SCell slot where SPS HARQ-ACK subject to another SPS config would be transmitted.

Proposal 2: When dynamic PUCCH cell switching is configured, if the DCI is for SPS PDSCH activation, the PDSCH-to-HARQ_feedback timing indicator field value maps to the Pcell’s K1 set.

Proposal 3: For semi-static PUCCH cell switching, a PUCCH repetition transmission on a different target PUCCH cell from the PUCCH cell of the first PUCCH repetition is not supported.

- A PUCCH slot mapped to different PUCCH cell is considered as invalid for PUCCH repetition and is not counted towards the total number of PUCCH repetitions / the repetition is post-poned as in Rel.16.

Reference

- R1-2200776, “Final moderator summary on HARQ-ACK feedback enhancements for NR Rel-17 URLLC/IIoT”

- 3GPP TS 38.213 V17.0.0 (2022-01), “Physical layer procedures for control”

Appendix A: Agreements made on the topic of “PUCCH carrier switching for HARQ feedback” in the RAN1 #106bis-e and RAN1 #107-e meeting

Agreement

For PUCCH carrier switching, support PUCCH carrier switching only among different TDD cells with PUCCH configured on the NUL carrier in Rel-17

Agreement

For PUCCH cell switching, support independent TPC per PUCCH cell including

- Separate P0 / TPC configuration per PUCCH cell

  - Note: This flexibility is already provided as PUCCH-config is per UL BWP of a PUCCH cell

- Accumulating closed loop power control commands only within the same PUCCH target cell by reusing Rel-15 procedure, i.e.

  - For dynamic PUCCH cell indication, the TPC command in the DCI scheduling the PUCCH only applies for the dynamically indicated PUCCH target cell

  - For semi-static / time-domain pattern, the TPC command in the DCI scheduling the PUCCH only applies for the determined PUCCH target (using the time-domain pattern)

- Separate TPC command indication using DCI format 2_2 for the individual PUCCH cells

  - Note: this requires configuration of individual TPC command starting points for each PUCCH cell within DCI format 2_2

Agreement

For semi-static and dynamic indication of PUCCH cell switching, the PUCCH repetition factor is determined based on the PUCCH format or PUCCH resource on the target PUCCH cell for the first repetition.

Agreement

PUCCH cell switching between 2 cells is supported in Rel-17.

Agreement

For PUCCH cell switching and a PUCCH transmission on the alternative PUCCH cell, the alternative PUCCH cell is used to derive the downlink pathloss estimate PLb,f,c(qd), i.e., replace in the main bullet of the PLb,f,c(qd) determination in Sec. 7.2.1 of 38.213 the ‘primary cell’ with ‘cell for PUCCH transmission’

Specific to PUCCH cell switching based on dynamic indication:

Agreement

UE does not expect overlapping PUCCH slots with dynamic PUCCH cell indication on more than one cell, i.e., gNB should only dynamically indicate a single PUCCH cell for a final PUCCH slot.

Agreement

For PUCCH cell switching based on dynamic indication in the DCI,  introduce a new, dedicated DCI field for the DCI scheduling PDSCH to indicate the target PUCCH cell.

Agreement

In addition, the dynamic target PUCCH cell indication also applies to HARQ-ACK corresponding to SCell dormancy indication without scheduling PDSCH.

Agreement

Support PUCCH cell switching based on dynamic indication in the DCI using DCI format 1_2 for a UE supporting DCI format 1_2.

- The presence of the ‘PUCCH carrier switching’ bitfield in DCI format 1_2 is RRC configured.

Conclusion

There is no consensus to support multiplexing of HARQ-ACK (without dynamic PUCCH cell indication), SR and P/SP-CSI on the dynamically indicated PUCCH cell (other than PCell / PSCell / PUCCH-SCell) in Rel-17.

- FFS: further handling, incl. e.g., UE does not expect overlapping HARQ-ACK (without dynamic PUCCH cell indication), SR and P/SP-CSI or overlapping HARQ-ACK (without dynamic PUCCH cell indication), SR and P/SP-CSI is to be dropped

- FFS: overlapping definition for SR and P/SP-CSI in terms of PUCCH slot or PUCCH resource

Conclusion

There is no consensus to support the simultaneous configuration of one-shot HARQ-ACK re-transmission and dynamic PUCCH cell switching in Rel-17.

Conclusion

For dynamic PUCCH cell switching, the UE does not expect a PUCCH slot with UCI on PCell /SPCell / PUCCH SCell to overlap with a PUCCH slot with HARQ-ACK on the dynamically indicated alternative PUCCH PUCCH cell.

The UCI on PCell /SPCell / PUCCH SCell dropped due to collision with semi-static DL symbols, SSB, and symbols indicated by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set is exempted and is not multiplexed on the PUCCH on the alternative PUCCH cell.

Specific to semi-static PUCCH cell switching:

Agreement

For semi-static PUCCH cell switching, PCell / PSCell / PUCCH-SCell is reference cell:

- The time domain pattern configurations are based on the numerology of the reference cell.

- The PDSCH to HARQ-ACK offset k1 is interpreted based on the numerology and PUCCH configuration of a reference cell to be able to apply the time-domain PUCCH cell switching pattern.

- Note: There may not be a need to define a ‘reference cell’ in the specification. This terminology is used for further clarifications of the procedure.

Agreement

For semi-static PUCCH cell switching, the time-domain pattern configuration is based on the following properties:

- A single time-domain pattern is configured per PUCCH cell group

- The granularity of the time-domain pattern is one slot of the PCell / PSCell / PUCCH-SCell reference cell

- The time-domain pattern is applied periodically

  - FFS on period / pattern length (e.g., 10ms, RRC configured, …).

- The pattern defines for each slot of the PCell / PSCell / PUCCH-SCell reference cell at least the applicable target PUCCH cell

Agreement

For semi-static PUCCH cell switching, the PUCCH resource indicator (PRI) is interpreted based on the PUCCH configuration of determined target PUCCH cell.

Agreement

The periodicity / length of the time-domain pattern for semi-static PUCCH cell switching is directly determined by the RRC configuraton of the time domain pattern pucchCellPattern

- Note: pucchCellPattern has a variable length of (1… maxNrofSlots)

Agreement

Down-select in RAN1#107-e from Alt. 1 & Alt. 3 below:

For PUCCH carrier switching based on semi-static operation, for the case the PCell slot to be longer than the target PUCCH cell slot or sub-slot (i.e. multiple target PUCCH cell slots overlapping with a single PCell slot),  the following PUCCH cell slot is used for UCI transmission:

- Alt. 1: the first target PUCCH slot overlapping with the PCell slot

- Alt. 3: using a relative slot-offset within the reference cell slot, the relative slot offset is configured in the time domain pattern (i.e. time domain pattern contains ‘cell index’ & ‘slot_offset’ for each reference cell slot)

  - Note: different relative slot offset can be configured for each reference cell slot in the time domain pattern, details see R1-2108829

Agreement

Down-select in RAN1#107-e from Alt. 2 & Alt. 4 below:

For PUCCH carrier switching based on semi-static operation, for the case the PCell slot to be shorter than the target PUCCH cell slot,

- Alt. 2: the UE does not expect the same UCI type (i.e. HARQ-ACK, SR or CSI) from more than one PCell PUCCH slot to be overlapping with a single dynamically indicated PUCCH cell slot

  - Note: there can be e.g. HARQ-ACK only be present in either of the overlapping slots, but not in more than one overlapping slot.

Alt. 4: the UE does not expect a semi-static PUCC cell configuration, where a single target PUCCH slot / sub-slot would be overlapping with more than one PCell slot/sub-slot.

Agreement

Support simultaneous configuration of SPS HARQ-ACK deferral and PUCCH cell switching based on the semi-static time domain pattern:

For the target slot determination of SPS HARQ-ACK deferral,

- Step 1: the UE first determines a next PUCCH slot on the cell for PUCCH transmission using the semi-static time-domain PUCCH cell pattern and the related rules for semi-static PUCCH cell switching, followed by

- Step 2: the UE determines based on the SPS HARQ-ACK deferral rules if this PUCCH slot on the PUCCH cell for transmission is the target PUCCH slot or not.

- Note: In step 1, k is increased on PCell/PScell/PUCCH-Scell. “The next PUCCH slot” represents the slot on the PUCCH cell based on PUCCH cell pattern, which is mapped from the PCell/PScell/PUCCH-Scell slot with increased K1.

Note: The maximum deferral limitation checking is based on the effective k + kdef value based on the granularity of PCell / PScell/PUSCCH-Scell

Agreement

Support simultaneous configuration of one-shot HARQ-ACK re-transmission and semi-static PUCCH cell switching:

the ‘backward HARQ-ACK slot-offset’ is interpreted with the granularity of a PUCCH slot of the respective PHY priority of PCell /PSCell / PUCCH SCell

Agreement

Support simultaneous configuration of enhanced Type 3 CB triggering and PUCCH cell switching.

Agreement

For PUCCH cell switching based on semi-static operation, for the case the PCell slot to be longer than the target PUCCH cell slot or sub-slot (i.e., multiple target PUCCH cell slots overlapping with a single PCell slot),  adopt Alt 1, i.e., the first target PUCCH slot overlapping with the PCell slot is used for UCI transmission.

Agreement

The time-domain pattern for semi-static PUCCH cell switching is separately configurable for the primary and secondary PUCCH cell group.

Agreement

The time-domain pattern for semi-static PUCCH cell switching is based on the reference SCS configuration provided by tdd-UL-DL-ConfigurationCommon and is common to every configured UL BWP (of PCell / SPCell / PUCCH SCell).

Agreement

For PUCCH cell switching based on semi-static operation, adopt Alt. 4, i.e., the UE does not expect a semi-static PUCCH cell configuration, where a single target PUCCH slot / sub-slot would be overlapping with more than one PCell slot/sub-slot.

Agreement

For semi-static PUCCH cell switching, if the alternative PUCCH cell (i.e. PUCCH sCell) is deactivated or the alternative PUCCH Cell is dormant, the UE does not apply time-domain pattern and the UCI is to be transmitted on PCell / SPCell / PUCCH SCell.

Joint operation of semi-static & dynamic PUCCH cell switching:

Conclusion

There is no consensus to support simultaneous configuration of semi-static PUCCH cell switching and dynamic PUCCH cell switching in Rel-17.