3GPP TSG-RAN WG1 Meeting #108-e	R1-2201649

# e-meeting, 21 February - 03 March, 2022

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.212 | CR | DRAFT | rev | - | Current version: | 15.13.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction to RI bitwidth for non-PMI based CSI feedback |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2022-02-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) |  |
| Reason for change: |  | RAN1 #107 agreed to a 38.212 CR 0079 on UCI bitwidth in R1-2112770. This clarified the RI field bitwidth in non-PMI (reportQuantity set to ‘cri-RI-CQI’). The introduced CR works fine as long as the UE is not confiugred with maxMIMO-Layers for PDSCH that is smaller than the number of antenna ports supported and all rank values are allowed. However, if the gNB configures the UE with a smaller number of allowed rank values, the rank indication field size deviates from that used for codebookType=typeI-SinglePanel, and the CR caused the specification not to match the reality in the field. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Restricting the RI field bitwidth for reportQuantity=’cri-RI-CQI’ by the maximum number of supported PDSCH layers. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The newly introduced clarification for the RI bitwitdth in the case of reportQuantity set to 'cri-RI-CQI' deviates from the reality in the field for reportQuantity set to 'cri-RI-CQI', when the gNB configures the UE maxMIMO-Layers for PDSCH that is smaller than the number of antenna ports Isolated impact analysis: This change impacts non-PMI CSI reporting and does not impact other functionalities. There is no impact if the UE is not configured with maxMIMO-Layers for PDSCH. In the case that the network configures the UE with maxMIMO-Layers for PDSCH that is smaller than the number of antenna ports: - If the UE is implemented according to the CR and the gNB is not, or vice versa, the UE and the gNB have a different understanding of the Rank Indication field size and thus the UCI payload size, leding to CSI decoding failulre |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.3.1.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

##### 6.3.1.1.2	CSI only

< unchanged part not inluded >

The bitwidth for RI/LI/CQI/CRI of codebookType=typeI-SinglePanel or reportQuantity set to 'cri-RI-CQI' is provided in Tables 6.3.1.1.2-3.

Table 6.3.1.1.2-3: RI, LI, CQI, and CRI of codebookType=typeI-SinglePanel, or reportQuantity set to 'cri-RI-CQI'

| Field | Bitwidth |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | 1 antenna port | 2 antenna ports | 4 antenna ports | >4 antenna ports |  |
|  |  |  |  | Rank1~4 | Rank5~8 |
| Rank Indicator when codebookType=typeI-SinglePanel | 0 |  |  |  |  |
| Rank Indicator when reportQuantity set to 'cri-RI-CQI' | 0 | min1,log2νmax | min2,log2νmax | log2νmax | log2νmax |
| Layer Indicator | 0 |  |  |  |  |
| Wide-band CQI for the first TB | 4 | 4 | 4 | 4 | 4 |
| Wideband CQI for the second TB | 0 | 0 | 0 | 0 | 4 |
| Subband differential CQI for the first TB | 2 | 2 | 2 | 2 | 2 |
| Subband differential CQI for the second TB | 0 | 0 | 0 | 0 | 2 |
| CRI |  |  |  |  |  |

in Table 6.3.1.1.2-3 is the number of allowed rank indicator values according to Clause 5.2.2.2.1 [6, TS 38.214]. νmax is the maximum reported rank given by the higher layer parameter maxMIMO-Layers of PDSCH-ServingCellConfig, if configured, or by the maximum number of layers for PDSCH supported by the UE for the serving cell.  is the value of the rank. The value of  is the number of CSI-RS resources in the corresponding resource set. The values of the rank indicator field are mapped to allowed rank indicator values with increasing order, where '0' is mapped to the smallest allowed rank indicator value. For higher layer parameter reportQuantity set to 'cri-RI-CQI', the values of the rank indicator field are mapped to rank indicator values with increasing order, where '0' is mapped to rank-1.

< unchanged part not inluded >