3GPP TSG RAN WG1 #108-e		                          			   R1-2201660

e-Meeting, February 21st – March 3rd, 2022

Agenda Item:	8.8.2

Source:	InterDigital, Inc.

Title:	Discussions on PUCCH enhancements

Document for:	Discussion and Decision

# Introduction

In RAN1#107b-e, RAN1 made agreements regarding details of enhancement for PUCCH enhancements [1]. In this contribution, we discuss dynamic PUCCH repetition factor as well as joint channel estimation related designs for frequency hopping.

# Dynamic PUCCH repetition factor indication

In Rel-15, the PUCCH resource indicated in DCI indicating SPS release and the PUCCH resource indicated in DCI activating SPS PDSCH are used to transmit the HARQ-ACK corresponding to SPS release and SPS PDSCH reception respectively. According to 38.213 v15.14.0, n1PUCCH-AN resource configured by RRC is only used when the UE transmits HARQ-ACK information corresponding only to a PDSCH reception without PDCCH:

“If a UE transmits HARQ-ACK information corresponding only to a PDSCH reception without a corresponding PDCCH, a PUCCH resource for corresponding PUCCH transmission with HARQ-ACK information is provided by n1PUCCH-AN.”

In Rel-16, SPS-PUCCH-AN-List was introduced. The UE can be configured with a PUCCH list for SPS and a PUCCH resource within that list can be used when the UE transmits only HARQ-ACK corresponding to a SPS PDSCH reception as it is stated in 38.213 v16.8.0:

“If a UE is not provided SPS-PUCCH-AN-List and transmits HARQ-ACK information corresponding only to a PDSCH reception without a corresponding PDCCH, a PUCCH resource for corresponding PUCCH transmission with HARQ-ACK information is provided by n1PUCCH-AN.”

“If the UE is provided SPS-PUCCH-AN-List and transmits OUCI UCI information bits that include only HARQ-ACK information bits in response to one or more SPS PDSCH receptions and SR, if any, the UE determines a PUCCH resource to be

-	a PUCCH resource provided by sps-PUCCH-AN-ResourceID obtained from the first entry in sps-PUCCH-AN-List if OUCI≤2 including 1 or 2 HARQ-ACK information bits and a positive or negative SR on one SR transmission occasion if transmission of HARQ-ACK information and SR occurs simultaneously, or

-	a PUCCH resource provided by sps-PUCCH-AN-ResourceID obtained from the second entry in sps-PUCCH-AN-List, if provided, if 2<OUCI≤N1,SPS where N1,SPS is either provided by maxPayloadSize obtained from the second entry in sps-PUCCH-AN-List or is otherwise equal to 1706, or

-	a PUCCH resource provided by sps-PUCCH-AN-ResourceID obtained from the third entry in sps-PUCCH-AN-List, if provided, if N1,SPS<OUCI≤N2,SPS where N2,SPS is either provided by maxPayloadSize obtained from the third entry in sps-PUCCH-AN-List or is otherwise equal to 1706, or

-	a PUCCH resource provided by sps-PUCCH-AN-ResourceID obtained from the fourth entry in sps-PUCCH-AN-List, if provided, if N2,SPS<OUCI≤N3,SPS where N3,SPS is equal to 1706.”

Thus, there is no ambiguity that the UE uses the indicated PUCCH resource in the DCI indicating the SPS release to acknowledge the SPS release for both Rel-15 and Rel-16. The gNB can indicate to the UE to use PUCCH repetition using dynamic PUCCH repetition factor in the DCI.

Proposal 1: The dynamic PUCCH repetition factor indication applies to HARQ-ACK corresponding to the SPS release DCI.

For the first SPS PDSCH received after activation DCI, according to Rel-16 specification, when SPS-PUCCH-AN-List is configured, the RRC configured PUCCH resource is used to transmit the HARQ-ACK corresponding to a SPS PDSCH receptions including the first one (in case no HARQ-ACK corresponding to dynamic PDSCH). In case SPS-PUCCH-AN-List is not configured, the UE uses the indicated PUCCH resource in the DCI to acknowledge the reception of the first SPS PDSCH after activation. The gNB can indicate to the UE to use PUCCH repetition using dynamic PUCCH repetition factor in the DCI.

Proposal 2: The dynamic PUCCH repetition factor indication applies to the HARQ-ACK corresponding to SPS PDSCH received after SPS activation DCI in case SPS-PUCCH-AN-List is not configured.

# Details for configuration of frequency hopping interval

In RAN1#107e, the following agreement related to frequency hopping (FH) for DMRS bundling for PUCCH/PUSCH was made [2].

Agreement For the interaction between inter-slot frequency hopping and DMRS bundling for PUCCH/PUSCH repetitions, a UE performs the “hopping intervals determination”, “configured TDW determination”, and “actual TDW determination” in a sequential ordering, based on the following option 1. - Option 1: “hopping intervals determination” -> “configured TDW determination” -> “actual TDW determination” - DMRS bundling shall be restarted at the beginning of each frequency hop - DMRS bunding is per actual TDW - FFS: Frequency hopping pattern is determined by physical slot indices. - FFS: different FH pattern determination for PUCCH and PUSCH - FFS: details of FH pattern design - Support separate RRC configuration(s) for hopping interval and configured TDW length. - if hopping interval is not configured, the default hopping interval is the same as the configured TDW length - FFS: if both hopping interval and TDW length are not configured - Note: hopping interval is only determined by the configuration of hopping interval if hopping interval is configured

According to the agreement when the frequency hopping interval is configured by RRC, the UE determines the length of TDW according to the frequency hopping interval. When the frequency hopping interval is not configured, the UE determines the frequency hopping interval based on the configured TDW length. According to the following agreement made in RAN1#107e, TDW length, “L”, and DMRS bundling are configured by RRC separately. If DMRS bundling is enabled but TDW length is not configured, the UE follows the default behavior where the UE to determine the TDW length via L = min (maximum duration, duration of all PUSCH repetitions).

Agreement Support Option 1’-a Option 1’-a: - If L is configured, the maximum value of window length L of the configured TDW should not exceed the maximum duration, which is reported as UE capability as the duration where UE is able to maintain power consistency and phase continuity subject to power consistency and phase continuity requirements. - If L is not configured, the default value of L = min (maximum duration, duration of all PUSCH repetitions)

When frequency hopping and DMRS bundling are enabled and the UE did not receive configuration for the TDW length nor FH (frequency hopping) interval via RRC, the UE needs to determine the hopping interval. According to the agreement for PUSCH, the TDW length is determined by L = min (maximum duration, duration of all PUSCH repetitions). Reusing the same mechanism, the TDW length can be determined using L = min (maximum duration, duration of all PUCCH repetitions). Once the TDW length is determined, the UE can determine the frequency hopping interval and pattern.

One of the supporting arguments presented to set the default FH interval to the half of the window length of the configured TDW is that it enables frequency hopping even when neither FH interval nor TDW length is configured [3]. However, such default configuration seems unnecessary when frequency hopping can be enabled by configuring the FH interval. Thus, the following proposal is made.

Proposal 3: If both hopping interval and TDW length are not configured, hopping interval is determined by L = min (maximum duration, duration of all PUSCH repetitions) and L = min (maximum duration, duration of all PUCCH repetitions) for PUSCH and PUCCH, respectively.

# Conclusion.

In this contribution, the following proposal is made.

Proposal 1: The dynamic PUCCH repetition factor indication applies to HARQ-ACK corresponding to the SPS release DCI.

Proposal 2: The dynamic PUCCH repetition factor indication applies to the HARQ-ACK corresponding to SPS PDSCH received after SPS activation DCI in case SPS-PUCCH-AN-List is not configured.

Proposal 3: If both hopping interval and TDW length are not configured, hopping interval is determined by L = min (maximum duration, duration of all PUSCH repetitions) and L = min (maximum duration, duration of all PUCCH repetitions) for PUSCH and PUCCH, respectively.

# Reference

[1] RAN1 Chairman’s notes, RAN1#107b-e, Jan. 2022.

[2] RAN1 Chairman’s notes, RAN1#107-e, Nov. 2021.

[3] R1-2200754, “FL summary #2 of PUCCH coverage enhancement,” Qualcomm, RAN1#107be, Jan. 2022.