3GPP TSG RAN WG1 #108-e	R1- 2201673

e-Meeting, February 21st – March 3rd, 2021

Agenda item:		8.10.1

Source:	Nokia, Nokia Shanghai Bell

Title:		Enhancements for resource multiplexing among IAB backhaul and access links

Document for:		Discussion and Decision

# 1	Introduction

IAB Rel.17 WID [1] has following objectives led by RAN1:

Duplexing enhancements [RAN1-led, RAN2, RAN3, RAN4]:

- Specification of enhancements to the resource multiplexing between child and parent links of an IAB node, including:

  - Support of simultaneous operation (transmission and/or reception) of IAB-node’s child and parent links (i.e., MT Tx/DU Tx, MT Tx/DU Rx, MT Rx/DU Tx, MT Rx/DU Rx).

  - Support for dual-connectivity scenarios defined by RAN2/RAN3 in the context of topology redundancy for improved robustness and load balancing.

- Specification of IAB-node timing mode(s), extensions for DL/UL power control, and CLI and interference measurements of BH links, as needed, to support simultaneous operation (transmission and/or reception) by IAB-node’s child and parent links.

In RAN1 #102-e/103-e/104-e/104b-e/105-e meetings [2-17], several agreements related to resource multiplexing were made and they are captured in Annex I. In this contribution, we discuss detailed enhancements to the resource multiplexing between child and parent links of an IAB node. The multiplexing cases for simultaneous DU and MT operation are named as

  - Multiplexing Case A: Simultaneous MT-Tx/DU-Tx

  - Multiplexing Case B: Simultaneous MT-Rx/DU-Rx

  - Multiplexing Case C: Simultaneous MT-Rx/DU-Tx

  - Multiplexing Case D: Simultaneous MT-Tx/DU-Rx

Section 2 focuses on simultaneous half-duplex operation of the child and parent links (Case A and B). Section 3 discusses resource allocation enhancements related to dual connectivity scenarios.

# 2	Multiplexing operations of the child and parent links

## Extension of multiplexing operation to spatial domain

The following agreement was made in RAN1 #107-e regarding SDM enhancement in Rel-17 IAB:

Agreement

The IAB-DU is expected to apply the indicated beam restriction within its soft resources that are not explicitly indicated as available.

This agreement makes it unambiguous that beam restrictions are not applied to resources with either a hard, not available, or soft resources with an associated dynamic indication of available. Additionally, the following agreement was made in RAN1#107-e regarding the application of beam restriction indications:

Agreement

The restricted beam indication from the parent node to the IAB node may be indicated to be associated with some combination (one or multiple) of the following IAB-node’s configurations:

- {MT CC, DU cell} pair and optionally may be indicated to be associated with only {DU cell} if independent of MT CC(s)

- Multiplexing mode info (i.e. multiplexing info in 38.473) and optionally may be indicated to be applicable to non-overlapping frequency resources

- Slot index

- Association with IAB-MT’s DL Rx beam via TCI state ID and RS ID (SSB ID and/or CSI-RS ID) or UL TX beam via SRI

When beam restrictions are associated with multiplexing mode info it is unambiguous which physical resources the beam restriction applies to, and therefore slot indication is unnecessary. If a beam restriction is associated with a slot index, there is still a potential ambiguity on which resources the beam restriction is intended to apply. For this reason, slot index is not necessary, but multiplexing mode info should be included in each beam restriction indication.

Proposal 2.1:	Restricted beam indication from the parent node to the IAB node is indicated to be associated with the following IAB node’s configurations:

- {MT CC, DU cell} pair and optionally may be indicated to be associated with only {DU cell} if independent of MT CC(s)

- Multiplexing mode info (i.e. multiplexing info in 38.473) and optionally may be indicated to be applicable to non-overlapping frequency resources

- Association with IAB-MT’s DL Rx beam via TCI state ID and RS ID (SSB ID and/or CSI-RS ID) or UL TX beam via SRI

Similar consideration is relevant for recommended beam indication from the IAB node to the parent node, given the following agreement made in RAN1#107-e:

Agreement

The recommended beam indication from the IAB node to the parent node may be indicated to be associated with some combination (one or multiple) of the following IAB-node’s configurations:

- {MT CC, DU cell} pair and optionally may be indicated to be associated with only {MT CC} if independent of DU cell(s)

- Multiplexing mode info (i.e. multiplexing info in 38.473) and optionally may be indicated to be applicable to non-overlapping frequency resources

- Slot index

As with beam restriction indication, indication of slot index is not necessary in addition to multiplexing mode info, and ambiguous when provided without multiplexing mode info.

Proposal 2.2:	Recommended beam indication from the IAB node to the parent node is indicated to be associated with the following IAB node’s configurations:

- {MT CC, DU cell} pair and optionally may be indicated to be associated with only {DU cell} if independent of MT CC(s)

- Multiplexing mode info (i.e. multiplexing info in 38.473) and optionally may be indicated to be applicable to non-overlapping frequency resources

## Signaling for applicability of a multiplexing capability

In RAN1 #107-e the following agreement was reached for guard symbol indication enhancement in support of enhanced timing modes.

Agreement

The following RAN1#106bis-e agreement is updated.

The MAC-CE signaling of Desired/Provided Guard Symbols is enhanced to optionally indicate the number of guard symbols required for switching between at least the following cases:

- A: Case #6 MT TX to/from Case #1 DU RX

- D: Case #7 MT TX (to support Case #7 at parent node) to/from Case #1 DU RX

- G: Case #7 MT TX (to support Case #7 at parent node) to/from Case #1 DU TX

- H: Case #6 MT TX to/from Case #1 DU TX

As discussed in previous RAN1 meeting, guard symbols may change with the application of different timing modes. For example, when switching between case#1 timing and case#7 timing modes, a child node making an UL transmission to the IAB node will need to adjust its timing reference. RAN 1 has agreed that timing for the child node making an UL transmission to an IAB node in case#7 timing may be symbol aligned but not slot aligned.  The symbol offset between the slot boundary for an UL transmission in case#1 timing and case#7 timing will not necessarily be known by the child node and so may need to be indicated to the child node.

In Rel-16, RAN1 introduced MAC-CE messages to indicate desired guard symbols and provided guard symbols to support transition of IAB-MT and IAB-DU operations. In particular, the following details are described in TS 38.321,

5.18.19	Guard symbols for IAB

For IAB operation, the MAC entity on the IAB-DU or IAB-donor-DU reserves a sufficient number of symbols at the beginning and/or the end of slots where the child IAB-node switches operation from its IAB-DU to its IAB-MT function and operation from its IAB-MT to its IAB-DU function. The MAC entity on the IAB-DU or IAB-donor-DU informs the child node about the number of guard symbols it provides via the Provided Guard Symbols MAC CE. The IAB-MT on the child node may inform the parent IAB-DU or IAB-donor-DU about the number of guard symbols desired via the Desired Guard Symbols MAC CE.

Upon reception of a Provided Guard Symbols MAC CE the MAC entity shall:

-	indicate to lower layers the number of provided guard symbols and the SCS configuration for the indicated Serving Cell.

The MAC entity may:

1>	if a Desired Guard Symbol query has not been triggered:

2>	trigger a Desired Guard Symbol query for this Serving Cell.

If the MAC entity has UL resources allocated for new transmission the MAC entity shall:

1>	for each Desired Guard Symbol query that has been triggered and not cancelled:

2>	if the allocated UL resources can accommodate a Desired Guard Symbols MAC CE plus its subheader as a result of LCP as defined in clause 5.4.3.1:

3>	instruct the Multiplexing and Assembly procedure to generate the Desired Guard Symbols MAC CE;

3>	cancel this Desired Guard Symbol query.

A separate value for the number of guard symbols is specified for each of the following eight switching scenarios (see Table 5.18.19-1).

Table 5.18.19-1: Switching scenarios and relevant guard symbols

| Switching scenario |  | Field for number of guard symbols in MAC CE |
| --- | --- | --- |
| IAB-MT operation to IAB-DU operation | MT Rx to DU Tx | NmbGS1 |
|  | MT Rx to DU Rx | NmbGS2 |
|  | MT Tx to DU Tx | NmbGS3 |
|  | MT Tx to DU Rx | NmbGS4 |
| IAB-DU operation to IAB-MT operation | DU Rx to MT Tx | NmbGS5 |
|  | DU Rx to MT Rx | NmbGS6 |
|  | DU Tx to MT Tx | NmbGS7 |
|  | DU Tx to MT Rx | NmbGS8 |

…. Text omitted ….

For each of the guard symbol fields, NmbGSi, the guard symbols MAC CD specifies 3 bits for a range of possible guard symbols from 0-7 symbols; however, values of 5-7 symbols are reserved.  The newly agreed guard symbol fields are included to account for timing offset between each of the various multiplexing modes.  For this reason, the required range of guard symbols is expected to be larger than legacy fields.  For case #6 timing, the MT transmits synchronously with the DU Tx timing and so therefore the timing offset between MT UL timing and parent DL timing can be expected to be roughly equal to the propagation latency.  Assuming that the field size of the guard symbol indication in the guard symbol MAC CE is unchanged, using the reserved entries, i.e., 5,6,7, would allow for an additional 3 symbols.  Propagation latencies for typical cell sizes would be constitute only a small fraction of a 3-symbol duration, and the addition of 3 symbols should be sufficient supporting new multiplexing cases.

Proposal 2.3:	Guard symbol indications for new Rel-17 guard symbol field should be in the range of 0-7 symbols.

# 3	Conclusions

Our observations and proposals on the FDM/SDM half-duplex operation of the child and parent links are:

Proposal 2.1:	Restricted beam indication from the parent node to the IAB node is indicated to be associated with the following IAB node’s configurations:

- {MT CC, DU cell} pair and optionally may be indicated to be associated with only {DU cell} if independent of MT CC(s)

- Multiplexing mode info (i.e. multiplexing info in 38.473) and optionally may be indicated to be applicable to non-overlapping frequency resources

- Association with IAB-MT’s DL Rx beam via TCI state ID and RS ID (SSB ID and/or CSI-RS ID) or UL TX beam via SRI

Proposal 2.2:	Recommended beam indication from the IAB node to the parent node is indicated to be associated with the following IAB node’s configurations:

- {MT CC, DU cell} pair and optionally may be indicated to be associated with only {DU cell} if independent of MT CC(s)

- Multiplexing mode info (i.e. multiplexing info in 38.473) and optionally may be indicated to be applicable to non-overlapping frequency resources

Proposal 2.3:	Guard symbol indications for new Rel-17 guard symbol field should be in the range of 0-7 symbols.

# References

- RP-193251, New WID on Enhancements to Integrated Access and Backhaul.

- Chairman’s notes RAN1#102-e

- R1-2007241, Summary #1 of [102-e-NR-eIAB-01],	Moderator (AT&T)

- R1-2007322, Summary #2 of [102-e-NR-eIAB-01], Moderator (AT&T)

- R1-2007287, Summary of [102-e-NR-eIAB-02], Moderator (Qualcomm)

- R1-2007352,	Summary #3 of [102-e-NR-eIAB-02],	Moderator (Qualcomm)

- Chairman’s notes RAN1#103-e

- R1-2009604	Summary #1 of [103-e-NR-eIAB-01]	Moderator (AT&T)

- R1-2009567	Summary #1 of [103-e-NR-eIAB-02]	Moderator (Qualcomm Incorporated)

- R1-2101878	Summary #1 of [104-e-NR-eIAB-02] on other enhancements for simultaneous operation of IAB-node’s child and parent links	Moderator (Qualcomm)

- R1-2101889	Summary of [104-e-NR-eIAB-01] – 1st Checkpoint	Moderator (AT&T)

- R1-2101890	Summary of [104-e-NR-eIAB-01] – 2nd Checkpoint	Moderator (AT&T)

- R1-2101891	Summary of [104-e-NR-eIAB-01] – 3rd Checkpoint	Moderator (AT&T)

- R1-2101909	Summary #2 of [104-e-NR-eIAB-02] on other enhancements for simultaneous operation of IAB-node’s child and parent links	Moderator (Qualcomm)

- R1-2102031	Summary #3 of [104-e-NR-eIAB-02] on other enhancements for simultaneous operation of IAB-node’s child and parent links	Moderator (Qualcomm)

- R1-2102172	Summary #4 of [104-e-NR-eIAB-02] on other enhancements for simultaneous operation of IAB-node’s child and parent links	Moderator (Qualcomm)

- R1-2103958	Summary of [104b-e-NR-eIAB-01]	Moderator (AT&T)

- R1-2106054	Feature Lead Summary #1 of 8.10.1	Moderator (AT&T)

- R1-2106056	Feature Lead Summary #3 of 8.10.1	Moderator (AT&T)

- R1-2108277	Feature Lead Summary #1 of 8.10.1	Moderator (AT&T)

- R1-2108279	Feature Lead Summary #3 of 8.10.1	Moderator (AT&T)

- R1-2110442	Feature Lead Summary #1 of 8.10.1	Moderator (AT&T)

- R1-2110443	Feature Lead Summary #2 of 8.10.1	Moderator (AT&T)

- R1-2112804 Feature lead Summary #3 of [107-e-NR-eIAB-01] Moderator (AT&T)

# Annex I

## RAN1 #107-e meeting

Agreement

Support indication of whether FDM is required or not for an enhanced multiplexing operation mode to donor-CU.

Agreement

The maximum number of non-overlapping RB sets configurable per DU cell is M

- where, M is to be selected from one of values from 4, 8, 16

- DU frequency configuration information should be provided to the parent node.

Agreement

The value of the maximum number of contiguous and non-overlapping RB sets configurable per DU cell, M is 8.

Agreement

For DCI format 2_5 indicating availability for the soft resources of the respective RB sets corresponding to a given time resource of the child IAB-DU cell:

- AvailabiltyCombination can be extended to include multiple resourceAvailabilty, where each resourceAvailabilty includes availability indication for one RB set group

  - One RB set group consists of one or multiple RB sets

Agreement

In addition to SSB ID, CSI-RS ID may be additionally used as the RS ID for a restricted beam indication from the parent node to the IAB node.

- STC index may be additionally indicated along with SSB ID if more than one STC is configured at the IAB node.

- Note: This does not mean that IAB-specific CSI-RS should be developed and requires no additional specification work

Agreement

- The maximum number of recommended beams per MT CC in a given indication (including all associated parameters/conditions) is 8.

- The maximum number of restricted beams per DU cell in a given indication (including all associated parameters/conditions) is 8.

Agreement

The recommended beam indication from the IAB-MT to the parent node are provided using the following:

- For DL Rx beam(s)

  - DL TCI state ID and RS ID (SSB ID and/or CSI-RS ID)

- For UL Tx beam(s)

  - SRI

Agreement

The restricted beam indication from the parent node to the IAB node may be indicated to be associated with some combination (one or multiple) of the following IAB-node’s configurations:

- {MT CC, DU cell} pair and optionally may be indicated to be associated with only {DU cell} if independent of MT CC(s)

- Multiplexing mode info (i.e. multiplexing info in 38.473) and optionally may be indicated to be applicable to non-overlapping frequency resources

- Slot index

- Association with IAB-MT’s DL Rx beam via TCI state ID and RS ID (SSB ID and/or CSI-RS ID) or UL TX beam via SRI

Agreement

The recommended beam indication from the IAB node to the parent node may be indicated to be associated with some combination (one or multiple) of the following IAB-node’s configurations:

- {MT CC, DU cell} pair and optionally may be indicated to be associated with only {MT CC} if independent of DU cell(s)

- Multiplexing mode info (i.e. multiplexing info in 38.473) and optionally may be indicated to be applicable to non-overlapping frequency resources

- Slot index

Agreement

The IAB-DU is expected to apply the indicated beam restriction within its soft resources that are not explicitly indicated as available.

Agreement

Whether or not an IAB node can operate under a given non-TDM multiplexing mode (i.e. multiplexing info in 38.473) is left to IAB implementation in Rel-17

Agreement

The following RAN1#106bis-e agreement is updated.

The MAC-CE signaling of Desired/Provided Guard Symbols is enhanced to optionally indicate the number of guard symbols required for switching between at least the following cases:

- Case#6 MT Tx and [Case #7] DU [Tx]/Rx

- Case#7 MT Tx (to support Case #7 at parent node) and DU Tx/Rx

- A: Case #6 MT TX to/from Case #1 DU RX

- D: Case #7 MT TX (to support Case #7 at parent node) to/from Case #1 DU RX

- G: Case #7 MT TX (to support Case #7 at parent node) to/from Case #1 DU TX

- H: Case #6 MT TX to/from Case #1 DU TX

Agreement

A child IAB-MT can inform a parent node via MAC-CE whether Case 6 timing is required for simultaneous operation.

Agreement

An IAB-MT is provided with a Timing Case Indication via MAC-CE that explicitly indicates a list of slots and their associated UL TX timing cases (i.e., one of {Case 1, Case 6, Case 7} for each slot).

Agreement

A Timing Case Indication received from a serving cell is applicable to all other cells in the same timing advance group (TAG).

## RAN1 #106bis-e meeting

Agreement

Select the following alternative to handle potential indication conflict of symbols configured as semi-static flexible by one parent node, but not the other in inter-donor DC scenarios if the IAB MT of the dual-connected IAB-node does not support simultaneous Tx and Rx on different carriers:

- Alt. 1. The IAB MT does not expect to receive conflicting DCI formats including DCI2_0 and dynamic scheduling grants from different parents. FFS: Explicitly captured in the specification or left as a network configuration error case without specification impact

Select the following alternative to handle potential indication conflict of symbols configured as semi-static flexible by both parent nodes in inter-donor DC scenarios if the IAB MT of the dual-connected IAB-node does not support simultaneous Tx and Rx on different carriers:

- Alt. 5: If a conflict occurs, the IAB MT is expected to perform as scheduled by MCG

Agreement

In DC scenarios, support per-child MT link-NA resource configuration.

- This configuration can be made available to IAB node as well

Agreement

The Rel-17 frequency domain H/S/NA configuration is provided across multiple slots and/or over a subset of slots only, with the same time-domain granularity and pattern duration as the Rel-16 H/S/NA configuration (i.e. gNB-DU Cell Resource Configuration (9.3.1.107 in TS 38.473 [8])).

- For a given slot, different H/S/NA resource types can be configured for different RB sets

- Additional signaling details (e.g. bitmap, slot pattern, etc.) can be left up to RAN3

- FFS: The number of different frequency domain configurations at a given time

Agreement

A single value for the RB set size, N, is configured for a given IAB-DU cell’s Rel-17 frequency domain H/S/NA configuration

Working Assumption

If both the Rel-16 time domain H/S/NA configuration and Rel-17 frequency domain H/S/NA configuration are provided for a given RB set within a slot, one of the following is selected:

- Alt. 1: An IAB node applies the frequency domain H/S/NA only if the IAB node is currently operating in a non-TDM multiplexing mode in the slot, otherwise the Rel-16 time domain H/S/NA configuration is applied.

For RAN1#107-e, companies to consider the following decision point:

An IAB node (or parent node) cannot operate under a given non-TDM multiplexing mode until:

- Alt. 1: All required conditions and parameters which have been directly indicated/requested to the parent node (e.g. via MAC-CE) are explicitly acknowledged by the parent node.

- Alt. 2: All required conditions and parameters which have been directly indicated/requested to the parent node (e.g. via MAC-CE) are implicitly acknowledged by the parent node or implicitly determined at the child node

Agreement

RS ID, based on the IAB node’s DU configurations, is used by a parent node to indicate beams of an IAB-DU in the direction of which simultaneous operation is restricted

- At least SSB ID and [STC index] are supported

- FFS: Whether restrictions are indicated to apply differently for H or S resources

- FFS: Informing the parent node of SRS configuration of the IAB-MT (if collocated with the IAB-DU)

Agreement

The restricted beam indication from the parent node to the IAB node may be indicated (or specified) to be associated with some combination (one or multiple) of the following IAB node’s parameters:

- [Multiplexing mode]

- [MT’s DL beam (e.g. TCI state id)] or MT’s UL beam (e.g., SRI id)

- [DU resource configuration (e.g. soft resources)]

- [Slot index]

Agreement

The MAC-CE signaling of Desired/Provided Guard Symbols is enhanced to optionally indicate the number of guard symbols required for switching between at least the following cases:

- Case#6 MT Tx and [Case #7] DU [Tx]/Rx

- Case#7 MT Tx (to support Case #7 at parent node) and DU Tx/Rx

Agreement

The recommended beam indication from the IAB-MT to the parent node are provided via MAC-CE:

- For DL Rx beam(s): using one or more of the following:

  - DL TCI state ID

    - FFS: UE/IAB-MT does not assume that DL Tx power adjustment (if provided) is applied to the SSB index (if supported) indicated as QCLed reference signal in DL TCI state ID.

  - [RS ID]

  - [R17 DL TCI, or joint DL/UL TCI ID]

- For UL Tx beam(s): using one or more of the following:

  - [Spatial relation]

  - [RS ID]

  - [R17 UL TCI, or joint DL/UL TCI ID]

  - [SRI]

Agreement

A single DCI format 2_5 can be received indicating availability for the soft resources of the respective RB sets corresponding to a given time resource of the child IAB-DU cell.

- FFS: Extension of AvailabiltyCombination to include multiple RB sets in a resourceAvailabilty indication

- FFS: Update resourceAvailability mapping table defined in TS38.213 so that the indication of availability can be applied over soft resources in frequency-domain for DL or UL or Flexible symbols.

- FFS: Need for extension of the maximum payload size of DCI format 2_5 to increase the number of IAB-DU cells that can be provided with availability information for Soft resources to accommodate the maximum number of possible RB sets for a given DU cell (if defined), or other backwards compatible signaling extensions in case the principal indication capabilities of DCI format 2_5 are increased.

## RAN1 #106-e meeting

Agreement

The following solutions are supported to handle potential indication conflict of overlapping flexible symbols between two parent IAB-nodes:

- In intra-donor DC scenarios, if the IAB MT does not support simultaneous Tx and Rx on different carriers, it does not expect to receive conflicting DCI 2_0 from different parents.

Agreement

The IAB-donor-CU can be made aware of the IAB-MT’s capability regarding simultaneous transmission and reception on multiple serving cells in a frequency band, configured by the two parent nodes in intra-donor DC scenarios.

Agreement

The semi-static configuration of H/S/NA resource type in frequency domain is provided per RB set, per D/U/F resource type within a slot.

Agreement

N is a configured number of PRBs, where the CU configures N

- N = {2, 4, 8, 16, 32, 64}

- FFS: Value(s) of N in case of multiple configured BWPs at the IAB-MT

- This agreement does not revert any existing RAN1 agreement

Agreement

For a given RB set at a symbol, if Rel-17 frequency domain H/S/NA configuration is not provided, the Rel-16 time domain H/S/NA is applied

Agreement

A Reference SCS is configured for frequency domain H/S/NA configuration.

Agreement

Spatial domain restrictions from a parent node or recommendations from a child node is limited to a subset of time resources in which simultaneous operation is applied.

- FFS: Handling of frequency resources in case of FDM operation

- FFS: Support for implicit/explicit indication of the simultaneous operation mode

Agreement

The child node indication of recommended beams to the parent node can include both IAB-MT DL beams and/or IAB-MT UL beams.

- FFS: Indication via MAC-CE or UCI transmission

- FFS: Definition of IAB-MT DL beams and/or IAB-MT UL beams (e.g. TCI state ID, Spatial relation information ID, RS ID (including CSI-RS, SRS, SSB, etc.))

- FFS: Whether indication of “not preferred” beams is supported

Agreement

MAC-CE signaling of Desired/Provided Guard Symbols is enhanced (e.g. using the same Rel-16 MAC-CE design) to support indication of guard symbols additionally required for Case #6 and Case #7 timing cases.

- FFS: Number of guard symbols associated with Case #6 and Case #7 timing modes

- FFS: Need for explicit indication of guard symbols switching between timing cases

Agreement

To support extension of CA TDD prioritization rules to NR-DC, the following resource coordination mechanisms between parents/donors are supported:

- For intra-donor and inter-donor DC scenarios, in addition to coordination at the donor CU(s), a parent-node can be made aware of the DU resource configuration (UL/DL/FL, H/S/NA) of the other peer parent node that connects to the same IAB-node.

- For intra-donor and inter-donor DC scenarios, coordinating the semi-static and/or cell-common higher layer configuration (e.g. SSB, CORESET 0, and RACH and configurations) from/for different parent nodes.

Agreement

To support soft resource availability in the frequency domain, the existing DCI 2_5 format is reused according to one of the following alternatives:

- Alt. 1: A single DCI format 2_5 can be received indicating availability for multiple RB sets which correspond to the same time resources of the child IAB-DU cell.

- Alt. 2: Multiple DCI format 2_5 can be received indicating availability with the granularity of one or more RB set(s) for different RB sets which correspond to the same time resources of the child IAB-DU cell.

- Alt. 3: A single DCI format 2_5 can be received indicating availability of all the soft resources which correspond to the same time resources of the child IAB-DU cell.

Agreement

MAC-CE signaling from a parent node is supported for indication of beams of an IAB-DU in the direction of which simultaneous operation is restricted

- FFS: Details of beam indication (e.g. TCI state ID, Spatial relation information ID, RS ID (including CSI-RS, SRS, SSB, etc.))

- FFS: Applicability to other beams

Agreement

Select from the following alternatives to handle potential indication conflict of symbols configured as semi-static flexible by one parent node, but not the other in inter-donor DC scenarios if the IAB MT of the dual-connected IAB-node does not support simultaneous Tx and Rx on different carriers:

- Alt. 1. The IAB MT does not expect to receive conflicting DCI formats including DCI2_0 and dynamic scheduling grants from different parents. FFS: Explicitly captured in the specification or left as a network configuration error case without specification impact

- Alt. 6. The IAB-MT is expected to operate according to the non-flexible configuration.

Agreement

Select from the following alternatives to handle potential indication conflict of symbols configured as semi-static flexible by both parent nodes in inter-donor DC scenarios if the IAB MT of the dual-connected IAB-node does not support simultaneous Tx and Rx on different carriers:

- Alt. 1: The IAB MT does not expect to receive conflicting DCI formats including DCI2_0 and dynamic scheduling grants from different parents. FFS: Explicitly captured in the specification or left as a network configuration error case without specification impact

- Alt. 5: If a conflict occurs, the IAB MT is expected to perform as scheduled by MCG

- FFS: Consideration of the impact of parent node’s H/S/NA when specifying the prioritization rules in case of UL/DL conflict

Conclusion

Decide in RAN1#106bis-e whether the parent IAB DUs of a DC connected IAB node can have per-backhaul-link resource configurations.

- FFS: whether the per-backhaul-link configuration is provided to the dual-connected IAB node.

## RAN1 #105-e meeting

Agreement

For frequency domain multiplexing, H/S/NA configurations for an IAB-node are provided separately in addition to the Rel-16 H/S/NA

Agreement

DCI Format 2_5 is reused to support soft resource availability indications for frequency-domain resources

- FFS: If additional enhancements are necessary

Agreement

The parent IAB-node is dynamically provided with conditions/parameters to facilitate adaptation between multiplexing operation modes:

- FFS: Required number of guard symbols for switching of multiplexing mode (FFS: per timing mode or per multiplexing mode) for IAB-DU

- FFS: Signaling procedure

- FFS: Required guard band for FDM

- FFS: other conditions, e.g. required timing mode, required power control parameters, and preferred TCI.

Agreement

If an IAB node is configured with a frequency-domain H/S/NA configuration down select between the following options:

- Alt. 1 Either the Rel-16 H/S/NA configuration or frequency domain configuration is applied for a given resource

  - FFS: Whether configurations are switched with per-slot, per-resource type within a slot, or per-symbol granularity

- Alt. 2 The Rel-16 H/S/NA configuration and frequency domain configuration are jointly applied

Agreement

The minimum resource size for configuring the frequency domain granularity is a set of N RBs:

- Candidate values for N: {4, 8, 16, other values TBD}

- N is at least the # PRBs that are corresponding to the MT’s # PRBs of an RBG).

- FFS: Scaling or configuration of N based on system BW or size of IAB-MT BWP

Agreement

In case of intra-band inter-carrier dual connectivity for both inter-donor and intra-donor scenarios the following are supported:

- Reusing the Rel-16 CA TDD prioritization rules in case of UL/DL conflict when applicable

  - FFS: Whether all prioritization rules apply in case of NR-DC

  - FFS: Need of new prioritization rules in case of NR-DC

- Coordinating the IAB-MT’s TDD configurations to avoid conflicts from different parent nodes in case the child IAB-MT does not support simultaneous TX and RX on different carries

  - FFS: Coordination for scheduling conflicts involving at least DCI Format 2_0 usage (e.g. usage of flexible symbols)

- Exchanging H/S/NA configurations between parent nodes/donors

Agreement

In case of simultaneous MT/DU operation,

- the parent node can dynamically indicate to the child node at least a set of restricted beams at the IAB-DU of the child node

- the child node can dynamically report to the parent node a set of recommended beams, not preferred beams, or both recommended and not preferred beams of the IAB-MT of the child node

  - FFS: Whether the specification supports all reporting combinations.

- FFS: Applicability to specific multiplexing cases or specific time-frequency resources

- FFS: Additional semi-static signaling

- FFS: Per-panel granularity in addition to per-beam granularity

- FFS: Relationship between child IAB-MT beam indication and parent IAB-DU beam indication

- Note: This does not preclude any enhancements for either DU or MT-based CLI measurement and reports

Agreement

For an IAB-MT with multiple serving cells (including the case with two parent nodes), a per-cell IAB-DU soft resource is considered as available if the resource is either explicitly indicated (via DCI 2_5), or implicitly determined as available with respect to all serving cells.

- If the IAB-DU per-cell soft resource neither explicitly indicated as Available, nor implicitly determined as Available by the IAB-DU with respect to at least one serving cell

  - Alt 1. The IAB-DU per-cell resource is assumed to be not available

- This agreement does not necessarily mean the Rel-16 spec does not support what is described in the main bullet

## RAN1 #104b-e meeting

Agreement

The extension of the semi-static DU resource type indication to frequency-domain resources within a carrier (in addition to existing Rel-16 per-carrier granularity) for H/S/NA resource types is supported

Agreement

For the semi-static DU resource configuration in the frequency domain within a carrier, the frequency-domain granularity is configurable

- FFS:  minimum resource size e.g. N PRBs/N RBGs

- FFS: Separate or joint TDM and FDM semi-static DU resource configurations

Agreement

Soft resource availability indications for frequency-domain resources are supported

- FFS enhancements to DCI Format 2_5

- FFS: Separate or joint TDM and FDM indications

Agreement

To facilitate simultaneous operations and interference management, dynamic indication for restriction/usage/availability of beams (in upstream and/or downstream directions) is supported

- FFS: Applicability to specific multiplexing cases or specific time-frequency resources

- FFS: Whether IAB-specific enhancements beyond the existing beam management framework are needed to the support the functionality

- FFS: Impact on the semi-static resource configurations (e.g., extending the H/S/NA resource attributes to the spatial domain)

- FFS: Whether panel-based granularity is additionally supported

Agreement

Adaptation of an IAB-node’s multiplexing operation is supported. The adaptation may be based on multiple factors, for example (not necessary to support all of the following):

- Resource type (D/U/F) at the IAB-DU and IAB-MT

- Specific sets of time/frequency resources

- Certain conditions being met (e.g. supported timing modes, power control enhancements (if supported), etc.)

FFS:  Mechanisms for informing/coordination the change in multiplexing operation(s) between child and parent nodes (including whether the adaptation is dynamic or semi-static)

FFS: Need for explicit linkage between indicated multiplexing operations and other features/enhancements – e.g. number of required guard symbols, supported timing modes, and power control enhancements (if supported)

Agreement

The following enhancements to support intra-band inter-carrier dual connectivity for both inter-donor and intra-donor scenarios are considered (in addition to reusing solutions for inter-band dual connectivity) to support simultaneous Tx and/or Rx at the child IAB-MT to/from both parent links:

- Extending the Rel-16 CA TDD conflict resolution framework for synchronous intra-band NR-DC operation

- Coordinating TDD configurations for the parent nodes (for both intra-donor and inter-donor operation) and coordinating H/S/NA configurations for the child node between donors (at least for inter-donor operation)

## RAN1 #104-e meeting

Agreement

Further study whether/how to manage resources in the spatial domain. Candidate solutions are:

- Dynamic signaling between parent and child nodes for using/restricting/sharing antenna panels/beams

- Beam management / multi-panel enhancements for simultaneous operations

- Extension of H/S/NA resource indication to the spatial domain

Other solutions are not precluded.

Agreement

Regardless of simultaneous operation, the same cell-specific/semi-static signals and channels of the IAB-DU considered as hard time/frequency resources in Rel-16 are also considered as hard time/frequency resources in Rel-17.

- FFS: IAB-MT behavior in case of conflicts between cell-specific signals/channels and other resource configurations of the IAB-MT (e.g., dedicated slot configurations)

Agreement

Support indication/reporting of information between an IAB node and its parent node to assist in the determination of the applicability of a given multiplexing capability in case of simultaneous operation. The following solutions are considered (other solutions not precluded):

- Temporal applicability of a given multiplexing capability

- Time/frequency resource restrictions (e.g. access vs. backhaul links, DL vs. UL resources)

- Indications of conditions/reporting information required to realize the given multiplexing capability, (e.g. timing mode, power control, guard symbols, etc.)

FFS: channels/signals used for indicating/reporting information

Agreement

Send LS response to RAN3 that both inter-donor multi-parent scenarios (Scenario 1 and Scenario 2) can be supported in Rel-17 with support for inter-donor resource coordination (e.g. DU H/S/NA and DL/UL resource configurations) in RAN3 specification.

- The reply LS to R1-210004 (RAN3) is endorsed in R1-2101880.

Agreement

The following are considered to support at least inter-band inter-carrier scenarios in Rel-17:

- Solutions to address resource coordination/scheduling collision issues between parent nodes including TDD configurations and resource type indications at least in case of intra-donor CU multi-parent scenarios

  - Consider Rel-16 CA framework as starting point

  - Solutions for scheduling collision between two parent DUs due to indication of the resource availability for soft symbol(s) to the IAB-DU(s) by DCI format 2_5

  - Solutions for scheduling collision between two parent DUs due to indication of the slot format by DCI format 2_0

  - FFS: Whether or not separate solutions are required for resource coordination in case of inter-donor CU multi-parent scenarios

- Per-backhaul link (e.g. per child IAB-MT link) resource configurations in addition to per-DU resource configurations

- FFS: Enhancements to indication of soft resource availability from child node to parent node(s)

- FFS: Additional restrictions on simultaneous operation and/or multiplexing

- FFS: Whether the above solutions are also applicable for intra-band inter-carrier scenarios and whether additional solutions are required (e.g. RAN2 and RAN4 work related to adding band configuration and RRM requirements for intra-band inter-carrier NR-DC or updating related UE/MT capabilities for NR-DC so that they are applicable for intra-band inter-carrier NR-DC)

Agreement

Further consider until RAN1#104bis-e whether to support the extension of the semi-static DU resource type indication to frequency-domain resources within a carrier (in addition to existing Rel-16 per-carrier granularity) for H/[S]/NA resource types, including the following aspects:

- Granularity for frequency domain resources within a carrier (starting point is a set of N RBs with FFS: value of N >=1)

- Relationship with Rel-16 DU resource type indications in case of coexistence between TDM and FDM operation, including time-granularity of switching between multiplexing options to ensure backwards compatibility with Rel-16 IAB nodes and avoid impact on access UEs and their RRC configurations at CU

- In case frequency-domain extension is supported for soft resources, enhancements for DCI format 2_5 to support dynamic indication of availability for soft frequency resources.

  - Alt. 1 Separate indication of time and frequency resources

    - FFS: different field, RNTI or different DCI

  - Alt. 2 Joint indication of time and frequency resources

    - FFS: backwards compatibility with Rel-16

- FFS: Extension of FDM across carriers

- FFS: Restrictions on band/minimum bandwidth for FDM operation (e.g. FR2 100MHz+ etc.)

## RAN1 #103-e meeting

Agreement

The Rel-16 IAB-DU resource types (Soft/Hard/NA) are the starting point for supporting resource multiplexing for simultaneous operation cases in Rel-17.

- FFS: Whether resource type definitions need to be extended to frequency domain resources

- FFS: Coexistence of simultaneous operation resources and TDM resources

- FFS: Whether new rules governing cell-specific/semi-static signals and channels at the IAB-DU and/or IAB-MT in case of simultaneous operation are necessary

Agreement

Further consider different applicability restrictions/conditions for simultaneous operation multiplexing cases:

- FFS: Whether a given case is only applicable for certain resource types or combinations: e.g. DL access, DL backhaul, UL access, UL backhaul

- FFS: Network (including parent node) awareness of a child IAB node’s ability to support simultaneous operation due to short-term and long-term factors including panel selection, interference, timing, transmit power, capability indication etc.

- FFS: Necessary differentiation for paired spectrum vs. unpaired spectrum

- FFS: Whether specific enhancements are defined for full-duplex cases vs. being left to implementation (as in Rel-16)

- Note: There should not be any impact on legacy UE behavior

Agreement

The Rel-16 explicit indication of soft resources by DCI Format 2_5 is supported for simultaneous operation cases in Rel-17.

FFS: Whether/how to extend DCI Format 2_5 to frequency domain resources and/or paired spectrum

FFS: Coexistence of simultaneous operation resources and TDM resources

Agreement

From a RAN1 perspective, at least intra-donor multi-parent operation is supported in Rel-17

FFS: Inter-donor operation pending additional input from RAN2/RAN3

Agreement

The explicit indication of soft resources by DCI Format 2_5 is supported for multi-parent scenarios in Rel-17.

- FFS: Whether additional enhancements over the Rel-16 solution are needed

Agreement

From a RAN1 perspective, resource multiplexing and coordination is supported for the following DC scenarios in Rel-17:

- Inter-carrier, inter-band

- Inter-carrier, intra-band is additionally supported at least for FR2

  - At least to the extent it reuses solutions for supporting Inter-carrier, inter-band

  - FFS: whether specific enhancements for inter-carrier, intra-band DC are introduced in Rel-17

## RAN1 #102-e meeting

Conclusion

At least the inter-carrier DC scenario can be considered in Rel-17. Further discussion in RAN3/RAN Plenary may be necessary for the intra-carrier DC scenario.

Agreement

Reuse by IAB-MT of existing Inter-frequency DC is considered as a starting point to support concurrent BH links to two parents.

- FFS: Reuse of multi-TRP transmission resource allocation features (if intra-freq DC scenario is supported for IAB)

- FFS: Additional specification effort to support IAB

For companies to further consider:

The following categories of enhancements have been proposed to support DC scenarios (not an exhaustive list):

- Inter-parent DU resource coordination mechanisms and signaling

- Resource allocation/scheduling conflict resolution rules at the parent or child node

- Per-link IAB-DU resource configurations at the parent node

Agreement

At least existing Rel-16 bands supporting IAB can be considered when evaluating the feasibility/impact of supporting different multiplexing cases.

Simultaneous operations

Agreement

The Rel-16 semi-static and dynamic resource allocation mechanisms are the starting point for supporting Rel-17 multiplexing cases.

- FFS: Applicability for different IAB-DU resource types

- FFS: Cell-specific/semi-static signals and channels at the IAB-DU and/or IAB-MT

Agreement

- Based on the WID, the following multiplexing cases are in scope for potential support in Rel-17:

  - Multiplexing Case A: Simultaneous MT-Tx/DU-Tx

  - Multiplexing Case B: Simultaneous MT-Rx/DU-Rx

  - Multiplexing Case C: Simultaneous MT-Rx/DU-Tx

  - Multiplexing Case D: Simultaneous MT-Tx/DU-Rx

- Further study for Case A and Case B at least the following scenarios:

  - Single or multi-panel IAB nodes operating in unpaired spectrum (FR1 and FR2 bands)

- Further study for Case C and Case D at least for the following scenarios:

  - Multi-panel IAB nodes operating in unpaired spectrum (FR1 and FR2 bands)

- FFS: Required level of specification impact to support the different cases. Any additional specification support in Rel-17 should be conditioned on feasibility from an interference and reliability perspective on a per-link and network basis

For companies to further consider:

Whether the following characteristics of the IAB node implementation will impact the operation of different resource multiplexing cases, including resource partitioning (i.e. identify whether there is a need for potential specification impact/enhancements compared to Rel-16 if the characteristic is or is not supported by an IAB node):

- Baseband (mis)timing alignment between IAB-MT and IAB-DU

- Separate vs. shared antenna panels/RF front-end for IAB-MT/IAB-DU

- Separate vs. shared baseband for IAB-MT/IAB-DU

- Transmitter/receiver implementation

- Self-interference cancellation

- Power control mechanisms

For companies to further consider:

Different resource partitioning scenarios for access and backhaul links, including their respective implication on interference, for different resource multiplexing cases. Examples include:

- Whether a given case is only applicable for certain resource types: e.g. DL only, UL only, DL + UL

- Whether a given case is only applicable for backhaul links or both access and backhaul links

- Note: This should have no impact on legacy UE behavior

Agreement

- Case 7 timing is supported in Rel-17 for IAB-nodes operating in multiplexing scenario Case 2 (simultaneous MT-Rx/DU-Rx)

- Case 6 timing is supported in Rel-17 for IAB-nodes operating in multiplexing scenario Case 1 (simultaneous MT-Tx/DU-Tx)

  - RAN1 should strive to minimize specification impact due to this feature

- FFS: Whether Case 7 timing is supported in Rel-17 for IAB-nodes operating in multiplexing scenario Case 4 (simultaneous MT-Tx/DU-Rx)