3GPP TSG RAN WG1 #108-e																		    R1-2201679

e-Meeting, February 21st – March 3rd, 2022

Source: 	Intel Corporation

Title:	Discussion on reply LS for separate BWP for RedCap UEs supporting SDT

Agenda item:	5

Document for:	Discussion and Decision

# Introduction

RAN1 received a reply LS from RAN2, which includes the following questions [1]:

ACTION: RAN2 respectfully asks RAN1 to take the above information into account for their specification work and to answer the following question: Question to RAN1: RAN2 would like RAN1 to confirm that the separate BWP in case of REDCAP may still be considered as the initial BWP and SDT resources can hence be configured on this BWP for REDCAP UEs.

In the contribution, we present our views on separate BWP for RedCap UEs supporting SDT.

# Discussion on response to RAN2 LS

At the RAN1#106-e meeting, the following agreement was made for separate BWP for RedCap UEs [2]. The separate BWP is mainly used for traffic offloading, especially during initial access procedure and to support the scenario where initial DL BWP for non-RedCap UEs is larger than maximum RedCap UE bandwidth.

Agreement For FR1, - For TDD, center frequencies are assumed to be the same for the initial DL (FFS: if it does not include CD-SSB and the entire CORESET#0) and UL BWPs used during random access for RedCap UEs. - FFS: For Option 1 and Option 2, whether the case that the center frequencies are different is also supported, and whether RedCap UE can expect CD-SSB and CORESET#0 in this case - For TDD, center frequencies are assumed to be the same for non-initial DL and UL BWPs with the same BWP id for a RedCap UE.

Following the similar design principle, separate BWP can be considered for RedCap UEs as the initial BWP in case of SDT operation. In this case, SDT resources can be configured on this BWP for RedCap UEs. Further, the agreements for the RedCap UEs can be applied, i.e., for TDD, center frequencies are assumed to be the same for the separate DL and UL BWPs used for RedCap UEs.

Proposal 1

- RAN1 to confirm that separate BWP can be considered for RedCap UEs as the initial BWP and SDT resources can be configured on this BWP for RedCap UEs.

- For TDD, center frequencies are assumed to be the same for the separate DL and UL BWPs used for RedCap UEs.

# Conclusions

In this contribution, we presented our views on remaining issue for physical layer aspects of small data transmission. Further, we summarize the proposals as follows:

Proposal 1

- RAN1 to confirm that separate BWP can be considered for RedCap UEs as the initial BWP and SDT resources can be configured on this BWP for RedCap UEs.

- For TDD, center frequencies are assumed to be the same for the separate DL and UL BWPs used for RedCap UEs.

# References

- R1-2200881, “Reply LS on the L1 aspects of small data transmission”, February 2022

- Chairman’s notes, RAN1 #10b e-Meeting, August 2021