3GPP TSG RAN WG1 #108-e                                                              	                R1-2201684

e-Meeting, February 21st – March 3rd, 2022

Agenda Item:	8.1.2.2

Source: 	Intel Corporation

Title:	Maintenance of multi-TRP inter-cell operation

Document for: 	Discussion/Decision

# Introduction

The feMIMO WID has been revised in the last RAN plenary to clarify the scope of this AI:

- Identify and specify QCL/TCI-related enhancements to enable inter-cell multi-TRP operations, assuming multi-DCI based multi-PDSCH reception based on Rel-15/16 TCI framework

# ssb-PositionsInBurst from non-serving cell

The following FL proposal was discussed in email discussions during RAN1#107-e:

Proposal 5-30 (version 2): In the set of symbols indicated to a UE by ssb -PositionsInBurst in SSB associated with the active additional PCI, - Option 1: The UE does not transmit any UL signal/channel. - Option 2: The UE can only transmit UL signal/channel associated with the serving cell PCI, and does not transmit UL signal/channel associated with the active additional PCI - Association of UL signal/channel with the serving cell PCI or the active additional PCI is derived based on PL-RS for the UL signal/channel - Further study the impact on The following Rel. 15/16 procedures are based on a selected option from Option 1 or 2 above: - Procedure 1: When SSB overlaps with UL channel/RS, UE does not transmit the UL channels/RS [38.213, Section 11.1]. - Procedure 2: UE does not expect the set of SSB symbols to indicated as uplink symbols either semi-statically or dynamically (by SFI) [38.213, Section 11.1 and Section 11.1.1]. - Procedure 3: SSB symbols are assumed to be invalid symbols in a nominal repetition for PUSCH repetition Type B [38.214, Section 6.1.2.1]. - Procedure 4: For determination of the NPUCCHRepeat slots in the case of PUCCH repetition, i.e., a slot is not counted toward the NPUCCHRepeat slots if the PUCCH resource in that slot overlaps with a SSB [38.213, Section 9.2.6]. - Other options are not precluded. - Down-select one Option in maintenance

In our interpretation option-1 extends current UE behavior of not transmitting UL channel/RS overlapping with SS/PBCH block indicated in ssb-PositionsInBurst (for the serving cell) to SS/PBCH blocks indicated in the union of ssb-PositionsInBurst for the serving cell and the configured ssb-PositionsInBurst associated with the active additional PCI. We think this defines clear UE behavior applicable for all relevant uplink procedures uniformly (without distinction between serving PCI or additional PCI) and quite easy for the NW and the UE to be in sync.

Option-2 defines different UE behaviors depending on whether the UL signal/channel is associated with the serving cell PCI or the additional PCI. Additionally, it is up to the NW to avoid overlap of SS/PBCH block and uplink transmission if they are not associated with the same PCI – this becomes problematic for scheduling certain procedures like PUSCH Type B repetitions. Therefore, we make the following proposal:

Proposal-1: Extend current UE behavior of not transmitting UL channel/RS overlapping with SS/PBCH block indicated in ssb-PositionsInBurst (for the serving cell) to SS/PBCH blocks indicated in the union of ssb-PositionsInBurst for the serving cell and the configured ssb-PositionsInBurst associated with the active additional PCI (option-1).

# Broadcast channels monitoring

We made the following agreement in RAN1#107-e

Agreement UE is not required to monitor a Type0/0A/1[/2] CSS in a CORESET when the active TCI state is associated with a PCI different from serving cell PCI.

Our understanding is that there is no serving cell change for inter-cell multi-TRP and therefore common configuration that includes paging etc. is associated with the serving cell. Consequently, a UE is not required to monitor a Type 2 CSS in a CORESET associated with a PCI different from the serving cell PCI

Proposal-2: Remove the brackets from “UE is not required to monitor a Type0/0A/1[/2] CSS in a CORESET when the active TCI state is associated with a PCI different from serving cell PCI”

# Conclusions

Proposal-1: Extend current UE behavior of not transmitting UL channel/RS overlapping with SS/PBCH block indicated in ssb-PositionsInBurst (for the serving cell) to SS/PBCH blocks indicated in the union of ssb-PositionsInBurst for the serving cell and the configured ssb-PositionsInBurst associated with the active additional PCI (option-1).

Proposal-2: Remove the brackets from “UE is not required to monitor a Type0/0A/1[/2] CSS in a CORESET when the active TCI state is associated with a PCI different from serving cell PCI”

# References

[1] RP-193133, New WID: Further enhancements on MIMO for NR, Samsung, RAN#86