3GPP TSG RAN WG1 #108-e                                                              	       R1-2201685

e-Meeting, February 21st – March 3rd, 2022

Agenda Item:	8.1.2.3

Source: 	Intel Corporation

Title:	Maintenance of multi-TRP beam management

Document for: 	Discussion/Decision

# Introduction

According to NR MIMO WID RP-193133, the following is the scope of work in this AI.

2. Enhancement on the support for multi-TRP deployment, targeting both FR1 and FR2: - Evaluate and, if needed, specify beam-management-related enhancements for simultaneous multi-TRP transmission with multi-panel reception

In this contribution we provide our views on the features that should be considered for beam-management-related enhancements for simultaneous multi-TRP transmission with multi-panel reception.

# Enhancements for beam measurement and reporting

## L1-SINR and interference measurement

RAN1#107-e FL proposal: Updated FL Proposal 1.2-2: Support L1-SINR for beam reporting option 2  IMR resource assumptions: explicit IMR configuration, including ZP or NZP IMR - Note: ZP or NZP IMR is configured per CMR for each of CMR sets

In order to obtain accurate beam-pairing information at the gNB, we think L1-SINR reporting should be an integral part of group-based beam reporting framework. Given that a UE identifies a beam-pair for reporting as part of group-based beam reporting framework, it can also calculate L1-SINR for such candidate beam-pairs. We are supportive of both the gNB configuring beam-pairs and also the UE being responsible for identifying potential beam pairs. Given that we are in the maintenance phase we are supportive of the FL proposal from RAN1#107-e.

Proposal-1: Support L1-SINR reporting as part of enhanced group-based beam reporting.

## UE reception during beam switching (resubmit)

UE panel architecture 1

Figure 1: UE with 2-TRP Rx panel combinations {0,1},{0,2},{0,3},{1,2},{1, 3},{2, 3}

Let us assume that each panel for the example UE above comprises of 2 Rx antennas and the UE is able to receive downlink transmissions from any 2 of the 4 Rx panels (4 Rx ports in base-band). Let us also assume that the UE reports beam pairs corresponding to panel-ids {0, 2} and {1, 3}. The gNB may consider a joint/time-synchronous beam switch (TRP-1 and TRP-2) leading to switching the UE panels from {0, 2} → {1, 3} or an independent/asynchronous beam switch (switching one beam at a time) leading to switching the UE panels from {0, 2} → {1, 2} → {1, 3}. In this situation, UE reception due to panel combination {1, 2}, which was not reported as a beam-pair) may lead to poor downlink reception quality during the transition.

UE panel architecture 2

Figure 2: UE with 2-TRP Rx panel combinations {0,2},{1,3} only

Let us assume that each panel for the example UE above comprises of 2 Rx antennas and the UE is able to receive downlink transmissions from only 2 combinations of the 4 Rx panels {0,2} and {1,3}. Let us also assume that the UE reports beam pairs corresponding to panel-ids {0, 2} and {1, 3}. The gNB may consider a joint/time-synchronous beam switch (TRP-1 and TRP-2) leading to switching the UE panels from {0, 2} → {1, 3} or an independent/time-asynchronous beam switch leading to switching the UE panels from {0, 2} → {1, 2} → {1, 3}. In this situation, UE reception due to panel combination {1, 2} during the transition (which was not reported) may not be possible due to UE architecture constraints (and may trigger BFR).

There could be different ways of resolving this, we propose a scheduling restriction:

Proposal-2: A UE is not expected to receive downlink signals/channels or transmit uplink signals/channels in a time overlapping manner with a beam pair combination that it has not reported within a certain interval of time.

# Beam failure recovery for multi-TRP

## Cell-specific and TRP specific BFR

Agreement - Support simultaneous configuration of cell-specific BFR and TRP-specific BFR in different CCs. - FFS: whether cell-specific and TRP-specific BFR can be configured in the same CC.

RAN1#107-e FL proposal: A UE can be configured with either “Rel-15/16 BFD” (i.e., 1 BFD-RS set per BWP) or “TRP-specific BFD” (i.e. 2 BFD-RS sets per BWP) on one BWP.

We do not see a use-case for the gNB to configure cell-specific (1 BFD-RS set) and TRP-specific BFR (2 BFD-RS sets) in the same BWP. Generally we think TRP specific BFR should be a standalone and complete procedure including RACH-based fall-back. Also it is not expected that gNB would not take into account certain TRPs (in a serving cell) while configuring TRP specific BFR. It would also be additional work to define interaction between both these procedures running in parallel.

Proposal-3: Cell-specific and TRP-specific BFR configuration in the same CC is not supported. A UE can be configured with either 1 BFD-RS set (Rel-15/16 BFD) or 2 BFD-RS sets (TRP-specific BFD)

## Resetting beam for recovery SDCI

Agreement For the case of all CORESETs with 1 activated TCI state per CORESET , after 28 symbols from receiving the BFR response, the QCL assumption of all CORESETs associated with CORESETPoolIndex k (k=0,1) is updated by the RS resource associated with the latest reported new candidate beam (if found) associated with the failed BFD -RS set k (k=0,1) in the MAC-CE for TRP -specific BFR - The above applies to Scell and SpCell - The above applies for the multi-DCI case

RAN1#107-e FL proposal: To associate CORESET(s) with failed BFD-RS set  For explicit BFD-RS configuration of S-DCI  Alt2: Support association configuration between CORESET and BFD-RS set

In a previous RAN1 meeting, association between BFD-RS set and CORESETPoolIndex is been established to enable recovery from a single TRP failure for mTRP BFR procedure.

The problem of resetting the beam for recovery also applies to single-DCI case. In this case, we propose to associate a CORESET and an explicitly configured BFD-RS set as proposed in the FL proposal in RAN1#107-e

Proposal-4: Support association between a CORESET and a BFD-RS set for single DCI operation to allow resetting beam for recovery.

## Beam/power update for PUCCH after gNB response

RAN1#107-e FL Proposal 2.6: Support beam/power update for PUCCH after receiving gNB response. - Introduce association between PUCCH and TRP, e.g. through BFD-RS set ID, CORESETPoolIndex, etc. Note: the term TRP is used only for the purposes of discussions

In RAN1#107-e it was discussed that an association between PUCCH and TRP is necessary to allow beam/power update to PUCCH for TRP-specific BFR. Similar functionality is already present in Rel-15. This makes sense for low-latency uplink control information carried over PUCCH.

Proposal-5: Support beam/power update for PUCCH after receiving gNB response by an association between PUCCH and TRP.

# Conclusions

Proposal-1: Support L1-SINR reporting as part of enhanced group-based beam reporting.

Proposal-2: A UE is not expected to receive downlink signals/channels or transmit uplink signals/channels in a time overlapping manner with a beam pair combination that it has not reported within a certain interval of time.

Proposal-3: Cell-specific and TRP-specific BFR configuration in the same CC is not supported. A UE can be configured with either 1 BFD-RS set (Rel-15/16 BFD) or 2 BFD-RS sets (TRP-specific BFD)

Proposal-4: Support association between a CORESET and a BFD-RS set for single DCI operation to allow resetting beam for recovery.

Proposal-5: Support beam/power update for PUCCH after receiving gNB response by an association between PUCCH and TRP.

# References

[1] RP-193133, New WID: Further enhancements on MIMO for NR, Samsung, RAN#86