3GPP TSG RAN WG1 Meeting #108-e	R1-2201689

e-Meeting, February 21 – March 03, 2022

Source:	Intel Corporation

Title:	Discussion on PDCCH monitoring enhancements for extending NR up to 71 GHz

Agenda item:	8.2.2

Document for:	Discussion/Decision

# Introduction

In this contribution, we provide our views on remaining open issues for PDCCH monitoring capability and search space set configuration for the PDCCH monitoring.

# Multi-slot PDCCH monitoring capability

## Single cell PDCCH monitoring capability

The multi-slot PDCCH monitoring is based on a fixed pattern of slot groups, where each group has X consecutive slots and the slot groups are consecutive and non-overlap. The group (1) SS and group (2) SS are configured in a slot group independently. Group (1) SS is only monitored in the Y consecutive slots in a slot group. The same position of the Y slots in a slot group maintains in all slot groups. The default value of Y is 1. Note that the slot/symbol position of a SS set with searchSpaceId = 0 is determined by the associated SS/PBCH block, while the other SS sets in group (2) can be anywhere in a slot group. The budget of maximum BD/CCE are shared by all configured PDCCHs in a slot group.

UE mandatorily supports X=4/8 for SCS 480/960kHz respectively. UE can optionally support X=4 for SCS 960kHz. One open issue is whether X=2 can be supported for SCS 480kHz. Since a slot group with a duration equals to half slot of SCS 120kHz is already supported for SCS 960kHz by allowing X=4, it is straightforward that same duration of slot group could also be supported for SCS 480kHz. The increase on UE complexity is marginal. Therefore, it is preferred to optionally support X=2 for SCS 480kHz. Correspondingly, a new combination (X, Y) = (2, 1) should be supported optionally. The BD/CCE budget for (2,1) can be half that of X=4 for SCS 480kHz.

Agreements from early RAN1 meetings: - Reporting the BD/CCE budget for X=4/8 slots (for 480/960 kHz resp.) is mandatory (if UE supports the corresponding SCS), and is optional for X=[2]/4 slots (for 480/960 kHz resp.) - Supported combinations of (X,Y) - A UE capable of multi-slot monitoring mandatorily supports - For SCS 480 kHz: (X,Y) = (4,1) - For SCS 960 kHz: (X,Y) = (8,1) - A UE capable of multi-slot monitoring optionally supports - For SCS 480 kHz: (X,Y) = (4,2) - For SCS 960 kHz: (X,Y) = (8,4), (4,2), (4,1) - Working assumption: BD/CCE budget for (4,2), (4,1) is half that of X=8

The following TP is proposed to capture additional X value 2 for SCS 480kHz.

------------------------------ TP#1: TS 38.213 ----------------------------------- *** Unchanged text is omitted *** ## 10.1 UE procedure for determining physical downlink control channel assignment *** Unchanged text is omitted *** Table 10.1-2B provides the maximum number of monitored PDCCH candidates, MPDCCHmax,Xs,Ys,μ, per slot group for combination (Xs,Ys) for a UE in a DL BWP with SCS configuration μ for operation with a single serving cell. Table 10.1-2B: Maximum number MPDCCHmax,(Xs,Ys),μ of monitored PDCCH candidates per slot group for combination Xs,Ys for a DL BWP with SCS configuration μ∈5, 6 for a single serving cell Maximum number MPDCCHmax,Xs,Ys,μ of monitored PDCCH candidates per combination Xs,Ys and per serving cell. μ; (2, 1); (4, 1); (4, 2); (8, 1); (8, 4). 5; 10; 20; 20; -; -. 6; -; 10; 10; 20; 20. *** Unchanged text is omitted *** Table 10.1-3B provides the maximum number of non-overlapped CCEs, CPDCCHmax,Xs,Ys,μ, for a DL BWP with SCS configuration μ that a UE is expected to monitor corresponding PDCCH candidates for combination Xs,Ys for operation with a single serving cell. Table 10.1-3B: Maximum number CPDCCHmax,(Xs,Ys),μ of non-overlapped CCEs in a slot group for combination Xs,Ys for a DL BWP with SCS configuration μ∈5, 6 for a single serving cell Maximum number of non-overlapped CCEs per combination Xs,Ys and per serving cell CPDCCHmax,(Xs,Ys),μ. μ; (2, 1); (4, 1); (4, 2); (8, 1); (8, 4). 5; 16; 32; 32; -; -. 6; -; 16; 16; 32; 32. *** Unchanged text is omitted ***

Proposal 1:

- X=2 can be optionally supported for SCS 480kHz, which corresponds to combination (X, Y) = (2, 1). The BD/CCE budget for (2,1) can be half that of X=4

- Agree on TP 1 to capture additional X value 2 for SCS 480kHz

Before discussing details of Group (2) SS handling, it is better to align the understanding of FG 3-1 especially ‘any of’ in the following bullet.

- For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, the monitoring occasion can be any OFDM symbol(s) of a slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, or 2 CSS configurations within a single span of three consecutive OFDM symbols within a slot

There are two interpretations.

- Interpretation #1: It is limited to single span in a slot for each Group (2) SS set, however the different Group (2) SS sets can be configured in different spans in the slot.

- Interpretation #2: It is limited to single span in a slot considering all configured Group (2) SS sets in the slot.

In our view, Interpretation #2 is the correct understanding. Interpretation #1 will not bring any real complexity reduction at UE side compared to Interpretation #2. With Interpretation #1, though a single Group (2) SS set is limited to single span per slot, there can still be multiple spans in a slot due to multiple configured Group (2) SS sets. Consequently, UE must prepare for the worst case, i.e., to decode multiple spans per slot for Group (2) SS sets.

Proposal 2:

- Clarify that Interpretation #2 is the right understanding for FG 3-1

  - Interpretation #2: It is limited to single span in a slot considering all configured Group (2) SS sets in the slot.

It was agreed in RAN1#107b-e that UE can support monitoring Group (2) SSs according to FG 3-1 within each of the Xs slots of a slot-group for Group (2) SS. However, it is still open whether to introduce certain limitation on Group (2) SS. The limitation should be clarified from two aspects. Firstly, whether to limit the number of slots/spans on which the SS set(s) can be simultaneously configured in a slot group for a UE. Secondly, whether to limit the number of slots/spans on which the SS set(s) can be simultaneously monitored in a slot group by a UE. Note: two or more spans may be configured in overlap/adjacent OFDM symbols, which effectively forms a single span.

Agreements from early RAN1 meetings: - A UE capable of multi-slot monitoring mandatorily supports the following PDCCH monitoring within Y slots - For Y>1: FG3-1 (monitoring Group (1) SSs in the first 3 OFDM symbols of each of the Y slots) - For 960 kHz SCS For Y=1: FG3-5b with set1 = (7, 3) - [FL Note: The first number is the minimum gap in symbols between the start of two spans, the second number is the span duration in symbols (cf. TS 38.822)] - For 480 kHz SCS For Y=1: FG3-5b with set2 = (4, 3) and (7, 3) with a modification with maximum two monitoring spans in a slot - [FL Note: The first number is the minimum gap in symbols between the start of two spans, the second number is the span duration in symbols (cf. TS 38.822)] - The following supersedes FG3-5b and FG3-1 definition: - Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per slot group of X slots per scheduled CC for FDD - Processing one unicast DCI scheduling DL and 2 unicast DCI scheduling UL per slot group of X slots per scheduled CC for TDD Agreement Clarify earlier agreement as follows: - A UE capable of multi-slot monitoring mandatorily supports monitoring Group (2) SSs according to FG 3-1 within each of the Xs slots of a slot-group, such that: - For type 1 CSS without dedicated RRC configuration and for type 0, 0A, and 2 CSS, the monitoring occasion can be any OFDM symbol(s) of each slot, with the monitoring occasions for any of Type 1- CSS without dedicated RRC configuration, or Types 0, 0A, or 2 CSS configurations within a single span of three consecutive OFDM symbols within each slot of the slot group. - Continue discussion on whether or not introducing other limitation for Group (2) SSs in RAN1#108-e.

The UE can skip monitoring a configured MO of group (2) SS in a slot group. For example, the MOs of Type0A/2 CSS sets with searchSpaceID non-zero are sequentially numbered and mapped to the actual transmitted SSBs according to ssb-PositionsInBurst in SIB1, where the actual transmitted SSBs are sequentially numbered in ascending order of their SSB indexes. That is, the [s×N+K]th PDCCH MO(s) of Type0A/2 CSS sets corresponds to the Kth transmitted SSB, where s = 0, 1, ...S-1, K = 1, 2, …N. Therefore, even gNB configures M>1 MOs for Type0A/2 CSS sets with searchSpaceID non-zero in a slot group, the UE may decide to monitor only one MO or even not monitor any MO at all in the slot group. Note: it is possible to configure Type0A/2 CSS sets with searchSpaceID non-zero in each of consecutive slots in existing NR.

In general, it is not preferred to limit the maximum number of slots that can be simultaneously configured for PDCCH monitoring in a slot group for Group (2) SS configuration flexibility. That is, the Group (2) SS can be simultaneously configured in all slots in a slot group. On the other hand, the maximum number of configured spans in a slot may remain to be 2 to align with Group (1) SS for the Y=1 slot.

Since UE mandatorily supports Y=1 for the multi-slot PDCCH monitoring capability, the design on handling group (2) SS can take existing agreement for Y=1 as baseline, i.e., FG3-5b extension. That is, UE must be capable to monitor up to two spans of Group (1) SS in a slot in a slot group. To avoid excessive increase of UE complexity, it is preferred that the maximum number of monitored spans per slot in a slot group for the two groups of SS sets remains to be 2. Further, the total number of monitored spans of SS sets in a slot group should be carefully designed. A larger number of spans provide a better flexibility for PDCCH transmission. However, it also increases UE complexity and power consumption. To balance various factors, it is preferred that UE should be capable to monitor up to 3 or 4 spans of all SS sets in a slot group which includes spans for all Group (1) and Group (2) SS sets.

Proposal 3:

- The limitation on Group (2) SS sets should be clarified from two aspects

  - Whether to limit the number of slots/spans on which the SS set(s) can be simultaneously configured in a slot group for a UE.

  - Whether to limit the number of slots/spans on which the SS set(s) can be simultaneously monitored in a slot group by a UE.

Proposal 4:

- For SS set configuration

  - The Group (2) SS sets can be simultaneously configured in all slots in a slot group.

  - The maximum number of configured spans in a slot remains to be 2 considering all SS sets.

- For PDCCH monitoring at UE

  - The maximum number of monitored spans per slot in a slot group remains to be 2 considering all SS sets

  - UE is capable to monitor up to 3 or 4 spans of SS sets in a slot group considering all SS sets.

- Discuss the exact number of limited configured/monitored spans in a slot or in a slot group in UE feature for WI NR_ext_to_71GHz

It is desirable if gNB can provide proper SS set configuration so that the total number of spans for all SS sets in any slot group does not exceed the defined maximum number of spans per slot group. However, there is also the case that the maximum number can be exceeded in a slot group. Further, if the SS/PBCH block for the UE is changed, the slot/symbol position of the SS set with searchSpaceId = 0 moves accordingly, which may impact the total number of spans in a slot group. A span of SS sets configured in a slot group, if it is not monitored by the UE based on a semi-static rule, may not be counted in the number of spans in the slot group. If the maximum number of spans in a slot group is exceeded, UE can drop one or more spans of at least group (1) SS sets so that the maximum number is not exceeded in the slot group.

Proposal 5:

- A span of SS sets configured in a slot group, if it is not monitored by the UE based on a semi-static rule, may not be counted in the number of monitored spans in the slot group.

- If the maximum number of spans in a slot group is exceeded, UE can drop one or more spans of at least the group (1) SS sets

The number of DL or UL unicast DCIs per slot group that can be processed by UE was also agreed in last meeting. Additionally, UE should be able to process one broadcast DCI for SI/RACH/paging in the slot group of X slots. If cross-carrier scheduling is configured, for example, the scheduling cell has SCS 15kHz, it needs clarification whether the agreement implies that UE would be able to detect up to 8 unicast DCIs in a slot on the scheduling cell that schedule a unicast transmissions on the scheduled cell with SCS 960kHz.

Agreements from early RAN1 meetings: - The following supersedes FG3-5b and FG3-1 definition: - Processing one unicast DCI scheduling DL and one unicast DCI scheduling UL per slot group of X slots per scheduled CC for FDD - Processing one unicast DCI scheduling DL and 2 unicast DCI scheduling UL per slot group of X slots per scheduled CC for TDD

Proposal 6:

- UE should be able to process one broadcast DCI for SI/RACH/paging in addition to the agreed number of processed unicast DCI in a slot group of X slots.

- To clarify whether a UE would be able to detect up to 8 unicast DCIs in a slot on the scheduling cell with SCS 15kHz

- The limitation on number of detected DCIs in a slot group should be discussed in UE feature for WI NR_ext_to_71GHz

Based on the agreed framework, UE may report the capability to support multiple combinations (X, Y) for SCS 480/960kHz. The configured search space sets for the UE must satisfy at least one supported combination (X, Y).

- For SCS 480 kHz: (X,Y) = (4,1), (4,2), (2,1)?

- For SCS 960 kHz: (X,Y) = (8,1), (8,4), (4,2), (4,1)

Further, if the configured search space sets of the UE satisfy two or more supported combinations (X, Y), the UE needs to identify the active combination (X, Y). Since the supported maximum numbers of BD/CCE is only determined by value X, UE can determine a combination (X, Y) with larger X which enables larger maximum numbers of BD/CCE for PDCCH monitoring for better flexibility. If multiple potential combinations (X, Y) have same value X, the active combination (X, Y) could be determined as the combination (X, Y) with smallest value Y.

The following TP is proposed to determine the active combination (X, Y).

------------------------------ TP#2: TS 38.213 ----------------------------------- *** Unchanged text is omitted *** # 10 UE procedure for receiving control information *** Unchanged text is omitted *** For SCS configuration μ=5 or μ=6, a UE can indicate a capability to monitor PDCCH according to one or more combinations Xs,Ys, where Xs and Ys are numbers of consecutive slots, groups of Xs slots are consecutive and non-overlapping, and the Ys slots are within the Xs slots. The first group of Xs slots starts from the beginning of a subframe. The start of two consecutive groups of Ys slots is separated by Xs slots. If a UE indicates a capability to monitor PDCCH according to multiple Xs,Ys combinations and a configuration of search space sets to the UE for PDCCH monitoring on a cell is allowed by one or more of the multiple combinations Xs,Ys, the UE monitors PDCCH on the cell according to the combination Xs,Ys with largest Xs and smallest Ys. *** Unchanged text is omitted ***

Proposal 7:

- For a UE capable of multiple combinations (X, Y), if the configured SS sets are aligned with more than one combination (X, Y), the active combination (X, Y) is determined that is associated with the largest X and smallest Y.

- Agree on TP 2 to determine the active combination (X, Y).

## Maximum number of BD/CCEs in CA

In this section, we discuss the issue with the maximum number of PDCCH candidates. The same handling can apply to the maximum number of non-overlapped CCEs. In the per slot PDCCH monitoring capability in Rel-15/16, the UE is capable to decode MPDCCHmax,slot,μ PDCCH candidates in a slot for a scheduled cell on the scheduling cell with SCS configuration μ. Further, another limit MPDCCHtotal,slot,μ in a slot applies to the scheduled cells that are configured with scheduling cells having same SCS configuration μ.

In details, assuming Ncells,0DL,μ cells are configured with scheduling cells having SCS configuration μ and one CORESET pool, and Ncells,1DL,μ cells are configured with scheduling cells having SCS configuration μ and two CORESET pools, assuming a UE reports BD/CCE capability of Ncellscap cells, the maximum BD/CCE for the serving cells can be determined in NR Rel-16 as below.

- If μ=03Ncells,0DL,μ+γ∙Ncells,1DL,μ≤Ncellscap, the UE is not required to monitor per slot for each scheduled cell,

-	MPDCCHtotal,slot,μ=MPDCCHmax,slot,μ PDCCH candidates if the scheduling cell is from the Ncells,0DL,μ cells, or

-	MPDCCHtotal,slot,μ=γ∙MPDCCHmax,slot,μ PDCCH candidates if the scheduling cell is from the Ncells,1DL,μ cells

-	MPDCCHmax,slot,μ PDCCH candidates for each CORESET pool when the scheduling cell is from the Ncells,1DL,μ downlink cells

- If μ=03Ncells,0DL,μ+γ∙Ncells,1DL,μ>Ncellscap, the UE is not required to monitor per slot

- MPDCCHtotal,slot,μ=Ncellscap⋅MPDCCHmax,slot,μ⋅Ncells,0DL,μ+γ∙Ncells,1DL,μj=03Ncells,0DL,j+γ∙Ncells,1DL,j PDCCH candidates on the scheduling cell(s) from the Ncells,0DL,μ+Ncells,1DL,μ downlink cells.

- For each scheduled cell from the Ncells,0DL,μ downlink cells, minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ PDCCH candidates

- For each scheduled cell from the Ncells,1DL,μ downlink cells,

  - minγ∙MPDCCHmax,slot,μ,MPDCCHtotal,slot,μ PDCCH candidates

  - minMPDCCHmax,slot,μ,MPDCCHtotal,slot,μ PDCCH candidates for each CORESET pool

In multi-slot PDCCH monitoring capability, the maximum numbers of BD/CCEs in a slot group with X=4/8 slots for SCS 480/960kHz are same as corresponding maximum numbers of SCS 120kHz. The duration of the slot group with X=4/8 slots for SCS 480/960kHz equals to the slot length of SCS 120kHz. For X=4 for SCS 960kHz, it is further agreed that the maximum numbers are half of that for X=8. These values are the limitation for single cell/TRP and are denoted as MPDCCHmax,(X,Y),μ, CPDCCHmax,(X,Y),μ.

Agreements from early RAN1 meetings: - The following limits apply for multi-slot monitoring within the group of X slots - The maximum number of monitored PDCCH candidates per X=4 slots for a DL BWP with 480 kHz SCS configuration for a single serving cell is 20. - The maximum number of monitored PDCCH candidates per X=8 slots for a DL BWP with 960 kHz SCS configuration for a single serving cell is 20. - The maximum number of non-overlapped CCEs per X=4 slots for a DL BWP with 480 kHz SCS configuration for a single serving cell is 32. - The maximum number of non-overlapped CCEs per X=8 slots for a DL BWP with 960 kHz SCS configuration for a single serving cell is 32. - Note: Search spaces monitoring within the group of X slots will be additionally defined - Supported combinations of (X,Y) - A UE capable of multi-slot monitoring mandatorily supports - For SCS 480 kHz: (X,Y) = (4,1) - For SCS 960 kHz: (X,Y) = (8,1) - A UE capable of multi-slot monitoring optionally supports - For SCS 480 kHz: (X,Y) = (4,2) - For SCS 960 kHz: (X,Y) = (8,4), (4,2), (4,1) - Working assumption: BD/CCE budget for (4,2), (4,1) is half that of X=8

When CA is considered, the total number of BD/CCEs that are shared by multiple cells/TRPs. MPDCCHtotal,(X,Y),μ, CPDCCHtotal,(X,Y),μ need to be determined considering both SCS configuration μ and the combination (X, Y) of the scheduling cells. Three options are discussed as below.

- Option 1: MPDCCHtotal,(X,Y),μ is determined and shared by all cells that are configured with scheduling cells having same SCS configuration μ and same combination (X, Y).

Based on the agreed framework, the slot group always starts from the subframe boundary. Consequently, the slot groups of difference cells must be aligned if same SCS configuration μ are configured. On the other hand, though the group (1) SS is limited to the Y slots, the group (2) SS can be configured in any slot in a slot group. Since the PDCCH monitoring for the two scheduled cells may not be fully aligned due to the arbitrary MOs for group (2) SS, it is not necessary to align the Y slots in the slot group for multiple cells for BD/CCE sharing. Based on the above discussions, option 1 can be considered without alignment of the Y slots.

- Option 2: MPDCCHtotal,(X,Y),μ is determined and shared by all cells that are configured with scheduling cells having SCS configuration μ and same value X in combinations (X, Y).

In the option 1, the BD/CCE for the scheduling cells with combination (X, Y) of same value X and different value Y are separately handled. In fact, since the maximum numbers of BD/CCE is only determined by SCS configuration μ and value X, but not the value Y, it is possible to share the BD/CCE for the scheduling cells with same value X of the combinations (X, Y). This option can provide gNB more freedom to share the PDCCH scheduling capability for more cells.

- Option 3: MPDCCHtotal,(X,Y),μ is determined and shared by all cells that are configured with scheduling cells having same duration of slot or slot group.

A slot of SCS 120kHz and a slot group of X=4/8 slots for SCS 480/960kHz are fully overlapped in time with same the maximum numbers of BD/CCE. It is then expected the UE implementation on the PDCCH detection could be common in some extent. Consequently, it can be considered to share the BD/CCE for the scheduling cells with SCS 120kHz and for SCS 480/960kHz if X=4/8 slots apply. Option 3 can provide even more flexibility on PDCCH transmission than Option 2. However, Option 3 also requires UE to jointly share maximum BD/CCE budget across cells with different numerology, which results in additional complexity.

Based on the above analysis, we prefer Option 2 since it provides a good balance between flexibility and complexity. The following TP is proposed to determine MPDCCHtotal,(X,Y),μ, CPDCCHtotal,(X,Y),μ.

------------------------------ TP#3: TS 38.213 ----------------------------------- *** Unchanged text is omitted *** ## 10.1 UE procedure for determining physical downlink control channel assignment *** Unchanged text is omitted *** If a UE - does not report pdcch-BlindDetectionCA or is not provided BDFactorR, γ=R - reports pdcch-BlindDetectionCA, the UE can be indicated by BDFactorR either γ=1 or γ=R If a UE is configured with Ncells,0DL,μ+Ncells,1DL,μ downlink cells for which the UE is provided monitoringCapabilityConfig = r17monitoringcapability and with associated PDCCH candidates monitored in the active DL BWPs of the scheduling cells using SCS configuration μ, and with Ncells,0DL,(Xs,Ys),μ+Ncells,1DL,(Xs,Ys),μ of the Ncells,0DL,μ+Ncells,1DL,μ downlink cells using combination Xs,Ys with same Xs for PDCCH monitoring, where μ=06Ncells,0DL,μ+γ∙Ncells,1DL,μ≤Ncellscap, the UE is not required to monitor, per a group of Xs slots on the active DL BWPs of the scheduling cells, - more than MPDCCHtotal,(Xs,Ys),μ=MPDCCHmax,(Xs,Ys),μ PDCCH candidates or more than CPDCCHtotal,(Xs,Ys),μ=CPDCCHmax,(Xs,Ys),μ non-overlapped CCEs for each scheduled cell when the scheduling cell is from the Ncells,0DL,(Xs,Ys),μ downlink cells, or - more than MPDCCHtotal,(Xs,Ys),μ=γ∙MPDCCHmax,(Xs,Ys),μ PDCCH candidates or more than CPDCCHtotal,(Xs,Ys),μ=γ∙CPDCCHmax,(Xs,Ys),μ non-overlapped CCEs for each scheduled cell when the scheduling cell is from the Ncells,1DL,(Xs,Ys),μ downlink cells - more than MPDCCHmax,(Xs,Ys),μ PDCCH candidates or more than CPDCCHmax,(Xs,Ys),μ non-overlapped CCEs for CORESETs with same coresetPoolIndex value for each scheduled cell when the scheduling cell is from the Ncells,1DL,(Xs,Ys),μ downlink cells If a UE - is configured with Ncells,0DL,μ+Ncells,1DL,μ downlink cells for which the UE is provided monitoringCapabilityConfig = r17monitoringcapability and with associated PDCCH candidates monitored in the active DL BWPs of the scheduling cells using SCS configuration μ, and - with Ncells,0DL,(Xs,Ys),μ+Ncells,1DL,(Xs,Ys),μ of the Ncells,0DL,μ+Ncells,1DL,μ downlink cells using combination Xs,Ys with same Xs for PDCCH monitoring, where μ=06Ncells,0DL,μ+γ∙Ncells,1DL,μ>Ncellscap, and - a DL BWP of an activated cell is the active DL BWP of the activated cell, and a DL BWP of a deactivated cell is the DL BWP with index provided by firstActiveDownlinkBWP-Id for the deactivated cell, the UE is not required to monitor, per a group of Xs slots on the active DL BWP with SCS configuration μ of the scheduling cells - for the Ncells,0DL,(Xs,Ys),μ+Ncells,1DL,(Xs,Ys),μ, downlink cells more than MPDCCHtotal,(Xs,Ys),μ=Ncellscap⋅MPDCCHmax,(Xs,Ys),μ⋅Ncells,0DL,(Xs,Ys),μ+γ∙Ncells,1DL,(Xs,Ys),μj=06Ncells,0DL,(Xs,Ys),j+γ∙Ncells,1DL,(Xs,Ys),j PDCCH candidates or more than CPDCCHtotal,(Xs,Ys),μ=Ncellscap⋅CPDCCHmax,(Xs,Ys),μ⋅Ncells,0DL,(Xs,Ys),μ+γ∙Ncells,1DL,(Xs,Ys),μj=06Ncells,0DL,(Xs,Ys),j+γ∙Ncells,1DL,(Xs,Ys),j non-overlapped CCEs. - for each scheduled cell from the Ncells,0DL,(Xs,Ys),μ downlink cells, more than minMPDCCHmax,(Xs,Ys),μ,MPDCCHtotal,(Xs,Ys),μ PDCCH candidates or more than minCPDCCHmax,(Xs,Ys),μ,CPDCCHtotal,(Xs,Ys),μ non-overlapped CCEs. - for each scheduled cell from the Ncells,1DL,(Xs,Ys),μ downlink cells, - more than minγ∙MPDCCHmax,(Xs,Ys),μ,MPDCCHtotal,(Xs,Ys),μ PDCCH candidates or more than minγ∙CPDCCHmax,(Xs,Ys),μ,CPDCCHtotal,(Xs,Ys),μ non-overlapped CCEs - more than minMPDCCHmax,(Xs,Ys),μ,MPDCCHtotal,(Xs,Ys),μ PDCCH candidates or more than minCPDCCHmax,(Xs,Ys),μ,CPDCCHtotal,(Xs,Ys),μ non-overlapped CCEs for CORESETs with same coresetPoolIndex value *** Unchanged text is omitted ***

Proposal 8:

- MPDCCHtotal,(X,Y),μ, CPDCCHtotal,(X,Y),μ needs to be determined and shared by all cells that are configured with scheduling cells having same SCS configuration μ and same value X in combinations (X, Y).

- Agree on TP 3 to determine MPDCCHtotal,(X,Y),μ, CPDCCHtotal,(X,Y),μ

## SSSG switching

The search space set group (SSSG) switching was firstly introduced in NR-U in Rel-16, which provides a means to balance the fast channel access right after LBT is successful and effective scheduling and power saving in the COT. Further, SSSG switching is extended in power saving WI in Rel-17. The feature is useful for FR2-2 too.

Working assumption The following values are adopted as minimum value of Pswitch for 120/480/960 kHz - Support only search space set group switching processing capability 1 with the following values μ; Minimum Pswitch value for UE processing capability 1 [symbols]. 3; 40. 5; 160. 6; 320. Conclusion The SSSG switching timer is in units of slots.

The time of SSSG switching after applying Pswitch may not be the boundary of a slot group. Since the second SSSG can provide better scheduling flexibility and the agreed value Pswitch are rather large, it would be helpful if the PDCCH monitoring following the second SSSG can start as soon as possible. Therefore, it is preferred that PDCCH monitoring following the second SSSG can start from the time of SSSG switching. It is harmful to wait until the boundary of a slot group for the SSSG switching.

The configured search space configuration in the two SSSGs may have different requirements on the PDCCH monitoring capability. Figure 1 provides two examples for the SSSG switching with corresponding switching between combination (4, 1) and (4, 2). In Figure 1A, at the time for SSSG switching, UE needs to decode more PDCCHs if no additional restriction is introduced. Since the agreed Pswitch values are much larger than the slot group size X=4/8, UE can know in a quite early time that there happens burst PDCCH detections at the time of SSSG switching. As a result, a simple solution is that UE can cancel the PDCCH detection in X slots before the time of SSSG switching. On the other hand, Figure 1B shows another example that there exists a distance between the slots for PDCCH monitoring before or after SSSG switching. There is practically no problem to monitor all PDCCHs around the time of SSSG switching. In this case, it is desired if PDCCH in the first SSSG in the X slots before the time of SSSG switching can still be detected. Based on Figure 1A/1B, a unified solution could be that UE can monitor the PDCCHs in the X slots before the time of SSSG switching in the slots that are the intersection of the slot patterns of combinations (X, Y) used before and after SSSG switching.

Figure 1: SSSG switching resulting in burst PDCCH detections

The following TP is proposed to do SSSG switching with different multi-slot PDCCH monitoring capability combinations (X, Y).

------------------------------ TP#4c: TS 38.213 ----------------------------------- *** Unchanged text is omitted *** ## 10.4 Search space set group switching *** Unchanged text is omitted *** If a UE is provided by SearchSpaceSwitchTrigger a location of a search space set group switching flag field for a serving cell in a DCI format 2_0, as described in clause 11.1.1; - if the UE detects a DCI format 2_0 and a value of the search space set group switching flag field in the DCI format 2_0 is 0, the UE starts monitoring PDCCH according to search space sets with group index 0, and stops monitoring PDCCH according to search space sets with group index 1, for the serving cell at a first slot that is at least Pswitch symbols after the last symbol of the PDCCH with the DCI format 2_0 - if the UE detects a DCI format 2_0 and a value of the search space set group switching flag field in the DCI format 2_0 is 1, the UE starts monitoring PDCCH according to search space sets with group index 1, and stops monitoring PDCCH according to search space sets with group index 0, for the serving cell at a first slot that is at least Pswitch symbols after the last symbol of the PDCCH with the DCI format 2_0, and the UE sets the timer value to the value provided by searchSpaceSwitchTimer - if the UE monitors PDCCH for a serving cell according to search space sets with group index 1, the UE starts monitoring PDCCH for the serving cell according to search space sets with group index 0, and stops monitoring PDCCH according to search space sets with group index 1, for the serving cell at the beginning of the first slot that is at least Pswitch symbols after a slot where the timer expires or after a last symbol of a remaining channel occupancy duration for the serving cell if indicated by DCI format 2_0 If a UE indicates a capability to monitor PDCCHs according to multiple combinations Xs,Ys, the search space sets with group index 0 and 1 are respectively associated with different combinations Xs0,Ys0 and Xs1,Ys1, the UE monitors PDCCHs of search space sets with group index g in the slot(s) that belongs to both the Ys0 slots of combinations Xs0,Ys0 and the Ys1 slots of combinations Xs1,Ys1 within the maxXs0,Xs1 slots prior to PDCCH monitoring according to search space sets with group index 1-g. *** Unchanged text is omitted ***

Proposal 9:

- Dynamic SSSG switching is supported for all SCSs 120, 480 and 960kHz.

- PDCCH monitoring following the second SSSG can start right after the time of SSSG switching

- The search space set configurations of the two SSSG can correspond to two different PDCCH monitoring capabilities combinations (X, Y)

- UE can monitor the PDCCHs in the X slots before the time of SSSG switching in the slots that are the intersection of the slot patterns of the two combinations (X, Y) before and after SSSG switching

- Agree on TP 4 to do SSSG switching with different multi-slot PDCCH monitoring capability combinations (X, Y).

# Search space set configuration

In RAN1#107b-e meeting, a good progress on SS set configuration was achieved.

Agreement For search space set configuration of multi-slot PDCCH monitoring: - monitoringSlotPeriodicityAndOffset and duration are appended with "-r17", and - For monitoringPeriodicityAndOffset-r17 - The values represent slots - Add periodicity values {32,64,128,5120,10240,20480} to the existing values in monitoringSlotPeriodicityAndOffset - Note: Total list of supported periodicity values: {1,2,4,5,8,10,16,20,32,40,64,80,128,160,320,640,1280,2560,5120,10240,20480} - For each periodicity value Xp - The value range for the offset O is {0 .. Xp-1} slots - Note: There may be no need to introduce the term "Xp" in the specifications - The configured periodicity at least for Group (1) SSs is restricted to be an integer multiple of Xs slots - FFS: details of offset - For duration-r17 - The values represent slots - The value range is { 8, 12, …, 20476} - The configured duration is restricted to be an integer multiple of Xs slots at least for Group (1) SSs - FFS: need to revise the definition of duration - monitoringSymbolsWithinSlot applies to each slot in a slot group configured for multi-slot PDCCH monitoring - Note: This parameter can be directly re-used from earlier releases. - Introduce new parameter monitoringSlotsWithinSlotGroup-r17 - Working assumption: - The size is 8 bits - Each bit in monitoringSlotsWithinSlotGroup-r17 represents a slot in a slot group - A slot in the slot group is configured for multi-slot PDCCH monitoring if the corresponding bit in the slot group is set to '1' - Note: Further configuration of the monitoring symbols in such a slot is done by monitoringSymbolsWithinSlot - The slots indicated in the bitmap should be consecutive at least for Group (1) SSs

According to the above WA, monitoringSlotsWithinSlotGroup-r17 has a size of 8 bits, which may exceed value X. for example, it is X=4 for SCS 480kHz, the monitoringSlotsWithinSlotGroup-r17 can indicate the slots with configured MOs in two consecutive slot groups of X=4. UE can expect that the configured MOs are in same position in both slot groups. The 8-bit bitmap allows full flexibility for SS set configuration, however, it is unnecessary for a typical SS set configuration if Y=1. Since the agreed value range for the offset O is {0 .. Xp-1} slots, the offset O can indicate a right slot index with configured MOs in a slot group, i.e., mod(offset, X). In this case, monitoringSlotsWithinSlotGroup-r17 can be omitted.

One more remaining issue is on the definition of duration. duration indicates the number of consecutive slots for PDCCH monitoring according to a combination (Xs, Ys) upon every period as given in the monitoringSlotPeriodicityAndOffset.

The following TP is proposed for SS set configuration.

------------------------------ TP#5: TS 38.213 ----------------------------------- *** Unchanged text is omitted *** ## 10.1 UE procedure for determining physical downlink control channel assignment *** Unchanged text is omitted *** For each DL BWP configured to a UE in a serving cell, the UE is provided by higher layers with S≤10 search space sets where, for each search space set from the S search space sets, the UE is provided the following by SearchSpace: - a search space set index s, 0<s<40 , by searchSpaceId - an association between the search space set s and a CORESET p by controlResourceSetId or by controlResourceSetId-v1610 - a PDCCH monitoring periodicity of ks slots and a PDCCH monitoring offset of os slots, by monitoringSlotPeriodicityAndOffset or by monitoringSlotPeriodicityAndOffset-r17 - a PDCCH monitoring pattern within a slot, indicating first symbol(s) of the CORESET within a slot for PDCCH monitoring, by monitoringSymbolsWithinSlot - a PDCCH monitoring pattern within a group of slots, indicating consecutive slots in the group of slots for PDCCH monitoring, by monitoringSlotsWithinSlotGroup-r17. If monitoringSlotsWithinSlotGroup-r17 is not configured, the slot for PDCCH monitoring is indicated by PDCCH monitoring offset os - a duration of Ts<ks slots indicating a number of slots that the search space set s exists by duration or by duration-r17 *** Unchanged text is omitted ***

Proposal 10:

- In search space set configuration,

  - duration indicates the number of consecutive slots for PDCCH monitoring according to a combination (Xs, Ys) upon every period as given in the periodicityAndOffset

  - Revise the WA:

    - monitoringSlotsWithinSlotGroup-r17 can indicate the slots with configured MOs in two consecutive slot groups of X=4. UE can expect that the configured MOs are in same position in both slot groups.

    - if monitoringSlotsWithinSlotGroup-r17 is not configured, the SS set is only configured in slot modoffset,X in a slot group

- Agree on TP 5 for SS set configuration

# Conclusions

In this contribution, we discussed the potential enhancements on PDCCH monitoring in 52.6-71GHz frequency. The following is a summary of proposals:

Proposal 1:

- X=2 can be optionally supported for SCS 480kHz, which corresponds to combination (X, Y) = (2, 1). The BD/CCE budget for (2,1) can be half that of X=4

- Agree on TP 1 to capture additional X value 2 for SCS 480kHz

Proposal 2:

- Clarify that Interpretation #2 is the right understanding for FG 3-1

  - Interpretation #2: It is limited to single span in a slot considering all configured Group (2) SS sets in the slot.

Proposal 3:

- The limitation on Group (2) SS sets should be clarified from two aspects

  - Whether to limit the number of slots/spans on which the SS set(s) can be simultaneously configured in a slot group for a UE.

  - Whether to limit the number of slots/spans on which the SS set(s) can be simultaneously monitored in a slot group by a UE.

Proposal 4:

- For SS set configuration

  - The Group (2) SS sets can be simultaneously configured in all slots in a slot group.

  - The maximum number of configured spans in a slot remains to be 2 considering all SS sets.

- For PDCCH monitoring at UE

  - The maximum number of monitored spans per slot in a slot group remains to be 2 considering all SS sets

  - UE is capable to monitor up to 3 or 4 spans of SS sets in a slot group considering all SS sets.

- Discuss the exact number of limited configured/monitored spans in a slot or in a slot group in UE feature for WI NR_ext_to_71GHz

Proposal 5:

- A span of SS sets configured in a slot group, if it is not monitored by the UE based on a semi-static rule, may not be counted in the number of monitored spans in the slot group.

- If the maximum number of spans in a slot group is exceeded, UE can drop one or more spans of at least the group (1) SS sets

Proposal 6:

- UE should be able to process one broadcast DCI for SI/RACH/paging in addition to the agreed number of processed unicast DCI in a slot group of X slots.

- To clarify whether a UE would be able to detect up to 8 unicast DCIs in a slot on the scheduling cell with SCS 15kHz

- The limitation on number of detected DCIs in a slot group should be discussed in UE feature for WI NR_ext_to_71GHz

Proposal 7:

- For a UE capable of multiple combinations (X, Y), if the configured SS sets are aligned with more than one combination (X, Y), the active combination (X, Y) is determined that is associated with the largest X and smallest Y.

- Agree on TP 2 to determine the active combination (X, Y).

Proposal 8:

- MPDCCHtotal,(X,Y),μ, CPDCCHtotal,(X,Y),μ needs to be determined and shared by all cells that are configured with scheduling cells having same SCS configuration μ and same value X in combinations (X, Y).

- Agree on TP 3 to determine MPDCCHtotal,(X,Y),μ, CPDCCHtotal,(X,Y),μ

Proposal 9:

- Dynamic SSSG switching is supported for all SCSs 120, 480 and 960kHz.

- PDCCH monitoring following the second SSSG can start right after the time of SSSG switching

- The search space set configurations of the two SSSG can correspond to two different PDCCH monitoring capabilities combinations (X, Y)

- UE can monitor the PDCCHs in the X slots before the time of SSSG switching in the slots that are the intersection of the slot patterns of the two combinations (X, Y) before and after SSSG switching

- Agree on TP 4 to do SSSG switching with different multi-slot PDCCH monitoring capability combinations (X, Y).

Proposal 10:

- In search space set configuration,

  - duration indicates the number of consecutive slots for PDCCH monitoring according to a combination (Xs, Ys) upon every period as given in the periodicityAndOffset

  - Revise the WA:

    - monitoringSlotsWithinSlotGroup-r17 can indicate the slots with configured MOs in two consecutive slot groups of X=4. UE can expect that the configured MOs are in same position in both slot groups.

    - if monitoringSlotsWithinSlotGroup-r17 is not configured, the SS set is only configured in slot modoffset,X in a slot group

- Agree on TP 5 for SS set configuration

# References

- Chairman notes for RAN1#107bis-e