3GPP TSG RAN WG1 Meeting #108-e	R1-2201692

e-Meeting, February 21st – March 3rd, 2022

Source:	Intel Corporation

Title:	Discussion on Channel Access Mechanism for extending NR up to 71 GHz

Agenda item:	8.2.6

Document for:	Discussion/Decision

# Introduction

In 3GPP TSG RAN Meeting #86, a new SID on studying the support of NR from 52.6 GHz to 71 GHz (RP-193228) [1] was approved. Updated working item (WI) has been approved with the aim to extending NR up to 71 GHz [2] in the 3GPP TSG RAN Meeting #93-e. As part of the objectives of the WI, the following aspects were included:

- Physical layer procedure(s) including [RAN1]: - Channel access mechanism assuming beam based operation in order to comply with the regulatory requirements applicable to unlicensed spectrum for frequencies between 52.6GHz and 71GHz. - Specify both LBT and No-LBT related procedures, and for No-LBT case no additional sensing mechanism is specified. - Study, and if needed specify, omni-directional LBT, directional LBT and receiver assistance in channel access - Study, and if needed specify, energy detection threshold enhancement - Radio interface protocol architecture and procedures [RAN2]: - For operation in this frequency range: Introduce higher layer support of enhancements listed above that are agreed to be specified. - Core specifications for UE, gNB and RRM requirements [RAN4]: - Specify new band(s) for the frequency range from 52.6GHz-71GHz. The band(s) definition should include UL/DL operation and excludes ITS spectrum in this frequency range. - Specify gNB and UE RF core requirements for the band(s) in the above frequency range, including a limited set of example band combinations (see Note 1). - Specify RRM/RLM/BM core requirements. Note 1: The WI can be completed provided requirements for at least one band combination involving a new NR-U band is specified as long as it is in line with country-specific regulatory directives. Note 2: UEs supporting a band in the range of 52.6GHz-71GHz are not required to support 480kHz SCS and 960kHz SCS. Note 3: The maximum FFT size required to operate the system in 52.6GHz-71GHz frequency is 4096, and the maximum of RBs per carrier is 275 RBs. Note 4: the system is designed to support both single-carrier and multi-carrier operation. Note 5: RAN plenary will decide whether new FR (e.g. FR3) shall be defined for the frequency range from 52.6GHz-71GHz or the existing FR2 shall be extended to cover frequency range from 52.6GHz-71GHz. Similar to regular NR and NR-U operations below 52.6GHz, NR/NR-U operation in the 52.6GHz to 71GHz can be in stand-alone or aggregated via CA or DC with an anchor carrier.

In this contribution, in the context of channel access to extend NR up to 71 GHz, and based on the agreements and conclusions made during the previous RAN1 meetings along this WI [2-9], the following aspects will be discussed:

- Channel access indication within Fall-back DCIs;

- Cyclic prefix extension for CG UL transmissions;

- Definition of operating BW and Pout within the EDT calculation;

- Details for the enhanced L3-RSSI scheme;

- Details related to the short signaling exemption;

- Details related to multi-beam channel access procedure;

- Details Related to Multi-carrier Channel Access procedure;

- Details Related to COT sharing procedure;

- Correction of editorials.

# Channel Access Indication within Fall-back DCIs

In RAN1 #107 meeting [8], it was agreed that for FR2-2 the field ChannelAccess-CPext carried in the non-fallback DCIs is reinterpreted to carry information indicating which channel access type to use. In particular, the following was agreed:

Agreement For Non-Fallback DCI formats, for FR2-2 operation, for the configuration of the ChannelAccess-CPext field in DCI to indicate the channel access type only, new tables are introduced indicating channel access types for FR2-2, with entries “Type 1 channel access in 4.4.1 of 37.213”, “Type 2 channel access in 4.4.2 of 37.213” and “Type 3 channel access in 4.4.3 of 37.213”.

However, no consensus was reach regarding the details of the channel access indication for both the fallback DCI format 0-0 and 1-0, and the RAR UL grant. In this matter, even though type 2 channel access mechanism may be only supported based on the UE’s capability, the gNB should be still allowed to configure/indicate this channel access type when supported.

Proposal 1: For fallback DCI formats 0_0/1_0 and RAR UL grant, for FR2-2 operation, the ChannelAccess-Cpext field indicates one of the entries of a table which entries are “Type 1 channel access in 4.4.1 of 37.213”, “Type 2 channel access in 4.4.2 of 37.213” and “Type 3 channel access in 4.4.3 of 37.213”.

In order to capture the above intent, the following text should be captured in Sec. 7.3.1.1.1, and 7.3.1.2.1 of TS 38.212:

------------------------------ TP#1: TS 38.212 ----------------------------------- *** Unchanged text is omitted *** ##### 7.3.1.1.1 Format 0_0 DCI format 0_0 is used for the scheduling of PUSCH in one cell. The following information is transmitted by means of the DCI format 0_0 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI: *** Unchanged text is omitted *** - ChannelAccess-CPext –2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4, or Table 7.3.1.1.1-4A if ChannelAccessMode-r16 = "semistatic" is provided, for operation in a cell with shared spectrum channel access for frequency range 1, and 2 bits indicating the channel access type as defined in Table 7.3.1.1.1-4B, if LBT-mode is provided and indicates that channel access procedures would be performed for operation in a cell in frequency range 2-2; 0 bit otherwise. *** Unchanged text is omitted *** The following information is transmitted by means of the DCI format 0_0 with CRC scrambled by TC-RNTI: *** < Unchanged parts are ommitted> *** - ChannelAccess-CPext –2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4, or Table 7.3.1.1.1-4A if ChannelAccessMode-r16 = "semistatic" is provided, for operation in a cell with shared spectrum channel access for frequency range 1, and 2 bits indicating the channel access type as defined in Table 7.3.1.1.1-4B, if LBT-mode is provided and indicates that channel access procedures would be performed for operation in a cell in frequency range 2-2; 0 bit otherwise. *** Unchanged text is omitted *** Table 7.3.1.1.1-4: Channel access type & CP extension for DCI format 0_0 and DCI format 1_0 in frequency range 1 Bit field mapped to index; Channel Access Type; The CP extension T_"ext" index defined in Clause 5.3.1 of [4, TS 38.211]. 0; Type2C-ULChannelAccess defined in [clause 4.2.1.2.3 in 37.213]; 2. 1; Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213]; 3. 2; Type2A-ULChannelAccess defined in [clause 4.2.1.2.1 in 37.213]; 1. 3; Type1-ULChannelAccess defined in [clause 4.2.1.1 in 37.213]; 0. Table 7.3.1.1.1-4A: Channel access type & CP extension if ChannelAccessMode-r16 = "semistatic" is provided in frequency range 1 Bit field mapped to index; Channel Access Type; The CP extension T_"ext" index defined in Clause 5.3.1 of [4, TS 38.211]. 0; No sensing as defined in Clause 4.3 in TS 37.213; 0. 1; No sensing as defined in Clause 4.3 in TS 37.213; 2. 2; 9us sensing within a 25us interval as defined in Clause 4.3 in TS 37.213; 0. 3; -; -. Table 7.3.1.1.1-4B: Channel access type in frequency range 2-2 Bit field mapped to index; Channel Access Type. 0; Type 1 channel access defined in clause 4.4.1 of 37.213. 1; Type 2 channel access defined in clause 4.4.2 of 37.213. 2; Type 3 channel access defined in clause 4.4.3 of 37.213. 3; Reserved. *** Unchanged text is omitted *** ##### 7.3.1.2.1 Format 1_0 DCI format 1_0 is used for the scheduling of PDSCH in one DL cell. The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by C-RNTI or CS-RNTI or MCS-C-RNTI: *** Unchanged text is omitted *** - ChannelAccess-CPext – 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4, or Table 7.3.1.1.1-4A if ChannelAccessMode-r16 = "semistatic" is provided, for operation in a cell with shared spectrum channel access for frequency range 1, and 2 bits indicating the channel access type as defined in Table 7.3.1.1.1-4B, if LBT-mode is provided and indicates that channel access procedures would be performed for operation in a cell in frequency range 2-2; 0 bits otherwise *** Unchanged text is omitted *** The following information is transmitted by means of the DCI format 1_0 with CRC scrambled by TC-RNTI: *** Unchanged text is omitted *** - ChannelAccess-CPext – 2 bits indicating combinations of channel access type and CP extension as defined in Table 7.3.1.1.1-4, or Table 7.3.1.1.1-4A if ChannelAccessMode-r16 = "semistatic" is provided, for operation in a cell with shared spectrum channel access for frequency range 1, and 2 bits indicating the channel access type as defined in Table 7.3.1.1.1-4B, if LBT-mode is provided and indicates that channel access procedures would be performed for operation in a cell in frequency range 2-2; 0 bits otherwise *** Unchanged text is omitted ***

Furthermore, the following text should be captured in Sec. 8.2 and 8.2A of TS 38.213:

------------------------------ TP#2: TS 38.213 ----------------------------------- *** Unchanged text is omitted *** ## 8.2 Random access response - Type-1 random access procedure *** Unchanged text is omitted *** The ChannelAccess-CPext field indicates a channel access type and CP extension for operation with shared spectrum channel access [15, TS 37.213] as defined in Table 7.3.1.1.1-4 in TS 38.212 or Table 7.3.1.1.1-4A in TS 38.212 if ChannelAccessMode-r16 = "semistatic" is provided for frequency range 1. For operation in a cell in frequency range 2-2 if LBT-mode is provided and indicates that channel access procedures would need to be performed [15, TS 37.213], the ChannelAccess-CPext field indicates a channel access type as defined in Table 7.3.1.1.1-4B in TS 38.212. Table 8.2-1: Random Access Response Grant Content field size RAR grant field; Number of bits. Frequency hopping flag; 1. PUSCH frequency resource allocation; 14, for operation without shared spectrum channel access 12, for operation with shared spectrum channel access. PUSCH time resource allocation; 4. MCS; 4. TPC command for PUSCH; 3. CSI request; 1. ChannelAccess-CPext; 0, for operation without shared spectrum channel access, or for operation with shared spectrum access in frequency range 2-2 when LBT-mode indicates that channel access procedures would not need to be performed or LBT-mode is not provided, 2, for operation with shared spectrum channel access in frequency range 1, or in frequency range 2-2 when LBT-mode is provided and indicates that channel access procedures would need to be performed.. ## *** Unchanged text is omitted *** ## 8.2A Random access response - Type-2 random access procedure *** Unchanged text is omitted *** If the UE detects the DCI format 1_0, with CRC scrambled by the corresponding MsgB-RNTI and LSBs of a SFN field in the DCI format 1_0, if applicable, are same as corresponding LSBs of the SFN where the UE transmitted PRACH, and the UE receives a transport block in a corresponding PDSCH within the window, the UE passes the transport block to higher layers. The higher layers indicate to the physical layer - an uplink grant if the RAR message(s) is for fallbackRAR and a random access preamble identity (RAPID) associated with the PRACH transmission is identified, and the UE procedure continues as described in clauses 8.2, 8.3, and 8.4 when the UE detects a RAR UL grant, or - transmission of a PUCCH with HARQ-ACK information having ACK value if the RAR message(s) is for successRAR, where - a PUCCH resource for the transmission of the PUCCH is indicated by PUCCH resource indicator field of 4 bits in the successRAR from a PUCCH resource set that is provided by pucch-ResourceCommon - a slot for the PUCCH transmission is indicated by a HARQ Feedback Timing Indicator field of 3 bits in the successRAR having a value k from {1, 2, 3, 4, 5, 6, 7, 8} and, with reference to slots for PUCCH transmission having duration Tslot, the slot is determined as n+k+∆+2μ∙Kcell,offset, where n is a slot of the PDSCH reception, ∆ is as defined for PUSCH transmission in Table 6.1.2.1.1-5 of [6, TS 38.214], μ is the SCS configuration of the active UL BWP, and Kcell,offset is provided by Koffset in ServingCellConfigCommon; otherwise, if not provided, Kcell,offset=0 - the UE does not expect the first symbol of the PUCCH transmission to be after the last symbol of the PDSCH reception by a time smaller than NT,1+0.5 msec where NT,1 is the PDSCH processing time for UE processing capability 1 [6, TS 38.214] - for operation with shared spectrum channel access, a channel access type and CP extension [15, TS 37.213] for a PUCCH transmission is indicated by a ChannelAccess-CPext field in the successRAR as defined in Table 7.3.1.1.1-4 in TS 38.212 or Table 7.3.1.1.1-4A in TS 38.212 if ChannelAccessMode-r16 = "semistatic" is provided for frequency range 1. For operation in a cell in frequency range 2-2 if LBT-mode is provided and indicates that channel access procedures would need to be performed [15, TS 37.213], the ChannelAccess-CPext field indicates a channel access type as defined in Table 7.3.1.1.1-4B in TS 38.212. - the PUCCH transmission is with a same spatial domain transmission filter and in a same active UL BWP as a last PUSCH transmission ## *** Unchanged text is omitted ***

Proposal 2: TP#1 and TP#2 should be supported.

# Cyclic Prefix Extension for CG UL Transmissions

In RAN1 #107 meeting [8], it was concluded that no cyclic prefix (CP) extension would be used for F2-2 for DCI scheduled UL transmissions, but it was left for further study whether a CP extension should be supported for CG-PUSCH transmissions:

Conclusion Rel.16 NR-U style Cyclic Prefix extension is not supported for FR2-2 at least for DCI scheduled UL transmission - FFS: If CP extension is supported for CG-PUSCH in FR2-2

In this matter, it is important to clarify that a CP extension applied in FR2-2 to a CG-PUSCH may not be intended to create a suitable gap for COT sharing, but may be still used to mitigate mutual interference among UEs so that UEs may start their transmission with a proper offset between them, but could also be used so that to prioritize UL scheduled transmissions over UL CG transmissions.

It is worth recalling that in Rel.16 NR-U a CP extension is applied to the first symbol of the configured resources based on a defined set of the offset values {16, 25, 34, 43, 52, 61, k=1CTsymb, (l-k)mod 7∙2μμ}, which is used both when a UE performs a CG PUSCH transmission within and outside of a gNB-initiated COT with the aim to mitigate mutual interference. Furthermore, a grant based PUSCH is prioritized over a CG PUSCH through proper scheduling, but also through proper configuration of the aforementioned offset. In this regards, the offset values were picked taking in mind that: i) the starting position offsets are not necessarily applied within symbol #0, ii) multiple subcarrier spacings are supported, iii) the slot granularity of 11ac is 9 us, and the offset values can be aligned across the same granularity, and iv) a gap of at least 16us must be left between a scheduled and a CG transmission to perform a 16us LBT. In this matter, the CP extension and the starting offsets were derived as follows:

Text=k=12μTsymb,  l-kmod 7∙2μ μ-Δi

where Δi is selected from the following set of values {16, 25, 34, 43, 52, 61, k=12μTsymb,  l-kmod 7∙2μ μ}.

With that said, similar approach can be used for FR2-2, with the difference that the slot granularity of 11ad/ay is 5us and for FR2-2 at most a gap of 8us is required given that this is the observation length for the type 2 LBT procedure. Therefore, while the small starting offset value should not be 8 us, the granularity among the values within the set should be 5us.

Proposal 3: Support cyclic prefix extension for CG-PUSCH transmissions in the FR2-2 frequency range using the same design principle as NR-U.

- The first starting offset value should be equal to 8us and the granularity among the set of starting offsets should be equal to 5us.

# Definition of Operating BW and Pout within the EDT Calculation

In prior RAN#1 meetings [3-8], the EDT threshold to use to determine whether a channel is busy or not has been defined for FR2-2 as per the following agreement:

Agreement: The baseline ED threshold can be computed as EDT=-80 dBm+10*log10PmaxPout+10*log10(Operating Channel BW in MHz) Where Pout is RF output power (EIRP) and Pmax is the RF output power limit, Pout≤Pmax. - FFS: Further adjustment on ED threshold based on the sensing beam and the transmission beam (further adjustment should not violate EDT requirements as per regulations) - FFS: If Pout is max output EIRP of the device or instantaneous output EIRP - FFS definition of Operating Channel BW - FFS: Whether ED threshold for NR-U and NR-U coexistence scenarios (eg, at regulation level) can be appropriately relaxed compared with the threshold of coexistence between NR-U and Wi-Fi. - FFS: EDT when the COT has time varying transmission beams and varying EIRP

While all the details have been discussed, the definition of the operating channel BW is still unresolved. In this matter, it has been agreed that regardless of whether the LBT procedure is performed for single or multi-carrier transmissions, the smaller granularity over which the LBT procedure is actually performed is over the active BWP of each channel bandwidth:

Agreement: - For LBT for single carrier transmission, gNB/UE performs LBT over the channel bandwidth (or BWP bandwidth) (Alt SC.1. in earlier agreements) - For LBT for multi-carrier transmission in intra-band CA, gNB/UE performs multiple LBT, one for each channel bandwidth separately (Alt CA.1. in earlier agreements) - FFS: Additional support of performing single LBT over all CCs (Alt CA.2. in earlier agreements)

Proposal 4: For single carrier transmission, a device performs LBT over a channel bandwidth, where for the case when the UE is the device performing LBT, then the channel bandwidth should include at least the active UL BWP.

Proposal 5: For LBT for multi-carrier transmission, gNB/UE performs multiple LBT, one for each channel bandwidth separately.

In order to capture the above intent, the following text should be captured in Sec. 4.4.7 of TS 37.213:

------------------------------ TP#3: TS 37.213 ----------------------------------- *** Unchanged text is omitted *** ### 4.4.7 Energy detection threshold adaptation procedures A gNB/UE accessing a channel on which transmission(s) on beam(s) are performed within a channel occupancy, shall set the energy detection threshold XThresh to be less than or equal to the maximum energy detection threshold XThresh_max that is determined as follows: XThresh_max=-80dBm+Pmax- Pout+ 10⋅log10(BW) where: - Pmax is the RF output power limit in dBm - Pout is the maximum EIRP of the intended transmission(s) by the gNB/UE to acquire a channel occupancy in dBm where Pout≤Pmax. The maximum EIRP used for the transmission(s) by the initiating gNB/UE during the channel occupancy is limited to Pout. - BW is the channel bandwidth in MHz for each carrier ci∈C, where for the case when the UE performs channel access the channel bandwidth includes at least the RBs of the active uplink BWP. *** Unchanged text is omitted ***

Proposal 6: TP#3 should be supported.

As for the definition of Pout, the following agreements have been made during the prior RAN1 meetings [3-9]:

Working assumption: For Pout in EDT determination, define Pout as the maximum EIRP of the node determining EDT during a COT. Agreement Confirm the WA with some clarifications Working assumption: - For Pout in EDT determination, define Pout as the maximum EIRP of the intended transmissions by the node determining EDT during a COT. - The node is not expected to transmit in the COT with higher Pout than the Pout used to determine the EDT used to acquire the COT Agreement For gNB initiated COT, for Pout in EDT determination at the initiating device (gNB), the Pout of the responding device (UE) is not considered

However, in case an initiating device may perform a per-beam LBT procedure, it is not yet clear how the Pout is calculated in case the maximum EIRP in different beams may be different. In this case, our view is that for the matter of the individual calculation of the ED threshold the Pout should be accounting only for the maximum EIRP in the transmission beams that are covered by that specific sensing beam, and should not account for all other beams.

Proposal 7: For a COT with MU-MIMO (SDM) transmission or TDM transmission of beams with beam switching, when independent per-beam LBT is performed at the start of the COT, define Pout as the maximum EIRP of the intended transmissions “covered” by the sensing beam by the node determining EDT during the COT.

# Details for the Enhanced L3-RSSI Scheme

In prior RAN1 meetings [7-9], it was concluded to extend the Rel.16 L3-RSSI for the FR2-2 frequency range. Furthermore, it was agreed to introduce an additional field within the RRC parameter RMTC-Config which is used to indicate the measurement bandwidth. Additionally, while the reference subcarrier spacings (SCS) have not been yet decided, for both 120, 480 and 960KHz SCS, RAN1 has defined the specifics of the possible measurement duration. In this matter, the following agreements were made:

Agreement: Support extending Rel.16 L3-RSSI to unlicensed operation in FR2-2 - Introduce RRC configuration for reference SCS, measurement duration, and measurement bandwidth - Extend the reference SCS/CP field (ref-SCS-CP-r16) and measurement duration field (measDurationSymbols-r16) in RMTC-Config - FFS value range and valid combinations for ref-SCS-CP-r16 and measDurationSymbols-r16 - Introduce parameter in RMTC-Config to indicate the measurement bandwidth - FFS: Value range for measurement bandwidth - For the QCL Type-D of L3-RSSI measurement, down-select one or both of the following alternatives - Alt 1: gNB configures the beam when configures the L3-RSSI measurement - Alt 2: Use the QCL type-D of the latest received PDSCH and the latest monitored CORESET Agreement Introduce new parameter in RMTC-Config for L3-RSSI to indicate measurement bandwidth. - The value range for the configured measurement bandwidth should include the maximum and the minimum channel bandwidth and the intermediate channel bandwidths defined by RAN4. Agreement On measDurationSymbols and reference SCS/CP for L3-RSSI - On measDurationSymbols-r16 with ref-SCS-CP-r16=120KHz, extend measDurationSymbols-r16 to {1,14,28,42,70,140} - On measDurationSymbols-r16 with ref-SCS-CP-r16=480KHz (if supported), extend measDurationSymbols-r16 to {1,14,28,42,70,140, 560} On measDurationSymbols-r16 with ref-SCS-CP-r16=960KHz (if supported), extend measDurationSymbols-r16 to {1,14,28,42,70,140, 560,1120}

As mentioned above and indicated in one of the agreement above, while RAN1 agreed to enhance the L3-RSSI measurements for FR2-2, it has not yet concludes on the reference SCS to support and on the specifics of the field ref-SCS-CP-r16 within RMTC-Config. In this context, ref-SCS-CP-r16 should be definitely extended to include the relevant subcarrier spacing for FR2-2 (i.e, 120, 480 and 960 KHz).

Proposal 8: ref-SCS-CP-r16 is extended to include all the supported SCS for FR2-2 (i.e., 120, 480 and 960 KHz).

As for the discussion related to the modality with which the QCL Type-D is selected for L3-RSSI measurements, Alt-2 seems to be sufficient.

Proposal 9: For the QCL Type-D of L3-RSSI measurement use the QCL type-D of the latest received PDSCH and the latest monitored CORESET.

# Details Related to the Short Signaling Exemption

Short signal exemption (SSE) is a mechanism to send relatively short signaling transmissions (compared to regular data transmissions) without sensing the channel for presence of other signals. For frequency ranges from 52.6 GHz up to 71 GHz, SSE is described in the most recent draft of EN 302 567 v2.2.0 [10] with application to short management and control frames. The total duration of the short control signaling transmissions is required to be constrained to less than 10 msec within 100 msec observation period per device. While this exemption is supported and can be applied in CEPT/EU regions, this may not be used in other regions even when LBT may be used as an adaptivity method. As an example while in Japan ARIB mandates the use of LBT, this hasn’t defined the concept of SSE. Therefore, this relaxation cannot be applied in the related regions. In this matter, it may be necessary to introduce a new RRC parameter to allow the network to indicate to a UE whether the SSE should be applied or not.

Proposal 10: The gNB indicates through a cell-specific RRC parameter in SIB1 whether the short signal exemption should be applied or not.

In previous RAN1 meetings [3-9], it has been concluded that the SSE rules can apply to the transmission of mg1 for the 4-step RACH and MsgA for the 2-step RACH for all supported SCS as long as the 10% duty cycle is met. However, it was argued how to actually interpret this constraint, and whether the 10% duty cycle is applied per cell or from the perspective of a UE. As indicated along this section, the EN 302 567 v2.2.0 [10] is written from the point of view of the device (equipment), and the limitations imposed for a SSE are meant to be applied per device. Therefore, when applying this to UL channels, the maximum 10 % transmission over an observation of 100ms should be measured from the UE’s perspective.

Proposal 11: The 10% over any observation period of 100ms is applicable to the msg1/msgA transmission from one UE perspective.

In order to capture the above intent, the following text should be captured in Sec. 4.4.5 of TS 37.213:

------------------------------ TP#4: TS 37.213 ----------------------------------- ### *** Unchanged text is omitted *** ### 4.4.5 Exempted transmissions from sensing In regions where channel sensing is required to access a channel for transmission and short control signalling exemption is allowed by regulation, if the higher layer parameter ShortControlSignalling-r17 is configured a gNB/UE may transmit the following transmission(s) without sensing the channel: - Transmission(s) of the discovery burst by the gNB; - Transmission(s) of the first message in a random access procedure by the UE. When the gNB/UE transmits the above transmission(s) without sensing on a channel by utilizing the exemption above, the total duration of such transmission(s) by the gNB/UE shall not occupy the corresponding channel more than 10ms over any 100ms interval from the perspective of the transmitting device. *** Unchanged text is omitted ***

Proposal 12: TP#4 should be supported.

# Details Related to Multi-beam Channel Access Procedure

In the context of multi-beam operation, in prior RAN1 meetings [3-9], the following agreements were made:

Agreement For a COT with MU-MIMO (SDM) transmission, support both Alt 1 and Alt 2 below: - Alt 1: Single LBT sensing at the start of the COT with wide beam ‘cover’ all beams to be used in the COT with appropriate ED threshold - Alt 2: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT, if the node can perform simultaneous sensing in different beams Note: On UE side, no UE capability will be introduced for this purpose. Agreement Within a COT with TDM of beams with beam switching, at least support Alt 1 - Alt 1 (from previous agreement): Single LBT sensing with wide beam ‘cover’ all beams to be used in the COT Agreement Within a COT with TDM of beams with beam switching, Alt 2 is supported if the node has the capability to perform simultaneous sensing in different beams. Alt 3 is allowed as node implementation choice if the node also supports Cat 2 LBT. The use of Alt 2 or Alt 3 is based on node’s implementation. - Alt 2 from previous agreement: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT - Alt 3 from previous agreement: Independent per-beam LBT sensing at the start of COT is performed for beams used in the COT with additional requirement on Cat 2 LBT before beam switch

While according to the agreements above a device can acquire a COT over multi-beams by either performing a wide-beam LBT or independent per-beam LBT for the latter the details of how the LBT procedure is done has not been yet defined. In fact, when independent per-beam LBT sensing is performed, it is not clear whether a COT may be established once all LBT procedures across all beams are successful, or a COT can be considered established as long the LBT procedure is successful for at least a single beam, and in this last case a transmission may be allowed only over the beam or beams for which the LBT procedure was found to be successful. In this matter, the discussion could be made separately based on whether multiplexing within a COT is done in spatial or time domain, and whether type 2 channel access type may be used. However, in all these cases it may be beneficial to increase spectral utilization and prevent to have procedure with single point of failure to allow a transmission to occur as long as the LBT procedure has been successful for at least a single beam, and disallow transmission on those beams for which the LBT was not successful.

Proposal 13: When independent per-beam LBT sensing is performed, a transmission may be allowed to occur as long as the LBT procedure has been successful before a channel occupancy for at least a single beam. However, a transmission (via either spatial or time multiplexing) may not be allowed on those beams for which the LBT procedure was not successful.

Given that counters for the per-beam LBT procedure may be independently maintained per each beam, another aspect that must be also clarified in this matter is on how and when to declare the channel clear or idle for a beam and when to reinitialize each counters for each beam. Additionally, another aspect that should be jointly clarified is on how to handle the likely situation that the independent per beam LBT procedures may terminate in different instances of time. Due to the latter issue, a device should be indeed prevented from transmitting over one beam while sensing on another in order to avoid self-interference and blocking. In this sense, a transmission should be always aligned among beams, and a common transmission starting time should be defined and established among beams. In this case, given that the per-beam LBT procedure may assess that a channel is idle before the common transmission starting time starts, in order to make sure that a beam would continue to be available, a device (gNB or UE) should behave as follows:

- If the backoff counter NBi for a sensing beam Bi reaches zero before the aligned transmission starting time, the device continues to decrement the counter NBi by continuing to sense the channel via sensing slots of 5us each and transmits in the corresponding beam at the aligned start time if the channel continues to be sensed idle in all of the additional sensing slot durations.

- If the backoff counter NBi for a sensing beam Bi does not reach zero before the aligned start time, or reaches zero but the channel has been sensed busy in any of the additional sensing slot durations, the transmission(s) in the corresponding beam is dropped, and the channel over that beam is declared to be busy.

Once the device performing the channel access is able to cease a transmission over at least one of the beams over which the channel access procedure was done, then device should reinitialize the counter for all those beams.

Proposal 14: When independent per-beam LBTs are performed to initiate a multi-beam COT with TDMed or SDMed transmission beams, independent counters are maintained per beam.

Proposal 15: When independent per-beam LBTs are performed to initiate a multi-beam COT with TDMed or SDMed transmission beams, support to align transmission starting time across the multiplexed beams such that a transmission on one beam does not start while sensing is ongoing on another beam. In this matter, a device should behave as follows:

- If the backoff counter NBi for a sensing beam Bi reaches zero before the aligned transmission starting time, the device continues to decrement the counter NBi by continuing to sense the channel via sensing slots of 5us each and transmits in the corresponding beam at the aligned start time if the channel continues to be sensed idle in all of the additional sensing slot durations.

- If the backoff counter NBi for a sensing beam Bi does not reach zero before the aligned start time, or reaches zero but the channel has been sensed busy in any of the additional sensing slot durations, the transmission(s) in the corresponding beam is dropped.

Proposal 16: After the gNB/UE ceases transmission in any of the beam for which the channel access procedure was done, the gNB/UE will reinitialize the counter for all beams.

Together with an independent per-beam LBT sensing where the sensing is done simultaneously across the beams, it should be also supported an LBT procedure where the LBT sensing is done in a sequential manner one at the time.

Proposal 17: When time-domain switching across beams within the same COT is supported, the per-beam LBT for different beams is also performed in a sequential manner. In particular, the initiating device may sense on a beam before either transmitting on that beam or switching to a separate beam to perform sensing.

Another aspect that also needs further discussions and clarification is related to when L1 must indicate and trigger the LBT failure recovery procedure in case an independent per-beam LBT sensing is performed. In this matter, RAN2 has made the following agreement:

Agreement From RAN2 point of view there is no need that PHY provides per-beam LBT failure indications to MAC in Rel-17. No need to send LS to RAN1 unless they request RAN2 view.

While this agreement has been made from a RAN2 point of view, RAN1 is expected to discuss this further and definitely conclude on this topic. Given the conclusion and discussion in RAN2 and the same technical reasons as described above, it may be preferable to leave the procedure as in Rel.16 and identify an LBT failure per transmission rather than per beam. Furthermore, once a conclusion is made in RAN1, it may be advisable for RAN1 to inform RAN2 of the decision made.

Proposal 18: When independent per-beam LBT sensing is performed, an LBT failure is counted per transmission, and an LBT failure is reported only if all per beam LBTs fail.

Proposal 19: RAN1 should send an LS to RAN2 to inform them about the decision made in terms of how an LBT failure should be counted.

# Details Related to Multi-carrier Channel Access Procedure

In the context of multi-carrier operation, in Rel. 16 NR for the for sub-6 GHz band, two modes of operation where introduced:

Type A: the gNB performs Cat-4 LBT on each carrier. In order to align the transmissions across carriers, the gNB performs a self-deferral. For this type of operation, the gNB may:

    - Type A1: Maintain a CWS counter and perform CWS adjustment independently for each carrier. In this case, if gNB ceases transmission over one carrier, and resumes to decrease the counter for all other carriers if the channel is idle for 4 CCA or after reinitializing the value of the counter.

    - Type A2: Maintain a common CWS counter and perform a common CWS adjustment for all the carriers. The CWS adjustment is performed by associating the highest CWS value across carriers to all carriers. In this case, if gNB ceases transmission over one carrier, then the gNB reinitializes the counter for all carriers.

Type B: the gNB selects a single carrier by either uniformly picking in a random fashion one carrier across all carriers or by configuring a specific carrier to operate as a so called “primary channel”. A primary channel cannot be changed more than once every second. The gNB can perform sensing over the other carriers by applying a Cat-2 LBT only if the gNB completes the Cat-4 LBT for the primary channel. For this type of operation, the gNB may:

    - Type B1: Maintain a CWS counter and perform CWS adjustment independently for each carrier. In this case the single carrier mechanism is used with the distinction that a CWS is increased if NACK is determined in all carriers within the reference subframe.

    - Type B2: Maintain a common CWS counter and perform a common CWS adjustment for all the carriers.

For NR operating above 52.6 GHz band, the following agreement was made:

Agreement Type A multi-channel channel access is supported. - FFS whether legacy mechanisms such as type A1 is supported

While as mentioned above the type A procedure is a well-established procedure for sub-6 GHz band, where the LBT with adjustable contention window is used and the concept of channel access priority class and contention window adjustment were introduced, these are no longer relevant for above 52.6 GHz band where type 1 LBT has been defined for which the contention window is fixed. Therefore, while some aspects could be reused some may require to be changed. In this matter, a similar procedure as that used for multi-beam operation which is based on type A procedure could be used. In this matter, in order to avoid any self-interference and blocking a common transmission starting time should be defined among carrier, so that a device may be prevent from transmitting in one carrier while sensing on another. Furthermore, since independent LBT will be performed for each carrier, then a device should maintain an independent counter for each carrier LBT procedure. Additionally, given that the per-carrier LBT procedure may assess that a channel is idle before the common transmission starting time starts, in order to make sure that a carrier would continue to be available, a device (gNB or UE) should behave as follows:

- If the backoff counter NCi for a carrier Ci reaches zero before the aligned transmission starting time, the device continues to decrement the counter NCi by continuing to sense the channel via sensing slots of 5us each and declares the channel for that carrier to be idle if the channel continues to be sensed idle in all of the additional sensing slot durations.

- If the backoff counter NCi for a carrier Ci does not reach zero before the aligned start time, or reaches zero but the channel has been sensed busy in any of the additional sensing slot durations, channel access procedure in carrier Ci is considered to have failed.

Once the device performing the channel access is able to cease a transmission over at least one of the carriers over which the channel access procedure was done, then device should reinitialize the counter for all those carries.

Proposal 20: For FR2-2, multi-carrier channel access procedure is employed through independent counters, one for each carrier.

Proposal 21: Support to align transmission starting time across the carrier such that a transmission on one carrier does not start while sensing is ongoing on another carrier. In this matter, a device should behave as follows:

- If the backoff counter NCi for a carrier Ci reaches zero before the aligned transmission starting time, the device continues to decrement the counter NCi by continuing to sense the channel via sensing slots of 5us each and declares the channel for that carrier to be idle if the channel continues to be sensed idle in all of the additional sensing slot durations.

- If the backoff counter NCi for a carrier Ci does not reach zero before the aligned start time, or reaches zero but the channel has been sensed busy in any of the additional sensing slot durations, channel access procedure in carrier Ci is considered to have failed.

Proposal 22: After the gNB/UE ceases transmission in any of the carrier for which the channel access procedure was done, the gNB/UE will reinitialize the counter for all channels.

# Details Related to COT Sharing Procedure

In prior RAN1 meeting [6], the following agreement has been made:

Agreement: On COT sharing from an initiating device transmission to responding device transmission, support both of the following two alternatives - Alt 1: No maximum gap defined between the initiating device transmission and responding device transmission. A responding device transmission can occur without LBT with any gap within the maximum COT duration - Alt 3: Define a maximum gap Y, such that a responding device transmission can occur without LBT only if the transmission starts within Y from the end of the initiating device transmission. If the responding device transmission starts after Y from the end of the initiating device transmission, a Cat 2 LBT is needed before the responding device transmission. - The Cat 2 LBT uses the same sensing structure as the 8 us initial deferral period as in eCCA - Further down-select between the following options: - Option 1: Y=8 us (motivated by need to operate in all regions) - Option 2: Y=a multiple number of OFDM symbols - Option 3: gNB determines Y (for example, according to local regulation) - Cat. 2 LBT is a UE capability - The usage of the two alternatives is a gNB choice and depends at least on local regulations. Note: Alt. 3 is motivated by the regulations in Japan, but use of Cat. 3 LBT is also an option for operation in Japan and Cat. 2 LBT is not restricted for use only in Japan. Note: Maximum gap allowed without Cat 2 LBT between two initiating device transmissions is to be separately discussed Note: Other use cases of Cat 2 LBT will be separately discussed

One aspect that should be discussed is related to the whether Alt-3 should be also applied to the initiating device when it transmits bursts which are not contiguous in time domain, and there may be pauses in between. In this matter, the regulatory requirement for Japan [11], which is mandated by ARIB, is very generic and does not define any concept of initiating or responding device, but rather distinguishes a device from a transmitter to a receiver.

- Interference mitigation function - Sending and receiving identification signals (Enforcement Article 6-2) (Facilities Article 9-4) Shall automatically transmit or receive identification codes. - Carrier Sense (Facilities Article 49-20) If the transmission power of the transmitter exceeds 10 mW, provide a carrier sense that will operate at beginning of the transmission.

Therefore, in our understanding the aforementioned agreement should be also extended in case of pauses.

Proposal 23: If an initiating device is capable to perform Cat-2 LBT, and if the initiating device performs an additional burst within the initiated COT which may be separated with any prior burst of at least a minimum gap Y, then under Alt-3 a Cat 2 LBT is needed before the initiating device transmission.

In Rel.16, the CG UE’s COT sharing procedure has two modes of operation:

- Mode A: If the higher layer parameter ul-toDL-COT-SharingED-Threshold-r16 is provided, then the UE is allowed to share its COT with the gNB. In this case, the DL transmission should contain a transmission to the UE that initiated the channel occupancy and can also include non-unicast  and /or unicast transmissions, where any unicast transmission that includes user plane data is only transmitted to the UE that initiated the channel occupancy. Under this mode of operation, the UE is configured by cg-COT-SharingList-r16 where cg-COT-SharingList-r16 provides a table configured by higher layer, where each row of the table provides jointly the following information:

  - Channel access priority class used by the UE when acquiring the COT

  - Slot from where the DL transmission could start, which is identified as x+O, where x is the current slot and O is an offset indicated in terms of slots

  - Maximum duration of the DL transmission, which is indicated by D, and provided in terms of slots.

In addition, cg-COT-SharingList-r16 also includes an entry indicating that no COT sharing would be allowed:

cg-COT-SharingList-r16 SEQUENCE (SIZE (1..1709)) OF CG-COT-Sharing-r16 CG-COT-Sharing-r16 ::= CHOICE { noCOT-Sharing-r16 NULL, cot-Sharing-r16 SEQUENCE { duration-r16 INTEGER (1.. 39), offset-r16 INTEGER (1.. 39), channelAccessPriority-r16 INTEGER (1..4) } }

An illustration of this modality of operation is provided in Figure 1.

Figure 1 – High level illustration of mode A

- Mode B: If the higher layer parameter ul-toDL-COT-SharingED-Threshold-r16 is not provided, then if 'COT sharing information' in CG-UCI indicates '1' the UE is allowed to share its COT only for the purpose of DL control transmission for the length of 2/4/or 8 OFDM symbol for 15/30 or 60 KHz subcarrier spacing, respectively. In this case, the gNB can share the UE channel occupancy and start the DL transmission X symbols from the end of the slot where CG-UCI is detected, where cg-COT-SharingOffset-r16 is provided by higher layer.

An illustration of this modality of operation is provided in Figure 2.

Figure 2 – High level illustration of mode B.

Moving forward to Rel.17 for 52.6 GHz band, the following agreement was made:

Agreement In Rel-17, the same ED threshold determination mechanism is used for UL to DL COT sharing and for UL transmission without COT sharing with UE as initiating device. - FFS: Spec impact for UL to DL COT sharing mechanism

However, based on the description of the Rel.16 UE’s COT sharing procedure, it is not clear whether both modes of operation will be supported or only mode B will be supported, which is in line with the agreement above and implies that ul-toDL-COT-SharingED-Threshold-r16 is not applicable for NR operated above 52.6 GHz band. Our view is that for this frequency range, there is no need to support mode B, and only mode A should be supported, given that mode B was defined specifically for sub-6 GHz band to coexist with 11ac.

Proposal 24: When a UE performing a CG transmission shares its COT with a gNB, the gNB is always allowed to perform both unicast and broadcast transmissions without any constrains, and cg-UCI may always indicate one entry of cg-COT-SharingList-r16.

# 10 Editorial Corrections

Along TS 37.213 a few editorials have been identified:

- Along the description of the general channel access procedure in Sec. 4.4, it would be preferrable to use “that” before “channel(s)” in order to indicate that we are referring to the channel over which sensing should be done before transmission(s) should be performed;

- In Sec. 4.4.1, “performing as least” should be substituted with “performing at least”;

- The measurement window definition is the same between type 1 and type 2 channel access procedure. Therefore, when describing independently the two procedure same language should be used when indicating how the measurement is done within the defer duration Td=8μs .

In order to capture the above intent, the following text should be captured in Sec. 4.4, 4.4.1 and 4.4.2 of TS 37.213:

------------------------------------ TP#5: Sec. 4.4, 4.4.1 and 4.4.2 of TS 37.213 ----------------------------------- ### *** Unchanged text is omitted *** ## 4.4 Channel access procedures for frequency range 2-2 When a gNB/UE(s) is required by regulations to sense channel(s) for availability for performing transmission(s) on that channel(s) or when a gNB provides UE(s) with higher layer parameters LBT-Mode by SIB1 or dedicated configuration indicating that the channel access procedures would be performed before transmission(s) on that channel(s), the channel access procedures described in this clause, for accessing the channel(s) on which the transmission(s) are performed by the gNB/UE(s), are applied. *** Unchanged text is omitted *** 4.4.1 Type 1 channel access procedures This clause describes channel access procedures to be performed by a gNB/UE where the time duration spanned by the sensing slots that are sensed to be idle before a transmission(s) is random based on a fixed contention window size. The clause is applicable to any transmission initiating a channel occupancy by the gNB/UE. *** Unchanged text is omitted *** The defer duration is Td=8μs that ends with a sensing slot of a Tsl=5μs for performing at least a single measurement to determine whether the channel is idle. A gNB/UE shall not transmit on a channel for a Channel Occupancy Time that exceeds 5ms. ### 4.4.2 Type 2 channel access procedures This clause describes channel access procedures to be performed by a gNB/UE where the time duration spanned by sensing slots that are sensed to be idle before a DL/UL transmission(s) is deterministic. A gNB/UE may transmit a transmission(s) on a channel immediately after Td that ends within a sensing slot of duration Tsl=5μs for performing at least a single measurement to determine whether the channel is sensed to be idle. *** Unchanged text is omitted ***

Proposal 25: TP#5 should be supported.

# Conclusions

In this contribution, we discussed several aspects related to channel access enhancements for extending NR up to 71 GHz, and we derived the following proposals, and observations:

Proposal 1: For fallback DCI formats 0_0/1_0 and RAR UL grant, for FR2-2 operation, the ChannelAccess-Cpext field indicates one of the entries of a table which entries are “Type 1 channel access in 4.4.1 of 37.213”, “Type 2 channel access in 4.4.2 of 37.213” and “Type 3 channel access in 4.4.3 of 37.213”.

Proposal 2: TP#1 and TP#2 should be supported.

Proposal 3: Support cyclic prefix extension for CG-PUSCH transmissions in the FR2-2 frequency range using the same design principle as NR-U.

- The first starting offset value should be equal to 8us and the granularity among the set of starting offsets should be equal to 5us.

Proposal 4: For single carrier transmission, a device performs LBT over a channel bandwidth, where for the case when the UE is the device performing LBT, then the channel bandwidth should include at least the active UL BWP.

Proposal 5: For LBT for multi-carrier transmission, gNB/UE performs multiple LBT, one for each channel bandwidth separately.

Proposal 6: TP#3 should be supported.

Proposal 7: For a COT with MU-MIMO (SDM) transmission or TDM transmission of beams with beam switching, when independent per-beam LBT is performed at the start of the COT, define Pout as the maximum EIRP of the intended transmissions “covered” by the sensing beam by the node determining EDT during the COT.

Proposal 8: ref-SCS-CP-r16 is extended to include all the supported SCS for FR2-2 (i.e., 120, 480 and 960 KHz).

Proposal 9: For the QCL Type-D of L3-RSSI measurement use the QCL type-D of the latest received PDSCH and the latest monitored CORESET.

Proposal 10: The gNB indicates through a cell-specific RRC parameter in SIB1 whether the short signal exemption should be applied or not.

Proposal 11: The 10% over any observation period of 100ms is applicable to the msg1/msgA transmission from one UE perspective.

Proposal 12: TP#4 should be supported.

Proposal 13: When independent per-beam LBT sensing is performed, a transmission may be allowed to occur as long as the LBT procedure has been successful before a channel occupancy for at least a single beam. However, a transmission (via either spatial or time multiplexing) may not be allowed on those beams for which the LBT procedure was not successful.

Proposal 14: When independent per-beam LBTs are performed to initiate a multi-beam COT with TDMed or SDMed transmission beams, independent counters are maintained per beam.

Proposal 15: When independent per-beam LBTs are performed to initiate a multi-beam COT with TDMed or SDMed transmission beams, support to align transmission starting time across the multiplexed beams such that a transmission on one beam does not start while sensing is ongoing on another beam. In this matter, a device should behave as follows:

- If the backoff counter NBi for a sensing beam Bi reaches zero before the aligned transmission starting time, the device continues to decrement the counter NBi by continuing to sense the channel via sensing slots of 5us each and transmits in the corresponding beam at the aligned start time if the channel continues to be sensed idle in all of the additional sensing slot durations.

- If the backoff counter NBi for a sensing beam Bi does not reach zero before the aligned start time, or reaches zero but the channel has been sensed busy in any of the additional sensing slot durations, the transmission(s) in the corresponding beam is dropped.

Proposal 16: After the gNB/UE ceases transmission in any of the beam for which the channel access procedure was done, the gNB/UE will reinitialize the counter for all beams.

Proposal 17: When time-domain switching across beams within the same COT is supported, the per-beam LBT for different beams is also performed in a sequential manner. In particular, the initiating device may sense on a beam before either transmitting on that beam or switching to a separate beam to perform sensing.

Proposal 18: When independent per-beam LBT sensing is performed, an LBT failure is counted per transmission, and an LBT failure is reported only if all per beam LBTs fail.

Proposal 19: RAN1 should send an LS to RAN2 to inform them about the decision made in terms of how an LBT failure should be counted.

Proposal 20: For FR2-2, multi-carrier channel access procedure is employed through independent counters, one for each carrier.

Proposal 21: Support to align transmission starting time across the carrier such that a transmission on one carrier does not start while sensing is ongoing on another carrier. In this matter, a device should behave as follows:

- If the backoff counter NCi for a carrier Ci reaches zero before the aligned transmission starting time, the device continues to decrement the counter NCi by continuing to sense the channel via sensing slots of 5us each and declares the channel for that carrier to be idle if the channel continues to be sensed idle in all of the additional sensing slot durations.

- If the backoff counter NCi for a carrier Ci does not reach zero before the aligned start time, or reaches zero but the channel has been sensed busy in any of the additional sensing slot durations, channel access procedure in carrier Ci is considered to have failed.

Proposal 22: After the gNB/UE ceases transmission in any of the carrier for which the channel access procedure was done, the gNB/UE will reinitialize the counter for all channels.

Proposal 23: If an initiating device is capable to perform Cat-2 LBT, and if the initiating device performs an additional burst within the initiated COT which may be separated with any prior burst of at least a minimum gap Y, then under Alt-3 a Cat 2 LBT is needed before the initiating device transmission.

Proposal 24: When a UE performing a CG transmission shares its COT with a gNB, the gNB is always allowed to perform both unicast and broadcast transmissions without any constrains, and cg-UCI may always indicate one entry of cg-COT-SharingList-r16.

Proposal 25: TP#5 should be supported.

# References

- RP-193228, “New SID Study on supporting NR above 52.6 GHz”, Intel Corporation, 3GPP TSG RAN Meeting #86, Dec. 2019.

- RP-212637, “Revised WID: Extending current NR operation to 71GHz,” Qualcomm, Intel, 3GPP TSG RAN Meeting #93-e, Dec. 2020.

- RAN1Chairman’s notes, 3GPP TSG RAN WG1 #104 e-Meeting, February 2021.

- RAN1Chairman’s notes, 3GPP TSG RAN WG1 #104b e-Meeting, April 2021.

- RAN1 Chairman’s note, 3GPP TSG RAN WG1 #105 e-Meeting, May, 2021.

- RAN1 Chairman’s note, 3GPP TSG RAN WG1 #106 e-Meeting, August, 2021.

- RAN1 Chairman’s note, 3GPP TSG RAN WG1 #106-bis e-Meeting, October, 2021.

- RAN1 Chairman’s note, 3GPP TSG RAN WG1 #107 e-Meeting, November, 2021.

- RAN1 Chairman’s note, 3GPP TSG RAN WG1 #107b e-Meeting, January, 2022.

- Draft ETSI EN 302 567 V2.2.0, “Multiple-Gigabit/s radio equipment operating in the 60 GHz band; Harmonised Standard for access to radio spectrum,” Dec. 2020.

- ARIB STD-T117, “Low Power Data Communication System / 60 GHz-band Wireless LAN for Very High Throughput Data Communications”, September 2016.