3GPP TSG RAN WG1 #108-e							    R1-2201693

e-Meeting, February 21st – March 3rd, 2022

Source:	Intel Corporation

Title:	Open issues on UE HARQ feedback enhancements

Agenda item:	8.3.1

Document for:	Discussion and Decision

Introduction

In this document, we provide discussion on remaining issues for UE HARQ feedback enhancements. Views on other aspects of Rel.17 URLLC/IIOT are presented in [1]-[3].

HARQ-ACK Retransmission

In the last meeting it was discussed whether/how to handle case when DCI(s) scheduling dropped PUCCH were missed and gNB requests PUCCH retransmission. The most flexible approach of signalling the retransmitted CB size was not agreed:

Conclusion There is no consensus on the support of HARQ-ACK CB size indication in the triggering DCI for HARQ-ACK re-transmission

The remaining open issue in this context is whether to handle the case when a UE received PUCCH retransmission request for a slot/sub-slot for which it did not generate a valid HARQ CB. For both Type 1 and Type 2 CB, it may be the case when no DCI was received. We think it is important and easy to clarify such scenario. The proposal from RAN1#107bis-e is a good starting point:

Proposal 1

- The UE ignores the one-shot triggering DCI for new PUCCH determination if it schedules a re-transmission of HARQ-ACK information from a PUCCH slot/sub-slot on which the UE did not generate a valid HARQ-ACK CB

PUCCH cell switching and PUCCH repetition

It was discussed whether PUCCH repetitions can be mapped to different cells, especially when semi-static pattern is provided. In our view, this is an overoptimization which complicates the system substantially, due to separate PUCCH configurations for different cells, PUCCH repetition deferral procedure, different numerology cases, unavailability of soft-combining of PUCCH when different rate-matching is allowed in different carriers, etc.

Alternatively, the two features may be incompatible at network configuration level, i.e. so that a UE does not expect to be scheduled with repeated PUCCH wherein a PUCCH repetition slot is determined to be in another cell/carrier than the initial PUCCH repetition slot.

Proposal 2

- For PUCCH carrier switching based on dynamic indication and/or semi-static pattern,

  - When the indicated PUCCH resource is associated with > 1 repetitions, the cell for PUCCH repetitions transmission is fixed to the same cell as the initial PUCCH repetition

  - Alt. 1:

    - A PUCCH slot mapped to different PUCCH cell is considered as invalid for PUCCH repetition and the PUCCH repetition is dropped

  - Alt.2:

    - A UE does not expect the cell switching pattern to indicate a different cell for a PUCCH repetition comparing to the initial PUCCH repetition

PUCCH cell switching DCI

In the last meeting it was discussed whether to optimize PUCCH cell switching for scenarios involving unlicensed carrier operation. The following was concluded:

Conclusion There is no consensus for introducing further specification support for the following - PUCCH cell switching between cells with shared spectrum channel access (in any mode) - PUCCH cell switching between a cell with licensed spectrum and a cell with shared spectrum channel access (in any mode)

Although optimized operation is precluded, the procedure for DCI monitoring still needs to be fixed in specification. Since in multiple cases involving shared spectrum channel access, ChannelAccess-CPext field size/presence between carriers may be different, we suggest specifying that a UE does not expect different field size configuration among involved carriers.

Proposal 3

- For PUCCH carrier switching based on dynamic indication and/or semi-static pattern, a UE does not expect the size/presence of ‘ChannelAccess-CPext’ DCI field to vary across PCell / SPCell / PUCCH SCell and sSCell

Furthermore, there is at least one more DCI field from other R17 items which require handling: ‘Second TPC command for scheduled PUCCH’ (controlled by SecondTPCFieldDCI-1-1 and SecondTPCFieldDCI-1-2 RRC parameters). This field presence/size depends on PUCCH configuration and is intended for multi-TRP operation. In general, we don’t see issues allowing R17 urllc/iiot to work on top of R17 multi-TRP design (or vice versa). Therefore, DCI size handling is required for this field.

Proposal 4

- For PUCCH carrier switching based on dynamic indication and/or semi-static pattern,

  - Apply the same handling for ‘Second TPC command for scheduled PUCCH’ field size determination and zero-bit padding as for PRI and PDSCH-to-HARQ_feedback

SPS HARQ-ACK with dynamic PUCCH cell switching

SPS HARQ-ACK bits transmission in the case of PUCCH cell switching by dynamic DCI was discussed. The latest version discussed in the meeting:

Alternative Proposal 5: For dynamic PUCCH cell switching, the HARQ-ACK of SPS PDSCH without associated DCI is to be transmitted on PCell / SPCell / PUCCH-SCell independently of the dynamically indicated PUCCH cell in the SPS activation DCI. Only the HARQ-ACK corresponding to the first SPS PDSCH activated by Activation DCI uses the PUCCH cell based on the indication in the activation DCI (based on the earlier agreement). - The UE does not expect to be dynamically indicated for PUCCH transmission on the PUCCH sSCell in the SPS activation DCI in a slot overlapping with a PCell / SPCell / PUCCH-SCell slot where SPS HARQ-ACK subject to another SPS config would be transmitted.

In general companies discuss the following combinations:

- Initial SPS HARQ-ACK only on PCell and other SPS HARQ-ACK only on PCell

  - This option in general violates the prior agreement that initial SPS HARQ-ACK follows DCI indication, however, it is simple for spec implementation and HARQ CB construction.

- Initial SPS HARQ-ACK on indicated cell and other SPS HARQ-ACK only on PCell

  - This option is aligned with the prior agreement, however the initial HARQ-ACK may be on a different cell. This creates HARQ CB construction issue with initial HARQ-ACK and non-initial HARQ-ACK. There are also proposals to introduce restrictions on overlap of initial SPS HARQ-ACK with other UCI / HARQ-ACK in order to avoid hybrid CB construction issues.

- Initial SPS HARQ-ACK on indicated cell and other SPS HARQ-ACK on indicated cell

  - Due to the conclusion that UCI without DCI could not be multiplexed on a dynamically indicated PUCCH cell, it is preferred that SPS HARQ-ACK is carried on PCell. Otherwise, there could be increased dropping or error cases.

Our preference is to keep SPS HARQ-ACK on PCell and decide how to treat initial SPS HARQ-ACK. We can either restrict the dynamic indication to be only PCell for the initial SPS HARQ-ACK or to restrict multiplexing with other UCI (at least carrying SPS HARQ-ACK) for the initial SPS HARQ-ACK.

Proposal 5

- For dynamic PUCCH cell switching, the HARQ-ACK of SPS PDSCH without associated DCI is to be transmitted on PCell / SPCell / PUCCH-SCell independently of the dynamically indicated PUCCH cell in the SPS activation DCI. Only the HARQ-ACK corresponding to the first SPS PDSCH activated by Activation DCI uses the PUCCH cell based on the indication in the activation DCI

  - Alt. 1 The UE does not expect to be dynamically indicated for PUCCH transmission on the PUCCH sSCell in the SPS activation DCI in a slot overlapping with a PCell / SPCell / PUCCH-SCell slot where SPS HARQ-ACK subject to another SPS config would be transmitted.

  - Alt.2 The UE does not expect to be dynamically indicated for PUCCH transmission on the PUCCH sSCell in the SPS activation DCI

Intra-UE multiplexing and SPS HARQ-ACK deferral

With intra-UE multiplexing, without SPS deferral, UCI multiplexing is first performed within each priority (step 1), and then, multiplexing or prioritization is performed across different priorities (step 2).

When intra-UE multiplexing for different priorities is enabled together with SPS HARQ-ACK deferral, the following options may be considered:

- Check the deferral before resolving overlapped UL channels between different priorities (i.e., deferral is determined after step 1 but before step 2).

In general, it conflicts with the principle that SPS deferral is determined based on the final output of the multiplexing procedure. Moreover, it leads to either transmitting deferred SPS HARQ-ACK in both initial slot and target slot, or increased latency for a SPS HARQ-ACK which can be transmitted after multiplexing onto a UL channel with different priority.

- Check the deferral after resolving overlapped UL channels between different priorities (i.e., deferral is determined after step 2).

(2-1) If the resultant UL channel is a PUCCH resource for SPS PUCCH for priority i, then, defer the SPS HARQ-ACK for priority i. That means, if LP SPS HARQ-ACK is multiplexed with HP SPS HARQ-ACK, and the resultant channel is HP SPS PUCCH resource which is invalid, only deferral of HP SPS HARQ-ACK is allowed, and LP SPS HARQ-ACK is dropped.

(2-2) If the resultant UL channel is a PUCCH resource for SPS PUCCH regardless of priority i, then defer the SPS HARQ-ACK. That means, if LP SPS HARQ-ACK is multiplexed with HP SPS HARQ-ACK, and the resultant channel is HP SPS PUCCH resource which is invalid, both LP and HP SPS HARQ-ACK are deferred

(2-2-a)	Treat the deferred LP and HP SPS HARQ-ACK as a joint HP SPS HARQ-ACK to find the target slot. Therefore, deferred LP SPS HARQ-ACK is always jointly coded with the deferred and initial HP HARQ-ACK in the target slot, while the initial LP SPS HARQ-ACK is separately coded. It would be a bit strange to use different coding chain for deferred and initial LP SPS HARQ-ACK. More importantly, always joint coded deferred LP SPS HARQ-ACK with HP HARQ-ACK would degrade the performance of HP HARQ-ACK.

(2-2-b)	Treat the deferred LP and HP SPS HARQ-ACK separately to find the target slot. Considering the resultant channel for LP and HP transmission in one slot can be different, e.g., the resultant channel for LP and HP is non-overlapped and only one of the channels is invalid, then, the slot is the target slot for the priority with valid channel while another priority may need to find a later slot as a target slot. Comparing with (2-2-a), the performance for HP would be better. Comparing with (2-1), the performance for LP would be better, but this option is more complicated for UE implementation.

Although it was not explicitly discussed/agreed that HARQ-ACK without DCI can follow R17 multiplexing procedures, assuming it is allowed, from the above alternatives, our preference is 2-1.

Proposal 6

- For the case when R17 intra-UE multiplexing is configured together with SPS HARQ-ACK deferral, the deferral is checked after resolving overlapped UL channels between different priorities (i.e., deferral is determined after step 2, if any).

  - If the resultant UL channel is a PUCCH resource for SPS PUCCH for priority i, then, defer the SPS HARQ-ACK for priority i

Intra-UE multiplexing and PUCCH carrier switching

The following conclusion may need to be further clarified:

Conclusion For dynamic PUCCH cell switching, the UE does not expect a PUCCH slot with UCI on PCell /SPCell / PUCCH SCell to overlap with a PUCCH slot with HARQ-ACK on the dynamically indicated alternative PUCCH cell. - The UCI on PCell /SPCell / PUCCH SCell dropped due to collision with semi-static DL symbols, SSB, and symbols indicated by pdcch-ConfigSIB1 in MIB for a CORESET for Type0-PDCCH CSS set is exempted and is not multiplexed on the PUCCH on the alternative PUCCH cell.

- We suggest clarifying that the valid PUCCH resource on Pcell means PUCCH resources before multiplexing on Pcell. This simplifies handling of different cases which arise if the PUCCH resource after multiplexing is considered.

- We suggest clarifying that for different priority UCI, any PUCCH resource before multiplexing/prioritization is considered. Similarly, to the first issue, in this case there are multiple additional considerations when PUCCH resource after multiplexing/prioritization is considered, e.g., multiplexing of LP HARQ-ACK with HP HARQ-ACK.

Proposal 7

- For the conclusion that “For dynamic PUCCH cell switching, the UE does not expect a PUCCH slot with UCI on PCell /SPCell / PUCCH SCell to overlap with a PUCCH slot with HARQ-ACK on the dynamically indicated alternative PUCCH cell”,

  - Clarify that the valid PUCCH resource on Pcell means PUCCH resources before multiplexing on Pcell

  - Clarify that for different priority UCI, any PUCCH resource before multiplexing/prioritization is considered

Intra-UE multiplexing and HARQ feedback retransmission

Enhanced Type 3 CB

For the enhanced Type 3 CB, when Release 17 intra-UE multiplexing is enabled, the constructed CB (of LP or HP) may be involved into prioritization with other UCI or PUSCH. Since the priority indicated in the triggering DCI is associated with the PUCCH for Type 3 CB, the procedures being finalized in Release 17 works well for this CB.

Overall, for eType3 CB with intra-UE multiplexing/prioritization the desired behaviour would be that HP HARQ-ACK or HP eType3 CB can be prioritized over LP eType3 CB or LP HARQ-ACK respectively.

The following cases can be considered:

- LP HARQ-ACK overlaps with LP eType3 CB

  - This case is already being handled by agreements, since it is an equal priority case

- HP HARQ-ACK overlaps with HP eType3 CB

  - This case is already being handled by agreements, since it is an equal priority case

- LP HARQ-ACK overlaps with HP eType3 CB

  - In this situation, LP HARQ-ACK may be dropped (R16 prioritization) or multiplexed with HP eType3 CB (R17 multiplexing). Current agreement says that UE does not expect that eType3 CB does not contain a HARQ process for a bit overlapping in the same PUCCH resource. This may need to be clarified for the case of different priorities, so that LP dropping/multiplexing is allowed.

- HP HARQ-ACK overlaps with LP eType 3 CB

  - In this situation, LP eType 3 CB may be dropped (R16 prioritization) or multiplexed with HP HARQ-ACK (R17 multiplexing).

To cover the above cases, the following is proposed:

Proposal 8

- For phy prioritization between LP/HP PUCCH carrying (e)Type3 CB and HP/LP PUCCH carrying HARQ-ACK using Release 16 dropping

  - UE may expect eType3 CB to not contain a HARQ process for a bit overlapping with the same PUCCH resource as the eType3 CB

  - LP PUCCH is dropped according to Release 16 procedures

- For phy prioritization between LP/HP PUCCH carrying (e)Type3 CB and HP/LP PUCCH carrying HARQ-ACK using Release 17 multiplexing, follow the agreed behavior

One-shot retransmission

For one-shot retransmission, the main issue would be in how/whether to trigger retransmission of a PUCCH which carries R17 multiplexed LP+HP codebooks. According to agreements, only single triggering DCI is allowed for a PUCCH. This may need to be lifted for R17 multiplexing case. Some options are below:

- Alt.1 Allow retransmission of a multiplexed LP+HP codebook by a single triggering DCI. In this case, the priority in the triggering DCI may need to be re-interpreted or ignored, since it does not relate to the LP+HP priority of the retransmitted CB.

- Alt.2 Allow retransmission only of a given priority j part of LP+HP PUCCH, which is indicated by the triggering DCI priority field j. It would allow independent retransmission of different priority parts of the multiplexed PUCCH.

Our preference is Alt.2, since it can respect current procedures for the same priority as much as possible.

Proposal 9

- For one-shot triggering of a PUCCH which is a mix of LP and HP HARQ-ACK according to R17 multiplexing, the one-shot trigger only instructs to retransmit HARQ-ACK of priority j which is indicated in the triggering DCI

When a CB is retransmitted, it is appended to the end of another ‘new’ Type 1 or Type 2 CB. With R17 intra-UE multiplexing / prioritization, if the priorities are different, then the multiplexing/prioritization procedures can be reused.

Conclusions

In this contribution the UE HARQ feedback enhancements targeting URLLC/IIOT scenarios in Release 17 have been presented. Based on analysis and discussion, the following proposals have been made:

Proposal 1

- The UE ignores the one-shot triggering DCI for new PUCCH determination if it schedules a re-transmission of HARQ-ACK information from a PUCCH slot/sub-slot on which the UE did not generate a valid HARQ-ACK CB

Proposal 2

- For PUCCH carrier switching based on dynamic indication and/or semi-static pattern,

  - When the indicated PUCCH resource is associated with > 1 repetitions, the cell for PUCCH repetitions transmission is fixed to the same cell as the initial PUCCH repetition

  - Alt. 1:

    - A PUCCH slot mapped to different PUCCH cell is considered as invalid for PUCCH repetition and the PUCCH repetition is dropped

  - Alt.2:

    - A UE does not expect the cell switching pattern to indicate a different cell for a PUCCH repetition comparing to the initial PUCCH repetition

Proposal 3

- For PUCCH carrier switching based on dynamic indication and/or semi-static pattern, a UE does not expect the size/presence of ‘ChannelAccess-CPext’ DCI field to vary across PCell / SPCell / PUCCH SCell and sSCell

Proposal 4

- For PUCCH carrier switching based on dynamic indication and/or semi-static pattern,

  - Apply the same handling for ‘Second TPC command for scheduled PUCCH’ field size determination and zero-bit padding as for PRI and PDSCH-to-HARQ_feedback

Proposal 5

- For dynamic PUCCH cell switching, the HARQ-ACK of SPS PDSCH without associated DCI is to be transmitted on PCell / SPCell / PUCCH-SCell independently of the dynamically indicated PUCCH cell in the SPS activation DCI. Only the HARQ-ACK corresponding to the first SPS PDSCH activated by Activation DCI uses the PUCCH cell based on the indication in the activation DCI

  - Alt. 1 The UE does not expect to be dynamically indicated for PUCCH transmission on the PUCCH sSCell in the SPS activation DCI in a slot overlapping with a PCell / SPCell / PUCCH-SCell slot where SPS HARQ-ACK subject to another SPS config would be transmitted.

  - Alt.2 The UE does not expect to be dynamically indicated for PUCCH transmission on the PUCCH sSCell in the SPS activation DCI

Proposal 6

- For the case when R17 intra-UE multiplexing is configured together with SPS HARQ-ACK deferral, the deferral is checked after resolving overlapped UL channels between different priorities (i.e., deferral is determined after step 2, if any).

  - If the resultant UL channel is a PUCCH resource for SPS PUCCH for priority i, then, defer the SPS HARQ-ACK for priority i

Proposal 7

- For the conclusion that “For dynamic PUCCH cell switching, the UE does not expect a PUCCH slot with UCI on PCell /SPCell / PUCCH SCell to overlap with a PUCCH slot with HARQ-ACK on the dynamically indicated alternative PUCCH cell”,

  - Clarify that the valid PUCCH resource on Pcell means PUCCH resources before multiplexing on Pcell

  - Clarify that for different priority UCI, any PUCCH resource before multiplexing/prioritization is considered

Proposal 8

- For phy prioritization between LP/HP PUCCH carrying (e)Type3 CB and HP/LP PUCCH carrying HARQ-ACK using Release 16 dropping

  - UE may expect eType3 CB to not contain a HARQ process for a bit overlapping with the same PUCCH resource as the eType3 CB

  - LP PUCCH is dropped according to Release 16 procedures

- For phy prioritization between LP/HP PUCCH carrying (e)Type3 CB and HP/LP PUCCH carrying HARQ-ACK using Release 17 multiplexing, follow the agreed behavior

Proposal 9

- For one-shot triggering of a PUCCH which is a mix of LP and HP HARQ-ACK according to R17 multiplexing, the one-shot trigger only instructs to retransmit HARQ-ACK of priority j which is indicated in the triggering DCI

- References

- R1-2201694, “Remaining Clarifications to Enable URLLC/IIoT in Unlicensed Band”, Intel Corporation, February-March 2022

- R1-2201695, “Remaining Open Details of Intra-UE Uplink Channel Multiplexing and Prioritization”, Intel Corporation, February-March 2022

- R1-2201696, “Open issues for RTT-based propagation delay compensation”, Intel Corporation, February-March 2022