3GPP TSG RAN WG1 #108-e                                                        R1-2201701

e-Meeting, February 21st – March 3rd, 2022

Source: 	Intel Corporation

Title: 	Maintenance of Rel.17 NR Positioning Solutions for RRC_INACTIVE UEs

Agenda item:	8.5.6

Document for: 	Discussion and Decision

# Introduction

In this contribution, we discuss remaining open details of NR positioning for UEs in RRC_INACTIVE state that were identified during the Rel.17 maintenance work.

PRS processing window for RRC_INACTIVE state

RAN1 has received LS from RAN4 WG [1] with the following question:

Based on previous RAN1 LS, RAN1 has introduced a PRS processing window to support PRS measurements outside measurement gaps. But when RAN4 is working on the PRS measurement requirements in RRC_INACTIVE state, it is unclear whether the PRS processing window can be applied in RRC_INACTIVE state, which may have impact on the measurement requirements. So RAN4 would like to ask RAN1 for the clarification on the following question: Q1: Whether the PRS processing window defined for PRS measurements outside measurement gaps can be also applied for PRS measurements in RRC_INACTIVE state? RAN4 kindly asks RAN1 to provide feedback on the question above.

The main motivation behind support of the PRS processing window is the support of the UE positioning measurements outside measurement gaps for the sake of latency reduction of RRC_CONNECTED UEs. The positioning latency was not optimized for UEs in RRC_INACTIVE state. The PRS processing window design implies that UE can be configured in different state in terms of DL PRS processing priority.

In addition, RAN1 made an agreement that reception of DL PRS has lower priority than other DL signals / channels [2]:

From RAN1 perspective, in RRC_INACTIVE state, reception of DL PRS has lower priority than other DL signals/channels (SSB, SIB1, CORESET0, MSG2/MSGB, paging, DL SDT)

Based on discussion above, we do not see strong motivation/benefits to support PRS processing window for UEs in RRC_INACTIVE state and have the following proposal:

  - Send reply to LS from RAN4 WG (cc to RAN2) and clarify that

    - PRS processing window is not supported for PRS measurements in RRC_INACTIVE state

Below, we provide content for the draft reply to LS from RAN4 based on proposal above:

RAN1 has discussed the LS from RAN4 on applicability of PRS processing window for DL PRS measurements in RRC_INACTIVE state and concluded that PRS processing window is not supported for PRS measurements in RRC_INACTIVE state RAN1 asks RAN4 to take the above conclusion into account in the Rel.17 work on NR positioning enhancements.

Conclusion

In this contribution, we provided our inputs for Rel.17 maintenance work on NR positioning enhancements for UEs in RRC_INACTIVE state. In summary we have the following proposals:

Proposal #1:

  - Send reply to LS from RAN4 WG (cc to RAN2) and clarify that

    - PRS processing window is not supported for PRS measurements in RRC_INACTIVE state

References

- R1-2200903, LS on the applicability of PRS processing window in RRC_INACTIVE state, RAN WG4, January 2022

- R1-2110644, LS on DL PRS reception priority by RRC_INACTIVE UEs, RAN WG1