3GPP TSG RAN WG1 Meeting #108-e			                       R1-2201705

e-Meeting, February 21st – March 3rd, 2022

Source:	Intel Corporation

Title:	Discussion on the remaining aspects of paging enhancements

Agenda item:	8.7.1.1

Document for:	Discussion and Decision

Introduction

In RAN1# 107bis-e[1], the following was agreed with regards to paging early indication design

Agreement A new PEI-RNTI is supported for DCI format 2_7. The following text proposals are adopted: TS 38.213 section 10.1: <Unchanged text is omitted> - a Type2A-PDCCH CSS set configured by peiSearchSpace in DownlinkConfigCommonSIB for a DCI format 2_7 with CRC scrambled by a PEI-RNTI on the primary cell of the MCG <Unchanged text is omitted>. TS 38.213 section 10.4B: <Unchanged text is omitted> A UE in RRC_IDLE state or RRC_INACTIVE state can be provided by TRS-ResourceSetConfig a set of TRS occasions [6, TS 38.214]. If TRS-ResourceSetConfig is provided, a DCI format 2_7 with CRC scrambled by PEI-RNTI or a DCI format 1_0 with CRC scrambled by P-RNTI includes a TRS availability indication field [4, TS 38.212] that provides a bitmap to groups of TRS resource sets where the configuration of each TRS resource set includes an association to a bit of the bitmap. <Unchanged text is omitted>. Agreement For whether and how to accommodate PEI-O location determination for the case POnumPerPEI is smaller than Ns, decide one of the following alternatives - Alt-2: It is supported, and UE applies the single value in PEI-F_offset for the frame-level offset and the -th value out of configured values in firstPDCCH-MonitoringOccasionOfPEI-O for the symbol-level offset Note: The number of PO mapping to one PEI should be multiple of Ns when POnumPerPEI is larger than Ns Agreement For PEI-O location determination: - Range of PEI-F_offset, in unit of frame, is - Range of firstPDCCH-MonitoringOccasionOfPEI-O, in unit of symbol, is decided as one of the following alternatives: - Alt-1: The same value range as firstPDCCH-MonitoringOccasionOfPO Agreement For the value PEI-RNTI, decide one of the following: - Alt-1: PEI-RNTI is of fixed value, and value design is up to RAN2. Conclusion For a UE supporting R17 paging enhancement feature, it is up to UE implementation whether the UE monitors the MO(s) for a PEI. - If UE decides to not to monitor PEI, it has to monitor UE’s PO as defined in 38.304. - Note: No specification impact Agreement - The PDCCH monitoring occasions defined by peiSearchSpace colliding with UL symbols are omitted from the determination of the PEI monitoring occasions. The PDCCH monitoring occasions for PEI which do not overlap with UL symbols (determined according to tdd-UL-DL-ConfigurationCommon) are sequentially numbered from zero starting from the first PDCCH monitoring occasion for PEI in the PEI-O. Agreement The transmission power of PEI is based on the same determination method as paging PDCCH, and the following TP is adopted. .............................................................. Text Proposal for 38.213.............................................................................. 4.1 Cell search Cell search is the procedure for a UE to acquire time and frequency synchronization with a cell and to detect the physical layer Cell ID of the cell. A UE receives the following synchronization signals (SS) in order to perform cell search: the primary synchronization signal (PSS) and secondary synchronization signal (SSS) as defined in [4, TS 38.211]. A UE assumes that reception occasions of a physical broadcast channel (PBCH), PSS, and SSS are in consecutive symbols, as defined in [4, TS 38.211], and form a SS/PBCH block. The UE assumes that SSS, PBCH DM-RS, and PBCH data have same EPRE. The UE may assume that the ratio of PSS EPRE to SSS EPRE in a SS/PBCH block is either 0 dB or 3 dB. If the UE has not been provided dedicated higher layer parameters, the UE may assume that the ratio of PDCCH DMRS EPRE to SSS EPRE is within -8 dB and 8 dB when the UE monitors PDCCHs for a DCI format 1_0 with CRC scrambled by SI-RNTI, P-RNTI, or RA-RNTI, and DCI format 2_7 with CRC scrambled by PEI-RNTI. ............................................................... Text Proposal for 38.213................................................................................... Agreement UE expects at most one PEI outcome from each PDCCH monitoring occasion of PEI-O, and the following text proposal is adopted. Section 10.1 of TS 38. 213 *** Unchanged text is omitted *** If a UE is provided - one or more search space sets by corresponding one or more ofsearchSpaceZero, searchSpaceSIB1, searchSpaceOtherSystemInformation,pagingSearchSpace, peiSearchSpace, ra-SearchSpace, or a CSS set byPDCCH-Config, and - a SI-RNTI, a P-RNTI, a PEI-RNTI, a RA-RNTI, a MsgB-RNTI, a SFI-RNTI, an INT-RNTI, a TPC-PUSCH-RNTI, a TPC-PUCCH-RNTI, or a TPC-SRS-RNTI then, for a RNTI from any of these RNTIs, the UE does not expect to process information from more than one DCI format with CRC scrambled with the RNTI per slot. *** Unchanged text is omitted ***. Agreement If one PEI-O is associated with POs of 2 PFs, - The two PFs are consecutive PFs configured in SIB - FFS: two PFs are consecutive PFs within the same paging cycle - Note: As an example, SFN of the first PF of the PF(s) associated with the PEI-O can be obtained by: (SFN of UE’s PF) - Agreement - LS to RAN2 for sharing agreements with potential RAN2 specification impacts - Draft LS in R1-2200788 Conclusion - For Rel-17, inclusion of SI change indication or ETWS/CMAS notification in DCI format 2_7 is not supported. Conclusion For Rel-17 DCI format 2_7, - Explicit starting position configuration for paging indication field or TRS availability indication field is not supported. - Paging indication field starts from the first bit, and TRS availability indication field starts from the next bit after the end of the paging indication field - UE does not expect the total size of paging indication filed and TRS availability indication field is larger than the DCI payload size Working assumption - SearchSpaceId = 0 can be configured forpeiSearchSpace for the case of CORESET multiplexing pattern 2 or 3 Agreement - Draft LS R1-2200788 is endorsed in principle - Include above working assumption in the LS on paging enhancement to RAN2 Agreement - Final LS R1-2200800 is endorsed.

In this contribution, we express our views on the remaining aspects of PDCCH base PEI design.

# Remaining aspects of design of paging early indication

It has been discussed during RAN1-107bis-e that currently sub-group index indication bit is undefined in the PEI for the UE that does not support UE sub grouping. In other words, supporting such behavior would require new UE behavior and new design for PEI, such as adding another bit. In our view, support of PEI and UE sub-grouping can be joint and use case/motivation of UE supporting one but not the other is not clear. Hence, we suggest to not spend time during maintenance phase for adding non-essential features in the PEI.

Proposal 1: RAN1 does not support revision of PEI design to indicate paging early indication to a UE not supporting UE sub-grouping.

Another proposal was discussed during last meeting regarding one PEI-O being associated with POs of 2 PFs, where the two PFs are consecutive PFs configured in SIB. It is still open whether the 2 PFs are part of same paging cycle. Current design does not preclude the configuration that one PEI is mapped to two PFs of different paging cycles. We think the common use case is one PEI addressing PFs within a same paging cycle. And it also keeps the design simple.

Proposal 2: If two consecutive PFs are associated with a PEI, the two PFs belong to the same paging cycle.

# Conclusions

In summary, we have the following list of proposals:

Proposal 1: RAN1 does not support revision of PEI design to indicate paging early indication to a UE not supporting UE sub-grouping.

Proposal 2: If two consecutive PFs are associated with a PEI, the two PFs belong to the same paging cycle.

References

- 3GPP RAN1# 107bis-e Chairman Notes