3GPP TSG RAN WG1 #108-e	R1-2201714

e-Meeting, February 21st – March 3rd, 2022

Agenda item:	8.10.2

Source: 	Intel Corporation

Title: 	Other Enhancements for Simultaneous Operation of IAB

Document for:	Discussion and decision

# Introduction

In this contribution, based on previous discussion and agreements, we further elaborate on timing alignment and power control to support IAB-node’s simultaneous operation.

# Support for Case#7 Timing for Simultaneous MT-RX/DU-RX

RAN1#107-e has made the following agreements for Case#7 timing.

- The dynamic range of the MAC CE case #7 timing offset indication is 12 bits.

  - FFS the numerical values of the endpoints of the range

Regarding the numerical values of Case#7 timing offset range endpoints, in Figure 2.1, we show an IAB-node’s timing relationship (slot-level alignment) with parent node operating in Case#7 timing:

- TAcase1: The original Case#1 TA at IAB-MT to make parent node operating at Case#1 timing mode

- TAcase7: The absolute Case#7 TA at IAB-MT to make parent node operating at Case#7 timing mode

- Tp0: parent-MT RX propagation delay

- Tp: IAB-MT RX propagation delay

The shifting offset from Case#1 TA to absolute Case#7 TA is defined as:

TAoffset, Case7=TACase1-TACase7                                                           (1)

We derive TAoffset, Case7 value for slot-level alignment (which can be considered as the maximum offset value comparing with symbol-level alignment) with the following steps.

        - IAB-MT RX timing is Tp behind DL TX timing

IAB-MT TX Case#1 timing is TACase1 ahead of IAB-MT RX timing

        - IAB-MT TX Case#1 timing is TACase1-Tp ahead of DL TX timing

        - Parent-DU RX Case#1 timing is TACase1-2Tp ahead of DL TX timing

        - Parent-MT RX timing is Tp0 behind DL TX timing

Parent-DU RX Case#7 timing is the same as parent-MT RX timing

        - Parent-DU RX Case#7 timing is Tp0 behind DL TX timing

        - Parent-DU RX Case#1 timing is TACase1-2Tp ahead of DL TX timing (from Step1)

Parent-DU RX Case#7 timing is Tp0 behind DL TX timing (from Step 2)

        - Parent-DU RX Case#1 timing is TACase1-2Tp+Tp0 ahead of Parent-DU RX Case#7 timing

        - IAB-MT TX Case#1 timing is TACase1-2Tp+Tp0 ahead of IAB-MT TX Case#7 timing

        - TAoffset, Case7=TACase1-2Tp+Tp0

        - TAcase1=2Tp+Tg                               from Rel-16 IAB discussion [1] as in Figure 2.2

Tdelta=-Tg2                                          from Rel-16 IAB discussion [1]

TAoffset, Case7=TACase1-2Tp+Tp0      from Step 3

        - TAoffset, Case7=Tp0+Tg=Tp0-2Tdelta                                                                     (2)

Figure 2.1: IAB-node timing relationship details with parent node operating at Case#7 timing

Figure 2.2: TAcase1/Tp/Tg relationship for Case#1 timing [1]

According to the derivation procedure, Case#7 timing offset numerical range endpoints can be designed according to TAoffset, Case7=Tp0-2Tdelta, e.g., the range of   TAoffset, Case7 can refer to [0, max⁡(Tp)-2×min⁡(Tdelta)]. Note that according to  TAoffset, Case7 definition in Eq.(1),  TACase7=TACase1-TAoffset, Case7  and  TAoffset, Case7 is always a positive value. This definition can interpret RAN1#106 Case#7 timing agreement as: for Case#7 timing at a parent node, the IAB-MT TX timing of the node is obtained via the legacy TA loop plus (e.g., and) an offset from the parent node.

Another definition of TAoffset, Case7 can be changed from Eq.(1) to Eq.(3) as follows:

TAoffset, Case7=TACase7-TACase1                                                           (3)

Which makes  TACase7=TACase1+TAoffset, Case7  and   TAoffset, Case7 is always a negative value. With similar the derivation procedure, we can have

TAoffset, Case7=-Tp0-Tg=2Tdelta-Tp0                                                               (4)

And Case#7 timing offset numerical range endpoints can be designed according to TAoffset, Case7=2Tdelta-Tp0, e.g., the range of   TAoffset, Case7 can refer to [2×minTdelta-max⁡(Tp), 0].

Therefore, RAN#1 needs to decide between two  TAoffset, Case7 definitions and numerical value range endpoints.

- Alt. 1: TAoffset, Case7=TACase1-TACase7 and TAoffset, Case7 is always a positive value.

The numerical range endpoints can be designed according to TAoffset, Case7=Tp0-2Tdelta, e.g., the range of   TAoffset, Case7 can refer to [0, max⁡(Tp)-2×min⁡(Tdelta)].

- Alt. 2:  TAoffset, Case7=TACase7-TACase1 and TAoffset, Case7 is always a negative value.

The numerical range endpoints can be designed according to TAoffset, Case7=2Tdelta-Tp0, e.g., the range of   TAoffset, Case7 can refer to [2×minTdelta-max⁡(Tp), 0].

Proposal 1: RAN#1 needs to decide between two  TAoffset, Case7 definitions and numerical value range endpoints.

- Alt. 1: TAoffset, Case7=TACase1-TACase7 and TAoffset, Case7 is always a positive value.

The numerical range endpoints can be designed according to TAoffset, Case7=Tp0-2Tdelta, e.g., the range of   TAoffset, Case7 can refer to [0, max⁡(Tp)-2×min⁡(Tdelta)].

- Alt. 2:  TAoffset, Case7=TACase7-TACase1 and TAoffset, Case7 is always a negative value.

The numerical range endpoints can be designed according to TAoffset, Case7=2Tdelta-Tp0, e.g., the range of   TAoffset, Case7 can refer to [2×minTdelta-max⁡(Tp), 0].

# Regarding LS on Power Control Parameters

Regarding the incoming LS from RAN#4 [2], besides replying RAN#1 on the desired DL TX power adjustment range, RAN#4 also raised a question about whether basic PSD difference is needed in addition to desired DL TX power adjustment range. There are RAN#4 requirements for an IAB-DU receiver by which the RX PSD difference between two FDM signals can be derived. Such RX PSD difference range could be regarded as basic PSD different can be tolerated by child IAB receiver and thus the desired DL TX power adjustment could be derived to avoid higher PSD difference to be tolerated by child IAB receiver compared to the basic PSD difference.

In RAN#1 perspective, desired DL TX power adjustment request from child-MT will implicitly include information regarding the basic PSD difference that can be tolerated by child IAB receiver. Hence, no need to additionally transmit basic PSD difference information.

Proposal 2: No need to transmit basic PSD difference information in addition to desired DL TX power adjustment range.

# Conclusion

In this contribution, we discussed mechanisms of timing alignment and power control to support IAB-node’s simultaneous operation.  It is summarized by the following proposals.

Proposal 1: RAN#1 needs to decide between two  TAoffset, Case7 definitions and numerical value range endpoints.

- Alt. 1: TAoffset, Case7=TACase1-TACase7 and TAoffset, Case7 is always a positive value.

The numerical range endpoints can be designed according to TAoffset, Case7=Tp0-2Tdelta, e.g., the range of   TAoffset, Case7 can refer to [0, max⁡(Tp)-2×min⁡(Tdelta)].

- Alt. 2:  TAoffset, Case7=TACase7-TACase1 and TAoffset, Case7 is always a negative value.

The numerical range endpoints can be designed according to TAoffset, Case7=2Tdelta-Tp0, e.g., the range of   TAoffset, Case7 can refer to [2×minTdelta-max⁡(Tp), 0].

Proposal 2: No need to transmit basic PSD difference information in addition to desired DL TX power adjustment range.

# References

[1] R1-1905842, “LS on clarification of OTA timing alignment for IAB”, Xi’an, China, April 2019.

[2] R1-2200906, “LS on range of power control parameters for eIAB”, RAN1#108e, Feb. 2022.