3GPP TSG RAN WG1 Meeting #108-e													         R1-2201725

e-Meeting, February 21 – March 03, 2022

Source:	Intel Corporation

Title:	UE features for DSS

Agenda item:	8.16.13

Document for:	Discussion/Decision

# Introduction

In this contribution, we discuss aspects related to UE capability signaling for NR DSS & NR DC.

# Discussion

## NR_DSS

The updated FG 34-1/34-2 for NR DSS were agreed for further discussion in RAN1#107-e [1], which are provided in Annex A. We provide our further views based on the last agreements made in the end of RAN1#107-e.

For FG 34-2 for Type B UE,

- The item 7) can be included as one aspect of 34-2.

- The item 8), 9) may be defined as separate FGs, since the basic feature CCS from sSCell to P(S)Cell works without support of sSCell dormancy/deactivation.

- The item 10) is not necessary since the search space configurations monitored on sSCell for cross-carrier scheduling to P(S)Cell may be in a span that is not the first 3 OFDM symbols of a P(S)Cell slot

- Regarding item 12), we prefer to differnet a separate FG for unaligned CA, since the basic feature CCS from sSCell to P(S)Cell works without support of unaligned CA.

- It is not clear why item 13) is necessary

For FG 34-1 for Type A UE,

- The item 2) could be included since it is aligned with agreed behavior.

- The item 3) can be replaced by ‘Configuration of scaling factor α for BD and CCE limit handling and PDCCH overbooking handling on P(S)Cell’ to align with 34-2, since it was agreed that BD/CCE limits for Type B UEs are applicable for Type A UEs.

- The item 7) can be included as one aspect of 34-1.

- The item 8) & 9) can be updated to reflect the following agreement in RAN1#107-e

Agreement - Following approaches for PDCCH monitoring and BD limit handling is supported for Type A UE - Additional simplifications to PDCCH monitoring - Type A UE as per RAN1#105-e agreement and - no simultaneous monitoring between ‘USS sets (for P(S)Cell scheduling) on sSCell’ and ‘Type 0/0A/1/2/CSS sets on P(S)Cell for DCI formats with CRC scrambled by C-RNTI/MCS-C-RNTI/CS-RNTI’ - simultaneous monitoring of ‘USS sets (for P(S)Cell scheduling) on sSCell’ and ‘Type 0/0A/1/2/CSS sets on P(S)Cell for DCI formats with CRC not scrambled by C-RNTI/MCS-C-RNTI/CS-RNTI’

- The item 10), 11) may be defined as separate FGs, since the basic feature CCS from sSCell to P(S)Cell works without support of sSCell dormancy/deactivation. It can be merged with the one for Type B UE.

- The item 12) is not necessary since the search space configurations monitored on sSCell for cross-carrier scheduling to P(S)Cell may be in a span that is not the first 3 OFDM symbols of a P(S)Cell slot

- Regarding item 14), it was agreed in last meeting unaligned CA can be supported for CCS from sSCell to P(S)Cell. We prefer to differnet a separate FG for unaligned CA. It can be merged with the one for Type B UE.

- It is not clear why item 15) is necessary

Proposal 1: For FG 34-2, it is proposed that

- The item 7) can be included as one aspect of 34-2.

- The item 8), 9), 12) may be defined as separate FGs

- The item 10), 13) are not necessary

Proposal 2: For FG 34-1, it is proposed that

- The item 2), 7) can be included

- The item 3) can be replaced by ‘Configuration of scaling factor α for BD and CCE limit handling and PDCCH overbooking handling on P(S)Cell’ to align with 34-2

- The item 8) & 9) can be updated to reflect the following agreement in RAN1#107-e

- The item 10), 11), 14) may be defined as separate FGs.

- The item 12), 15) are not necessary

# Conclusions

In this contribution, we discussed issues related to UE capability signaling for DSS and NR DC. The following is a summary of proposals:

Proposal 1: For FG 34-2, it is proposed that

- The item 7) can be included as one aspect of 34-2.

- The item 8), 9), 12) may be defined as separate FGs

- The item 10), 13) are not necessary

Proposal 2: For FG 34-1, it is proposed that

- The item 2), 7) can be included

- The item 3) can be replaced by ‘Configuration of scaling factor α for BD and CCE limit handling and PDCCH overbooking handling on P(S)Cell’ to align with 34-2

- The item 8) & 9) can be updated to reflect the following agreement in RAN1#107-e

- The item 10), 11), 14) may be defined as separate FGs.

- The item 12), 15) are not necessary

# References

- R1-2111812, “Summary of UE features for DSS” Moderator (AT&T)

- R1-2109636, “On efficient activation/de-activation for SCells” Intel Corporation

- R1-2109637, “On efficient activation/de-activation for SCells” Intel Corporation

# ANNEX A: UE features on NR_DSS [1]

| 34. NR_DSS | 34-1 | Cross-carrier scheduling from SCell to PCell/PSCell [with search space restrictions] (Type A) | Support of Cross-carrier scheduling from sSCell to PCell/PSCell [with search space restrictions] (Type A) - Cross-carrier scheduling from sSCell to PCell/PSCell with CIF - FFS: sSCell USS set(s) (for CCS from sSCell to PCell/PSCell) and at least following search space sets on PCell/PSCell can only be configured such that UE does not monitor them in same [slot/symbol] of PCell/PSCell and sSCell - USS sets for DCI formats 0_1,1_1,0_2,1_2 (if supported) - USS sets for DCI formats 0_0,1_0 - Type3-CSS set(s) for DCI formats 1_0/0_0 with C-RNTI/CS-RNTI/MCS-C-RNTI - FFS: BD limit handling and any configuration of associated parameters and UE reporting of any associated parameters - FFS: #unicast DCI limits for PCell/PSCell scheduling - Processing one unicast DCI scheduling DL on PCell/PSCell per PCell/PSCell slot and its aligned N consecutive sSCell slot(s) - Processing one unicast DCI scheduling UL on PCell/PSCell per PCell/PSCell slot and its aligned N consecutive sSCell slot(s) - N={1, 2, 4} for {15kHz, 30kHz, 60kHz} SCS for sSCell N is based on pair of (PCell/PSCell SCS, sSCell SCS): N=1 for(15,15), (30,30), (60,60) and N=2 for (15,30), (30,60) and N=4 for (15, 60) - FFS: PDCCH overbooking on sSCell USS set(s) is not allowed - FFS: Same numerology or different numerologies between sSCell and P(S)Cell or sSCell SCS is larger than P(S)Cell SCS - FFS: USS set(s) for DCI format 0_1,1_1,0_2,1_2 (if supported) configured on sSCell for CCS from sSCell to Pcell/PSCell - FFS: sSCell USS set(s) (for CCS from sSCell to Pcell/PSCell) and Type0/0A/1/2 CSS sets on Pcell/PSCell can be configured so that the UE monitors them in overlapping [slot/symbol] of Pcell/PSCell and sSCell. FFS overlap handling - FFS: Support of monitoring DCI formats 0_1,1_1,0_2,1_2 on PCell/PSCell USS set(s) - FFS: Support of sSCell deactivation/activation when sSCell cross carrier scheduling to PCell/PSCell is configured - FFS: Support of sSCell dormancy when sSCell cross carrier scheduling to PCell/PSCell is configured - FFS: PDCCH monitoring occasion(s) is within the first 3 OFDM symbols of a PCell/PSCell slot - FFS: Numbers of CORESET configurations and search space sets on sSCell (for PCell/PSCell cross-carrier scheduling) per BWP are 1 and 3, respectively - FFS: frame boundary alignment between PCell/PSCell and sSCell - FFS: Precoder granularity of REG-bundle CORESET size when CCS from sSCell to PCell/PSCell is configured - FFS: Whether the PCell/PSCell SCS other than 15kHz can be allowed | FFS 6-5 | Yes |  |  | FFS Per BC | No | FFS Applicable to FR1 only |  | FFS: UE should be able to indicate following two as part of this per-BC capability [Candidate value set 1: One or more of supported SCS combinations ({P(S)Cell SCS in kHz, sSCell SCS in kHz}) from following set are indicated by the UE: {15,15}, {15,30}, (15, 60) for N=4, {30,30}, {30,60},{60,60}) (1) subcarrier spacing for sSCell and/or subcarrier spacing for Pcell, (2) Candidate value set 2: frequency band pair(s) for {PCell/PSCell, sSCell}] Candidate value set for component 6: {Scheduling CC of higher SCS and scheduled CC of lower SCS, same SCS for scheduling CC and scheduled CC, both} Note: The CCS from sSCell to PCell is applicable to FR1 only but there can be other SCells in FR2 configured for the UE. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 34. NR_DSS | 34-2 | Cross-carrier scheduling from SCell to Pcell/PSCell (Type B) | [Support of Cross-carrier scheduling (CCS) from sSCell to PCell/PSCell (Type B)] - Cross-carrier scheduling from sSCell to PCell/PSCell with CIF - sSCell USS set(s) (for CCS from sSCell to PCell/PSCell) and search space sets on PCell/PSCell can be configured so that the UE monitors them in overlapping [slot/symbol] of PCell/PSCell and sSCell - Configuration of scaling factor[s] α [and α_cce] for BD and CCE limit handling and PDCCH overbooking handling on P(S)Cell FFS: BD limit handling between [Option A/C] or [Option C] and any configuration of associated parameters and UE reporting of any associated parameters - FFS: #unicast DCI limits for PCell/PSCell scheduling - Processing one unicast DCI scheduling DL on PCell/PSCell per PCell/PSCell slot and its aligned N consecutive sSCell slot(s) - Processing one unicast DCI scheduling UL on PCell/PSCell per PCell/PSCell slot and its aligned N consecutive sSCell slot(s) - N is based on pair of (PCell/PSCell SCS, sSCell SCS): N=1 for(15,15), (30,30), (60,60) and N=2 for (15,30), (30,60) and N=4 for (15, 60) - FFS: PDCCH overbooking on sSCell USS set(s) is not allowed - FFS: Same numerology or different numerologies between sSCell and P(S)Cell or sSCell SCS is larger than P(S)Cell SCS - FFS: USS set(s) for DCI format 0_1,1_1,0_2,1_2 (if supported) configured on sSCell for CCS from sSCell to PCell/PSCell - FFS: Support of sSCell deactivation/activation when sSCell cross carrier scheduling to PCell/PSCell is configured - FFS: Support of sSCell dormancy when sSCell cross carrier scheduling to PCell/PSCell is configured - FFS: PDCCH monitoring occasion(s) is within the first 3 OFDM symbols of a PCell/PSCell slot - FFS: Numbers of CORESET configurations and search space sets on sSCell (for PCell/PSCell cross-carrier scheduling) - FFS: frame boundary alignment between PCell/PSCell and sSCell - FFS: Precoder granularity of REG-bundle CORESET size when CCS from sSCell to PCell/PSCell is configured - FFS: Whether the PCell/PSCell SCS other than 15kHz can be allowed Note: The SCell configured with Cross-carrier scheduling to PCell/PSCell is referred to as ‘sSCell’ | FFS 6-5 [, 34-1] | Yes |  |  | FFS Per BC | No | FFS Applicable to FR1 only |  | FFS: UE should be able to indicate following two as part of this per-BC capability [Candidate value set 1: One or more of supported SCS combinations ({P(S)Cell SCS in kHz, sSCell SCS in kHz}) from following set are indicated by the UE: {15,15}, {15,30}, (15, 60) for N=4, {30,30}, {30,60},{60,60}) (1) subcarrier spacing for sSCell and/or subcarrier spacing for Pcell, (2) Candidate value set 2: frequency band pair(s) for {Pcell/PSCell, sSCell}] Candidate value set for component 6: {Scheduling CC of higher SCS and scheduled CC of lower SCS, same SCS for scheduling CC and scheduled CC, both} Note: The CCS from sSCell to Pcell is applicable to FR1 only but there can be other Scells in FR2 configured for the UE. | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

# ANNEX B: UE features on LTE_NR_DC_enh2 [1]

| 35. LTE_NR_DC_enh2 | 35-1 | [TRS\Temporary] RS [based/for] SCell activation | [SCell activation with the assistance of temporary RS/TRS] - [TRS/Temporary RS] for SCell activation is aperiodic and triggered by MAC CE - FFS: Temporary RS is based on aperiodic TRS and FFS other signals - FFS: Temporary RS is triggered only configured within the BWP indicated by firstActiveDownlinkBWP-Id for the sSCell - FFS: A P-TRS SSB of the to-be-activated Scell can be is indicated as a QCL source for the temporary RS in case of known Scell same as existing specification - FFS: Maximum number of temporary RS resource sets that can be configured to UE per CC {1 … 16} - FFS: Maximum number of temporary RS resource sets that can be configured to UE across CCs {1 … 256} - FFS: Maximum number of triggering states for temporary RS based Scell activation by a MAC-CE {1 … 64} - FFS: Maximum number of temporary RS resource sets that can be associated with a triggering state {1 … 16} - FFS: Support of temporary RS based SCell activation on one or more from {FR1 FDD, FR1 TDD, FR1 unlicensed, FR2} [Note: following are reported via the legacy feature, FG2-33 - Maximum number of configured NZP-CSI-RS resources per CC - Maximum total number of simultaneous NZP-CSI-RS resources in active BWPs across all CCs - Maximum number simultaneous NZP-CSI-RS resources per CC - Maximum total number of CSI-RS ports in simultaneous NZP-CSI-RS resources in active BWPs across all CCs] | 6-5 | Yes | N/A |  | [Per UE/Per BC/Per band] | [No/Yes] | [No/Yes] |  | [The NZP-CSI-RS configured as temporary RS for fast SCell activation are not considered when counting the maximum NZP-CSI-RS configurations of FG2-33.] | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |