3GPP TSG RAN WG1 #108-e       																	    	R1-2201726

e-Meeting, February 21st – March 3rd, 2022

Source:	Intel Corporation

Title:	On UE features for IOT over NTN

Agenda item:	8.16.14

Document for:	Discussion and Decision

# Introduction

In this contribution, we provide discussion on UE features for IoT-NTN based on the latest proposal in [1]. Table with the details on UE features for IoT-NTN is captured in the Appendix (copied from [1] with proposed changes highlighted by red font).

# Discussion

We have the following comments on the UE feature list for IoT-NTN in [1].

- Word Satellite can be removed since it is already clear that operation with satellite is assumed for NTN.

- UE capability for NTN can be indicated per band to allow more flexibility for UE implementation.

- Separate signaling of UE capabilities for eMTC and NB-IoT can be used to allow more flexibility for UE implementation.

- For Component 1-1 in FG 2-1, brackets can be removed since indication of validity duration was agreed in RAN1.

- For FG 2-2 and FG 2-3, differentiation based on orbits such as LEO/MEO/GEO is not needed since the corresponding UE feature may be supported or not for a band without considering particular deployment characteristics. Also, in our view the corresponding FGs should be supported for all UEs supporting NB-IoT/eMTC NTN since UE-specific slot offset and reporting of UE’s TA are important features to enable half-duplex FDD devices.

- For FG 2-3, many aspects of UE reporting of information about the UE specific TA pre-compensation were defined in RAN2 including reporting of TA during initial access and in RRC_CONNECTED state and reporting of UE location information. Thus, in our view RAN2 can define the details on the UE capability signaling for the exact content of UE reporting of information about the UE specific TA pre-compensation.

# Conclusion

In this contribution, we provide discussion on UE features for IoT-NTN. The following proposal is made.

Proposal:

- Consider changes to the UE feature list for IoT-NTN marked below using red font

# References

- R1-2112900, Updated RAN1 UE features list for Rel-17 LTE after RAN1 #107-e

# Appendix

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the eNB to know if the feature is supported | [Need for the UE to know if the feature is supported (only for V2X WI, where the PC5-RRC capability signalling is delivered between the UEs)] | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Capability interpretation for mixture of FDD/TDD | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 2. LTE_NBIOT_eMTC_NTN | 2-1 | Basic IoT over NTN support | 1. UE derives its position based on its GNSS measurements [1-1. UE report the validity duration of GNSS] 2. Receive serving satellite ephemeris in either state vector format or orbital element format 4. UE specific TA calculation in RRC_IDLE and RRC_CONNECTED state based on its GNSS-acquired position and the serving satellite ephemeris 6. UE applies common TA in RRC_IDLE and RRC_CONNECTED according to the parameters provided by the network (UE considers common TA as 0 if the parameter is not provided) [7. For TA update in RRC_CONNECTED state, combination of both open (i.e. UE autonomous TA estimation, and common TA estimation) and closed (i.e., received TA commands) control loops] 8. In RRC_IDLE and RRC_CONNECTED state, UE calculates frequency pre-compensation to counter shift the Doppler experienced on the service link [in DL] [and] [in UL] 10. Support a validity timer of UL synchronization is configured by the network 13. UE applies cell specific K_offset in timing relationship enhancements 14.UE estimates UE-gNB RTT and delays the starts of ra-ResponseWindow by UE-gNB RTT [15. delaying the starts of ra-ResponseWindow] [16. Delaying the UE action and assumption on a downlink configuration carried by MAC CE command by K_mac if it is indicated] 17. In RRC_IDLE state and RRC_CONNECTED state, pre-compensate the calculated frequency offset and TA in uplink transmissions |  | Yes | N/A | Release 17 UE cannot access [NTN/satellite] | [per UE/per band] | No | No | FFS: whether this feature group needs to be separate for eMTC and NB-IoT Separate signalling for eMTC and NB-IoT | Optional with capability signalling For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported [Note: This UE feature group is applicable only for IoT-NTN cell, for terrestrial cell this feature is not supported] |
| 2. LTE_NBIOT_eMTC_NTN | 2-1a | Segment duration configuration | Support segment duration configuration by the network |  | Yes | N/A | Release 17 UE cannot access [NTN/satellite] | [Per UE/per band] | No | No | For UEs support NSGO scenarios, it must indicate this FG is supported. Separate signalling for eMTC and NB-IoT | Optional with capability signalling [Note: This UE feature group is applicable only for IoT-NTN cell, for terrestrial cell this feature is not supported] |
| 2. LTE_NBIOT_eMTC_NTN | 2-2 | Enhancing timing relationships using a time offset | - UE applies UE specific K_offset in timing relationship enhancements | 2-1 [, 2-3] | Yes | N/A | UE does not know the offset to apply for UL transmission | [per UE/per band] | No | No | The K_offset is a scheduling offset used for the identified timing relationships that need to be modified for IoT NTN. For IoT NTN, support cell-specific Koffset configuration for use during initial access. For IoT NTN, support the use of UE-specific Koffset in CONNECTED mode. FFS: whether this feature group needs to be separate for eMTC and NB-IoT FFS: differentiation based on orbits such as LEO/MEO/GEO Separate signalling for eMTC and NB-IoT | Optional with capability signalling FFS: For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported |
| 2. LTE_NBIOT_eMTC_NTN | 2-3 | UE specific TA pre-compensation reporting | - [Support reporting of information about the UE specific TA pre-compensation] - The UE capability signalling for the exact content of UE reporting of information about the UE specific TA pre-compensation is up to RAN2 | 2-1 | Yes | No | UL scheduling for FDD-HD: Use of UE-specific TA and/or K_offset to avoid UL-DL collisions in FDD-HD | [per UE/per band] | No | No | UE-specific TA reporting is supported in IoT-NTN FFS: Detailed contents of report FFS: whether this feature group needs to be separate for eMTC and NB-IoT FFS: differentiation based on orbits such as LEO/MEO/GEO Separate signalling for eMTC and NB-IoT | Optional with capability signalling FFS: For UEs supporting NB-IoT/eMTC NTN, it must indicate this FG is supported |