3GPP TSG RAN WG1 #108-e			R1-2201756

e-Meeting, February 21st – March 3rd, 2022

Agenda Item:	7.2.5

Source:	Apple Inc.

Title:	Remaining issues on intra-UE multiplexing/prioritization for eURLLC

Document for:	Discussion/Decision

# 1. Introduction

In this contribution, we discuss the remaining issues on intra-UE multiplexing/prioritization for eURLLC, targeting for a clear common understanding among companies on the procedure.

We had the following working assumption:

Working assumption (RAN1#102-e)

- Multiplexing/overriding/etc. is performed similar to Rel.15 as if HP channels do not exist; this means that LP operations, multiplexing/overriding/etc., are performed before cancellation.

- A UE cancels the transmission of a LP channel including any intermediate scheduled LP transmission that does not overlap with any LP channel, if any DCI schedules an overlapping HP transmission with the LP channel, before performing multiplexing/overriding HP channels if any.

- Multiplexing/overriding of HP channels is performed as if LP channels do not exist.

- A final HP channel is prioritized if it overlaps with a final LP channel, after performing multiplexing of HP channels

In addition, we had the following agreement:

Agreement: (RAN1#101-e)

- If a UE is expected to cancel a scheduled low priority PUCCH/PUSCH due to a first DCI scheduling an overlapping high priority channel, the UE is not expected to transmit the scheduled low priority PUCCH/PUSCH due to a second DCI scheduling PUCCH/PUSCH that is received after the first DCI.

  - Note: The collision between HP PUSCH and LP PUSCH is not covered by this agreement.

However, there are still different understandings regarding which HP channels should be used to cancel LP channels, and there was also concern on the involved UE complexity if the UE needs to perform all the intermediate multiplexing. This was extensively discussed in a few meetings but not concluded [1][2][3].

# 2. Intra-UE multiplexing/prioritization

The email discussion [107-e-NR-L1enh-URLLC-05] ([3]) had discussed the options proposed earlier, and seemed to be fine with keeping the following options (which includes a newly proposed Option 5) for further discussion. In the end the main focus was on Option 2 and Option 5, especially Option 5 which had attracted quite some interest.

- Option 2 (v2): The UE does not use the outcome of intermediate multiplexing for HP channels to cancel LP channels.

  - Any HP channel with a corresponding DCI that overrides or overlaps with a HP channel that overlaps with a LP channel shall meet the cancellation timeline, namely all HP DCIs corresponding to these HP channels must arrive Tproc,2+d1 before the earliest symbol of the LP channel that would be cancelled by the any of these HP channels.

  - All HP PUCCH/PUSCH channels except the final HP PUCCH/PUSCH that gets transmitted by the UE are intermediate channel

- Option 3: [No change from the spec is needed.] Clarify that the “before or after” term in Claus 9 in 38.213 is interpreted as:

  - the UE checks overlapping between HP and LP channel for each HP grant it receives, including any intermediate HP channel that results from UCI multiplexing and PUCCH overriding triggered by each of the HP grant.

  - For Option 3, the handling of negative SR should be clarified. E.g., a HP PUCCH with negative SR is not considered when cancelling LP transmission.

- Option 3b:

  - Cancellation timeline needs to be satisfied for a group of overlapping HP channels as long as one of the HP channels overlaps with a LP channel.

  - HP PUCCH for HARQ-ACK indicated by each DCI can cancel LP.

  - Final HP PUCCH or PUSCH is used to cancel LP.

- Option 5: The UE makes a determination about canceling the LP channel at the cancellation deadline. This determination is based on the multiplexing/overriding of the HP channels that are determined up to the cancellation deadline.

- Multiplexing/overriding of the HP channels are performed based on their associated timelines defined in R15.

- Each and every dynamically scheduled HP channel as well as the HP channels that are the result of the HP channel multiplexing/overriding and are overlapping with a LP channel should satisfy the cancellation timeline, i.e., the gap between the ending symbol of the HP DCI to the starting symbol of that HP channel should be at least Tproc,2+d1.

- The UE cancels the LP channel starting from the first symbol that overlaps with the HP channel at the latest, i.e., the current specification wording is kept.

- Once a LP channel is determined to be cancelled at the cancellation deadline, a UE is not expected to revert its decision.

The pros and cons of Option 2/3/3b had been heavily discussed previously. For Option 5, we think it can be considered as a well balanced approach that takes into account both UE implementation complexity and gNB scheduling flexibility, even though the additional flexibility might not be strictly necessary from our perspective. Overall, Option 2 is still our first preference, because it makes a clean/simple procedure. But Option 3b and Option 5 could also be acceptable to us.

One issue that seems not clear regarding the cancellation timeline is the starting symbol of which HP channels are considered. Consider the example in Figure 1: at least for options 2/3b/5, the LP channel is considered being cancelled by HP PUSCH. However, when considering the cancellation timeline, the first symbol of HP PUCCH should be used instead of the first symbol of HP PUSCH, because the UE needs to receive the HP DCI earlier enough to know whether HP PUCCH is cancelling LP channel or not. As a general principle, the earliest symbol among all the HP channels that are involved in the overriding/multiplexing procedure should be used to define the cancellation timeline, not just the HP channel that actually cancels the LP channel.

Figure 1 Definition of cancellation timeline

Proposal 1: When a HP channel cancels a LP channel, the cancellation timeline is defined w.r.t. the earliest symbol among all the HP channels that are involved in the overriding/multiplexing procedure.

One concern that had been raised is how these options would impact the specifications. So we provide example TPs for Option 2 and Option 5 to be considered.

Proposal 2: Adopt Option 2 (1st preference) or Option 5 (2nd preference).

TS 38.213 TP for Option 2 # 9 UE procedure for reporting control information <unchanged parts omitted> When a UE determines overlapping for PUCCH and/or PUSCH transmissions of different priority indexes other than PUCCH transmissions with SL HARQ-ACK reports before considering limitations for UE transmission as described in clause 11.1 and clause 11.1.1, including repetitions if any, the UE first resolves the overlapping for PUCCH and/or PUSCH transmissions of smaller priority index as described in clauses 9.2.5 and 9.2.6. Then, - if a transmission of a first PUCCH of larger priority index scheduled by a DCI format in a PDCCH reception would overlap in time with a repetition of a transmission of a second PUSCH or a second PUCCH of smaller priority index, the UE cancels the repetition of a transmission of the second PUSCH or the second PUCCH before the first symbol that would overlap with the first PUCCH transmission - if a transmission of a first PUSCH of larger priority index scheduled by a DCI format in a PDCCH reception would overlap in time with a repetition of the transmission of a second PUCCH of smaller priority index, the UE cancels the repetition of the transmission of the second PUCCH before the first symbol that would overlap with the first PUSCH transmission where - the overlapping is applicable after resolving overlapping among channels of larger priority index, if any, as described in clauses 9.2.3, 9.2.5 and 9.2.6 - any remaining PUCCH and/or PUSCH transmission after overlapping resolution is subjected to the limitations for UE transmission as described in clause 11.1 and clause 11.1.1 - Tproc,2the UE expects the last symbol of the PDCCH reception for the DCI format to arrive at least Tproc,2 before the earliest symbol of all the involved PUCCH or PUSCH transmissions of the larger priority index during the overlapping resolution. - Tproc,2 is the PUSCH preparation time for a corresponding UE processing capability assuming d2,1= d1 [6, TS 38.214], based on μ and N2 as subsequently defined in this clause, and d1 is determined by a reported UE capability If a UE detects a DCI format scheduling a first PUCCH transmission of larger priority index that overrides, according to clause 9.2.3, a second PUCCH transmission of larger priority index, or if a UE detects a DCI format scheduling a first PUSCH transmission of larger priority index that overlaps a second PUCCH or a second PUSCH transmission of larger priority index, and the second PUCCH or second PUSCH transmission overlaps with a third PUCCH or third PUSCH transmission of smaller priority index, the UE expects the end of the PDCCH for the DCI format to arrive at least Tproc,2 , as defined in this clause, before the earliest symbol of all the involved PUCCH or PUSCH transmissions of the larger priority index. <unchanged parts omitted>

TS 38.213 TP for Option 5 # 9 UE procedure for reporting control information <unchanged parts omitted> When a UE determines overlapping for PUCCH and/or PUSCH transmissions of different priority indexes other than PUCCH transmissions with SL HARQ-ACK reports before considering limitations for UE transmission as described in clause 11.1 and clause 11.1.1, including repetitions if any, the UE first resolves the overlapping for PUCCH and/or PUSCH transmissions of smaller priority index as described in clauses 9.2.5 and 9.2.6. Then, - if a transmission of a first PUCCH of larger priority index scheduled by a DCI format in a PDCCH reception would overlap in time with a repetition of a transmission of a second PUSCH or a second PUCCH of smaller priority index, the UE cancels the repetition of a transmission of the second PUSCH or the second PUCCH before the first symbol that would overlap with the first PUCCH transmission - if a transmission of a first PUSCH of larger priority index scheduled by a DCI format in a PDCCH reception would overlap in time with a repetition of the transmission of a second PUCCH of smaller priority index, the UE cancels the repetition of the transmission of the second PUCCH before the first symbol that would overlap with the first PUSCH transmission where - the overlapping is applicable after resolving overlapping among channels of larger priority index, if any, as described in clauses 9.2.3, 9.2.5 and 9.2.6, considering all the DCI formats that have the last symbol being received no later than the cancellation deadline, where the cancellation deadline is defined as Tproc,2 before the earliest symbol of all the involved PUCCH or PUSCH transmissions of the larger priority index during the overlapping resolution. - any remaining PUCCH and/or PUSCH transmission after overlapping resolution is subjected to the limitations for UE transmission as described in clause 11.1 and clause 11.1.1 - Tproc,2 - Tproc,2 is the PUSCH preparation time for a corresponding UE processing capability assuming d2,1= d1 [6, TS 38.214], based on μ and N2 as subsequently defined in this clause, and d1 is determined by a reported UE capability <unchanged parts omitted>

# 4. Conclusions

In this contribution, we have discussed the few options for intra-UE multiplexing/prioritization in eURLLC and proposed:

Proposal 1: When a HP channel cancels a LP channel, the cancellation timeline is defined w.r.t. the earliest symbol among all the HP channels that are involved in the overriding/multiplexing procedure.

Proposal 2: Adopt Option 2 (1st preference) or Option 5 (2nd preference).

Example TPs are provided Option 2 and Option 5.

# References

[1] R1- 2106359, Summary of [105-e-NR-L1enh-URLLC-03] Email discussion/approval on remaining issues on Scheduling & HARQ enhancements, Moderator (Qualcomm), RAN1#105-e, May 2021.

[2] R1- 2108655, Summary of [106-e-NR-L1enh-URLLC-06] Issue #10: UE Procedures for UCI Multiplexing and Prioritization, Moderator (Qualcomm), RAN1#106-e, Aug. 2021.

[3] R1-212889, Summary of email discussion [107-e-NR-L1enh-URLLC-05] on UCI multiplexing and prioritization in Rel-16, Moderator (Apple), RAN1#107-e, Nov. 2021.