3GPP TSG RAN WG1 #107bis-e	                                    	   R1-2201769

e-Meeting, February 21st – March 3rd, 2022

Agenda Item:	8.3.1

Source:	Apple Inc.

Title:	Remaining issues in UE feedback enhancements for HARQ-ACK

Document for:	Discussion/Decision

# Introduction

In this contribution, we address the remaining issues of HARQ-ACK. Feedback enhancements for Rel-17 URLLC.

# Discussion

In Rel-17 URLLC, multiple features to handle HARQ-ACK feedback enhancements are specified. Mainly 4 scenarios are identified as use cases:

HARQ-ACK feedback can be dropped in a number of cases:

- Scenario 1:    on a TDD carrier, SPS HARQ feedback can be dropped due to PUCCH’s collision with DL symbols

- Scenario 2a:  with Rel-16 intra-UE prioritization, LP HARQ-ACK can be dropped if the LP PUCCH collides with a HP channel

- Scenario 2b:  with Rel-16 intra-UE prioritization, LP PUSCH can be dropped if the  LP PUSCH collides with a HP channel

- Scenario 3:    with Rel-16 inter-UE preemption, HARQ-ACK carried over LP PUSCH and even HP HARQ-ACK carried over PUSCH can be dropped.

In a fourth scenario, HARQ feedback’s latency can be un-necessarily long for inter-band TDD

- Scenario 4:

  - In the example below, CC1(in Band Y) is the PCell, where PUCCH is carried, and the HARQ feedback latency can be long.

  - If PUCCH can be carried over CC2 (in Band Z) for some HARQ feedback, then feedback latency can be reduced.

|  | Key benefits | Limitations |
| --- | --- | --- |
| SPS HARQ deferral | Avoid dropping SPS HARQ-ACK on a TDD carrier to better system capacity & UE power consumption | Applicable to TDD only |
| HARQ retransmission with enhanced Type 3 HARQ codebook(s) | Handling SPS HARQ-ACK dropping due to collision with DL symbols on a TDD carrier, LP channel’s dropping, inter-UE prioritization | It does help alleviate dropping of LP PUSCH |
| HARQ retransmission with Type 1/Type 2 codebook |  |  |
| PUCCH carrier switching | Reduce HARQ feedback latency | Applicable to inter-band TDD only |
| Simultaneous PUCCH/PUSCH transmission | Avoid un-necessary dropping | Applicable to inter-band CA only |
| Enhanced Type 1 HARQ codebook | Reduce feedback overhead |  |
| Intra-priority UCI multiplexing | Avoid dropping LP HARQ-ACK and LP PUSCH and protect unequal protection for HP HARQ-ACK and LP HARQ-ACK |  |

In the table below, the status for joint operation of two features is enumerated.

Figure 1 Joint operation of features

## Joint operation of intra-UE MUX and SPS HARQ deferral

Since deferred SPS HARQ-ACK can be handled per physical layer priority follows existing agreements, the payload at LP or HP with deferred SPS HARQ-ACK can be handled by the intra-UE MUX framework and it is natural to support the joint operation of SPS HARQ deferral and Rel-17 intra-UE MUX. In some cases, the payload size may exceed the capacity of UCI part 1 or UCI part 2, then some dropping rules can be defined to handle those cases. For example, with HP HARQ-ACK mapped to UCI part 1, and LP HARQ-ACK which includes DG HARQ-ACK and SPS HARQ-ACK is mapped to UCI part 2, then SPS HARQ-ACK can be dropped if UCI part 2’s capacity is not sufficient.

## Joint operation of intra-UE MUX and one-shot HARQ retransmission

Since one-short HARQ retransmission can be handled per physical layer priority follows existing agreements, the payload at LP or HP with retransmitted one-shot HARQ-ACK can be handled by the intra-UE MUX framework and it is natural to support the joint operation of one-shot HARQ retransmission and Rel-17 intra-UE MUX. In some cases, the payload size may exceed the capacity of UCI part 1 or UCI part 2, then some dropping rules can be defined to handle those cases. For example, with HP HARQ-ACK mapped to UCI part 1, and LP HARQ-ACK which includes initial DG HARQ-ACK and retransmitted one-shot HARQ-ACK is mapped to UCI part 2, then one-shot HARQ retransmission can be dropped if UCI part 2’s capacity is not sufficient.

## Summary

In summary, we have

Observation 1: joint operation of intra-UE MUX and SPS HARQ deferral can be supported.

Observation 2: joint operation of intra-UE MUX and one-shot HARQ retransmission can be supported.

Proposal 1: When UCI part 1 or UCI part 2’s capacity is exceeded, part of the HARQ-ACK feedback (initial HARQ bits / Type 1 or Type 2 codebook/deferred SPS HARQ-ACK) can be dropped.

Proposal 2: When UCI part 1 or UCI part 2’s capacity is exceeded, part of the HARQ-ACK feedback (initial HARQ codebook/retransmitted HARQ codebook) can be dropped.

# Conclusion

In this contribution, we discuss the joint operation of SPS HARQ deferral and dynamic PUCCH carrier switching. We have

Observation 1: joint operation of intra-UE MUX and SPS HARQ deferral can be supported.

Observation 2: joint operation of intra-UE MUX and one-shot HARQ retransmission can be supported.

Proposal 1: When UCI part 1 or UCI part 2’s capacity is exceeded, part of the HARQ-ACK feedback (initial HARQ bits / Type 1 or Type 2 codebook/deferred SPS HARQ-ACK) can be dropped.

Proposal 2: When UCI part 1 or UCI part 2’s capacity is exceeded, part of the HARQ-ACK feedback (initial HARQ codebook/retransmitted HARQ codebook) can be dropped.