3GPP TSG RAN WG1 #108-e		                                R1-2201773

e-Meeting, February 21st – March 3rd, 2022

Agenda Item:	8.4.3

Source:	Apple

Title:	Remaining Issue of HARQ Enhancements for NR NTN

Document for:	Discussion/Decision

# Introduction

It was concluded [1] that release 17 RAN1 work on NR NTN is completed. The maintenance on solutions for NR NTN is conducted in RAN1 #108-e meeting.

One of the RAN1-lead objectives in release 17 NR NTN is HARQ enhancements. In this contribution, we provide our views on one remaining issue related to HARQ enhancements for NR NTN.

# Discussion

## HARQ-ACK feedback for SPS PDSCH

It was agreed [2] that HARQ feedback for SPS activation may be enabled by the network by RRC configuration. If enabled, UE always reports ACK/NACK for the first SPS PDSCH after activation. Otherwise, UE follows configuration of HARQ feedback enabled/disabled corresponding to the first SPS PDSCH after activation. Regarding the first SPS PDSCH after activation, it is open between two alternatives. In the first alternative, UE follows the per-process configuration of HARQ feedback enabled/disabled for the associated HARQ process. In the second alternative, UE follows the feedback-enabled/disabled configuration of the SPS PDSCH.

In release 16 terrestrial network, a SPS configuration may include multiple HARQ processes. In NTN, enabling/disabling on HARQ feedback for downlink transmission is configurable per HARQ process. If this agreement is applicable to the HARQ processes in a SPS configuration, then it is possible that the HARQ processes in a SPS configuration have different HARQ feedback settings.

However, this configuration is not preferred. Considering that a SPS configuration is aimed to serve a certain type of data, it makes sense to ensure all HARQ processes in a SPS configuration have the same feedback setting (i.e., enabled or disabled HARQ feedback) to achieve the QoS requirement of the SPS traffic.

On the other hand, in the existing SPS configuration, the associated HARQ processes have consecutive HARQ process IDs. It is not easy from network implementation to guarantee that all the HARQ processes in a SPS configuration have the same feedback setting.

Hence, we propose that a SPS configuration is able to set the enabling/disabling of HARQ-ACK feedback for all HARQ processes associated with this SPS configuration. This SPS configuration overwrites the per HARQ process configuration of HARQ feedback enabling/disabling, when determining the HARQ-ACK feedback for SPS PDSCH. UE follows the HARQ feedback enabling/disabling in SPS configuration for SPS PDSCH transmissions.

Proposal 1: UE follows the HARQ feedback enabling/disabling in SPS configuration for SPS PDSCH transmissions.

# Conclusion

In this contribution, we provided our views on the remaining issue of HARQ enhancements for NR NTN. Our proposal is as follows:

Proposal 1: UE follows the HARQ feedback enabling/disabling in SPS configuration for SPS PDSCH transmissions.

# References

- RP-212803, “Status report for WI: NR-NTN-solutions,” Dec. 2021.

- R1-2112664, “Summary#3 of AI 8.4.3 for HARQ in NTN,” Nov. 2021.