3GPP TSG RAN WG1 #108-e		R1-2201796

e-Meeting, February 21st – March 3rd, 2022

Source: 	Apple Inc.

Title:                     UE features for RedCap

Agenda item:	8.16.6

Document for:	Discussion and Decision

# 1 Introduction

In RAN1 107 e-meeting, the following agreement was made for Redcap UE features [1]:

Agreement Following features are not added into FG 28-1 - Supported Rx branches and corresponding maximum DL MIMO layers - RedCap UE must indicate this capability from RAN1 perspective - Supported FDD operation - Supported maximum DL modulation order Agreement Inform RAN2 that “From RAN1 perspective, it would be enough to indicate the maximum number of PDSCH MIMO layers per band for RedCap UEs, but RAN1 notes that the type of FG2-3 (maxNumberMIMO-LayersPDSCH) is currently per FSPC and that it is up to RAN2 whether to signal per band or per FSPC” - Note: If RAN2 decides to reuse the existing signaling (FG2-3) with modification for RedCap, then FG 28-2 is not needed from RAN1 perspective. If RAN2 decides to keep FG28-2, a RedCap UE must indicate FG28-2 from RAN1 perspective. - 1st bullet is captured in the LS to RAN2 being discussed in [107-e-R17-UE-features-REDCAP-02] Agreement - FG 28-5 is removed - Add a note in FG 1-5 (256QAM for PUSCH) that “For RedCap UEs, the 256QAM MCS table for PUSCH is only supported if the UE supports 256QAM for PUSCH” Agreement: - RedCap UE supports FG1-4 (256QAM for PDSCH) as optional with capability signalling both for FR1 and FR2 - Add a note in FG 1-4 (256QAM for PDSCH) that “For RedCap UEs, the 256QAM MCS table for PDSCH and CQI table 2 are only supported if the UE supports 256QAM for PDSCH” Agreement: - Add a note in FG 1-5 (256QAM for PUSCH) that “For RedCap UEs, the 256QAM MCS table for PUSCH is only supported if the UE supports 256QAM for PUSCH” Agreement - Inform RAN2 that “From RAN1 perspective, it would be enough to indicate the maximum number of PDSCH MIMO layers per band for RedCap UEs, but RAN1 notes that the type of FG2-3 (maxNumberMIMO-LayersPDSCH) is currently per FSPC and that it is up to RAN2 whether to signal per band or per FSPC” - Note: If RAN2 decides to reuse the existing signaling (FG2-3) with modification for RedCap, then FG 28-2 is not needed from RAN1 perspective. If RAN2 decides to keep FG28-2, a RedCap UE must indicate FG28-2 from RAN1 perspective. - 1st bullet is captured in the LS to RAN2 being discussed in [107-e-R17-UE-features-REDCAP-02] Agreement - For early indication of RedCap UE, - The capability of early indication of RedCap UE in Msg.1 for 4-step RACH is added as a component in FG 28-1 - FFS other early indication schemes

This contribution discusses the open issues related to UE capabilities for Rel-17 Redcap WI listed in [1].

# 2. Discussions

## 2.1	Rel-17 Redcap UE features

## FG 28-1

One FFS aspect for FG 28-1 is whether to add other UE features [2]. In RAN1 107 e-meeting, it was agreed that SSB, either CD-SSB or NCD-SSB, can be expected for basic Redcap UE but not CORESET#0 [1]. Therefore, the following features can be added into basic feature group FG 28-1 for Redcap:

- 1 UE-specific RRC configured DL BWP per carrier

- 1 UE-specific RRC configured UL BWP per carrier

- RRC reconfiguration of any parameters related to BWP

- BW of a UE-specific RRC-configured BWP includes BW of SSB

- BW of UE-specific RRC-configured BWP may not include BW of MIB-configured CORESET#0.

We therefore proposed the following:

Proposal 1: Consider adding the following basic FGs into FG 28-1:

- 1 UE-specific RRC configured DL BWP per carrier

- 1 UE-specific RRC configured UL BWP per carrier

- RRC reconfiguration of any parameters related to BWP

- BW of a UE-specific RRC-configured BWP includes BW of SSB

- BW of UE-specific RRC-configured BWP may not include BW of MIB-configured CORESET#0.

## FG 28-x

In addition, we propose to introduce an optional UE feature FG 28-x to indicate the support of DL BWP without SSB in RRC_CONNECTED state, analogous to legacy FG 6-1A.

Proposal 2: Introduce a new FG 28-x that is used to indicate the support of a UE-specific DL BWP without SSB when UE is in RRC_CONNECTED state.

# 3. Conclusion

Based on the discussions in the previous sections, we made the following proposal:

Proposal 1: Consider adding the following basic FGs into FG 28-1:

- 1 UE-specific RRC configured DL BWP per carrier

- 1 UE-specific RRC configured UL BWP per carrier

- RRC reconfiguration of any parameters related to BWP

- BW of a UE-specific RRC-configured BWP includes BW of SSB

- BW of UE-specific RRC-configured BWP may not include BW of MIB-configured CORESET#0.

Proposal 2: Introduce a new FG 28-x that is used to indicate the support of a UE-specific DL BWP without SSB when UE is in RRC_CONNECTED state.

# References

- RAN1 107 e-meeting Chairman notes

- R1-2200780	Updated RAN1 UE features list for Rel-17 NR after RAN1 #107 bis-e, 	Moderator (AT&T, NTT DOCOMO, INC).

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 28. NR_redcap | 28-1 | RedCap UE | 1. Maximum FR1 RedCap UE bandwidth is 20 MHz. 2. Maximum FR2 RedCap UE bandwidth is 100 MHz. 3. Early indication of RedCap UE in Msg.1 for 4-step RACH FFS whether to add any other basic features for RedCap UE |  | Yes |  | Network assumes the UE is not a RedCap UE | Per UE | No | No |  | RedCap UEs do not support carrier aggregation or dual connectivity. | Optional with capability signaling RedCap UE must indicate this FG is supported |
| 28. NR_redcap | 28-2 | Number of UE Rx branches and DL MIMO layers for RedCap UE | 1. For a RedCap UE with 1 Rx branch, 1 DL MIMO layer is supported. 2. For a RedCap UE with 2 Rx branches, 2 DL MIMO layers are supported. | 28-1 | Yes |  | Impact on UE complexity | Per band | No | No |  | For UE capability signalling, the number of Rx branches for RedCap is implicitly indicated by the corresponding capability parameter maxNumberMIMO-LayersPDSCH in the existing UE capability framework. Detailed signalling is up to RAN2. | Optional with capability signaling |
| 28. NR_redcap | 28-3 | Half-duplex FDD operation type A for RedCap UE | 1. Half-duplex FDD operation (instead of full-duplex FDD operation) type A for RedCap UE | 28-1 | Yes |  | UE is assumed to support FD-FDD in FDD bands | Per band | FDD only | FR1 only |  |  | Optional with capability signaling |