3GPP TSG-RAN WG1 Meeting #108-e	R1- 2201809

e-Meeting, February 21st – March 3rd, 2022

Agenda Item:	8.14.2

Source:	Ericsson

Title:	On timing relationship maintenance issues for IoT NTN

Document for:	Discussion

# 1	Introduction

In the Status Report for IoT NTN WI [1], the following is stated regarding remaining open issues in RAN1:

All issues in 8.15.1 Enhancements to time and frequency synchronization and 8.15.2 Timing relationship enhancements were closed. RAN1 has specified designs that are sufficient for generating RRC parameters, UE feature definitions, and specifications.

The FFS as captured in the agreements in 8.15.1 in RAN1#107-e are on how to write the agreements into the specifications if needed and configuration values for RRC parameters. RAN1 will resolve these FFS if needed in the maintenance phase.

One objective of the IoT NTN WI is to specify timing relationship enhancements for NTN. In this contribution, we discuss the maintenance issues of timing relationship enhancements for IoT NTN.

# 2	TP for Timing in 36.213

The following specification excerpts are from Clause 6.1.1 and Clause 16.3.2 in TS 36.213, v17.0.0:

[Clause 6.1.1, TS 36.213, v17.0.0]

In case a random access procedure is initiated by a "PDCCH order" reception ending in subframe n for BL/CE UEs, the UE shall, if requested by higher layers, transmit random access preamble in the first subframe n+k2+Kcell_offset,, where a PRACH resource is available.

[Clause 16.3.2, TS 36.213, v17.0.0]

In case a random access procedure is initiated by a "PDCCH order" ending in subframe n, the UE shall, if requested by higher layers, start transmission of random access preamble at the end of the first subframe n+k2+Koffset, , where a NPRACH resource is available.

There is an inconsistency between the two clauses for eMTC and NB-IoT in the yellow highlighted text.

Adopt the following TP for Clause 16.3.2 in TS 36.213:

[Clause 16.3.2, TS 36.213, v17.0.0]

In case a random access procedure is initiated by a "PDCCH order" ending in subframe n, the UE shall, if requested by higher layers, start transmission of random access preamble at the end of the first subframe n+k2+Kcell_offset, , where a NPRACH resource is available.

# 2	TP for MPDCCH assignment procedure in 36.213

The following specification text occurs in Clause 9.1.5 in TS 38.213, v17.0.0:

[Clause 9.1.5, TS 36.213, v17.0.0]

If the UE has initiated a PUSCH transmission using preconfigured uplink resource ending in subframe n, the UE shall monitor the MPDCCH UE-specific search space in a search space window starting in subframe n+4+Kmac with duration given by higher layer parameter pur-MPDCCH-SS-window-duration where Kmac is provided by higher layer parameter K-mac, otherwise Kmac=0.

The subscript in the yellow highlighted text should not be in italics.

Adopt the following TP for Clause 9.1.5 in TS 36.213: Kmac should be replaced by Kmac.

# Conclusion

Based on the discussion in the previous sections we propose the following:

- Adopt the following TP for Clause 16.3.2 in TS 36.213:

[Clause 16.3.2, TS 36.213, v17.0.0]

In case a random access procedure is initiated by a "PDCCH order" ending in subframe n, the UE shall, if requested by higher layers, start transmission of random access preamble at the end of the first subframe n+k2+Kcell_offset, , where a NPRACH resource is available.

Adopt the following TP for Clause 9.1.5 in TS 36.213: Kmac should be replaced by Kmac.

# References

RP-213472, "Status report for WI: NB-IoT/eMTC support for Non-Terrestrial Networks (NTN)", MediaTek, RAN#94, Dec 2021