3GPP TSG RAN WG1 #108-e			                                                                 R1-2201837

e-Meeting, February 21st – March 3rd, 2022

Agenda item:	5

Source: 	CMCC

Title: 	Discussion on RAN4 LS on Rel-17 FeMIMO SRS related impact

Document for:	Discussion and Decision

Introduction

In LS [1] from RAN4, whether other signals could be transmitted between two SRS sets and if a guard period should be defined between SRS and other signals. In this contribution, we provide our view on those two questions.

Discussion

The questions and the main information is copied below.

Question 1: Is UE expected to transmit other signals between SRSs belonging to different set where usage is set to antenna switching? Question 2: Should there be a guard period defined in RAN1 specs or a switching time defined in ran4 specs in between SRSs and other signals? It is noticed that during RAN1#107-e meeting, similar issues have been discussed as summarized in [2]. Thus RAN4 would like to inform RAN1 the following initial discussion outcomes: - RAN4 thinks not sending PUSCH and PUCCH between SRS resources belonging to different sets where usage is set to antenna switching is inefficient. - RAN4 clarifies that the transient period between SRS resources is 15us. Further, RAN4 will continue working on these issues.

Question 1

Question 1 was discussed in #107e meeting but without any conclusion. As in Rel-15 when two slots carrying SRS in each, the symbols between two SRS sets will be used for other transmissions. And even in the case of 1T4R antenna switching, transmission between the two sets of SRS is allowed. Then when the gap between two SRS sets is larger than GP, it should be allowed to transmit other signals or channels. But if the gap between two SRS sets equals to the GP, or only Y symbols as defined for GP exists, it should not be allowed. In addition, we share the similar view that if the gap symbols between two sets of SRS are not allowed for other transmissions, it is too limited for gNB’s scheduling and it is inefficient from the perspective of resource usage.

Proposal 1:

- When the gap between two sets of SRS is larger than the GP of Y symbols, it should be allowed to transmit other signals or channels in the gap.

- When the gap between two sets of SRS equals to the GP of Y symbols, it should not be allow ed to transmit other signals or channels in the gap.

Question 2

For the Question 2, the case is that whether we should define a guard period between other signals and SRS symbols. Usually, UE will have main transmit antennas and the others used only for DL receiving. Raising 2T4R as an example, 2 antennas would be used for both UL transmission and DL receptions. And the other 2 antennas will be used only for DL reception. SRS antenna switching between the 2 sets of antennas is for DL CSI acquisition for all the 4 antennas. But when the SRS for antenna switching locates besides the symbols for other UL transmission, such as PUSCH or PUCCH, the antenna switching between the main transmit antennas and the receiving antennas would also happen. As illustrated in the following figure, it is assuming that antenna #1 and #2 are for both transmission and reception, and the antenna #3 and #4 are only for DL receptions. Between the SRS #2 and the PUSCH transmission in the next slot, there would be an antenna switching from antenna #3 and #4 to antenna #1 and #2. This is the similar procedure as SRS antennas switching which need some time to proceed the procedure. On the other side, in the slot n+1, if the SRS #1 is used for UE antenna #3 and #4 which are only for DL receptions, there would be an antenna switching procedure between the PUSCH/PUCCH transmission and the SRS#1 transmission. But if the SRS #1 are mapped with antenna #1 and #2 which are used for UL transmission, there is no need to insert guard period between PUSCH/PUCCH transmission and the SRS.

Observation 1:

The antenna switching procedure does happen between the adjacent SRS transmission and PUSCH/PUCCH transmissions depending mapping of UE antenna to the SRS ports for DL CSI acquisitions.

Figure 1, 2T4R SRS antenna switching and other UL transmission in the gaps

In addition, the mapping between the 4 antennas and the SRS resources are based on UE’s implementation. The gNB does not have the knowledge of which two SRS ports are mapped to the main transmission antennas of the UE. That means gNB does not know whether an antenna switching would be executed between the PUSCH/PUCCH transmission and SRS transmissions.

Observation 2：

The gNB does not know whether an antenna switching would be executed between the PUSCH/PUCCH transmissions and SRS transmissions.

As mentioned in RAN4’s LS, the transient period between SRS resources is 15us. If this is the same case for transient period between other transmission and SRS and there is no guard period, it will impact the SRS transmission or the other transmissions. But since the UE implementation is unknown to gNB, it cannot guarantee there should be a insertion of GP symbols. It need more discussion from RAN1.

Observation 3:

If the transient time is similar as that of the SRS, it may impact the performance of other transmissions such as PUSCH or PUCCH and SRS.

Proposal 2:

The transient period would be needed between PUSCH/PUCCH transmission and SRS. It needs more discussion about whether or how to define a GP to cover the transient period.

Conclusions

In this contribution, we provide our view on those two questions. The observations and proposals are as below.

Observation 1:

The antenna switching procedure does happen between the adjacent SRS transmission and PUSCH/PUCCH transmissions depending mapping of UE antenna to the SRS ports for DL CSI acquisitions.

Observation 2：

The gNB does not know whether an antenna switching would be executed between the PUSCH/PUCCH transmissions and SRS transmissions.

Observation 3:

If the transient time is similar as that of the SRS, it may impact the performance of other transmissions such as PUSCH or PUCCH and SRS.

Proposal 1:

- When the gap between two sets of SRS is larger than the GP of Y symbols, it should be allowed to transmit other signals or channels in the gap.

- When the gap between two sets of SRS equals to the GP of Y symbols, it should not be allow ed to transmit other signals or channels in the gap.

Proposal 2:

The transient period would be needed between PUSCH/PUCCH transmission and SRS. It needs more discussion about whether or how to define a GP to cover the transient period.

References

- [1] R1-2200895, LS on Rel-17 FeMIMO SRS related impact, RAN4, 3GPP TSG RAN WG1 #108-e, e-Meeting, February 21st – March 3rd, 2022.