3GPP TSG RAN WG1 #108-e			                                                                               R1-2201845

e-Meeting, February 21st - March 3rd, 2022

Agenda item:	8.1.2.1

Title:	Remaining issues of enhancements on Multi-TRP for PDCCH, PUCCH and PUSCH

Source:	CMCC

Document for:	Discussion and Decision

- Introduction

In RAN#86 meeting [1], it is agreed that the work item aims to identify and specify features to improve reliability and robustness for channels other than PDSCH (that is, PDCCH, PUSCH, and PUCCH) using multi-TRP and/or multi-panel, with Rel.16 reliability features as the baseline. SDM, TDM, and FDM based PDSCH enhancements have been specified in Rel-16 to improve reliability and robustness for multi-TRP transmission. The remaining issues for PUSCH are discussed in this contribution.

- Remaining issues on multi-TRP for PUSCH

## 4.1 PHR reporting for SUL

Agreement @RAN1#106-e For option 4, support the following: - When PHR MAC-CE is reported in slot n, for a CC that is configured with mTRP PUSCH repetition, second PHR value is determined as, - If the first PHR value is actual PHR (based on Rel. 15/16) corresponding to a repetition among mTRP PUSCH repetitions associated with a given TRP, the second PHR value, select Alt. 2A - Alt.2A: Is actual only when a repetition associated with the other TRP is transmitted in slot n. Otherwise, it is virtual. - If there are multiple repetitions associated with the other TRP in slot n, the earliest one in slot n is selected. - If the first PHR value is actual PHR (based on Rel. 15/16) but not corresponding to a repetition among mTRP PUSCH repetitions (corresponds to sTRP PUSCH), select Alt. 1B - Alt1B: a second PHR value is reported as virtual PHR. - If the first PHR value is virtual, select Alt. 1C - Alt1C: a second PHR value is reported as virtual PHR. - Note: It was agreed that when second PHR is virtual, it is calculated based on a set of default power control parameters defined for the other TRP (that is not associated with the first PHR) - Note: It was agreed that the above is applicable to both single entry and multi-entry PHR reports TS 38.213 ## 7.7 Power headroom report <omitted> If a UE - is configured with two UL carriers for a serving cell, and - determines a Type 1 power headroom report and a Type 3 power headroom report for the serving cell the UE - provides the Type 1 power headroom report if both the Type 1 and Type 3 power headroom reports are based on respective actual transmissions or on respective reference transmissions - provides the power headroom report that is based on a respective actual transmission if either the Type 1 report or the Type 3 report is based on a respective reference transmission <omitted> ### 7.7.1 Type 1 PH report If a UE is configured with two UL carriers for a serving cell and the UE determines a Type 1 power headroom report for the serving cell based on a reference PUSCH transmission, the UE computes a Type 1 power headroom report for the serving cell assuming a reference PUSCH transmission on the UL carrier provided by pusch-Config. If the UE is provided pusch-Config for both UL carriers, the UE computes a Type 1 power headroom report for the serving cell assuming a reference PUSCH transmission on the UL carrier provided by pucch-Config. If pucch-Config is not provided to the UE for any of the two UL carriers, the UE computes a Type 1 power headroom report for the serving cell assuming a reference PUSCH transmission on the non-supplementary UL carrier. <omitted> ### 7.7.3 Type 3 PH report <omitted> If a UE is configured with two UL carriers for a serving cell and the UE determines a Type 3 power headroom report for the serving cell based on a reference SRS transmission and a resource for the reference SRS is provided by SRS-Resource, the UE computes a Type 3 power headroom report for the serving cell assuming a reference SRS transmission on the UL carrier provided by pucch-Config. If pucch-Config is not provided to the UE for any of the two UL carriers, the UE computes a Type 3 power headroom report for the serving cell assuming a reference SRS transmission on the non-supplementary UL carrier.

In Rel-15, if a UE is configured with two UL carriers for a serving cell (non-SUL, SUL), UE provides the Type 1 PHR if both the Type 1 PHR and Type 3 PHR are actual PHRs or virtual PHRs, or UE provides the actual PHR if either the Type 1 PHR or the Type 3 PHR is actual PHR. If UE provides Type 1 virtual PHR, the UE computes Type 1 virtual PHR assuming a reference PUSCH transmission on the UL carrier provided by pusch-Config. If the UE is provided pusch-Config for both UL carriers, the UE computes Type 1 virtual PHR assuming a reference PUSCH transmission on the UL carrier provided by pucch-Config. If pucch-Config is not provided to the UE for any of the two UL carriers, the UE computes Type 1 virtual PHR for the serving cell assuming a reference PUSCH transmission on the non-SUL carrier.

When M-TRP is applied for two UL carriers for a serving cell (non-SUL, SUL) and gNB indicates UE to report two PHRs, the above Rel-15 criteria could be applied to determine the PHR for each TRP, and then Rel-17 M-TRP criteria could be applied for reporting two PHRs. For example, if one of the UL carriers is provided pusch-Config, for TRP1, UE provides the Type 1 virtual PHR if both the Type 1 PHR and Type 3 PHR are virtual PHRs, and UE computes Type 1 virtual PHR assuming a reference PUSCH transmission for TRP1 on the UL carrier provided by pusch-Config. For TRP2, based on Rel-17 M-TRP criteria, if the first PHR value is virtual, a second PHR value is reported as virtual PHR, then UE computes Type 1 virtual PHR assuming a reference PUSCH transmission for TRP2 on the UL carrier provided by pusch-Config.

Proposal 1: When M-TRP is applied for two UL carriers for a serving cell (non-SUL, SUL) and gNB indicates UE to report two PHRs, Rel-15 criteria could be applied to determine the PHR for each TRP, and then Rel-17 M-TRP criteria could be applied for reporting two PHRs.

## 4.2 TP for 38.213

The TP for 38.213 is shown below:

TS38.213 ### 7.1.1 UE behaviour <omitted> - For a PUSCH (re)transmission configured by ConfiguredGrantConfig, j=1, PO_NOMINAL,PUSCH,f,c1 is provided by p0-NominalWithoutGrant, or PO_NOMINAL,PUSCH,f,c1=PO_NOMINAL,PUSCH,f,c0 if p0-NominalWithoutGrant is not provided. - If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook', and is provided p0-PUSCH-Alpha2, for a retransmission of a configured grant Type 1 PUSCH, or for activation or retransmission of a configured grant Type 2 PUSCH, scheduled by a DCI format that includes a SRS resource set indicator field, and for active UL BWP b of carrier f of serving cell - If the SRS resource set indicator value is 00, first PO_UE_PUSCH,b,f,c1 value is provided by the value of p0-PUSCH-Alpha in ConfiguredGrantConfig. - If the SRS resource set indicator value is 01, second PO_UE_PUSCH,b,f,c1 value is provided by the value of p0-PUSCH-Alpha2 in ConfiguredGrantConfig. - If the SRS resource set indicator value is 10 or 11, first and second PO_UE_PUSCH,b,f,c1 values are respectively provided by the values of p0-PUSCH-Alpha and by p0-PUSCH-Alpha2 in ConfiguredGrantConfig. - else, PO_UE_NOMINAL,PUSCH,b,f,c1 is provided by p0 obtained from p0-PUSCH-Alpha in ConfiguredGrantConfig that provides an index P0-PUSCH-AlphaSetId to a set of P0-PUSCH-AlphaSet, or by p0-PUSCH for a PUSCH (re)transmission as described in clause 19.1, for active UL BWP b of carrier f of serving cell c

- Conclusions

Based on the above discussions, the proposals are as follows:

Proposal 1: When M-TRP is applied for two UL carriers for a serving cell (non-SUL, SUL) and gNB indicates UE to report two PHRs, Rel-15 criteria could be applied to determine the PHR for each TRP, and then Rel-17 M-TRP criteria could be applied for reporting two PHRs.

Proposal 2: The TP for 38.213 is shown below:

TS38.213 ### 7.1.1 UE behaviour <omitted> - For a PUSCH (re)transmission configured by ConfiguredGrantConfig, j=1, PO_NOMINAL,PUSCH,f,c1 is provided by p0-NominalWithoutGrant, or PO_NOMINAL,PUSCH,f,c1=PO_NOMINAL,PUSCH,f,c0 if p0-NominalWithoutGrant is not provided. - If the UE is provided two SRS resource sets in srs-ResourceSetToAddModList or srs-ResourceSetToAddModListDCI-0-2 with usage set to 'codebook' or 'nonCodebook', and is provided p0-PUSCH-Alpha2, for a retransmission of a configured grant Type 1 PUSCH, or for activation or retransmission of a configured grant Type 2 PUSCH, scheduled by a DCI format that includes a SRS resource set indicator field, and for active UL BWP b of carrier f of serving cell - If the SRS resource set indicator value is 00, first PO_UE_PUSCH,b,f,c1 value is provided by the value of p0-PUSCH-Alpha in ConfiguredGrantConfig. - If the SRS resource set indicator value is 01, second PO_UE_PUSCH,b,f,c1 value is provided by the value of p0-PUSCH-Alpha2 in ConfiguredGrantConfig. - If the SRS resource set indicator value is 10 or 11, first and second PO_UE_PUSCH,b,f,c1 values are respectively provided by the values of p0-PUSCH-Alpha and by p0-PUSCH-Alpha2 in ConfiguredGrantConfig. - else, PO_UE_NOMINAL,PUSCH,b,f,c1 is provided by p0 obtained from p0-PUSCH-Alpha in ConfiguredGrantConfig that provides an index P0-PUSCH-AlphaSetId to a set of P0-PUSCH-AlphaSet, or by p0-PUSCH for a PUSCH (re)transmission as described in clause 19.1, for active UL BWP b of carrier f of serving cell c

- References

- 3GPP RAN#86, RP-193133, “New WID: Further enhancements on MIMO for NR, December 2019.