3GPP TSG RAN WG1 #108-e																																																		  R1-2201852

e-Meeting, February 21st – March 3rd, 2022

Source:	CMCC

Title:	Remaining issues on timing relationship enhancements for NTN

Agenda Item:	8.4.1

Document for:	Discussion and Decision

Introduction

In RAN1#107-e meeting, agreements were achieved on timing relationship enhancements for NTN [1].

Agreement For defining value range(s) of K_offset, specify one value range of K_offset covering all scenarios. Agreement For defining value range(s) of K_mac, specify one value range of K_mac covering all scenarios. Agreement For determining UE specific K_offset - Option 2: MAC CE provides a differential UE specific K_offset value. The full UE specific K_offset value equals the cell specific K_offset value minus the differential UE specific K_offset value. - FFS: whether/how to resolve ambiguity of which cell-specific K_offset value to use during the SIB modification period Agreement 15 kHz is used as the reference subcarrier spacing value for the unit of TA reported in FR1. Agreement The reported TA is the least integer number of slots greater than or equal to the corresponding TA value. Proposal 3-A is endorsed as a basis to reply to RAN2 LS on TA pre-compensation (R1-2104230). Proposal 3-A: RAN1 to conclude the following as a basis to reply to RAN2: RAN1 inputs for the following RAN2 agreements (in italic) are given below: - Event-triggers for reporting on the information about UE specific TA in connected mode is supported. FFS on the details. Confirmation by RAN1 is also needed [RAN1]: RAN1 confirms that event-triggers for reporting on the information about UE specific TA in connected mode can be supported. - The event-triggers for reporting information about UE specific TA are based on TA values (confirmation from RAN1 is needed) [RAN1]: RAN1 confirms that the event-triggers for reporting information about UE specific TA can be based on TA values. - Under the work assumption "the UE location information cannot be reported in connected mode", the content of UE specific TA reported in connected mode is UE specific TA pre-compensation(for the details of the TA value, confirmation from RAN1 is needed). Under the work assumption "the UE location information can be reported in connected mode", for TA reporting purposes in connected mode, the network can configure the UE to send either the UE specific TA pre-compensation (for the details of the TA value, confirmation from RAN1 is needed) or the UE location information [RAN1]: RAN1 made the following further agreements for the details of the TA value: Agreement 15 kHz is used as the reference subcarrier spacing value for the unit of TA reported in FR1. Agreement The reported TA is the least integer number of slots greater than or equal to the corresponding TA value. R1-2112652 [DRAFT] LS reply on TA pre-compensation Moderator (Ericsson) Final LS endorsed in R1-2112766. Agreement The value range of cell specific K_offset is 0 – 1023 ms. Agreement The value range of K_mac is 1 – 512 ms. Agreement The value range of the differential UE specific K_offset provided in MAC CE is 0 – 63 ms. Agreement The K_offset value signaled in system information is always used for PDCCH ordered PRACH timing relationship. Agreement Adopt the following TP (38.213, 8.1): For a PRACH transmission triggered by a PDCCH order, the PRACH mask index field [5, TS 38.212], if the value of the random access preamble index field is not zero, indicates the PRACH occasion for the PRACH transmission where the PRACH occasions are associated with the SS/PBCH block index indicated by the SS/PBCH block index field of the PDCCH order. If UE is provided with KoffsetKoffset, for a PDCCH order received in downlink slot n, the available PRACH occasion is after uplink slot n+Koffsetn+Koffset. Note: Editor can make further adjustment as appropriate. Agreement On beam failure recovery procedure, for PRACH transmission in uplink slot n, UE monitors the corresponding PDCCH starting from downlink slot “n + K_mac + 4” within a corresponding RAR window.

In this contribution, we will discuss some remaining issues on timing related aspects for NTN.

Discussion

Issue #1: K_offset update

In RAN1#107-e meeting, the following agreement on MAC CE for updating UE specific K_offset was achieved, while whether/how to handle the ambiguity during the SIB modification period need some further discussions. Besides, note that there is an ambiguity period for MAC CE update as discussed in the past RAN1 meetings.

Agreement For determining UE specific K_offset - Option 2: MAC CE provides a differential UE specific K_offset value. The full UE specific K_offset value equals the cell specific K_offset value minus the differential UE specific K_offset value. - FFS: whether/how to resolve ambiguity of which cell-specific K_offset value to use during the SIB modification period

In our view, update of system information may be a very low frequent event. Although according to the specification, the SIB modification period may be configured with a minimum value of 640ms, NW may seldomly update it in the terrestrial network.

There was a concern in the last meeting that “there is nothing in specifications that forces a UE to read an updated SIB at a specific time when using the system information modification procedure. Also, the UE may not be able to decode the first instance of the updated SIB in the new SIB modification period (potentially due to being in poor coverage)”, nevertheless, considering SIB modification period is a very low frequent event, it is preferred to leave to gNB implementation, e.g., scheduling restriction, to address the ambiguity issue during the SIB modification period.

Proposal 1: To address the ambiguity issue during the SIB modification period, it is preferred to leave to gNB implementation, e.g., scheduling restriction.

On the other hand, we would like to suggest for more attention on the ambiguity period issue for MAC CE updating UE specific K_offset. In line with MAC CE signalling, update of UE specific K_offset seems not like a low frequency event.

Figure 1: Ambiguity period for MAC CE updating UE specific K_offset.

As shown in Figure 1, assume gNB send PDSCH carrying MAC CE to indicate differential UE-specific K_Offset in DL slot x, UE may send HARQ-ACK at UL slot n if it successfully decoded the PDSCH, and new MAC CE is applied after UL slot m, where, m = n + 3ms.

From gNB perspective,

- In Period 1, gNB assures that old UE specific K_offset is applied at UE side, where, Period 1 ends with x + T1, wherein,

T1=Koffsetold+k1+3ms−RTT−Kmac=Koffsetold+k1+3ms−TA

- In Period 3, gNB can assure whether new or old UE specific K_offset is applied at UE side, based on UE’s HARQ-ACK feedback corresponding to the PDSCH carrying MAC CE, where, Period 1 ends with x + T2, wherein,

T2=Koffsetold+k1+1slot+Kmac

- In Period 2, gNB can NOT assure whether new or old UE specific K_offset is applied at UE side, which depends on whether the PDSCH carrying MAC CE is successfully decoded or not. Note that the duration of Period 2 is

duration of P2=x+T2−x+T1=Koffsetold+k1+1slot+Kmac−Koffsetold+k1+3ms−TA=1slot+Kmac−3ms+TA≈RTT

As discussed above, the duration of the ambiguity period is about RTT. In order to address the ambiguity period issue, we suggest to support cell-specific K_offset can be always used for the additional transmission timings related to fallback DCI format. In this case, gNB can schedule UL transmission in the ambiguity period via fallback DCI. As a result, cell-specific K_offset may be used in the ambiguity period up to network implementation to eliminate the uncertainty.

Observation 1: For MAC CE updating UE specific K_offset, there may be an ambiguity period with a duration about RTT. In this ambiguity period, gNB can NOT assure whether new or old UE specific K_offset is applied at UE side.

Proposal 2: In order to address the ambiguity period issue for MAC CE updating UE specific K_offset, support if cell-specific K_offset is always used for the additional transmission timings related to fallback DCI format, use fallback DCI in the ambiguity period.

Issue #2: K_offset usage

In RAN1#105-e meeting, several agreements were achieved on K_offset usage issues [2].

Agreement: The K_offset value signaled in system information is always used for - The transmission timing of RAR / fallbackRAR grant scheduled PUSCH - The transmission timing of Msg3 retransmission scheduled by DCI format 0_0 with CRC scrambled by TC-RNTI - The transmission timing of HARQ-ACK on PUCCH to contention resolution PDSCH scheduled by DCI format 1_0 with CRC scrambled by TC-RNTI - FFS: The transmission timing of HARQ-ACK on PUCCH to contention resolution PDSCH scheduled by DCI format 1_0 with CRC scrambled by C-RNTI - The transmission timing of HARQ-ACK on PUCCH to MsgB scheduled by DCI format 1_0 with CRC scrambled by MsgB-RNTI - FFS: The transmission timing of HARQ-ACK on PUCCH to MsgB scheduled by DCI format 1_0 with CRC scrambled by C-RNTI FFS: how to treat additional transmission timings related to fallback DCI formats FFS: how to update this formulation with beam-specific K_offset if beam-specific K_offset is agreed to be supported

Regarding the “FFS: how to treat additional transmission timings related to fallback DCI formats”, there was a discussion in the RAN1 #106-e meeting on the following two options [3].

- Option 1: Always use the cell-specific K_offset (i.e., the K_offset value signaled in system information)

- Option 2: UE-specific K_offset if provided (otherwise, use the cell-specific K_offset)

As discussed in Issue #1, in order to address the ambiguity period issue for MAC CE updating UE specific K_offset, we prefer that if cell-specific K_offset is always used for the additional transmission timings related to fallback DCI format, use fallback DCI in the ambiguity period. To enable Option 2, it is preferred to support always use the cell-specific K_offset for the additional transmission timings related to fallback DCI format.

Proposal 3: Support always use the cell-specific K_offset for the additional transmission timings related to fallback DCI format.

Issue #3: Beam-specific K_offset in initial access

In RAN1#103-e meeting, a cell specific K_offset configuration was agreed, while beam specific K_offset was left FFS [4].

Agreement: - For K_offset configured in system information and used in initial access, at least a cell specific K_offset configuration, which is used in all beams of a cell, should be supported. - FFS: Beam specific K_offset configured in system information and used in initial access.

We prefer the beam specific K_offset with the following consideration.

- Beam specific K_offset is beneficial for finer granularity.

- Compared to repeating a list of K_offset values across beams, beam specific SIB, which is only applicable for NTN, can be further considered to reduce signaling overhead. The specification impact to support beam specific SIB seems minimal.

  - If beam-specific K_offset is carried in SIB1,

    - Support beam specific SIB1.

    - UE behavior (no change): Note that each Type0-PDCCH CSS is associated with a SSB. Then PDCCH for beam specific SIB1 is inherently beam specific, and UE can determine it according to its associated SSB.

  - If beam-specific K_offset is carried in other SIBs except SIB1,

    - Add beamScope field in SIB-TypeInfo IE (TS 38.331)

- SIB-TypeInfo ::= SEQUENCE { - type ENUMERATED {sibType2, sibType3, sibType4, sibType5, sibType6, sibType7, sibType8, sibType9, - sibType10-v1610, sibType11-v1610, sibType12-v1610, sibType13-v1610, sibType14-v1610, - spare3, spare2, spare1,... }, - valueTag INTEGER (0..31) OPTIONAL, -- Cond SIB-TYPE - areaScope ENUMERATED {true} OPTIONAL -- Need S - beamScope ENUMERATED {true} OPTIONAL -- Need S - }

    - UE behavior (minor change): UE can determine PDCCH monitoring occasion (s) associated with a given SSB from current specification. To support beam-specific SIBs except SIB1, specification can be enhanced to restrict UE only monitoring in PDCCH monitoring occasion (s) associated with its SSB.

[Section 5.2.2.3.2, TS 38.331] The [x×N+K]th PDCCH monitoring occasion (s) for SI message in SI-window corresponds to the Kth transmitted SSB, where x = 0, 1, ...X-1, K = 1, 2, …N, N is the number of actual transmitted SSBs determined according to ssb-PositionsInBurst in SIB1 and X is equal to CEIL(number of PDCCH monitoring occasions in SI-window/N). The actual transmitted SSBs are sequentially numbered from one in ascending order of their SSB indexes. The UE assumes that, in the SI window, PDCCH for an a beam specific SI message is transmitted in at least one PDCCH monitoring occasion corresponding to each transmitted SSB and thus the selection of SSB for the reception SI messages is up to UE implementation it is not transmitted in a PDCCH monitoring occasion corresponding to SSBs other than the transmitted one.

Proposal 4: gNB has the flexibility of configuring cell-specific or beam specific value of K_offset.

- Beam specific SIB can be supported, i.e., different beam specific SIB may carry different beam specific values (e.g., K_offset).

Conclusions

In this contribution, we share our views on related issues on timing relationship enhancements for NTN. The observations and proposals are summarized as follows:

Observation 1: For MAC CE updating UE specific K_offset, there may be an ambiguity period with a duration about RTT. In this ambiguity period, gNB can NOT assure whether new or old UE specific K_offset is applied at UE side.

Proposal 1: To address the ambiguity issue during the SIB modification period, it is preferred to leave to gNB implementation, e.g., scheduling restriction.

Proposal 2: In order to address the ambiguity period issue for MAC CE updating UE specific K_offset, support if cell-specific K_offset is always used for the additional transmission timings related to fallback DCI format, use fallback DCI in the ambiguity period.

Proposal 3: Support always use the cell-specific K_offset for the additional transmission timings related to fallback DCI format.

Proposal 4: gNB has the flexibility of configuring cell-specific or beam specific value of K_offset.

- Beam specific SIB can be supported, i.e., different beam specific SIB may carry different beam specific values (e.g., K_offset).

References

- Chairman's Notes RAN1#107-e, November 11th – 19th, 2021.

- Chairman's Notes RAN1#105-e, May 10th – 27th, 2021.

- R1- 2108555, “Feature lead summary#5 on timing relationship enhancements”, e-Meeting, August 16th – 27th, 2021.

- Chairman's Notes RAN1#103-e, October 26th – November 13th, 2020.