3GPP TSG RAN WG1 #108-e			R1-2201857

e-Meeting, February 21st – March 3rd, 2022

Agenda item:	8.5.2

Source: 	CMCC

Title: 	Remaining issues on UL-AoA enhancements

Document for:	Discussion and Decision

Introduction

On the issue of supporting of multiple SRS-RSRP values together with multiple UL-AoA values associated with the first arrival path, no consensus was reached during the last RAN1 meeting. In the maintenance phase, this remaining issue needs to be wrapped up, and our views are provided in this contribution.

Remaining issues on UL-AoA enhancements

In RAN1#106b-e meeting, the following agreement was made on UL-AoA reporting enhancements [1]:

Agreement: - For the first arrival path measurements on SRS for positioning resource, - gNB can report to LMF the following set of measurements {one SRS-RSRP, multiple UL-AOAs (AoA/ZoA pairs), one UL-RTOA} - gNB can report to LMF the following set of measurements {one SRS-RSRP, multiple UL-AOAs (AoA/ZoA pairs), one-gNB Rx-Tx time difference} - FFS additional option: gNB can report to LMF the following set of measurements {multiple SRS-RSRP, multiple UL-AOAs (AoA/ZoA pairs), one UL-RTOA, one-gNB Rx-Tx time difference} - All gNB measurements above are associated with SRS resource ID and timestamp, which are also reported to LMF - For the first arrival path measurements on SRS for MIMO resource, - gNB can report to LMF the following set of measurements {one SRS-RSRP, multiple UL-AOAs (AoA/ZoA pairs), one UL-RTOA} - FFS: gNB can report to LMF the following set of measurements {multiple SRS-RSRP, multiple UL-AOAs (AoA/ZoA pairs), one UL-RTOA} - All gNB measurements above are associated with SRS resource ID and timestamp, which are also reported to LMF - Note: The operation of SRS for MIMO is transparent to the UE

During the discussion, companies shared diverse views on whether to support reporting of multiple first path SRS-RSRP values together with multiple UL-AoA values and one timing measurement (UL-RTOA or gNB Rx-Tx time difference). In addition, for measurements on SRS for positioning resources, another FFS point was whether to allow the gNB to include both UL-RTOA and gNB Rx-Tx time difference in the measurement set.

In RAN1#107-e meeting, the first remaining issue above was further discussed and the following two intermediate alternatives were proposed [2]:

Proposal 3.4-2 - Select one of the following alternatives - Alt.1 - For the first arrival path measurements on SRS for positioning resource, gNB can report to LMF multiple sets of the following measurement tuples {one SRS-RSRPP, multiple UL-AOAs (AoA/ZoA pairs), one UL-RTOA or one gNB Rx-Tx time difference} - For the first arrival path measurements on SRS for MIMO resource, gNB can report to LMF multiple sets of the following measurement tuples {one SRS-RSRPP, multiple UL-AOAs (AoA/ZoA pairs), one UL-RTOA or one-gNB Rx-Tx time difference} - Alt.2 - For the first arrival path measurements on SRS for positioning resource, gNB can report to LMF multiple sets of the following set of measurements {multiple SRS-RSRPP, multiple UL-AOAs (AoA/ZoA pairs), one UL-RTOA or one gNB Rx-Tx time difference} - For the first arrival path measurements on SRS for MIMO resource, gNB can report to LMF multiple sets of the following set of measurements {multiple SRS-RSRPP, multiple UL-AOAs (AoA/ZoA pairs), one UL-RTOA or one-gNB Rx-Tx time difference} - Above gNB measurements are associated with SRS resource ID and timestamp, which are also reported to LMF

The main bullets of the above two alternatives allow the gNB to report multiple sets of measurements to the LMF, which we believe is within RAN3 scope. From RAN1 perspective, we should focus on whether the gNB can report multiple SRS-RSRP values together with multiple UL-AoAs and one timing measurement. For Alt. 1, though it was clarified that the intention was to support reporting {multiple path-RSRPPs, multiple AoA, single TOA} by reporting of multiple sets of {single path-RSRP, multiple AoA, single TOA}, we don’t think it is equal to support {multiple SRS-RSRPPs, multiple UL-AoAs, single ToA}. In our views, at a given TOA, there would be multiple angles associated with different power, and hence the gNB should be allowed to report multiple SRS-RSRPPs in such a case to the LMF.

Proposal 1:

    - For the first arrival path measurements on SRS for positioning resource, gNB can report to LMF the following measurement tuples {multiple SRS-RSRPPs, multiple UL-AOAs (AoA/ZoA pairs), one UL-RTOA or one gNB Rx-Tx time difference};

    - For the first arrival path measurements on SRS for MIMO resource, gNB can report to LMF the following measurement tuples {multiple SRS-RSRPPs, multiple UL-AOAs (AoA/ZoA pairs), one UL-RTOA or one-gNB Rx-Tx time difference}.

In addition, there is another remaining issue on whether gNB can report both UL-RTOA and gNB Rx-Tx time differences. From our perspective, there seems no additional information or significant benefits that the LMF can obtain if both RTOA and gNB Rx-Tx time difference are reported by the gNB. Therefore, we propose that:

Proposal 2: For the first arrival path measurements on SRS for positioning resource, do NOT support gNB to report to LMF the following set of measurements {multiple SRS-RSRP, multiple UL-AOAs (AoA/ZoA pairs), one UL-RTOA, one-gNB Rx-Tx time difference}.

Conclusions

In this contribution, we provide our views on the remaining issues on UL-AoA reporting enhancements, and the following proposals are made:

Proposal 1:

    - For the first arrival path measurements on SRS for positioning resource, gNB can report to LMF the following measurement tuples {multiple SRS-RSRPPs, multiple UL-AOAs (AoA/ZoA pairs), one UL-RTOA or one gNB Rx-Tx time difference};

    - For the first arrival path measurements on SRS for MIMO resource, gNB can report to LMF the following measurement tuples {multiple SRS-RSRPPs, multiple UL-AOAs (AoA/ZoA pairs), one UL-RTOA or one-gNB Rx-Tx time difference}.

Proposal 2: For the first arrival path measurements on SRS for positioning resource, do NOT support gNB to report to LMF the following set of measurements {multiple SRS-RSRP, multiple UL-AOAs (AoA/ZoA pairs), one UL-RTOA, one-gNB Rx-Tx time difference}.

References

- Draft report of 3GPP RAN1#106bis-e meeting v0.2.0.

- Draft report of 3GPP RAN1#107-e meeting v0.1.0