3GPP TSG RAN WG1 #108-e																																																	   	R1-2201881

e-Meeting, February 21st – March 3rd, 2022

Source:	CMCC

Title:	Remaining issues on timing relationship enhancements for IoT NTN

Agenda Item:	8.14.2

Document for:	Discussion and Decision

Introduction

In RAN1#107-e meeting, agreements were achieved on time and frequency synchronization for IoT NTN [1].

Agreement For IoT NTN, signalling one value for cell-specific K_offset in system information is supported. Agreement For IoT NTN, the unit of K_offset is subframe based on a 15kHz subcarrier spacing (i.e. 1 ms). - Further discuss the case where UL is using 3.75 kHz SCS Agreement For IoT NTN, the UE specific K_offset is provided and updated by the network using MAC CE. Agreement For IoT NTN, the information of K_mac is carried in system information. Agreement For IoT NTN, the unit of K_mac is subframe based on a 15kHz subcarrier spacing (i.e. 1 ms). - Further discuss the case where UL is using 3.75 kHz SCS Agreement Modification of the designation of subframes with NPDCCH monitoring restrictions is needed for at least Cases 1 to 6. Agreement Whether/how the “indicated value” of K_offset is translated into number of slots for different numerologies (i.e., 15 kHz and 3.75 kHz) is left to the spec-editor. - This resolves the bullet from previous agreement: Further discuss the case where UL is using 3.75 kHz SCS Agreement For IoT NTN, adopt the NR NTN agreement without modification for FR1: (a) the value range (i.e. 1 ms), (b) the quantity signalled (e.g. a differential UE specific K_offset) for the UE specific K_offset. Agreement For IoT NTN, adopt the NR NTN agreement without modification for FR1 for the value range of Kmac. Leave it to spec editor to formulate in the specs the NPDCCH monitoring restrictions for Cases 1 to 6. Explanatory Note for editor When the UE changes from receiving on the DL to transmitting on the UL (or vice versa), immediately before/after the DL/UL switch the UE is not required to monitor an NPDCCH candidate in some DL subframes. The designation of these subframes in the spec needs to take the “effect” of the TA into consideration. There may be multiple ways to capture this in the specifications for (at least) Cases 1 to 6. Two options (in principle) are described below, to guide the spec editor to capture this as best he/she sees it. Examples of where the changes may apply for cases 1 to 6 can be found as examples in appendix A in R1-2112554. Option 1: The DL subframes during which the UE is not required to monitor an NPDCCH candidate are described in terms of downlink subframe timing. This would typically involve inserting a “-TA” term in their indexing. Option 2: The DL subframes during which the UE is not required to monitor an NPDCCH candidate are described in terms of uplink subframe timing using the indexing of the UL subframes that coincide in time with the DL subframes in question. Agreement Network can configure UE-specific TA reporting either a TA or UE location for connected mode UE - In case a TA is configured, NR NTN solutions are a baseline for the following UE-specific TA handling issues, - Signaling – quantity (full or delta), range, number of bits - Granularity of report - Frequency of reporting - Means of reporting - NOTE: Any changes needed for IoT NTN can be made. - In case the UE location is configured, RAN2 will design solutions for the UE location information, and it is left to RAN2 to decide whether to support UE location reporting

In this contribution, we will discussion on timing relationship enhancements for IoT over NTN.

Discussion

Outstanding Timing Relationships for IoT NTN

PRACH Preamble Retransmission

In RAN1#106b-e meeting, agreements were achieved on PRACH preamble retransmission [2]:

Agreement: For eMTC in IoT NTN, if the UE determines that a preamble retransmission is necessary, the choice of a suitable preamble retransmission subframe shall be delayed by Koffset as compared to current specifications.

Nevertheless, there was no consensus on NB-IoT in IoT NTN according to the discussion in the RAN1#107-e meeting. According to the specification description for PRACH preamble retransmission for NB-IoT in IoT NTN,

From section 16.3.2 of TS 36.213, it says for NB-IoT: b) If a random access response is received and the corresponding DL-SCH transport block ending in subframe n does not contain a response to the transmitted preamble sequence, the UE shall, if requested by higher layers, be ready to transmit a new preamble sequence no later than the NB-IoT UL slot starting 12 milliseconds after the end of subframe n. c) If no NPDCCH scheduling random access response is received in subframe n, where subframe n is the last subframe of the random access response window, the UE shall, if requested by higher layers, be ready to transmit a new preamble sequence no later than the NB-IoT UL slot starting 12 milliseconds after the end of subframe n. d) If an NPDCCH scheduling random access response with associated RA-RNTI is detected and the corresponding DL-SCH transport block reception ending in subframe n cannot be successfully decoded, the UE shall, if requested by higher layers, be ready to transmit a new preamble sequence no later than the NB-IoT UL slot starting 12 milliseconds after the end of subframe n.

It can be observed that the time limit of 12ms is an absolute time, and the subframe n should refer to a DL subframe, thus, additional delayed by K_offset seems not needed for NB-IoT in IoT NTN.

Proposal 1: For NB-IoT in IoT NTN, if the UE determines that a preamble retransmission is necessary, further enhancement on the timing relationship as compared to current specifications is not needed.

PDCCH Order to PRACH

In RAN1#107-e meeting, there was a discussion on primacy of UE-specific K_offset [3], and we support that the solutions achieved in NR NTN with respect to which case K_offset is used can be re-used for IoT NTN. Therefore, cell-specific K_offset for PDCCH ordered PRACH is preferred, since in the case that PDCCH order RACH functionality is applied, UE-specific K_offset may be uncertain.

Proposal 2: Support cell-specific K_offset in the enhanced PDCCH ordered PRACH timing relationship. If UE is provided with K_offsetKoffset, for a PDCCH order received in downlink slot n, the available PRACH occasion is after uplink slot n+K_offsetn+Koffset.

WUS Configuration

There was a discussion on WUS configuration in RAN1#106b-e meeting [4].

For RAN1 perspective, there seems no enhancement on WUS configuration is needed. Regarding the proposal of extending the time interval between WUS and PO, we think UE does not need to perform GNSS measurement between WUS and PO. In fact, UE can perform GNSS measurement after PO, as discussed in our company’s contribution [5].

Observation 1: For sporadic DL traffic, UE may perform GNSS measurements after a paging occasion and only if it has been paged to reduce battery consumption. The existing timers (e.g., T3413/T3415) can be configured large enough to ensure a sufficient gap to accommodate GNSS acquisition after decoding the paging message and before initiating UL transmission. Proposal 1: Support the following conclusion. - Acquisition of GNSS position fix during paging procedure is up to UE implementation and network configuration of paging timers considering GNSS measurement duration (e.g. GNSS Time To First Fix with cold start of typically 10 seconds) impact in NTN scenario. These paging timers are not specified in 3GPP in legacy paging procedure (i.e. T3413 / T3415).

Proposal 3: Deprioritize further enhancement on WUS configuration.

Conclusions

In this contribution, we share our views on related issues on timing relationship enhancements for IoT over NTN. The observations and proposals are summarised as follows:

Proposal 1: For NB-IoT in IoT NTN, if the UE determines that a preamble retransmission is necessary, further enhancement on the timing relationship as compared to current specifications is not needed.

Proposal 2: Support cell-specific K_offset in the enhanced PDCCH ordered PRACH timing relationship. If UE is provided with K_offsetKoffset, for a PDCCH order received in downlink slot n, the available PRACH occasion is after uplink slot n+K_offsetn+Koffset.

Proposal 3: Deprioritize further enhancement on WUS configuration.

References

- Chairman's Notes RAN1#107-e, November 11th – 19th, 2021.

- Chairman's Notes RAN1#106bis-e, October 11th – 19th, 2021.

- R1-2112554, “FL summary 3 of AI 8.15.2: Timing relationships for IoT-NTN”, e-meeting, November 11th-19th, 2021.

- R1-2110534, “FL summary 4 of AI 8.15.2 Timing relationship for IoT-NTN”, e-meeting, October 11th – 19th, 2021.

- R1-2201880, “Remaining issues on enhancements to time and frequency synchronization for IoT NTN”, e-meeting, February 21st – March 3rd, 2022.