3GPP TSG RAN WG1 #108-e			R1-2201883

e-Meeting, February 21st – March 3rd, 2022

Agenda item:	8.16.5

Source: 	CMCC

Title: 	Discussion on UE features for NR positioning enhancement

Document for:	Discussion and Decision

Introduction

In RAN1#107-e meeting, the agreements listed in Section 6 of R1-2111810 was endorsed [1]. In this contribution, we provide our views on some of the remaining issues.

Discussion

## FG 27-1-1

| 27. NR_pos_enh | 27-1-1 | Maximum number Support of UE-RxTEGs [for UE-assisted DL TDOA and/or Multi-RTT positioning] | The maximum number of UE-RxTEG, which is supported and reported by UE [for UE assisted DL TDOA and/or Multi-RTT positioning] FFS: the values (>1) FFS: whether to have a value=1 to indicate UE Rx timing errors is well calibrated FFS: whether to have separate values/FGs for DL TDOA and/or Multi-RTT positioning [If UE supports this capability with the values > 1, the UE supports including one UE Rx TEG ID for the RSTD reference time and one UE Rx TEG ID for each DL RSTD measurement (including each additional DL RSTD measurement), in a DL TDOA measurement report] | 13-1 [,one or more of {13-3, 13-4}], 27-1-1b | No |  | Mitigation of UE Rx timing delays is not supported UE-RxTEG reporting is not supported and no assumption can be made on the mitigation of UE Rx timing delays for the measurements | FFS: Per UE or [per band or FS] | n/a | n/a | n/a | [The candidate values are {[1,]2[, 3],4,6,8[,12,16,24,32]}] Need for location server to know if the feature is supported FFS: Separate row for “Support of UE-RxTEG reporting for DL-TDOA”, and “Support of UE-RxTEG reporting for M-RTT” If UE supports this capability with the values > 1, and if the UE does not include RxTEG-ID associated with a measurement, no assumption can be made on the mitigation of UE Rx timing delays for this measurement [If value=1 is indicated by the UE, the UE Rx timing errors differences between two measurements are within a margin only if the UE reports an the same Rx-TEG-ID associated with both the measurements, otherwise, no assumption can be made about the timing error differences between these measurements.] Note: The “per band” reporting on this capability does not imply, that the RxTEG IDs in the measurement report are grouped per band; In the measurement report, the RxTEG ID can span from 0, up to 31 | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

For mitigating UE Tx/Rx timing errors for DL+UL positioning, it was agreed that subject to UE capability, either one or both of the following options are supported:

- Option 1: Reporting of a TRP RxTx TEG ID, and optionally a TRP Tx TEG ID

- Option 2: Reporting of a TRP Rx TEG ID and a TRP Tx TEG ID

A UE may only be capable of reporting TRP RxTx TEG ID. In this sense, we prefer to split two rows for “Support of UE TxTEGs for DL-TDOA” and “Support of UE TxTEGs for Multi-RTT”.

Proposal 1: Split two rows for “Support of UE TxTEGs for DL-TDOA” and “Support of UE TxTEGs for Multi-RTT”.

## FG 27-1-2

| 27. NR_pos_enh | 27-1-2 | Maximum number Support of UE-TxTEGs [for UL TDOA and/or Multi-RTT positioning] | The maximum number of UE-TxTEG, which is supported and reported by UE [for UL TDOA and/or Multi-RTT positioning] FFS: the values (>1) FFS: whether to have a value=1 to indicate UE Tx timing errors is well calibrated FFS: whether to have different values/FGs for UL TDOA and/or Multi-RTT positioning FFS: Separate row for “Support of UE-TxTEG reporting for MRTT”, and a separate row for the “maximum number of TxTEGs for RTT” [If UE supports this capability with the values > 1, the UE supports to provide the association information of UL SRS resources for positioning with Tx TEGs to the LMF. - FFS: Whether the association information is sent directly from UE to LMF, or is first provided to gNB and then forwarded to LMF] | [13-4, ] 13-8], 27-1-2b | FFS Yes |  | Mitigation of UE Tx timing delays is not supported UE-TxTEGs for UL TDOA is not supported and no assumption can be made on the mitigation of UE Tx timing for the SRS” and “UE-TxTEGs for RTT is not supported and no assumption can be made on the mitigation of UE Tx timing for the SRS | FFS: Per UE or per band or per FS | n/a | n/a | n/a | [The candidate values are {[1,] 2, 4, 6, 8, 16, 32}] [Need for location server to know if the feature is supported] Note: It should support the serving gNB to request the UE to provide the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB for UL TDOA [if UL TDOA is supported by UE] [Note: It should support the LMF to request the UE to provide the association information of UL SRS resources for positioning with Tx TEGs directly to the LMF for Multi-RTT if Multi-RTT is supported by UE] | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 27-1-2a | Support of UE-TxTEGs for Multi-RTT [and/or UL TDOA] positioning | The maximum number of UE-TxTEG, which is supported and reported by UE for Multi-RTT positioning | 13-4, 13-8 | No |  | UE-TxTEGs for Multi-RTT positioning is not supported | [per band per FS] | n/a | n/a | n/a | The candidate values are {[1, ] 2, 4, 6, 8} Need for location server to know if the feature is supported If UE supports this capability with the values > 1, and if if the UE does not include TxTEG-ID associated with a measurement, no assumption can be made on the mitigation of UE Tx timing delays for this SRS resource [If value=1 is indicated by the UE, the UE Tx timing errors differences between two SRS resources are within a margin only if the UE reports an Tx-TEG-ID associated with the SRS resources, otherwise, no assumption can be made about the timing error differences between these SRS resources.] [Note: It should support the serving gNB to request the UE to provide the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB for UL TDOA if UL TDOA is supported by UE] [Note: It should support the LMF to request the UE to provide the association information of UL SRS resources for positioning with Tx TEGs directly to the LMF for Multi-RTT if Multi-RTT is supported by UE] | Optional with capability signaling |

The mechanisms of reporting Tx TEGs for UL-TDOA and Multi-RTT are different, and when both UL-TDOA and Multi-RTT, or UL-TDOA and DL-TDOA are used, the UE should provide the association information of UL SRS resources for positioning with Tx TEGs, the UE should report the Tx TEG to the entity that initiates the request. Hence, for UL-TDOA, the LMF also needs to know if the feature is supported.  The following notes should be kept in both rows:

- Need for location server to know if the feature is supported.

- Note (for FG 27-1-2): It should support the serving gNB to request the UE to provide the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB for UL TDOA [if UL TDOA is supported by UE]

- [Note (for FG 27-1-2a): It should support the serving gNB to request the UE to provide the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB for UL TDOA]

- [Note: It should support the LMF to request the UE to provide the association information of UL SRS resources for positioning with Tx TEGs directly to the LMF for Multi-RTT if Multi-RTT is supported by UE]

Proposal 2:  The following notes should be kept in both rows of FG 27-1-2 and FG 27-1-2a.

- Need for location server to know if the feature is supported.

- Note (for FG 27-1-2): It should support the serving gNB to request the UE to provide the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB for UL TDOA [if UL TDOA is supported by UE]

- [Note (for FG 27-1-2a): It should support the serving gNB to request the UE to provide the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB for UL TDOA]

- [Note: It should support the LMF to request the UE to provide the association information of UL SRS resources for positioning with Tx TEGs directly to the LMF for Multi-RTT if Multi-RTT is supported by UE]

## FG 27-1-4

| 27. NR_pos_enh | 27-1-4 | The maximum Number Support of UE Rx TEGs for measuring the same DL PRS resource | The maximum number of different UE-RxTEGs that a UE can support to measure the same DL PRS of a TRP. FFS: The values (>1) | 27-1-1 | No |  | Mitigation of UE Rx timing delays by using different Rx TEGs are not supported Up to 1 RxTEG is used to measure the same DL PRS resource of a TRP | FFS: Per UE or per band | n/a | n/a | n/a | [The candidate values are {1, 2, 3, 4, 6, 8}] Need for location server to know if the feature is supported [Note: UE measures the same instance of the DL PRS with multiple RX TEGs] FFS: Separate row for “Support measuring the same DL PRS of a TRP with different UE-RxTEGs” FFS: Separate row for “The maximum Number of UE Rx TEGs for measuring the same DL PRS resource simultaneously” | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 27. NR_pos_enh | 27-1-4a | Support of UE Rx TEGs for measuring the same DL PRS resource simultaneously | The maximum number of UE Rx TEGs for measuring the same DL PRS resource simultaneously |  | No |  |  | Per band | n/a | n/a | n/a | [The candidate values are {1,2, 4, 8}] | Optional with capability signaling |

If a UE has multiple Rx TEGs, to mitigate the timing error difference between different Rx TEGs, the UE can be requested to optionally measure the same DL PRS resource of a TRP with N different UE Rx TEGs and report the corresponding multiple RSTD measurements. The timestamp of the multiple RSTD measurements in the same measurement report can be the same or different. It indicates that the UE may be capable of measuring the same DL PRS resource with N different Rx TEGs at the same time, or the UE can only measure the same DL PRS resource from different repetitions or occasions. Therefore, a separate capability of simultaneously measure the same DL PRS resource with different Rx TEGs is required.

Proposal 3: Keep the FG 27-1-4a.

## FG 27-2-1

| 27. NR_pos_enh | 27-2-1 | [UE-assisted] DL PRS RSRP of the first [or additional] path [for DL-AoD] | 1.) Support of [measuring and reporting the] PRS RSRP of the first path [or additional] [for DL-AoD positioning method] 2.) The maximum number of first path PRS RSRP per TRP [Note: Applicable for DL-TDOA and Multi-RTT] | [13-2 or 13-3], 13-4, 13-5, 13-8] | No |  | PRS RSRP of the first path is not supported | FFS: Per UE or per band | n/a | n/a | n/a | Component 2 candidate values: [2,4,8,16,24] Need for location server to know if the feature is supported | Optional with capability signaling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Since no DL PRS RSRP measurement is required to be reported to the LMF for UE-based positioning, the “UE-assisted” should be kept in the name of the FG.

Proposal 4: Keep “UE-assisted” in the name of the FG 27-2-1.

Conclusions

In this contribution, we provide our comments on some remaining issues of UE features for positioning enhancements, and the following proposal were made:

Proposal 1: Split two rows for “Support of UE TxTEGs for DL-TDOA” and “Support of UE TxTEGs for Multi-RTT”.

Proposal 2:  The following notes should be kept in both rows of FG 27-1-2 and FG 27-1-2a.

- Need for location server to know if the feature is supported.

- Note (for FG 27-1-2): It should support the serving gNB to request the UE to provide the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB for UL TDOA [if UL TDOA is supported by UE]

- [Note (for FG 27-1-2a): It should support the serving gNB to request the UE to provide the association information of UL SRS resources for positioning with Tx TEGs to the serving gNB for UL TDOA]

- [Note: It should support the LMF to request the UE to provide the association information of UL SRS resources for positioning with Tx TEGs directly to the LMF for Multi-RTT if Multi-RTT is supported by UE]

Proposal 3: Keep the FG 27-1-4a.

Proposal 4: Keep “UE-assisted” in the name of the FG 27-2-1.

References

- R1-2111810, Summary of UE features for NR positioning enhancements, Moderator (AT&T), 3GPP RAN1#107-e.