3GPP TSG RAN WG1 #108-e			                          R1-2201885

e-Meeting, February 21st – March 3rd, 2022

Source: 	CMCC

Title:	Discussion on UE features for UE power saving enhancements

Agenda item:	8.16.7

Document for:	Discussion & Decision

- Introduction

There were some discussions about UE features for UE power saving enhancements, but still some FFS in FL’s summary [1], we will further discuss the remaining FFS parts in this contribution.

- Discussion

- 29-1: Paging enhancement

One of remaining issues is that whether the type of FG 29-1 should be per UE or per band. From our point of view, we think per UE is reasonable, since whether the UE is a power consumption sensitive UE is independent of band categories, it makes no sense that one UE needs power saving enhancement in one band but not need in another one.

Proposal 1. The type of FG 29-1 should be per UE.

- 29-2: TRS resources for idle/inactive UEs

The similar issue is as paging enhancement that whether the type of FG 29-2 should be per UE or per band. As the discussion above, we also thin this feature should also be per UE not per band.

Proposal 2. The type of FG 29-2 should be per UE.

Another issue is that whether the feature is mandatory or optional. There were some discussions in last meeting, whether this should be left to RAN2 decision. Considering this feature is a RAN1-led feature, to make a decision in RAN1 is more reasonable. Because this feature is applied for RRC_IDLE/IANCTIVE UEs without UE capability report, we think the FG 29-2 should be optional without capability signaling.

Proposal 3. The signaling of FG 29-2 should be optional without capability signaling.

There were some discussions about the clarification the L1 signalling for TRS availability indication in the components and whether to put FG 29-1 as the prerequisite feature group to FG 29-2. In last RAN1 meeting, it was agreed that both PEI and paging DCI should convey the TRS availability indication.

Agreement

If SIB configures TRS resource, TRS availability indication field is present in DCI format 2_7 (if configured) with CRC scrambled by PEI-RNTI and DCI format 1_0 with CRC scrambled by P-RNTI

For UE doesn’t support FG 29-1, it can only receive the TRS availability indication in paging DCI. For UE supports FG 29-1, it can receive the TRS availability indication both in paging DCI and PEI. From this point of view, it is not reasonable to put FG 29-1 as the prerequisite feature group to the whole FG 29-2. An additional note to clarify UE can receive L1 indication for TRS availability when supporting PEI can be adopted for FG 29-2.

Proposal 4. FG 29-2 is updated as follows:

| 29. NR_UE_pow_sav_enh | 29-2 | TRS resources for idle/inactive UEs | TRS occasions for idle/inactive UEs 1. Support reading TRS configuration from SIB 2. Support receiving L1 indication for TRS availability |  | N |  | UE cannot receive TRS resources for idle/inactive mode | Per UE | N | N | N | Receiving L1 indication via DCI format 2_7 is supported only if the UE supports FG 29-1 | Optional without capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

- 29-3: PDCCH monitoring adaptation within an active BWP

Similar issue as the above two feature groups, we also think FG 29-3a/29-3b/29-3c/29-3d should also be per UE.

Proposal 5. The type of FG 29-3a/29-3b/29-3c/29-3d should be per UE.

- Conclusions

In this contribution, remaining issues UE features for UE power saving enhancements are discussed and the following proposals are made.

Proposal 1. The type of FG 29-1 should be per UE.

Proposal 2. The type of FG 29-2 should be per UE.

Proposal 3. The signaling of FG 29-2 should be optional without capability signaling.

Proposal 4. FG 29-2 is updated as follows:

| 29. NR_UE_pow_sav_enh | 29-2 | TRS resources for idle/inactive UEs | TRS occasions for idle/inactive UEs 1. Support reading TRS configuration from SIB 2. Support receiving L1 indication for TRS availability |  | N |  | UE cannot receive TRS resources for idle/inactive mode | Per UE | N | N | N | Receiving L1 indication via DCI format 2_7 is supported only if the UE supports FG 29-1 | Optional without capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Proposal 5. The type of FG 29-3a/29-3b/29-3c/29-3d should be per UE.

- References

- R1-2200678	Summary on UE features for UE power saving enhancements	Moderator (NTT DOCOMO, INC.)