3GPP TSG-RAN WG1 Meeting#108-e                                       R1-2201983

E-meeting, February 21st – March 3rd, 2022

Title:	[Draft] LS Reply to RAN2 on Inter-UE coordination

Response to:	R1-2200880/R2-2201809

Release:	Rel-17

Work Item:	NR_SL_enh_Core

Source:	RAN1

To:	RAN2

Cc:

Contact Person:

Name:	Emad Farag

E-mail Address:	e.farag@samsung.com

# Overall description

RAN1 would like to thank RAN2 for the LS on “Inter-UE coordination”. In the LS, identified issues on which RAN2 mainly relies on RAN1. In this reply LS, we provide feedback on these issues

1: 	Inter-UE coordination (IUC) issues (on which) RAN2 mainly relies on RAN1:

- HARQ retransmission number for inter-UE coordination information

- Information and length of information of IUC MAC CE. The information indicated in RAN1 LS should be taken into account as baseline.

- UE-B procedure (e.g. final selection of resources) to the (non-)preferred resource set in IUC

- Scheme 2 inter-UE coordination design

- Condition for the UE-A to transmit IUC

- Signaling design and trigger conditions for the request from UE-B to UE-A

- Cast types (UC/GC/BC) of inter-UE coordination

- Transmission of inter-UE coordination MAC CE on dedicated resource

- L1 parameters/configurations for IUC in Uu RRC (including L1 configurations per resource pool)

- Whether UE-A can be in mode1 or mode2 (interested companies are invited to raise/discuss the issue directly in RAN1)

RAN1 has discussed these issues and came to the following answers:

- HARQ retransmission number for inter-UE coordination information

As inter-UE co-ordination information and inter-UE co-ordination request is time-sensitive, it should be transmitted with little delay. Therefore, the maximum of HARQ re-transmissions should be small, e.g., a maximum of 4 HARQ retransmissions including the initial transmission. There should also be the possibility (through (pre-)configuration), to transmit inter-UR co-ordination information and inter-UE co-ordination request with no HARQ retransmissions.

- Information and length of information of IUC MAC CE. The information indicated in RAN1 LS should be taken into account as baseline

The inter-UE co-ordination information includes the following fields:

- Flag to differentiate between IUC message and IUC request (1-bit)

For IUC message, the following fields are included:

- Resource set type: This is 1 bit, and can indicate preferred resource set type or non-preferred resource set type.

- Reference slot location: This is a combination of DFN index and slot index. The size of this field is 14+μ bits, where μ is the sub-carrier spacing configuration, μ=0,1,2,3 for sub-carrier spacing 15,30,60,120 kHz respectively.

- N resource combination, wherein the size of each resource combination is:  log2(N subChannel SLN subChannel SL + 12N subChannel SL + 16)+9+log2Nrsv_period . N subChannel SL is the number of sub-channels in the SL BPW, and Nrsv_period is the number of configured periodicities in a resource pool. log2Nrsv_period is only included in case of non-preferred resources.

- N first resource. This is the location of the first resource of a resource combination in logical slots relative to the reference location. The size of this field is [8] bits.

- The maximum number of resource combinations is N=[100].

For the IUC request, the following fields are provided:

- Resource type: 1 bit to indicate preferred or non-preferred resources.

- Priority: Priority of the SL data for which inter-UE co-ordination information is being requested. Size is 3 bits.

- Number of subchannels of SL data transmission. Size is log2N subChannel SL bits.

- Resource reservation period of the SL data for which inter-UE co-ordination information is being requested log2Nrsv_period bits.

- Starting time of resource selection window:  This is a combination of DFN index and slot index. The size of this field is 14+μ bits, where μ is the sub-carrier spacing configuration, μ=0,1,2,3 for sub-carrier spacing 15,30,60,120 kHz respectively.

- Ending time of resource selection window: The ending time is relative to the starting time of the resource selection window and is in units of 0.5ms with a size of 10 bits.

- UE-B procedure (e.g. final selection of resources) to the (non-)preferred resource set in IUC

RAN1 has made the following agreements.

Agreement RAN1#107-e

For Scheme 1 with non-preferred resource set,

- Physical layer at UE-B excludes in its resource (re-)selection, candidate single-slot resource(s) obtained after Step 6) of Rel-16 TS 38.214 Section 8.1.4 overlapping with the non-preferred resource set

Agreement RAN1#107b-e

For Scheme 1 with preferred resource set Option A,

  - MAC layer selects resources using S_A and the received preferred resource set

    - MAC layer firstly selects resources for transmissions within the intersection of S_A and the preferred resource set until it becomes impossible to select a resource within the intersection under the constraint defined in Rel-16.

      - It is up to the UE whether to use the preferred resource set from SCI format 2-C and/or MAC CE

    - After this, if the number of selected resources is smaller than the required number of transmissions for a TB, MAC layer selects resources for the remaining transmissions outside the intersection but inside S_A under the constraint defined in Rel-16.

Agreement RAN1#107b-e

For Scheme 1 with preferred resource set Option B,

  - MAC layer selects resources belonging to the received preferred resource set under the constraint defined in Rel-16

    - It is up to the UE whether to use the preferred resource set from SCI format 2-C and/or MAC CE

Note that: In option A, UE-B’s resource(s) to be used for its transmission resource (re-)selection is based on both UE-B’s sensing results (if available) and the received coordination information. In option B, UE-B’s resource(s) to be used for its transmission resource (re-)selection is based only on the received coordination information.

- Scheme 2 inter-UE coordination design

For the purpose of RAN2 design, RAN1 has made the following agreements:

Agreement RAN1#107b-e

For Scheme 2,

- The PHY layer reports S_A after Step 7) of TS 38.214 Section 8.1.4 to higher layer.

- When UE-B receives a conflict indicator for resource(s) indicated by its SCI,

  - PHY layer at UE-B reports resources overlapping with the next reserved resource indicated by the corresponding UE-B’s SCI for current TB transmission to higher layer.

    - If (pre)configured, the PHY layer reports resources in a slot including the next reserved resource indicated by the corresponding UE-B’s SCI for current TB transmission to higher layer.

  - Higher layer at UE-B re-selects the resource(s) indicated by the conflict indicator among the S_A excluding the reported resources.

- FFS: Whether/How the conflict in periodic transmission is indicated by UE-A and handled by UE-B

- Condition for the UE-A to transmit IUC

RAN1 has made the following agreement:

Agreement RAN1#107b-e

For inter-UE coordination triggered by an explicit request in Scheme 1, whether or not to transmit the inter-UE coordination information upon the request reception is determined by UE-A’s implementation subject to the following procedures.

- Rel-16 procedure of UL/SL prioritization, LTE SL/NR SL prioritization, and congestion control

Agreement RAN1#107b-e

For inter-UE coordination triggered by a condition rather than request reception in Scheme 1,

- A resource pool level (pre-)configuration can enable one of the following alternatives:

  - Alt 1: it is up to UE-A’s implementation whether or not to trigger the inter-UE coordination information generation.

  - Alt 2: the inter-UE coordination information generation can be triggered only when UE-A has data to be transmitted together with the inter-UE coordination information to UE-B

- Note: Rel-16 procedure of UL/SL prioritization, LTE SL/NR SL prioritization, and congestion control is applied to the transmission of the inter-UE coordination information triggered by a condition.

- Signaling design and trigger conditions for the request from UE-B to UE-A

RAN1 has made the following agreement:

Agreement RAN1#107b-e

For inter-UE coordination triggered by UE-B’s explicit request in Scheme 1,

- A resource pool level (pre-)configuration can enable one of the following alternatives:

  - Alt 1: it is up to UE-B’s implementation whether or not to trigger the request generation

  - Alt 2: the request generation can be triggered only when UE-B has data to be transmitted to UE-A

- Note: Rel-16 procedure of UL/SL prioritization, LTE SL/NR SL prioritization, and congestion control is applied to the transmission of the request transmission.

- Cast types (UC/GC/BC) of inter-UE coordination

RAN1 has made the following agreement:

Agreement RAN1#107b-e

For Scheme 1, unicast is supported for an explicit request transmission for inter-UE coordination information

- Unicast is used for the inter-UE coordination information transmission triggered by the explicit request

Working Assumption RAN1#107b-e

For Scheme 1, following cast type(s) are supported for inter-UE coordination information transmission triggered by a condition other than explicit request reception

- Groupcast/Broadcast for non-preferred resource set, FFS for preferred resource set

  - FFS: Under which conditions groupcast/broadcast can be supported

- Unicast

- FFS: Under which conditions unicast can be supported

- Transmission of inter-UE coordination MAC CE on dedicated resource

Inter-UE co-ordination MAC CE is not transmitted on a dedicated resource. The resource selection procedure for resource exclusion and SL resource selection is used for inter-UE coordination MAC CE. Furthermore, MAC CE carrying IUC message or IUC request can be multiplexed with other SL data.

Agreement RAN1#107b-e

- For sidelink transmission carrying inter-UE coordination information in Scheme 1,

  - UE-A performs its resource (re)selection according to the same procedure in TS 38.214 Section 8.1.4 to transmit the inter-UE coordination information to UE-B.

- For sidelink transmission carrying request in Scheme 1,

  - UE-B performs its resource (re)selection according to the same procedure in TS 38.214 Section 8.1.4 to transmit the request for the inter-UE coordination information to UE-A if UE-B performs sensing/resource exclusion. Otherwise, at least UE-B can perform random selection

- Note: RAN1 does not pursue specific enhancement of Rel-17 resource (re)selection for the transmission of inter-UE coordination information and its request.

- L1 parameters/configurations for IUC in Uu RRC (including L1 configurations per resource pool)

Please refer to the RRC parameter list.

- Whether UE-A can be in mode1 or mode2 (interested companies are invited to raise/discuss the issue directly in RAN1)

Inter-UE co-ordination is only supported for mode 2.

# Actions

To: RAN2

ACTION: 	RAN1 kindly asks RAN2 to take the above into consideration.

# Dates of next TSG WG1 meetings

RAN1#109-e	May 16th – 27th, 2022	E-meeting

RAN1#110	August 22nd –26th, 2022	Toulouse, FR