3GPP TSG RAN WG1 #108-e	        		             R1-2201988

e-Meeting, February 21st – March 3rd, 2022

Agenda item:	7.1

Source: 	Samsung

Title: 	UCI Multiplexing in PUSCH with repetitions

Document for:	Discussion and Decision

# Introduction

This document provides the reason/justification for the draft CR in [1].

# Number of UCI symbols in PUSCH with repetitions

The number of UCI coded modulation symbols in a PUSCH transmission in a slot depends on the spectral efficiency of either the UL-SCH or, if no UL-SCH, of the CSI. Relevant agreements from RAN1#91 and RAN1#92bis are as below.

Agreement: (RAN1#91) - For UCI on PUSCH with UL-SCH, the amount of resources used for HARQ-ACK is calculated based on the following equation. Q'=O⋅MscPUSCH⋅NsymbPUSCH⋅βoffsetHARQ-ACKr=0C-1Kr where O is the number of ACK/NACK bits, MscPUSCH is the scheduled bandwidth for PUSCH transmission in the current PUSCH transmission period for the transport block, expressed as a number of subcarriers. C, and Kr are obtained from the PDCCH scheduling the PUSCH transmission. NsymbPUSCH is the number of OFDM symbols in the PUSCH transmission duration excluding DMRS. REs occupied by PTRS are also excluded. - FFS: if an upper bound on the number of symbols for HARQ-ACK resource is needed Agreement: (RAN1#92bis) For HARQ-ACK, CSI part 1, and CSI part 2 (if exists) transmission on PUSCH without UL-SCH, the number of coded modulation symbols per layer for HARQ-ACK, CSI part 1, and CSI part 2 (exists), are determined as follows: QACK'=minOACK+LACK⋅βoffsetPUSCHSE,α⋅l=l0Nsymb,allPUSCH-1MscUCIl QCSI-1'=minOCSI-1+LCSI-1⋅βoffsetPUSCHSE,l=0Nsymb,allPUSCH-1MscUCIl-QACK' QCSI-2'=l=0Nsymb,allPUSCH-1MscUCIl-QACK'-QCSI-1' SE is the spectrum efficiency which is code rate * modulation order

The original agreement is from LTE Rel-8 where instead of  O⋅MscPUSCH⋅NsymbPUSCHr=0C-1Kr, the equivalent expression OQm⋅R was originally used (e.g. [2]) where Qm⋅R= SE (same as for the case of no UL-SCH in the above agreements and in [3]). Basically, regardless of whether or not there is UL-SCH, the number of UCI coded modulation symbols is inversely proportional to the spectral efficiency of the PUSCH transmission. In other words, as captured in the first of the above agreements, “NsymbPUSCH is the number of OFDM symbols in the PUSCH transmission duration excluding DMRS”, NsymbPUSCH is not the number of symbols for one repetition of the PUSCH transmission as captured in [3].

The Rel-15 and Rel-16 specifications do not implement the RAN1 agreement in case of PUSCH repetitions as, for Ns repetitions (nominal repetitions in case of type B), the code rate R (or the spectral efficiency) is reduced by 1Ns or, equivalently, the number of symbols for the PUSCH transmission is Ns⋅NsymbPUSCH, not NsymbPUSCH. Then, for the RAN1#91 agreement and the case of repetitions, the number of UCI coded modulation symbols for the PUSCH transmission should be Q'=O⋅βoffsetHARQ-ACKQm⋅R=O⋅MscPUSCH⋅NsymbPUSCH⋅βoffsetHARQ-ACK⋅Nsr=0C-1Kr.

As a result of the incorrect specification, reliability of UCI multiplexing in the PUSCH is now lost and, depending on the number or repetitions, the BLER can be as large as 1 (e.g. ~9 dB is lost in case of 8 repetitions, ~15 dB in case of 32 repetitions). It is noted that in Rel-17 for TBoMS PUSCH, the LTE Rel-8/NR Rel-15 agreement is properly implemented by scaling Qm⋅R with the inverse of the number of slots Ns as above [3] – i.e. the “PUSCH transmission duration” of the Rel-15 agreement is correctly applied and the number of symbols is Ns⋅NsymbPUSCH, not NsymbPUSCH.

The problem could also be avoided if [3] defined Nsymb,allPUSCH for type A repetitions, or Nsymb,nominalPUSCH for type B repetitions, based on the Rel-15 agreement as the number of symbols for the “PUSCH transmission duration” for the TB, instead of the number of symbols in one slot for a single (nominal) repetition of the PUSCH transmission – that is what created the problem. That approach is also fine and can solve the issue but appears more complex because frequency hopping in Rel-15/Rel-16 is per slot and is defined in [3] with respect to Nsymb,allPUSCH.

# Conclusions

This document provided reasons/justifications for the draft CR in [1]. The current specifications are not according to RAN1 agreements and result to unreliable reception of UCI multiplexed in a PUSCH.

The proposed change is applicable to both Rel-15 and Rel-16 but it is recommended only for Rel-16 (the change is a simple software one and has no impact on hardware).

# References:

- R1-2201989, “Draft CR on UCI multiplexing in PUSCH with repetitions”

- R1-082730, “Linking of control resources in PUSCH to data MCS”

- TS 38.212 v15.14.0, “NR; Multiplexing and channel coding”