3GPP TSG RAN WG1 #108-e	R1-2202004

e-Meeting, February 21st – March 3rd, 2022

Agenda item:	8.2.1

Source:	Samsung

Title:	Maintenance on initial access aspects for NR from 52.6 GHz to 71 GHz

Document for:	Discussion and Decision

# Introduction

In RAN1#107bis-e, the following agreements and conclusion were made regarding the initial access aspects for NR from 52.6 to 71 GHz:

Conclusion:

RRC parameters list to capture changes identified below:

- Add the following note to the comment section of discoveryBurstWindowLength-r17 row in RRC parameter list

  - “Note: This parameter is to be included in both SIB1 and the common serving cell configuration parameters”.

- Support adding “SSB-PositionQCL-Relation-r17” to RRC parameter list as both UE-specific and cell-specific parameter.

- Inform RAN2 (by adding notes to RRC parameter list) that the either the value range of the information element SSB-PositionQCL-Relation-r16 needs to be extended, or a new Rel-17 IE need to be defined to allow configuration of Q = 16, 32, or 64 in SIB2, SIB3, SIB4, MeasObjectNR, and ServingCellConfigCommon for RRM measurements when operating with shared spectrum channel access in FR2-2.

The TP below for TS38.213v17.0.0 is endorsed as a working assumption

=========== Unchanged Text Omitted =========== Table 13-15A: PDCCH monitoring occasions for Type0-PDCCH CSS set - SS/PBCH block and CORESET multiplexing pattern 3 and {SS/PBCH block, PDCCH} SCS {480, 480} kHz or {960, 960} kHz Index; PDCCH monitoring occasions (SFN and slot number); First symbol index (k = 0, 1, …, 31). 0; SFNc=SFNSSB,i nc=nSSB,i; 2, 9 in i=2k, i=2k+1. ============ Unchanged Text Omitted ============

The TP below for TS38.211v17.0.0 is endorsed

*** Unchanged text omitted *** 5.3.2 OFDM baseband signal generation for PRACH The time-continuous signal on antenna port p for PRACH is defined by sl(p,μ)t=k=0LRA-1ak(p,RA)ej2πk+Kk1+kΔfRAt-NCP,lRATc-tstartRA K=ΔfΔfRA k1=k0μ+NBWP,istart-Ngridstart,μNscRB-Ngridsize,μNscRB2+nRAstartNscRB+nRANRBRANscRBif LRA∈139, 839 or LRA∈571, 1151 in FR2-2NRB,UL,n0+nRAstart,μ-NRB,UL,n0start,μNscRBif LRA∈571, 1151 in FR1 k0μ=Ngridstart,μ+Ngridsize,μ2NscRB-Ngridstart,μ0+Ngridsize,μ02NscRB2μ0-μ *** Unchanged text omitted ***

The TP below for TS38.214v17.0.0 is endorsed

*** Unchanged text omitted *** 7 UE procedures for transmitting and receiving on a carrier with intra-cell guard bands For operation with shared spectrum channel access in FR1, when the UE is configured with any of IntraCellGuardBandsPerSCS for UL carrier and for DL carrier with SCS configuration μ, the UE is provided with NRB-set,x-1 intra-cell guard bands on a carrier with μ, each defined by start CRB and size in number of CRBs,GB s,xstart,μ andGB s,xsize,μ , provided by higher layer parameters startCRB and nrofCRBs, respectively, wheres∈0,1,…,NRB-set,x-2. The subscript x is set to DL and UL for the downlink and uplink, respectively. Where there is no risk of confusion, the subscript x can be dropped. The intra-cell guard bands separateNRB-set,x RB sets, each defined by start and end CRB,RB s,xstart,μ andRB s,xend,μ, respectively. The UE does not expect that nrofCRBs is configured with non-zero value smaller than the applicable intra-cell guard bands as specified in [8, TS 38.101-1] corresponding toμ and carrier sizeNgrid,xsize,μ. The UE determines the start and end CRB indices for s∈0,1,…,NRB-set,x-1 as *** Unchanged text omitted ***

TP# 8-1a for TS38.213 in section 3 of R1-2200689 is endorsed.

13 UE procedure for monitoring Type0-PDCCH CSS sets ============= Unchanged Text Omitted ============= For operation without shared spectrum channel access and for operation with shared spectrum channel access in FR2-2, a UE assumes that the offset in Tables 13-1 through 13-10C is defined with respect to the SCS of the CORESET for Type0-PDCCH CSS set from the smallest RB index of the CORESET for Type0-PDCCH CSS set to the smallest RB index of the common RB overlapping with the first RB of the corresponding SS/PBCH block. The SCS of the CORESET for Type0-PDCCH CSS set is provided by subCarrierSpacingCommon for FR1 and FR2-1 and same as the SCS of the corresponding SS/PBCH block for FR2-2. In Tables 13-7, 13-8, and 13-10 kSSB is defined in [4, TS 38.211]. For operation with shared spectrum channel access in FR1, a UE determines an offset from a smallest RB index of the CORESET for Type0-PDCCH CSS set to a smallest RB index of the common RB overlapping with a first RB of the corresponding SS/PBCH block ============= Unchanged Text Omitted =====================

Conclusion

RRC parameters list to capture changes identified below

- New parameter, ra-ResponseWindow-r17, under sub-feature group SSB and RACH

  - Value range {sl240, sl320, sl640, sl960, sl1280, sl1920, sl2560}

  - Based on previous conclusion:

    - For FR2-2, support the same mechanism as in Rel-16 for extended RAR window for both 4-step and 2-step RACH.

- New parameter, msgB-ResponseWindow-r17, under sub-feature group SSB and RACH

  - Value range { sl240, sl640, sl960, sl1280, sl1920, sl2560}

  - Based on previous conclusion:

    - For FR2-2, support the same mechanism as in Rel-16 for extended RAR window for both 4-step and 2-step RACH.

- Existing parameter, msgA-PRACH-RootSequenceIndex-r16, under sub-feature group SSB and RACH

  - Description:

    - May not need to change the IE, but need to add in the note on the limitation to be used with SCS. Field description requires updating to capture that L = 1151 is not supported for SCS 480 and 960 kHz and L = 571 is not supported for 960 kHz.

  - Value range:

    - CHOICE { l571 INTEGER {0..569}, l1151 INTEGER {0..1149}}

- Cell-specific

This contribution discusses the remaining issues for the initial access aspects for NR 52.6 to 71 GHz, including the following aspects, and the TPs, if needed, associated with the proposals are listed in the appendix:

- Remaining issues for DBTW

- Remaining issues for CORESET#0 configuration

- Remaining issues for NR carrier RSSI measurement

# Remaining issues for DBTW

## 2.1	DBTW for licensed operation

In current specification TS 38.213, DBTW is supported for FR2-2 with unlicensed operation only, and the related signaling of QCL parameter NSSBQCL is also supported for FR2-2 with unlicensed operation only. There is one remaining issue to clarify – whether the DBTW and related signaling are also supported for FR2-2 with licensed operation.

DBTW was introduced in Rel-16 NR-U, and its functionality is to mitigate the impact of channel access failure for the components defined as “discovery burst” (e.g. SSB and configured RMSI and CSI-RS). The concept of “discovery burst” for licensed operation is not well defined yet in the specification, and its motivation to support such a concept for licensed operation is not clear (e.g. no need for channel access procedure). Hence, supporting DBTW for licensed operation will first need to introduce the concept of DB for licensed operation.

Meanwhile, even without DBTW for licensed operation, current specification can already work functionally. The concern on UE’s unclear behavior for frequency range with both licensed and unlicensed operation can be resolved by UE’s implementation. For instance, for a UE detecting a SSB over the frequency range with both licensed and unlicensed operation in FR2-2, it can either blind detect the SSB from licensed or unlicensed operations, or assume a reasonable value of NSSBQCL to detect the SSB. Further introducing the specification impact to restrict the UE behavior for licensed operation may not help to reduce the UE complexity.

Proposal 1: No need to support DBTW for licensed operation in FR2-2.

- No specification impact.

## 2.2 	Indication of NSSBQCL

In a RAN2 LS (R2-2201720), RAN2 decided not to agree to using the spare bit in MIB for signaling of NSSBQCL, and the following RAN1 working assumption cannot be confirmed. RAN1 needs to discuss the indication of NSSBQCL again.

Agreement

- Support DBTW with 480 and 960 kHz SCS.

- For licensed and unlicensed operation, support 64 candidate SSB positions in a half frame

- Working assumption: Use 2 bits for Q:

  - SubcarrierSpacingCommon

  - spare bit in MIB

- Send LS to RAN2 for confirming the use of the spare bit in MIB

  - The use of 2 bits for Q can be revisited if RAN2 tells RAN1 that the spare bit cannot be used

Based on current progress, other than SubcarrierSpacingCommon, almost all the bits in MIB have been utilized and may not be available for indication of NSSBQCL. The only field that could be possibly to be reinterpreted for indication of NSSBQCL is ssb-SubcarrierOffset, but may need RAN4’s confirmation on whether it can be used by RAN1 for this purpose. Hence, we believe the following options are feasible now for the indication of NSSBQCL:

Proposal 2: Down-select one option for the indication of NSSBQCL:

- Option 1: Keep using 2 bits in MIB for the indication of NSSBQCL, and send a LS to RAN4 for confirming the use of the LSB of ssb-SubcarrierOffset. Adopt TP#1 for TS 38.213.

- Option 2: Use 1 bit in MIB for the indication of NSSBQCL. Adopt TP#2 for TS 38.213.

- Option 3: Don’t indicate NSSBQCL in MIB. Adopt TP#3 for TS 38.213.

# Remaining issues for CORESET#0 configuration

In a RAN4 LS (R4-2200081), RAN4 agreed to support different channelization for licensed band(s) and unlicensed band(s), wherein fixed sync raster is supported for unlicensed bands, and floating sync raster is supported for licensed bands. Based on this agreement, we observe that floating channelization is the bottleneck for designing the frequency offset in CORESET#0 configuration, since it has smaller step size of the sync raster interval, and a configuration table working for floating channelization in licensed band can automatically work for fixed channelization in unlicensed band. Hence, the following analysis is based on the design for floating channelization in licensed band, but applicable to both licensed and unlicensed bands.

Proposal 3: Support the same CORESET#0 configuration table for licensed and unlicensed bands.

## 3.1	CORESET#0 configuration for 120 kHz SCS

The design of CORESET#0 configuration highly relies on the type of channelization and synchronization raster interval, and in NR Rel-15 FR2, CORESET#0 configuration was designed based on the Type 1 channelization (e.g. floating channel raster) with a synchronization raster interval of 12 RB for 120 kHz SCS of SS/PBCH block (i.e., 17.28 MHz) and 12 RB for 240 kHz SCS of SS/PBCH block (i.e., 34.56 MHz).

If Type 1 channelization is reused for NR 52.6 GHz to 71 GHz, since the minimum channel bandwidth is enlarged from FR2-1 (e.g. 50 MHz), the synchronization raster interval is expected to be larger than FR2-1, even using 120 kHz SCS of SS/PBCH block. Hence, the CORESET#0 configuration, especially the frequency offset between SS/PBCH block and CORESET#0 in multiplexing Pattern 1, should be enhanced accordingly. For example, for 120 kHz, 100 MHz minimum channel bandwidth has been agreed in RAN4, which results in a choice of synchronization raster step size as 2 or 3 GSCN values, wherein such step size for Rel-15 FR2-1 is 1 GSCN. A comparison on the number of RB offsets required to support such enlarged step size is shown in Table 1. It can be observed that at least for 48 RB of CORESET#0 bandwidth, enhancement is needed on the number of frequency offsets.

Table 1 Minimum number of frequency offsets for each step size of synchronization raster in Pattern 1.

| Minimum number of frequency offsets |  | Step size for synchronization raster (GSCN) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 1 (Rel-15 FR2-1) | 2 | 3 |
| CORESET#0 bandwidth (RB) | 24 | 2 | 1 | 1 |
|  | 48 | 1 | 2 | 2 |
|  | 96 | NaN | 1 | 1 |

Based on above discussion, we have the following proposal for CORESET#0 configuration for 120 kHz SCS:

Proposal 4: For 120 kHz SCS:

- Support one RB offset for 24 RB CORESET#0 bandwidth in Pattern 1;

- Support two RB offsets for 48 RB CORESET#0 bandwidth in Pattern 1;

- Support one RB offset for 96 RB CORESET#0 bandwidth in Pattern 1;

- Support same RB offsets as Rel-15 FR 2-1 in Pattern 3;

- Adopt TP#4 for TS 38.213.

## 4.2	CORESET#0 configuration for 480 kHz and 960 kHz SCS

For 480 kHz and 960 kHz SCS, the CORESET#0 configuration table can be the same, since 960 kHz is not utilized for initial cell search.

Moreover, since SS/PBCH block and CORESET#0 multiplexing pattern 3 has been supported for 120 kHz, it would be beneficial to support such pattern for 480 kHz and 960 kHz as well, with the same RB offset as in 120 kHz to save design effort.

For SS/PBCH block and CORESET#0 multiplexing pattern 1, if Type 1 channelization is reused for NR 52.6 GHz to 71 GHz, based on the minimum channel bandwidth agreed as 400 MHz in RAN4, the maximum step size of synchronization raster can be 15 GSCN values, and actual values from 8 to 15 are under discussion in RAN4. A comparison on the number of RB offsets required to support each step size is shown in Table 2. It can be observed that for 24 RB of CORESET#0 bandwidth, 1 to 2 frequency offsets may be needed; for 48 RB of CORESET#0 bandwidth, 2 to 3 frequency offsets may be needed; for 96 RB of CORESET#0 bandwidth, 1 to 2 frequency offsets may be needed. Since RAN1 may not have the detailed design on the synchronization raster from RAN4, it would be good to design in a conservative way, such that any RAN4 design can be applicable.

Table 2 Minimum number of frequency offsets for each step size of synchronization raster in Pattern 1.

| Minimum number of frequency offsets |  | Step size for synchronization raster (GSCN) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | 8 | 9 | 10 | 12 | 14 | 15 |
| CORESET#0 bandwidth (RB) | 24 | 1 | 1 | 1 | 1 | 1 | 2 |
|  | 48 | 2 | 2 | 2 | 2 | 3 | 3 |
|  | 96 | 1 | 1 | 1 | 1 | 2 | 2 |

Based on above discussion, we have the following proposal:

Proposal 5: For 480 kHz and 960 kHz SCS:

- Support the same CORESET#0 configuration table;

- Support multiplexing pattern 3 with same RB offsets as in Rel-15 FR2-1;

- Support two RB offsets for 24 RB CORESET#0 bandwidth in Pattern 1;

- Support three RB offsets for 48 RB CORESET#0 bandwidth in Pattern 1;

- Support two RB offsets for 48 RB CORESET#0 bandwidth in Pattern 1;

- Support 1 symbol for CORESET#0 when the bandwidth of CORESET#0 is 96;

- Adopt TP#5 for TS 38.213.

# Remaining issues for NR carrier RSSI measurement

NR carrier RSSI measurement is used for calculating SS-RSRQ, as specified in TS 38.215, and a UE can be indicated with an ending symbol index to calculate NR carrier RSSI measurement. The intention of introducing a flexible ending symbol is to allow flexible determination of SSB transmission, such that the measured symbols can be as DL symbols as possible. Meanwhile, the flexible symbols for NR carrier RSSI measurement also provide the feasibility to configure measured symbols either purely from serving cell (e.g. measuring serving cell DL transmissions with potential interference) or for neighboring cell transmission (e.g. measuring interference only). To achieve such purpose, the ending symbols for NR carrier RSSI measurement were carefully chosen to be compatible with the SSB pattern (as shown in Figure 1). Since Rel-17 introduces a new SSB pattern for 480 kHz and 960 kHz, it is essential to make the corresponding change for the ending symbols.

Figure 1 Illustration of symbols for NR carrier RSSI measurement.

Proposal 6: For 480 and 960 kHz:

- Support the following 4 configurations for NR carrier RSSI measurement:

- Configuration #0: {0, 1};

- Configuration #1: {0, 1, …, 5};

- Configuration #2: {0, 1, …, 8};

- Configuration #3: {0, 1, …, 12};

- Adopt TP#6 for TS 38.215.

# Conclusion

The proposals made in this contribution are summarized below (associated TPs are listed in the appendix):

Proposal 1: No need to support DBTW for licensed operation in FR2-2.

- No specification impact.

Proposal 2: Down-select one option for the indication of NSSBQCL:

- Option 1: Keep using 2 bits in MIB for the indication of NSSBQCL, and send a LS to RAN4 for confirming the use of the LSB of ssb-SubcarrierOffset. Adopt TP#1 for TS 38.213.

- Option 2: Use 1 bit in MIB for the indication of NSSBQCL. Adopt TP#2 for TS 38.213.

- Option 3: Don’t indicate NSSBQCL in MIB. Adopt TP#3 for TS 38.213.

Proposal 3: Support the same CORESET#0 configuration table for licensed and unlicensed bands.

Proposal 4: For 120 kHz SCS:

- Support one RB offset for 24 RB CORESET#0 bandwidth in Pattern 1;

- Support two RB offsets for 48 RB CORESET#0 bandwidth in Pattern 1;

- Support one RB offset for 96 RB CORESET#0 bandwidth in Pattern 1;

- Support same RB offsets as Rel-15 FR 2-1 in Pattern 3;

- Adopt TP#4 for TS 38.213.

Proposal 5: For 480 kHz and 960 kHz SCS:

- Support the same CORESET#0 configuration table;

- Support multiplexing pattern 3 with same RB offsets as in Rel-15 FR2-1;

- Support two RB offsets for 24 RB CORESET#0 bandwidth in Pattern 1;

- Support three RB offsets for 48 RB CORESET#0 bandwidth in Pattern 1;

- Support two RB offsets for 48 RB CORESET#0 bandwidth in Pattern 1;

- Support 1 symbol for CORESET#0 when the bandwidth of CORESET#0 is 96;

- Adopt TP#5 for TS 38.213.

Proposal 6: For 480 and 960 kHz:

- Support the following 4 configurations for NR carrier RSSI measurement:

- Configuration #0: {0, 1};

- Configuration #1: {0, 1, …, 5};

- Configuration #2: {0, 1, …, 8};

- Configuration #3: {0, 1, …, 12};

- Adopt TP#6 for TS 38.215.

# Appendix

## TP#1 for TS 38.213

============================== Start of TP #1 for TS 38.213 ==================================

4.1  Cell search

=============================== Unchanged Text Omitted ===================================

For operation with shared spectrum channel access in FR2-2, a UE assumes that SS/PBCH blocks in a serving cell that are within a same discovery burst transmission window or across discovery burst transmission windows are quasi co-located with respect to average gain, quasi co-location 'typeA' and 'typeD' properties, when applicable, if a value of imodNSSBQCL is same among the SS/PBCH blocks, where i is the candidate SS/PBCH block index. NSSBQCL is either provided by ssb-PositionQCL or, if ssb-PositionQCL is not provided, obtained from a MIB provided by a SS/PBCH block according to Table 4.1-2. The UE can determine an SS/PBCH block index according to i modNSSBQCL. The UE assumes that within a discovery burst transmission window, a number of transmitted SS/PBCH blocks on a serving cell is not larger than NSSBQCL and a number of transmitted SS/PBCH blocks with a same SS/PBCH block index is not larger than one.

Table 4.1-2: Mapping between the combination of subCarrierSpacingCommon and LSB of ssb-SubcarrierOffset to NSSBQCL for operation with shared spectrum channel access in FR2-2

| subCarrierSpacingCommon | LSB of ssb-SubcarrierOffset | NSSBQCL |
| --- | --- | --- |
| scs15or60 | 0 | 16 |
| scs15or60 | 1 | 32 |
| scs30or120 | 0 | 64 |
| scs30or120 | 1 | reserved |

=============================== Unchanged Text Omitted ===================================

============================== End of TP #1 for TS 38.213 ==================================

## TP#2 for TS 38.213

============================== Start of TP #2 for TS 38.213 ==================================

4.1  Cell search

=============================== Unchanged Text Omitted ===================================

For operation with shared spectrum channel access in FR2-2, a UE assumes that SS/PBCH blocks in a serving cell that are within a same discovery burst transmission window or across discovery burst transmission windows are quasi co-located with respect to average gain, quasi co-location 'typeA' and 'typeD' properties, when applicable, if a value of imodNSSBQCL is same among the SS/PBCH blocks, where i is the candidate SS/PBCH block index. NSSBQCL is either provided by ssb-PositionQCL or, if ssb-PositionQCL is not provided, obtained from a MIB provided by a SS/PBCH block according to Table 4.1-2. The UE can determine an SS/PBCH block index according to i modNSSBQCL. The UE assumes that within a discovery burst transmission window, a number of transmitted SS/PBCH blocks on a serving cell is not larger than NSSBQCL and a number of transmitted SS/PBCH blocks with a same SS/PBCH block index is not larger than one.

Table 4.1-2: Mapping between subCarrierSpacingCommon to NSSBQCL for operation with shared spectrum channel access in FR2-2

| subCarrierSpacingCommon |  | NSSBQCL |
| --- | --- | --- |
| scs15or60 |  | 32 |
| scs30or120 |  | 64 |

=============================== Unchanged Text Omitted ===================================

============================== End of TP #2 for TS 38.213 ==================================

## TP#3 for TS 38.213

============================== Start of TP #3 for TS 38.213 ==================================

4.1  Cell search

=============================== Unchanged Text Omitted ===================================

For operation with shared spectrum channel access in FR2-2, a UE assumes that SS/PBCH blocks in a serving cell that are within a same discovery burst transmission window or across discovery burst transmission windows are quasi co-located with respect to average gain, quasi co-location 'typeA' and 'typeD' properties, when applicable, if a value of imodNSSBQCL is same among the SS/PBCH blocks, where i is the candidate SS/PBCH block index. NSSBQCL is provided by ssb-PositionQCL. The UE can determine an SS/PBCH block index according to i modNSSBQCL. The UE assumes that within a discovery burst transmission window, a number of transmitted SS/PBCH blocks on a serving cell is not larger than NSSBQCL and a number of transmitted SS/PBCH blocks with a same SS/PBCH block index is not larger than one.

NSSBQCL

|  |  | NSSBQCL |
| --- | --- | --- |

=============================== Unchanged Text Omitted ===================================

============================== End of TP #3 for TS 38.213 ==================================

## TP#4 for TS 38.213

============================== Start of TP #4 for TS 38.213 ==================================

13 	UE procedure for monitoring Type0-PDCCH CSS sets

=============================== Unchanged Text Omitted ===================================

Table 13-10A: Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {120, 120} kHz for FR2-2

| Index | SS/PBCH block and CORESET multiplexing pattern | Number of RBs NRBCORESET | Number of Symbols NsymbCORESET | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 24 | 2 | 2 |
| 1 | 1 | 48 | 1 | 0 |
| 2 | 1 | 48 | 1 | 28 |
| 3 | 1 | 48 | 2 | 0 |
| 4 | 1 | 48 | 2 | 28 |
| 5 | 1 | 96 | 1 | 38 |
| 6 | 1 | 96 | 2 | 38 |
| 7 | 3 | 24 | 2 | -20 if kSSB=0, -21 if kSSB>0 |
| 8 | 3 | 24 | 2 | 24 |
| 9 | 3 | 48 | 2 | -20 if kSSB=0, -21 if kSSB>0 |
| 10 | 3 | 48 | 2 | 48 |
| 11 | Reserved |  |  |  |
| 12 | Reserved |  |  |  |
| 13 | Reserved |  |  |  |
| 14 | Reserved |  |  |  |
| 15 | Reserved |  |  |  |

=============================== Unchanged Text Omitted ===================================

============================== End of TP #4 for TS 38.213 ==================================

## TP#5 for TS 38.213

============================== Start of TP #5 for TS 38.213 ==================================

13 	UE procedure for monitoring Type0-PDCCH CSS sets

=============================== Unchanged Text Omitted ===================================

Table 13-10B: Set of resource blocks and slot symbols of CORESET for Type0-PDCCH search space set when {SS/PBCH block, PDCCH} SCS is {480, 480} kHz or {960, 960} for FR2-2

| Index | SS/PBCH block and CORESET multiplexing pattern | Number of RBs NRBCORESET | Number of Symbols NsymbCORESET | Offset (RBs) |
| --- | --- | --- | --- | --- |
| 0 | 1 | 24 | 2 | 0 |
| 1 | 1 | 24 | 2 | 4 |
| 2 | 1 | 48 | 1 | 0 |
| 3 | 1 | 48 | 1 | 14 |
| 4 | 1 | 48 | 1 | 28 |
| 5 | 1 | 48 | 2 | 0 |
| 6 | 1 | 48 | 2 | 14 |
| 7 | 1 | 48 | 2 | 28 |
| 8 | 1 | 96 | 1 | 0 |
| 9 | 1 | 96 | 1 | 76 |
| 10 | 1 | 96 | 2 | 0 |
| 11 | 1 | 96 | 2 | 76 |
| 12 | 3 | 24 | 2 | -20 if kSSB=0, -21 if kSSB>0 |
| 13 | 3 | 24 | 2 | 24 |
| 14 | 3 | 48 | 2 | -20 if kSSB=0, -21 if kSSB>0 |
| 15 | 3 | 48 | 2 | 48 |

|  |  | NRBCORESET | NsymbCORESET |  |
| --- | --- | --- | --- | --- |

=============================== Unchanged Text Omitted ===================================

============================== End of TP #5 for TS 38.213 ==================================

## TP#6 for TS 38.215

============================== Start of TP #6 for TS 38.215 ==================================

5.1.3 SS reference signal received quality (SS-RSRQ)

=============================== Unchanged Text Omitted ===================================

Table 5.1.3-1: NR Carrier RSSI measurement symbols

| OFDM signal indication endSymbol | Symbol indexes |
| --- | --- |
| 0 | {0,1} |
| 1 | For 480 kHz and 960 kHz {0,1,2,..,10,12}; otherwise {0,1,2,..,10,11} |
| 2 | {0,1,2,…, 5} |
| 3 | For 480 kHz and 960 kHz {0,1,2,..,8}; otherwise {0,1,2,…, 7} |

=============================== Unchanged Text Omitted ===================================

============================== End of TP #6 for TS 38.215 ==================================