3GPP TSG RAN WG1 #108-e			R1-2202007

e-Meeting, February 21st – March 3rd, 2022

Agenda item:	8.2.5

Source:	Samsung

Title:	Maintenance on PDSCH/PUSCH enhancements for NR from 52.6 GHz to 71 GHz

Document for:	Discussion and Decision

- Introduction

This contribution discusses the remaining issues for multi-PDSCH/PUSCH scheduling by a single DCI and the TPs, if needed, associated with the proposals are listed in Appendix.

- Multi-PDSCH/PUSCH scheduling and HARQ

3.1 Multi-PDSCH/PUSCH scheduling

3.1.1 SPS PDSCH / CG PUSCH

In the last RAN1#107b-e meeting, RAN1 made the following agreements.

Agreement A UE validates, for scheduling activation or scheduling release, a DL SPS assignment PDCCH or a configured UL grant Type 2 PDCCH if the PDCCH indicates a TDRA row index including only one SLIV.

For SPS PDSCH/CG PUSCH retransmission (i,e., DCI format with CRC scrambled with CS-RNTI with NDI=1) , RAN1 should decide whether/how to support multi-PDSCH/PUSCH scheduling.

It was discussed in [103-e-NR-NRU-06] that whether multi-PUSCH based CG PUSCH retransmission is allowed or not. For this discussion, RAN1 did not adopt TPs for multi-PUSCH based CG PUSCH retransmission. Then, if RAN1 supports multi-PDSCH/PUSCH scheduling form SPS/CG retransmission, there is a different UE behaviour on Rel-16 multi-PUSCH scheduling and Rel-17 multi-PUSCH scheduling in terms of CG PUSCH retransmission.

If we support multi-PDSCH/PUSCH scheduling for SPS PDSCH/CG PUSCH retransmission, further specification works are expected to clarify “NDI=1” since there are more than one bits for NDI field. So, it should be replaced with “NDI = all 1s” when TDRA is indicated to multi-PDSCH/PUSCH scheduling.

Also, there are no clear use cases to retransmit more than one SPS PDSCH/CG PUSCH by a single DCI considering their periodicities. Hence, it is also desirable to use a row with single SLIV by a DCI for SPS PDSCH/CG PUSCH retransmission.

If multi-PDSCH/PUSCH scheduling for SPS PDSCH/CG PUSCH retransmission is supported, there are potential possibilities for a UE to determine the HARQ process ID not dedicated to SPS PDSCH/CG PUSCH due to the HARQ process increment rule. Consider two cases.

- Case 1: a UE is configured with a SPS configuration with HARQ process IDs of 0, 1, 2, and 3. If a UE fails to receive SPS PDSCHs with HARQ process IDs of 0 and 2, then how does gNB retransmit these two SPS PDSCHs by using multi-PDSCH scheduling? Is it allowed to schedule 3 SPS PDSCHs with HARQ process IDs of 0, 1, and 2? If allowed, what is a UE behaviour for SPS PDSCH with HARQ process ID of 1?

- Case 2: a UE is configured with two SPS configurations, where the first SPS configuration has HARQ process IDs of 0,1 and the second SPS configuration has HARQ process IDs of 3,4. If a UE fails to receive SPS PDSCHs with HARQ process IDs of 1 and 3, again how does gNB retransmit the two SPS PDSCHs by using multi-PDSCH scheduling? Also, is it allowed to schedule 3 SPS PDSCHs with HARQ process IDs of 1, 2, and 3? If allowed, what is a UE behaviour for SPS PDSCH with HARQ process ID of 2?

For the first case, our understanding is that gNB may schedule retransmission of 3 SPS PDSCHs with HARQ process IDs of 0, 1, and 2 and consider the SPS PDSCH of HARQ process ID 1 is retransmitted regardless of it was ACKed or NACKed. That is, the UE will report ACK again for the retransmitted SPS PDSCH of HARQ process ID 1.

For the second case, it is preferred to not allow multi-PDSCH scheduling for SPS PDSCH/CG PUSCH for simplicity, which means that gNB should use single-PDSCH scheduling for SPS PDSCH/CG PUSCH retransmission for the second case. That is, the first alternative is that

- Alt 1(Case 2 is treated as an error case): A UE does not expect to be receive a DCI format scheduling multiple SPS PDSCHs (or CG PUSCHs) retransmission with a HARQ process ID not dedicated for any SPS configurations (CG configurations).

Another alternative is to allow the multi-PDSCH scheduling for the second case but define UE behaviours, for example,

- Alt 2(define UE behaviours for Case 2): A UE does not receive (or transmit) a SPS PDSCH (or CG PUSCH) retransmission with a HARQ process ID not dedicated for any SPS configurations (CG configurations).

Note that Alt 2 is similar as an invalid PDSCH (or invalid PUSCH) due to semi-static UL symbol (or DL symbol). In Alt 2, HARQ process ID is further used to determine whether a PDSCH is received or a PUSCH is transmitted. Also, it only depends on the configured SPS configurations (or CG configurations) then there are no additional timeline issues. It is worth noting that if activated SPS configurations are considered instead of configured SPS configurations, then a new timeline (for activation/deactivation) is required so that it should be avoided at this late stage of Rel-17 WI.

The other alternative is to only allow the single-PDSCH scheduling by using the first or last SLIV in the TDRA table row as we discussed in SPS/CG release.

Proposal 1: For SPS/CG retransmission, down select one of options.

- Option 1. It is allowed for gNB to indicate a TDRA row with single SLIV only.

- Option 2. It is allowed for gNB to indicate a TDRA row with more than one SLIVs, where

  - Alt 1: A UE does not expect to be receive a DCI format scheduling multiple SPS PDSCHs (or CG PUSCHs) retransmissions with a HARQ process ID not dedicated for any SPS configurations (CG configurations).

  - Alt 2: A UE does not receive (or transmit) a SPS PDSCH (or CG PUSCH) retransmission with a HARQ process ID not dedicated for any SPS configurations (CG configurations).

  - Alt 3: A UE receives (or transmit) a SPS PDSCH (or CG PUSCH) retransmission corresponding to [the first/last SLIV] in the TDRA table.

3.1.2 Scell Dormancy Indication and TCI-State indication

Scell dormancy indication can be indicated by DCI format 1_1. If the DCI format 1_1 is used for Scell dormancy indication, the DCI format 1_1 does not schedule a PDSCH reception and some of bit fields in the DCI format 1_1 is repurposed for a bitmap to indicate Scell dormancy states. The relevant specification text is copied below:

TS38.213 v16.7.0 If a UE is provided search space sets to monitor PDCCH for detection of DCI format 1_1, and if - the CRC of DCI format 1_1 is scrambled by a C-RNTI or a MCS-C-RNTI, and if - a one-shot HARQ-ACK request field is not present or has a '0' value, and if - the UE detects a DCI format 1_1 on the primary cell that does not include a carrier indicator field, or detects a DCI format 1_1 on the primary cell that includes a carrier indicator field with value equal to 0, and if - resourceAllocation = resourceAllocationType0 and all bits of the frequency domain resource assignment field in DCI format 1_1 are equal to 0, or - resourceAllocation = resourceAllocationType1 and all bits of the frequency domain resource assignment field in DCI format 1_1 are equal to 1, or - resourceAllocation = dynamicSwitch and all bits of the frequency domain resource assignment field in DCI format 1_1 are equal to 0 or 1 the UE considers the DCI format 1_1 as indicating SCell dormancy, not scheduling a PDSCH reception, and for transport block 1 interprets the sequence of fields of - modulation and coding scheme - new data indicator - redundancy version and of - HARQ process number - antenna port(s) - DMRS sequence initialization as providing a bitmap to each configured SCell, in an ascending order of the SCell index, where […]

The repurposed bit fields in the DCI format 1_1 include NDI and RV field. If the DCI format 1_1 is configured with multi-PDSCH scheduling (i.e., at least one row includes more than one SLIVs), then the bit size of NDI field and RV field is determined by which a row is indicated. In other words, if a row with single SLIV is indicated, there are 1-bit NDI field and 2-bit RV field in the DCI format 1_1, but if a row with more than one SLIVs indicated, there are Npdsch,max-bit NDI field and Npdsch,max-bit RV field in the DCI format 1_1, where Npdsch,max denotes the maximum number of SLIVs in a row in TDRA table.

Proposal 2: Clarify that for Scell dormancy indication, a UE repurposes Npdsch,max-bit NDI and Npdsch,max-bit RV fields if TDRA indicates multi-PDSCH scheduling or 1-bit NDI and 2-bit RV fields if TDRA indicates single-PDSCH scheduling.

- If Npdsch,max-bit NDI and Npdsch,max-bit RV fields are repurposed, the sequence order for a bitmap is 5-bit MCS, Npdsch,max-bit NDI, Npdsch,max-bit RV, HPN, antenna port(s), and DMRS sequence initialization fields

In Rel-17 Unified TCI indication, TCI-State indication is introduced. The TCI-State indication can be carried through DCI format 1_1, which is capable of multi-PDSCH scheduling. The relevant text are copied below.

TS38.214 v17.0.0 The UE with activated [TCI-State] configured with [tci-StateId_r17] receives DCI format 1_1/1_2 providing indicated TCI-State with [tci-StateId_r17] for a CC or all CCs in the same CC list configured by [simultaneousTCI-UpdateList1 or simultaneousTCI-UpdateList2]. The DCI format 1_1/1_2 can be with or without, if applicable, DL assignment. If the DCI format 1_1/1_2/ is without DL assignment, the UE can assume the following: - CS-RNTI is used to scramble the CRC for the DCI - The values of the following DCI fields are set as follows: - RV = all '1's - MCS = all '1's - NDI = 0 - Set to all '0's for FDRA Type 0, or all '1's for FDRA Type 1, or all '0's for dynamicSwitch (same as in Table 10.2-4 of [6, TS 38.213]).

For the TCI-State indication, it is preferred to use a row with single SLIV same as SPS/CG activation/deactivation. If a row with more than one SLIVs is used for the TCI-State indication, the NDI field should be all ‘0’s instead of ‘0’.

Proposal 3: For a DCI format carrying TCI-State indication, a UE does not expect that the DCI format indicates a TDRA row with more than one SLIVs.

3.1.3 Restriction on the number of PDSCH receptions/PUSCH transmissions in a slot

We have the following agreements for 480/960 kHz SCS scenario.

Agreement: (RAN1#106-e) - For single TRP operation, for 480/960 kHz SCS, - A UE does not expect to be scheduled with more than one unicast PDSCH in a slot, by a single DCI or multiple DCIs. - A UE does not expect to be scheduled with more than one PUSCH in a slot, by a single DCI or multiple DCIs. Agreement (RAN1#107-e) - For multi-TRP operation, for 480/960 kHz SCS, - A UE does not expect to be scheduled with more than one unicast PDSCH in a slot, by a single DCI or multiple DCIs, from the same TRP. - A UE does not expect to be scheduled with more than one PUSCH in a slot, by a single DCI or multiple DCIs, from the same TRP. - Note: This does not preclude a UE being scheduled with two PDSCHs (or two PUSCHs) in the same slot from two different TRPs for multi-DCI based multi-TRP mechanism.

If there is SPS PDSCH/CG PUSCH in the slot, UE behaviour is not clear. For example, whether UE can receive a unicast PDSCH and a SPS PDSCH in a slot? The intention of the agreement is to restrict that there is up to 1 PDSCH/PUSCH per slot. Same rule should apply to SPS PDSCH/CG PUSCH. In addition, whether the scheduled PDSCH/PUSCH is a valid one should be clarified.

Proposal 4: For single TRP or multi-TRP operation, for 480/960 kHz SCS,

  - A UE does not expect to receive more than one unicast PDSCH in a slot on a serving cell from the same TRP.

  - A UE does not expect to transmit more than one PUSCH in a slot on a serving cell from the same TRP.

3.1.4 DL/UL collision

For a single PUSCH scheduled by a DCI format, UE does not expect the scheduled PUSCH colliding with semi-static DL symbols/SSB. However, for multiple PUSCHs scheduling, colliding with semi-static DL symbols/SSB can happen. The order of checking semi-static DL collision and performing UCI multiplexing will impact the reliability of HARQ-ACK transmission.

For Rel-16 intra UE multiplexing we have the following agreements in RAN1#104e.

Agreement To address collision with semi-static DL symbols and SSB, the following easy way is suggested: - Step1: Perform intra UE prioritization (including multiplexing, overriding) according to related working assumption in 102 e-meeting and produce final PUCCHs/PUSCHs. - Step 2: Final PUCCHs/PUSCHs is cancelled by semi-static DL symbols and SSB symbols.

If a HARQ-ACK PUCCH overlaps with an invalid PUSCH which has collision with semi-static DL symbols, following Rel-16 multiplexing rules, the HARQ-ACK will be multiplexed in the invalid PUSCH before checking DL collision and MAC layer will generate a MAC PDU for the invalid PUSCH. Then, the HARQ-ACK will be dropped with the invalid PUSCH. When MAC layer generates MAC PDU it will have an associated HARQ ID, however, HARQ process number increment is skipped for the invalid PUSCH. Determining of the HARQ ID for the invalid PUSCH is not defined. A simple solution could be check semi-static DL collision before multiplexing. Dropping HARQ-ACK can be avoided and MAC layer does not need to generate a MAC PDU.

A CG PUSCH/SPS PDSCH can be cancelled by a DG PDSCH or a DG PUSCH on a same cell in Rel-15. For single PDSCH/PUSCH scheduling, the dynamic scheduled PDSCH/PUSCH can always be received/transmitted. However, for multiple PDSCHs/PUSCHs scheduling, a PDSCH/PUSCH may not be received/transmitted due to colliding with semi-static UL/DL symbols. Whether invalid PDSCH/PUSCH can cancel a CG PUSCH/SPS PDSCH on a same cell needs to be clarified. Not allowing an invalid PDSCH/PUSCH cancelling CG PUSCH/SPS PDSCH is preferred. Similarly, invalid PDSCH should not cancel a PUCCH without a corresponding DCI format on a same cell.

To resolve the issues above, a unified solution can be that UE first resolves the collision of PDSCH/PUSCH and semi-static UL/DL symbols and then UE resolves the collision among PDSCHs, PUSCHs and PUCCHs.

Proposal 5: For resolving collision of overlapping PDSCHs and/or PUSCHs and/or PUCCHs in case of M-PDSCH/M-PUSCH scheduling, UE first resolves the collision of PDSCHs/PUSCHs and semi-static UL/DL symbols and then UE resolves the collision among PDSCHs, PUSCHs and PUCCHs.

3.1.5 Clarification on pdsch-TimeDomainAllocationListForMultiPDSCH-r17 and pusch-TimeDomainAllocationListForMultiPUSCH-17

5.1.2.1 Resource allocation in time domain If a UE is configured with pdsch-TimeDomainAllocationListForMultiPDSCH-r17 in which one or more rows contain multiple SLIVs for PDSCH, the UE does not expect to be configured with higher layer parameter repetitionNumber in pdsch-TimeDomainAllocationListForMultiPDSCH-r17. If a UE is configured with pdsch-TimeDomainAllocationListForMultiPDSCH-r17 in which one or more rows contain multiple SLIVs for PDSCH on a DL BWP of a serving cell, the UE does not apply pdsch-AggregationFactor in PDSCH-config, if configured, to DCI format 1_1 on the DL BWP of the serving cell. 6.1.2.1 Resource allocation in time domain If a UE is configured with pusch-TimeDomainAllocationListForMultiPDSCH-r17 in which one or more rows contain multiple SLIVs for PUSCH on a UL BWP of a serving cell, the UE does not apply pusch-AggregationFactor, if configured, to DCI format 0_1 on the UL BWP of the serving cell and the UE does not expect to be configured with numberOfRepetitions in pusch-TimeDomainAllocationListForMultiPDSCH-r17.

In Clause 5.1.2.1 and Clause 6.1.2.1of TS38.214 v17.0.0, “in which one of more rows contain multiple SLIVs for PDSCH (PUSCH)” is used to indicate multi-PDSCH (PUSCH) scheduling. However, it is clear that pdsch-TimeDomainAllocationListForMultiPDSCH-r17 includes at least one row with more than one SLIVs for PDSCH. That is, it is an error case where pdsch-TimeDomainAllocationListForMultiPDSCH-r17 only includes rows with single SLIV for PDSCH because the RRC parameter pdsch-TimeDomainAllocationList has been used for single PDSCH scheduling. So, we suggest to remove “in which one of more rows contain multiple SLIVs for PDSCH (PUSCH).”

Proposal 6: Adopt TP#1 in Appendix for Clause 5.1.2.1 and Clause 6.1.2.1 of TS38.214.

3.2 HARQ-ACK feedback for Multi-PDSCH scheduling

3.2.1 Remaining issues on Type-1 HARQ-ACK codebook

Support PDSCH repetitions when time-domain bundling is configured

In the last RAN1#107-e meeting, RAN1 agreed the following agreements.

Agreement - If a UE is configured with a TDRA table in which one or more rows contain multiple SLIVs for PDSCH for DCI format 1_1, the UE does not expect to be configured with repetitionNumber for the TDRA table, and if pdsch-AggregationFactor is configued in PDSCH-config, it does not apply to DCI format 1_1. - Note: repetitionNumber cannot be configured with pdsch-TimeDomainAllocationListDCI-1-2 as in Rel-16. - Note: Under agenda item 8.2.4, in RAN1#106-bis, it was already agreed that within the TDRA table for multi-PDSCH scheduling, the UE does not expect to be configured with the higher layer parameter repetitionNumber. - Note: These does not preclude pdsch-AggregationFactor can be configured and applies to DCI format 1_2 - If a UE is configured with a TDRA table in which one or more rows contain multiple SLIVs for PUSCH for DCI format 0_1, the UE does not expect to be configured with numberOfRepetitions for the TDRA table, and if pusch-AggregationFactor is configued in PUSCH-config, it does not apply to DCI format 0_1. - Note: These does not preclude numberOfRepetitions is configured for TDRA table corresponding to DCI format 0_2 - Note: These does not preclude pusch-AggregationFactor can be configured and applies to DCI format 0_2

Based on the following agreements, we can observe that 1) a UE can be provided pdsch-TimeDomainAllocationListForMultiPDSCH-r17 and pdsch-AggregationFactor at the same time and if the UE are provided the both, then DCI format 1_1 may schedule more than one PDSCHs by pdsch-TimeDomainAllocationListForMultiPDSCH-r17 and DCI format 1_2 may schedule PDSCH repetition by pdsch-AggregationFactor. Therefore, if the UE should make type-1 HARQ-ACK CB for multi-PDSCH scheduling and PDSCH repetitions.

In Clause 9.1.2.1 of TS38.213 v17.0.0, the pseudo-code was introduced to support multi-PDSCH scheduling with/without time domain bundling.

If time domain bundling is not configured (see the green highlight below), PDSCH occasions are generated by taking into account multi-PDSCH scheduling and PDSCH repetition. Note that for multi-PDSCH scheduling, the row r of the set R contains single SLIVs decomposing multiple SLIVs in a TDRA table (i.e., SLIV decomposition) and for PDSCH repetition, the single SLIV is assumed to be repeated over NPDSCHrepeat,max slots. Therefore, some of rows associated with DCI format 1_1 does not need to be repeated but all of rows are assumed to be repeated over NPDSCHrepeat,max slots, which make some overhead in type-1 HARQ-ACK CB. However, it can be acceptable since it does not bring any scheduling restrictions.

If time domain bundling is configured (see the yellow highlight below), PDSCH occasions are generated by taking into account multi-PDSCH scheduling only, but not PDSCH repetition. As a results, the type-1 HARQ-ACK CB with time domain bundling does not includes PDSCH occasions for PDSCH repetition. Therefore, gNB may not schedule PDSCH repetitions by using DCI format 1_2.

while r<CR if the UE is not provided enableTimeDomainHARQ-Bundling and is provided tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot from slot n0,k+nD-NPDSCHrepeat,max+1 to slot n0,k+nD, at least one symbol of the PDSCH time resource derived by row r is configured as UL where K1,k is the k-th slot timing value in set K1, where n0,k is a DL slot with a smallest index among DL slots overlapping with UL slot nU-K1,k, or subslotLengthForPUCCH is provided for the HARQ-ACK codebook and the end of the PDSCH time resource for row r is not within any UL slot nU-K1,l, 0≤l<CK1, or if HARQ-ACK information for PDSCH time resource derived by row r in slot nU-K1,k⋅2μDL-μUL+nD cannot be provided in slot nU R=R\r; elseif the UE is provided enableTimeDomainHARQ-Bundling and tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot nU-K1,k⋅2μDL-μUL+nD- ∆K0,r(d), at least one symbol of the PDSCH time resource derived by row r of set R' is configured as UL, where d = 0,1,…,C∆K0,r-1, ∆K0,r=maxK0K0,r-K0,r, and C∆K0,r is the cardinality of ∆K0,r. R=R\r; R'=R'\r; else r=r+1; end if end while

Observation 1: If time domain bundling is configured, Type-1 HARQ-ACK CB does not cover PDSCH repetitions scheduled by DCI format 1_2.

To address this issue, we suggest to consider the following two options.

- Option 1. A row r is removed when two conditions are met

  - Condition 1 for multi-PDSCH scheduling: each SLIVs of the TDRA row r overlapped with a semi-static UL symbol

  - Condition 2 for PDSCH repetition: the last SLIV of the TDRA row r over K slots overlapped with a semi-static UL symbol.

    - Note that the last SLIV is used for PDSCH repetition since the TDRA row r may include more than one SLIVs. Note that this may result in some overhead because the TDRA rows only for multi-PDSCH scheduling are also used for PDSCH repetition.

- Option 2. Treat it as an error case. i.e., a UE does not expect to be configured with multi-PDSCH scheduling with time domain bundling and pdsch-AggregationFactor at the same time.

Since type-1 HARQ-ACK CB is mainly used for a coverage limited scenario where a PDCCH scheduling PDSCH may be missed often, support of PDSCH repetitions scheduled by a DCI format 1_2 would be beneficial. Therefore, we prefer to support that type-1 HARQ-ACK CB covers both multi-PDSCH scheduling and PDSCH repetition.

Proposal 7: To support multi-PDSCH scheduling by DCI format 1_1 and PDSCH repetition by DCI format 1_2 in type-1 HARQ-ACK CB, a row r in the set R’ and the set R is removed when the both conditions are met

  - Condition 1 for multi-PDSCH scheduling) each SLIVs of the TDRA row r overlapped with a semi-static UL symbol

  - Condition 2 for PDSCH repetition) the last SLIV of the TDRA row r over K slots overlapped with a semi-static UL symbol.

Proposal 8: Adopt TP#2 in Appendix for TS38.213

3.2.2 Remaining Issues on Type-2/Enhanced Type-2 HARQ-ACK codebook

Valid SLIVs vs Configured SLIVs for bundling grouping

Agreement For multi-PDSCH scheduling with a single DCI - Introduce a new RRC parameter, e.g., numberOfHARQ-BundlingGroups, to configure the number of HARQ bundling groups with value range {1, 2, 4} for type-2 HARQ-ACK codebook per serving cell. - If the RRC parameter is not configured for a serving cell, time domain bundling for type-2 HARQ-ACK codebook is not enabled for the serving cell. - The maximum number of PDSCHs allocated to each bundling group is ceil(NPDSCH,MAX/NHBG) where NHBG is the number of bundling groups configured by numberOfHARQ-BundlingGroups for a serving cell and NPDSCH,MAX is the maximum configured number of PDSCHs for the serving cell. - The PDSCHs corresponding to [configured or valid] SLIVs in a TDRA row index indicated by multi-PDSCH scheduling DCI are allocated to the bundling groups, e.g., if NHBG =4, NPDSCH,MAX =8, and 5 PDSCHs are scheduled, then 2/1/1/1 PDSCHs are assigned to each group, by reusing CBG grouping method. - For a group that is empty or is filled with only invalid PDSCH(s), HARQ-ACK bits for the bundling group is set to NACK (same principle as when no time bundling configured) - Logical AND operation is applied to across all valid PDSCHs within the same bundling group to generate 1 HARQ-ACK bit per group, at least for 1-TB case - If the number of HARQ bundling groups is configured as 1 for a serving cell, HARQ-ACK bits corresponding to any DCI for the serving cell belong to the first sub-codebook. - At least for 1-TB case, if the number of HARQ bundling groups is configured as larger than 1 for a serving cell, HARQ-ACK bits corresponding to multi-PDSCH scheduling case (which implies a multi-PDSCH DCI schedules more than one PDSCH) for the serving cell belong to the second sub-codebook, - Where the number of HARQ-ACK bits corresponding to a multi-PDSCH DCI is determined based on the maximum of Q value across all serving cells within the same PUCCH cell group, and Q=maximum configured number of PDSCHs for a cell without numberOfHARQ-BundlingGroups configured or Q=number of configured HARQ bundling groups for a cell with numberOfHARQ-BundlingGroups configured

In the last RAN1#107-e meeting, RAN1 agreed to support time domain bundling for Type-2 HARQ-ACK CB. The remaining issues is how to make a bundling group. Two options were discussed; one is to use configured SLIVs in a TDRA row, in which SLIVs are grouped without taking into account the SLIV’s validity. Therefore, a bundling group may contain invalid SLIVs only, which means that HARQ-ACK information for the bundling group is always set to ‘NACK’. Another option is to use valid SLIVs (not overlapping semi-static UL symbols) only to make a bundling group. Since every bundling groups consist valid SLIVs, all HARQ-ACK information for the bundling groups is meaningful. We compare two option in Table 1, where it is assumed that a TDRA row contains 8 SLIVs and 4 bundling groups are generated. 2 or 3 consecutive invalid PDSCHs are considered since the semi-static UL symbols are consecutive within a TDD period (if single TDD period is configured).

Table 1. Configured PDSCH vs Valid PDSCH

|  | 8 scheduled PDSCHs are denoted as 01234567 and x denotes invalid one | Configured PDSCH-based grouping | Valid PDSCH-based grouping |
| --- | --- | --- | --- |
| 2 consecutive invalid PDSCHs | 012345xx 01234xx7 0123xx67 012xx567 01xx4567 0xx34567 xx234567 | {01},{23},{45},{xx} {01},{23},{4x},{x7} {01},{23},{xx},{67} {01},{2x},{x5},{67} {01},{xx},{45},{67} {0x},{x3},{45},{67} {xx},{23},{45},{67} | {01},{23},{4},{5} {01},{23},{4},{7} {01},{23},{6},{7} {01},{25},{6},{7} {01},{45},{6},{7} {03},{45},{6},{7} {23},{45},{6},{7} |
| 3 consecutive invalid PDSCHs | 01234xxx 0123xxx7 012xxx67 01xxx567 0xxx4567 xxx34567 | {01},{23},{4x},{xx} {01},{23},{xx},{x7} {01},{2x},{xx},{67} {01},{xx},{x5},{67} {0x},{xx},{45},{67} {xx},{x3},{45},{67} | {01},{2},{3},{4} {01},{2},{3},{7} {01},{2},{6},{7} {01},{5},{6},{7} {04},{5},{6},{7} {34},{5},{6},{7} |

Based on Table 1, the configured PDSCH-based grouping generates un-necessary and meaningless HARQ-ACK bits. For example, in case of 2 consecutive invalid PDSCHs, there are a bundling group consisting of invalid PDSCHs only in half of cases. More severely, in case of 3 consecutive invalid PDSCHs, there are a bundling group consisting of invalid PDSCHs only in all cases. It is evident that the valid PDSCH-based grouping provides finer granularity for bundling grouping so that gNB can avoid un-necessary PDSCH retransmission.

In the last meeting, RAN1 discussed the performance of valid PDSCH-based grouping and configured PDSCH-based grouping and the proponent of the configured PDSCH-based grouping argued that the configured PDSCH-based grouping may provide batter performance due to that the valid PDSCH-based grouping puts non-consecutive PDSCHs in a bundling group. However, we cannot agree this argument based on the following analysis.

For reference, the “yellow” marked bundling group is empty as a result of configured SLIV based construction and the “cyan” marked bundling group is a group with non-consecutive SLIVs. Based on these analysis, we observe that

- For 2 consecutive invalid PDSCHs, 4 cases include empty bundling group, but 2 cases include non-consecutive SLIVs

- For 3 consecutive invalid PDSCHs, 6 cases include empty bundling group, but only 1 case includes non-consecutive SLIVs

Also, decoding results of two consecutive SLIVs can be less correlated. For example, since there is a conclusion to not limit maximum gap between two adjacent SLIVs, the decoding results of two adjacent SLIVs might be less correlated. Furthermore, if we consider initially transmitted PDSCH and retransmitted PDSCH, the decoding results of two PDSCHs (even if two are consecutive) are less correlated. So, for 2 or 3 consecutive invalid PDSCHs, in average sense, the valid SLIV is not worse than the configured one. Also, its benefit becomes significant when more SLIVs are invalid.

In the last meeting, RAN1 also discussed specification impact and UE implementation complexity. For specification impact, it was identified that either of alternatives may require additional specification works. For UE implementation complexity, for HARQ process ID determination, a UE first determines which SLIVs are valid regardless of two options. After determining the valid SLIVs, two options may have different complexity. For valid SLIVs, the HARQ-ACK information for valid SLIVs is considered to apply HARQ-ACK bundling and any invalid SLIVs are not considered. However, for configured SLIVs, the HARQ-ACK information for both valid SLIVs and invalid SLIVs is considered to apply HARQ-ACK bundling. Also, for configured SLIVs, when applying the HARQ-ACK bundling, the HARQ-ACK information for the invalid SLIV is considered as ACK if at least one SLIV in the bundling group is valid. Otherwise (i.e., all SLIVs in the bundling group are invalid), the HARQ-ACK information for the invalid SLIV is considered as NACK. So, it is not clear configured PDSCH-based grouping is simpler than valid PDSCH-based grouping in terms of UE implementation.

Proposal 9: Support to use valid PDSCH-based grouping for Type-2 HARQ-ACK CB with time-domain bundling

Proposal 10: Adopt TP#3 in Appendix for TS38.213

RM power control for Type-2 HARQ-ACK CB

From Clause 9.1.3.1 in TS38.213 v17.0.0, nHARQ-ACK is determined for using PUCCH transmission power if OACK+OSR+OCSI≤11. However, nHARQ-ACK is not specified when time domain bundling is configured. To support time-domain bundling for type-2 HARQ-ACK CB, we propose to adopt the TP#4 as a baseline.

Proposal 11: Adopt TP#4 in Appendix for TS38.213

C-DAI/T-DAI counting order

In RAN1-105e, the following agreement is endorsed:

Agreement: If Alt 2 (C-DAI/T-DAI is counted per PDSCH) is adopted for generating type-2 HARQ-ACK codebook corresponding to a DCI that can schedule multiple PDSCHs, - PDSCH(s) scheduled by a single DCI is counted firstly, serving cell(s) in the same PUCCH cell group and same PDCCH monitoring occasion is counted secondly, and PDCCH monitoring occasion(s) is counted thirdly. - The bit width of counter DAI field in fallback DCI (i.e., DCI formats 0_0 and 1_0) remains the same as in Rel-15 NR. - Note: The DAI bit width and number of sub-codebooks shall ensure that at most 3 consecutive missed DCIs can be resolved, same as in Rel-15/16 NR - This shall not impose additional gNB’s scheduling restriction. - In case where CBG retransmission is not configured for any serving cell in a same PUCCH cell group, the number of bits for each of counter DAI and total DAI in non-fallback DCI is extended (if needed) at least based on - The number of SLIVs associated with the row indexes in TDRA table - FFS: details - FFS: the case with configuration of CBG retransmission - FFS: the number of sub-codebooks FFS: for the UE indicating by type2-HARQ-ACK-Codebook support for more than one PDSCH reception on a serving cell that are scheduled from a same PDCCH monitoring occasion

This FFS is targeted for the situation when UE supports type2-HARQ-ACK-Codebook capability (FG 18-9: Type2 HARQ-ACK codebook for>1 unicast DL DCIs in same Monitoring Occasions) in cross-slot scheduling, how to count the PDSCHs when two DCIs belongs to the same MOs. Note that this FFS is under Alt 2 in the agreement and Alt 1 is adopted in the last RAN1 meeting but the DCI counting order can be an issue in Alt 1 as well.

In Rel-16, when two PDSCHs scheduled by two DCIs from the same MO, the counting order is determined by the PDSCH reception starting time. Now consider the case when DCI can schedule multiple PDSCHs, it is possible that PDSCHs scheduled by different DCIs same MO are interlaced in time domain, as shown in Figure 5. To be specific, the first PDSCH scheduled by DCI2 (PDSCH3) comes between 2 PDSCH scheduled by DCI1. If we follow the Rel-16 rule, the PDSCH3 should be counted after PDSCH1 but before PDSCH2. It is unclear how c-DAI/t-DAI in DCI1 and DCI2 are indicated. One solution is to divide PDSCH into sets by the scheduling DCIs. The PDSCHs in each set are counted separately. And the order between different set of PDSCHs are determined by the reception time of the first PDSCH in each set. By this rule, the counting order for the PDSCHs in Figure 5 is {PDSCH1, PDSCH2, PDSCH3, PDSCH4}.

Figure 1 DCIs in same MO both scheduling multiple PDSCHs, PDSCHs interlaced

Proposal 12: when a UE supports UE capability type2-HARQ-ACK-Codebook (FG 18-9), and there are >1 DCIs belonging to the same MOs and scheduling PDSCHs to the same serving cell. And these DCIs are configured to be able to schedule multiple PDSCHs. The counting procedure for the PDSCHs scheduled by these DCIs are:

- PDSCHs are separated into different sets and each set of PDSCHs are scheduled by the same DCI. PDSCHs are counted separately for different sets.

- The counting order between different sets of PDSCHs are based on the reception time of the first PDSCH in each set.

- The valid PDSCHs are only considered.

3.2.3 Remaining issues on HARQ Process numbers

RAN1 agreed the following agreements, where up to 32 HARQ processes for 120/480/960kHz SCS can be used, subject to UE capability.

Agreement: (RAN1#106-e) For NR FR2-2 at least for 480/960 kHz SCS, support 32 as the maximum number of HARQ processes for DL and UL, subject to UE capability. - Note: Up to 32 maximal supported HARQ process number is already agreed in Rel-17 NTN WI. - Working assumption: The same solution to support up to 32 HARQ process number in Rel-17 NTN WI is reused for NR FR2-2. Agreement (RAN1#107b-e) - In NR FR2-2, a UE supporting 32 maximum number of HARQ processes for 480/960 kHz SCS for DL (or for UL) shall support 32 as the maximum number of HARQ processes for 120 kHz SCS for DL (or UL), subject to UE capability.

In Clause 5.1 of TS38.214 v17.0.0, HARQ process ID skipping is defined, where modulo operation of nrofHARQ-ProcessesForPDSCH is applied. However, a UE may not be configured with nrofHARQProcessesForPDSCH. In this case, the UE assumes 8 HARQ processes. Similarly, if a UE is not configured with nrofHARQProcessesForPUSCH, the UE assumes up to 16 HARQ processes. These are not captured in the specification.

Proposal 13: Adopt TP#5 in Appendix for TS38.214

- Conclusion

The observations and proposals made in this contribution are summarized below:

Observation 1: If time domain bundling is configured, Type-1 HARQ-ACK CB does not cover PDSCH repetitions scheduled by DCI format 1_2.

Proposal 1: For SPS/CG retransmission, down select one of options.

- Option 1. It is allowed for gNB to indicate a TDRA row with single SLIV only.

- Option 2. It is allowed for gNB to indicate a TDRA row with more than one SLIVs, where

  - Alt 1: A UE does not expect to be receive a DCI format scheduling multiple SPS PDSCHs (or CG PUSCHs) retransmissions with a HARQ process ID not dedicated for any SPS configurations (CG configurations).

  - Alt 2: A UE does not receive (or transmit) a SPS PDSCH (or CG PUSCH) retransmission with a HARQ process ID not dedicated for any SPS configurations (CG configurations).

  - Alt 3: A UE receives (or transmit) a SPS PDSCH (or CG PUSCH) retransmission corresponding to [the first/last SLIV] in the TDRA table.

Proposal 2: Clarify that for Scell dormancy indication, a UE repurposes Npdsch,max-bit NDI and Npdsch,max-bit RV fields if TDRA indicates multi-PDSCH scheduling or 1-bit NDI and 2-bit RV fields if TDRA indicates single-PDSCH scheduling.

- If Npdsch,max-bit NDI and Npdsch,max-bit RV fields are repurposed, the sequence order for a bitmap is 5-bit MCS, Npdsch,max-bit NDI, Npdsch,max-bit RV, HPN, antenna port(s), and DMRS sequence initialization fields

Proposal 3: For a DCI format carrying TCI-State indication, a UE does not expect that the DCI format indicates a TDRA row with more than one SLIVs.

Proposal 4: For single TRP or multi-TRP operation, for 480/960 kHz SCS,

  - A UE does not expect to receive more than one unicast PDSCH in a slot on a serving cell from the same TRP.

  - A UE does not expect to transmit more than one PUSCH in a slot on a serving cell from the same TRP.

Proposal 5: For resolving collision of overlapping PDSCHs and/or PUSCHs and/or PUCCHs in case of M-PDSCH/M-PUSCH scheduling, UE first resolves the collision of PDSCHs/PUSCHs and semi-static UL/DL symbols and then UE resolves the collision among PDSCHs, PUSCHs and PUCCHs.

Proposal 6: Adopt TP#1 in Appendix for Clause 5.1.2.1 and Clause 6.1.2.1 of TS38.214.

Proposal 7: To support multi-PDSCH scheduling by DCI format 1_1 and PDSCH repetition by DCI format 1_2 in type-1 HARQ-ACK CB, a row r in the set R’ and the set R is removed when the both conditions are met

  - Condition 1 for multi-PDSCH scheduling) each SLIVs of the TDRA row r overlapped with a semi-static UL symbol

  - Condition 2 for PDSCH repetition) the last SLIV of the TDRA row r over K slots overlapped with a semi-static UL symbol.

Proposal 8: Adopt TP#2 in Appendix for TS38.213

Proposal 9: Support to use valid PDSCH-based grouping for Type-2 HARQ-ACK CB with time-domain bundling

Proposal 10: Adopt TP#3 in Appendix for TS38.213

Proposal 11: Adopt TP#4 in Appendix for TS38.213

Proposal 12: when a UE supports UE capability type2-HARQ-ACK-Codebook (FG 18-9), and there are >1 DCIs belonging to the same MOs and scheduling PDSCHs to the same serving cell. And these DCIs are configured to be able to schedule multiple PDSCHs. The counting procedure for the PDSCHs scheduled by these DCIs are:

- PDSCHs are separated into different sets and each set of PDSCHs are scheduled by the same DCI. PDSCHs are counted separately for different sets.

- The counting order between different sets of PDSCHs are based on the reception time of the first PDSCH in each set.

- The valid PDSCHs are only considered.

Proposal 13: Adopt TP#5 in Appendix for TS38.214

- Appendix

TP#1 for TS 38.214

============================== Start of TP #1 for TS 38.214 =================================

5.1.2.1	Resource allocation in time domain

=============================== Unchanged Text Omitted ===================================

If a UE is configured with pdsch-TimeDomainAllocationListForMultiPDSCH-r17, the UE does not expect to be configured with higher layer parameter repetitionNumber in pdsch-TimeDomainAllocationListForMultiPDSCH-r17.

If a UE is configured with pdsch-TimeDomainAllocationListForMultiPDSCH-r17 on a DL BWP of a serving cell, the UE does not apply pdsch-AggregationFactor in PDSCH-config, if configured, to DCI format 1_1 on the DL BWP of the serving cell.

=============================== Unchanged Text Omitted ===================================

6.1.2.1	Resource allocation in time domain

If a UE is configured with pusch-TimeDomainAllocationListForMultiPUSCH-r17 on a UL BWP of a serving cell, the UE does not apply pusch-AggregationFactor, if configured, to DCI format 0_1 on the UL BWP of the serving cell and the UE does not expect to be configured with numberOfRepetitions in pusch-TimeDomainAllocationListForMultiPUSCH-r17.

============================== End of TP #1 for TS 38.214 =================================

TP#2 for TS 38.213

============================== Start of TP #2 for TS 38.213 ==================================

9.1.2.1	Type-1 HARQ-ACK codebook in physical uplink control channel

=============================== Unchanged Text Omitted ===================================

while r<CR

if the UE is not provided enableTimeDomainHARQ-Bundling and is provided tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot from slot n0,k+nD-NPDSCHrepeat,max+1 to slot n0,k+nD, at least one symbol of the PDSCH time resource derived by row r is configured as UL where K1,k is the k-th slot timing value in set K1, where n0,k is a DL slot with a smallest index among DL slots overlapping with UL slot nU-K1,k, or subslotLengthForPUCCH is provided for the HARQ-ACK codebook and the end of the PDSCH time resource for row r is not within any UL slot nU-K1,l, 0≤l<CK1, or if HARQ-ACK information for PDSCH time resource derived by row r in slot n0,knU-K1,k⋅2μDL-μUL+nD cannot be provided in slot nU

R=R\r;

elseif the UE is provided enableTimeDomainHARQ-Bundling and tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated and, for each slot n0,knU-K1,k⋅2μDL-μUL+nD- ∆K0,r(d), at least one symbol of the PDSCH time resource derived by row r of set R' is configured as UL, where d = 0,1,…,C∆K0,r-1, ∆K0,r=maxK0K0,r-K0,r, and C∆K0,r is the cardinality of ∆K0,r and for each slot from n0,k+nD-NPDSCHrepeat,max+1 to slot n0,k+nD, at least one symbol of the PDSCH time resource derived by row r of set R is configured as UL

R=R\r;

R'=R'\r;

else

r=r+1;

end if

end while

============================== End of TP #2 for TS 38.213 ==================================

TP#3 for TS 38.213

============================== Start of TP #3 for TS 38.213 ==================================

9.1.3.1	Type-2 HARQ-ACK codebook in physical uplink control channel

=============================== Unchanged Text Omitted ===================================

If a UE is provided numberOfHARQ-BundlingGroups for a serving cell c, the UE generates HARQ-ACK information over transport block groups (TBGs) for PDSCH receptions where, for a maximum number of NPDSCHmax PDSCH receptions scheduled by a DCI format on the serving cell, a maximum number of TBGs NHARQ-ACK,cTBG,max is provided by numberOfHARQ-BundlingGroups. If the UE detects a DCI format scheduling NPDSCH,c PDSCH receptions, not overlapping with a UL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, on the serving cell c, the UE generates NHARQ-ACK,cTBG,max HARQ-ACK information bits for first TBs and, if applicable, generates NHARQ-ACK,cTBG,max HARQ-ACK information bits for second TBs in the NPDSCH,c PDSCH receptions as described in clause 9.1.1 by setting NHARQ-ACKCBG/TB,max=NHARQ-ACK,cTBG,max and C=NPDSCH,c.

============================== End of TP #3 for TS 38.213 ==================================

TP#4 for TS 38.214

============================== Start of TP #4 for TS 38.214 =================================

5.1	UE procedure for receiving the physical downlink shared channel

=============================== Unchanged Text Omitted ===================================

A UE shall upon detection of a PDCCH with a configured DCI format 1_0, 1_1 or 1_2 decode the corresponding PDSCHs as indicated by that DCI. When the UE is scheduled with multiple PDSCHs by a DCI, HARQ process ID indicated by this DCI applies to the first PDSCH not overlapping with a UL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, HARQ process ID is then incremented by 1 for each subsequent PDSCH(s) in the scheduled order, with modulo operation of nrofHARQ-ProcessesForPDSCH applied if nrofHARQ-ProcessesForPDSCH is provided, or with modulo operation of 8 applied, otherwise.

=============================== Unchanged Text Omitted ===================================

6.1	UE procedure for transmitting the physical uplink shared channel

=============================== Unchanged Text Omitted ===================================

A UE shall upon detection of a PDCCH with a configured DCI format 0_0, 0_1 or 0_2 transmit the corresponding PUSCH as indicated by that DCI unless the UE does not generate a transport block as described in [10, TS38.321]. Upon detection of a DCI format 0_1 or 0_2  with 'UL-SCH indicator' set to '0' and with a non-zero 'CSI request' where the associated reportQuantity in CSI-ReportConfig set to 'none' for all CSI report(s) triggered by 'CSI request' in this DCI format 0_1 or 0_2, the UE ignores all fields in this DCI except the 'CSI request' and the UE shall not transmit the corresponding PUSCH as indicated by this DCI format 0_1 or 0_2. When the UE is scheduled with multiple PUSCHs by a DCI, HARQ process ID indicated by this DCI applies to the first PUSCH not overlapping with a DL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided, or a symbol of an SS/PBCH block with index provided by ssb-PositionsInBurst, HARQ process ID is then incremented by 1 for each subsequent PUSCH(s) in the scheduled order, with modulo operation of nrofHARQ-ProcessesForPUSCH applied if nrofHARQ-ProcessesForPUSCH is provided, or with modulo operation of 16 applied, otherwise.

============================== End of TP #4 for TS 38.214 =================================

TP#5 for TS 38.213

============================== Start of TP #5 for TS 38.213 =================================

9.1.3.1	Type-2 HARQ-ACK codebook in physical uplink control channel

=============================== Unchanged Text Omitted ===================================

If a UE

-	is provided PDSCH-TimeDomainResourceAllocationListForMultiPDSCH and, if provided, numberOfHARQ-BundlingGroups with value NHARQ-ACKTBG,max>1 for NcellsDL,TBG serving cells; and

-	is not provided PDSCH-TimeDomainResourceAllocationListForMultiPDSCH or is provided numberOfHARQ-BundlingGroups with value NHARQ-ACKTBG,max=1, for NcellsDL,TB serving cells where NcellsDL,TB+NcellsDL,TBG=NcellsDL

the UE determines the o0ACK, o1ACK,⋯,oOACK-1ACK according to the previous pseudo-code with the following modifications

-	NcellsDL is used for the determination of a first HARQ-ACK sub-codebook for

-	SPS PDSCH reception,

-	any DCI format having associated HARQ-ACK information without scheduling PDSCH reception, and

-	PDSCH reception scheduled by a DCI format scheduling one PDSCH

-	PDSCH reception with NHARQ-ACKTBG,max=1 for TBG-based HARQ-ACK information on the NcellsDL,TB serving cells,

-	NcellsDL is replaced by NcellsDL,TBG for the determination of a second HARQ-ACK sub-codebook corresponding to the NcellsDL,TBG serving cells for TBG-based HARQ-ACK information, or for TB-based HARQ-ACK information corresponding to multiple PDSCH receptions scheduled by a single DCI format, and

-	if, for an active DL BWP of a serving cell, the UE is not provided coresetPoolIndex or is provided coresetPoolIndex with value 0 for one or more first CORESETs and is provided coresetPoolIndex with value 1 for one or more second CORESETs, and is provided ackNackFeedbackMode = joint, the serving cell is counted as two times where the first time corresponds to the first CORESETs and the second time corresponds to the second CORESETs, and

-	instead of generating one HARQ-ACK information bit per transport block for a serving cell from the NcellsDL,TBG serving cells, the UE generates NHARQ-ACK,maxTBG,max HARQ-ACK information bits, where NHARQ-ACK,maxTBG,max is the maximum value between NTB,cDL⋅NHARQ-ACK,cTBG,max across all NcellsDL,TBG serving cells if the UE is provided numberOfHARQ-BundlingGroups, and NTB,cDL⋅NPDSCH,cmax across all NcellsDL,TBG serving cells where the UE is not provided numberOfHARQ-BundlingGroups, and NTB,cDL is the value of maxNrofCodeWordsScheduledByDCI for serving cell c. If for a serving cell c where the UE is provided numberOfHARQ-BundlingGroups, it is NTB,cDL⋅NHARQ-ACK,cTBG,max<NHARQ-ACK,maxTBG,max, the UE generates NACK for the last NHARQ-ACK,maxTBG,max-NTB,cDL⋅NHARQ-ACK,cTBG,max HARQ-ACK information bits for serving cell c. If for a serving cell c where the UE is not provided numberOfHARQ-BundlingGroups, it is NTB,cDL⋅NPDSCH,cmax <NHARQ-ACK,maxTBG,max, the UE generates NACK for the last NHARQ-ACK,maxTBG,max-NTB,cDL⋅NPDSCH,cmax  HARQ-ACK information bits for serving cell c.

-	The pseudo-code operation when PDSCH-CodeBlockGroupTransmission is provided is not applicable.

-	The counter DAI value and the total DAI value apply separately for each HARQ-ACK sub-codebook.

-	The UE generates the HARQ-ACK codebook by appending the second HARQ-ACK sub-codebook to the first HARQ-ACK sub-codebook.

If OACK+OSR+OCSI≤11 and NcellsDL,TBG>0, the UE also determines nHARQ-ACK=nHARQ-ACK,TB+nHARQ-ACK,TBG for obtaining a PUCCH transmission power, as described in clause 7.2.1, with

nHARQ-ACK,TBG=VDAI,mlastDL-c=0NcellsDL,TBG-1UDAI,cTBGmodTDNHARQ-ACK,maxTBG,maxNTB,maxDL+c=0NcellsDL,TBG-1m=0M-1Nm,creceived,TBG

where

-	if NcellsDL=1, VDAI,mlastDL is the value of the counter DAI in the last DCI format scheduling more than one PDSCH reception with TBG-based HARQ-ACK information that the UE detects within the M PDCCH monitoring occasions

-	if NcellsDL>1, VDAI,mlastDL is the value of the total DAI in the last DCI format scheduling more than one PDSCH reception with TBG-based HARQ-ACK information for any serving cell c of NcellsDL,TBG serving cells that the UE detects within the M PDCCH monitoring occasions

-	VDAI,mlastDL=0, if the UE does not detect any DCI format scheduling more than one PDSCH reception with TBG-based HARQ-ACK information for any serving cell c of NcellsDL,TBG serving cells in any of the M PDCCH monitoring occasions

-	UDAI,cTBG is the total number of DCI formats scheduling more than one PDSCH reception with TBG-based HARQ-ACK information for any serving cell c of NcellsDL,TBG serving cells that the UE detects within the M PDCCH monitoring occasions. UDAI,cTBG=0 if the UE does not detect any such DCI format

-	if harq-ACK-SpatialBundlingPUCCH is not provided,

-	Nm,creceived,TBG is the number of TBGs including at least one PDSCH not overlapping with a UL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided the UE receives in serving cell c of NcellsDL,TBG serving cells in PDCCH monitoring occasion m and the UE reports corresponding HARQ-ACK information in the PUCCH

-	if harq-ACK-SpatialBundlingPUCCH is provided,

-	Nm,creceived,TBG is the number of groups of the PDSCHs including at least one PDSCH not overlapping with a UL symbol indicated by tdd-UL-DL-ConfigurationCommon or tdd-UL-DL-ConfigurationDedicated if provided the UE receives in serving cell c of NcellsDL,TBG serving cells in PDCCH monitoring occasion m and the UE reports corresponding HARQ-ACK information in the PUCCH

-	NTB,maxDL=2 if the value of maxNrofCodeWordsScheduledByDCI is 2 for any serving cell c of NcellsDL,TBG serving cells and harq-ACK-SpatialBundlingPUCCH is not provided; otherwise, NTB,maxDL=1

============================== End of TP #5 for TS 38.213 =================================