3GPP TSG RAN WG1 Meeting #108-e                                          R1- 2202016

e-Meeting, February 21th – March 3rd, 2022

Agenda Item:		8.5.3

Source:				Samsung

Title:					Discussion on accuracy improvements for DL-AoD positioning solutions

Document for:	Discussion

# Introduction

In the approved WI [1] for positioning enhancement, the angle-based method was recognized as one important direction to improve the positioning accuracy.

- Specify methods, measurements, signalling, and procedures for improving positioning accuracy of the Rel-16 NR positioning methods by mitigating UE Rx/Tx and/or gNB Rx/Tx timing delays, including [RAN1, RAN2, RAN3, RAN4] - DL, UL and DL+UL positioning methods - UE-based and UE-assisted positioning solutions

In this paper, we will discuss accuracy improvements for DL-AoD positioning solutions by improving the measurement reporting.

# AoD measurement for first arrival path

In RAN1 #104 meeting, the following agreement was reached regarding the first arrival path measurement for DL-AoD positioning:

Agreement: For both UE-based and UE-assisted DL-AOD, the UE can be requested subject to UE capability to measure and report (for UE-assisted) the PRS RSRP of the first path - FFS: Details of measurement and reporting of PRS RSRP of the first path

In Rel-16, a UE is expected to report the DL PRS-RSRP as defined in TS38.215, thus including all paths. For Rel-17, it is preferred to report, for at least some cases, the measurements computed on the first path only since this path has a higher probability of being LOS. Thus, for backward compatibility with Rel-16 AoD positioning, an indicator in the configuration for DL-AoD to specify if the reported measurement is for the first arrival path or for all the paths is necessary. While the exact signaling is up to RAN2, the indicator may be included in IE NR-DL-AoD-MeasElement alongside with DL PRS-RSRP measurement report. If this indicator is not present, the network assumes that the measurement is performed on all paths.

Proposal 1: An indicator of whether the report for PRS RSRP includes all the paths or the first arrival path only is supported.

# Enhancement on beam reporting

In RAN1 #107 meeting, the following agreement was reached for DL-AoD positioning:

Agreement For UE-assisted DL-AOD positioning method, to enhance the signaling to the UE for the purpose of PRS resource(s) reporting, the LMF may indicate in the assistance data (AD), one or both the following: - option 1: subject to UE capability, for each PRS resource, a subset of PRS resources for the purpose of prioritization of DL-AOD reporting: - a UE may include the requested PRS measurement for the subset of the PRS in the DL-AoD additional measurements if the requested PRS measurement of the associated PRS is reported - The requested PRS measurement can be DL PRS RSRP and/or path PRS RSRP. - UE may report PRS measurements only for the subset of PRS resources. - Note: The subset associated with a PRS resource can be in a same or different PRS resource set than the PRS resource - option 2: subject to UE capability, for each PRS resource, the boresight direction information. - Note: Either case does not imply any restriction on UE measurement - FFS: prioritization of the PRS resources and resource subsets to be measured

The left FFS issue is that whether there should be prioritization of the PRS resource and resource subsets to be measured.

For option 1, a subset of PRS resource has been associated with one PRS resource. By the agreed UE reporting on the subset of PRS resource associated with the given PRS resource, which somehow already indicate the prioritization of the subset of PRS resource for the reported associated PRS resource. A reasonable UE could already prioritize the subset PRS resource for the reported associated PRS resource.

For option 2, given the boresight direction, the UE could build the connection between QCLed SSB, PRS resource and the boresight direction. Then for a preferred SSB known by UE, the UE could prioritize the reception of PRS within the boresight direction (plus a range) corresponding to that preferred SSB. This is because the preferred SSB implies a relatively more accurate direction between UE and gNB, such that focus on these PRS under the promising direction could be more efficient.

Proposal 2: The PRS within the boresight direction (plus a range) corresponding to the QCLed SSB which is preferred by UE.

# Conclusion

This contribution discusses the accuracy improvements for DL-AoD positioning solutions. Observations and proposals are summarized as follows:

Proposal 1: An indicator of whether the report for PRS RSRP includes all the paths or the first arrival path only is supported.

Proposal 2: The PRS within the boresight direction (plus a range) corresponding to the QCLed SSB which is preferred by UE.

# References

- RP-210903, New WID on NR Positioning Enhancements, RAN#91-e, Mar, 2021.