3GPP TSG RAN WG1 #108                        	                                 R1-2202022

e-Meeting, Feb 21th – March 3rd, 2022

Agenda Item:		8.7.1.1

Source:			Samsung

Title:			Maintenance on paging enhancements

Document for:		Discussion and decision

# Introduction

In RAN1#107bis-e [1], the following were agreed:

Agreement

A new PEI-RNTI is supported for DCI format 2_7.

Agreement

For whether and how to accommodate PEI-O location determination for the case POnumPerPEI is smaller than Ns, decide one of the following alternatives

- Alt-2: It is supported, and UE applies the single value in PEI-F_offset for the frame-level offset and the (flooris/POnumPerPEI+1) -th value out of Ns/POnumPerPEI configured values in firstPDCCH-MonitoringOccasionOfPEI-O for the symbol-level offset

Note: The number of PO mapping to one PEI should be multiple of Ns when POnumPerPEI is larger than Ns

Agreement

For PEI-O location determination:

- Range of PEI-F_offset, in unit of frame, is 0, 1,2, …, 16

- Range of firstPDCCH-MonitoringOccasionOfPEI-O, in unit of symbol, is decided as one of the following alternatives:

  - Alt-1: The same value range as firstPDCCH-MonitoringOccasionOfPO

Agreement

For the value PEI-RNTI, decide one of the following:

- Alt-1: PEI-RNTI is of fixed value, and value design is up to RAN2.

Conclusion

For a UE supporting R17 paging enhancement feature, it is up to UE implementation whether the UE monitors the MO(s) for a PEI.

- If UE decides to not to monitor PEI, it has to monitor UE’s PO as defined in 38.304.

- Note: No specification impact

Agreement

The PDCCH monitoring occasions defined by peiSearchSpace colliding with UL symbols are omitted from the determination of the PEI monitoring occasions. The PDCCH monitoring occasions for PEI which do not overlap with UL symbols (determined according to tdd-UL-DL-ConfigurationCommon) are sequentially numbered from zero starting from the first PDCCH monitoring occasion for PEI in the PEI-O.

Agreement

The transmission power of PEI is based on the same determination method as paging PDCCH, and the following TP to TS 38.213 is adopted.

4.1 Cell search Cell search is the procedure for a UE to acquire time and frequency synchronization with a cell and to detect the physical layer Cell ID of the cell. A UE receives the following synchronization signals (SS) in order to perform cell search: the primary synchronization signal (PSS) and secondary synchronization signal (SSS) as defined in [4, TS 38.211]. A UE assumes that reception occasions of a physical broadcast channel (PBCH), PSS, and SSS are in consecutive symbols, as defined in [4, TS 38.211], and form a SS/PBCH block. The UE assumes that SSS, PBCH DM-RS, and PBCH data have same EPRE. The UE may assume that the ratio of PSS EPRE to SSS EPRE in a SS/PBCH block is either 0 dB or 3 dB. If the UE has not been provided dedicated higher layer parameters, the UE may assume that the ratio of PDCCH DMRS EPRE to SSS EPRE is within -8 dB and 8 dB when the UE monitors PDCCHs for a DCI format 1_0 with CRC scrambled by SI-RNTI, P-RNTI, or RA-RNTI, and DCI format 2_7 with CRC scrambled by PEI-RNTI.

Agreement

UE expects at most one PEI outcome from each PDCCH monitoring occasion of PEI-O, and the following text proposal is adopted.

Section 10.1 of TS 38. 213 *** Unchanged text is omitted *** If a UE is provided one or more search space sets by corresponding one or more ofsearchSpaceZero, searchSpaceSIB1, searchSpaceOtherSystemInformation,pagingSearchSpace, peiSearchSpace, ra-SearchSpace, or a CSS set byPDCCH-Config, and a SI-RNTI, a P-RNTI, a PEI-RNTI, a RA-RNTI, a MsgB-RNTI, a SFI-RNTI, an INT-RNTI, a TPC-PUSCH-RNTI, a TPC-PUCCH-RNTI, or a TPC-SRS-RNTI then, for a RNTI from any of these RNTIs, the UE does not expect to process information from more than one DCI format with CRC scrambled with the RNTI per slot. *** Unchanged text is omitted ***

Agreement

If one PEI-O is associated with POs of 2 PFs,

- The two PFs are consecutive PFs configured in SIB

  - FFS: two PFs are consecutive PFs within the same paging cycle

- Note: As an example, SFN of the first PF of the PF(s) associated with the PEI-O can be obtained by: (SFN of UE’s PF) - iPO/Ns⋅T/N

Agreement

- LS to RAN2 for sharing agreements with potential RAN2 specification impacts

Conclusion

For Rel-17, inclusion of SI change indication or ETWS/CMAS notification in DCI format 2_7 is not supported.

Conclusion

For Rel-17 DCI format 2_7,

- Explicit starting position configuration for paging indication field or TRS availability indication field is not supported.

  - Paging indication field starts from the first bit, and TRS availability indication field starts from the next bit after the end of the paging indication field

- UE does not expect the total size of paging indication filed and TRS availability indication field is larger than the DCI payload size

Decision: As per email decision posted on Jan 27th,

Working assumption

SearchSpaceId = 0 can be configured forpeiSearchSpace for the case of CORESET multiplexing pattern 2 or 3

This contribution considers remaining issues on supporting PDCCH based paging early indication.

# 2 Remaining issues of PDCCH monitoring for early indication of paging

## 2.1 Clarification on PEI-O

Agreement If one PEI-O is associated with POs of 2 PFs, - The two PFs are consecutive PFs configured in SIB - FFS: two PFs are consecutive PFs within the same paging cycle - Note: As an example, SFN of the first PF of the PF(s) associated with the PEI-O can be obtained by: (SFN of UE’s PF) - iPO/Ns⋅T/N …

According to the above agreement in RAN1#107-bis meeting, it’s FFS whether to restrict two PFs are consecutive PFs if a PEI-O is associated with POs from 2 PFs. We think the restriction is necessary; otherwise UE has to support both the case when two PFs are from same DRX and the other case when two PFs are from different DRX cycles. It will increase unnecessary UE implementation complexity. Also, we think the restriction has been supported according to previous agreement that POnumPerPEI is a factor of  (total PO number in a paging cycle). For the formula in the note, we think it’s related to UE implementation. UE is provided with sufficient information to identify the first PF associated with PEI monitoring. So, we think the note is not needed.

Conclusion: if one PEI-O is associated with POs of 2 PFs, the two PFs are consecutive PFs within the same paging cycle.

# 3. Conclusion

This contribution discussed remaining issues for supporting PDCCH based PEI. Following proposals are made:

Conclusion: if one PEI-O is associated with POs of 2 PFs, the two PFs are consecutive PFs within the same paging cycle.

# 4. Reference

[1] RAN1 Chairman’s Note, 3GPP TSG RAN WG1 Meeting #107bis-e