3GPP TSG RAN WG1 #108                   	                                        R1-2202023

e-Meeting, February 21th –  March 3rd, 2022

Agenda Item:		8.7.1.2

Source:			Samsung

Title:			Maintenance on TRS/CSI-RS for idle/inactive UEs

Document for:		Discussion and decision

# 1 Introduction

In RAN1#107bis e-meeting [1], the following were agreed:

Agreement

Support a configuration parameter for the number of, X, TRS resources for a TRS resource set at least for FR2

- Applicable values for X: {2, 4}

- For FR1, X is based on configuration parameter with applicable value {2, 4}

Note: Configuration follows restriction specified in sub-clause 5.1.6.1.1 in TS38.214 for connected mode TRS

Agreement

One or more scrambling IDs is configured for a TRS resource set.

- If a single scrambling ID is configured, it applies to all the TRS resources.

- Otherwise, each TRS resource is provided with a scrambling ID.

Agreement

The applicable values:{64, 128, 256, 512} are supported for the validity duration configured by higher layer.

- Note: If UE is provided a configuration of validity duration longer than 10.24s, and the UE does not support eDRX, it is up to UE implementation whether to assume the validity duration length is no larger than 10.24s

Agreement

UE can receive L1 based signaling for TRS availability indication before the expiration/end of validity duration associated with previous L1 based signaling for TRS availability indication

- For each bit indicated as ‘1’ in the availability indication field of the current L1 based signaling, the UE assumes the corresponding TRS resource set(s) are available from the reference point until the end of the validity duration associated with the current L1 based signaling.

- For each bit indicated as ‘0’ in the availability indication field of the current L1 based signaling, the UE keeps the existing assumption on the availability or unavailability of the corresponding TRS resource set(s).

Note: the validity duration for different group of TRS resources sets correspond to different bits in the availability indication field can be different and are maintained independently.

Note: CATT has concern on the power saving gain based on this agreement.

Agreement

Confirm the following working assumption

If TRS resource is configured in SIB, L1 based availability indication is always enabled based on the configuration.

Agreement

The parameter of periodicityAndOffset is used to determine the location of the first slot of TRS resource set.

Agreement

If SIB configures TRS resource, TRS availability indication field is present in DCI format 2_7 (if configured) with CRC scrambled by PEI-RNTI and DCI format 1_0 with CRC scrambled by P-RNTI.

This contribution discusses remaining issues for supporting TRS/CSI-RS at the configured occasion(s) to the idle/inactive UEs.

# 2 Remaining issues

## Clarification on reference point for TRS validity duration

According to the following agreement made in RAN1#107-e meeting, the reference point is the first SFN of the first PF from the current DRX cycle where UE receives the L1 signaling based TRS availability indication.

From RAN1#107-e: Agreement The reference point for start of the validity duration is SFN of the first PF from the current default DRX cycle where UE receives the availability indication - FFS: Whether the availability indication is transmitted [only once] during the validity duration

In latest CR for TS 38.213, a formula to determine the SFN was captured according to the agreement. However, the description of the formula doesn’t match the agreement well. In our understanding, the current description of the reference point is the frame where UE receives the L1 signalling based TRS availability indication rather than the radio frame of the DRX cycle where UE receives the L1 signalling. Therefore, we propose the following TP to clarify the reference point for TRS validity duration.

Proposal 1: support the following TP#1 for TS 38.213.

Proposed TP # 1 for TS 38.213

## 10.4B Indication of TRS resources A UE in RRC_IDLE state or RRC_INACTIVE state can be provided by TRS-ResourceSetConfig a set of TRS occasions [6, TS 38.214]. If TRS-ResourceSetConfig is provided, a DCI format 2_7 or a DCI format 1_0 with CRC scrambled by P-RNTI includes a TRS availability indication field [4, TS 38.212] that provides a bitmap to groups of TRS resource sets where the configuration of each TRS resource set includes an association to a bit of the bitmap. The UE can be additionally provided a multiple, by validityDuration, for a number of frames provided by defaultPagingCycle for TRS resource sets with indicated presence; if validityDuration is not provided, the multiple is equal to 2. A value of '1' for a bit of the bitmap indicates presence of associated TRS resource sets for the multiple of the number of frames, starting from a SFN determined from SFN+PF_offsetmodT=0 [17, TS 38.304] that corresponds to the first frame of the DRX cycle that includes a PDCCH providing the DCI format 2_7, or the DCI format 1_0 with CRC scrambled by P-RNTI, with the TRS availability indication field indicating the TRS resource sets, where T is provided by defaultPagingCycle. A value of '0' for a bit of the bitmap is ignored by the UE.

# 5 Reference

[1] RAN1 Chairman’s Note, 3GPP TSG RAN WG1 Meeting #107bis-e