3GPP TSG RAN WG1 #108-e	R1-2202033

e-Meeting, January 21st – March 3rd, 2022

Agenda item:	8.11.2

Source: 	Samsung

Title: 	Discussion on Sidelink Enhancement

Document for:	Discussion and Decision

# Introduction

In RAN#86, a work item on “NR Sidelink enhancement” was approved [1]. This work item was further updated in RAN#88-e [2] to account for the delay in starting the work item due to travel restrictions. In RAN#90-e [3], the work item was further updated to continue studying the feasibility and benefit of reliability and reduced latency enhancements of mode 2 resource allocation..

In this contribution, we provide our views on RRC parameters for SL enhancements.

# Views on RRC Parameters

In RAN1#107b-e, there was a discussion on RRC parameters [9][10] for SL enhancements building on earlier discussions in RAN1#107-e [4][5][6][7][8].

The outcome of the discussion is summarized in Table 1.

Table 1: RRC parameters for SL enhancements agreed after RAN1#107b-e.

The following is a list of open issues after the discussion in RAN1#107bis-e.

- contiguousSensingWindowPeriodic: This parameter indicates the size of contiguous partial sensing window in logical slot units for a resource (re)selection procedure triggered by periodic transmission. If not configured, the size of contiguous partial sensing window in logical slot units is 31. The valid range of this parameter is [TBD]… 30. We suggest to have the valid range of this parameter from 0 to 30 slots. Setting this value to 0 allows the (pre-)configuration to disable continuous based partial sensing for periodic transmissions.

- contiguousSensingWindowAperiodic: This parameter indicates the minimum size of contiguous partial sensing window in logical slot units for a resource (re)selection procedure and re-evaluation/pre-emption checking triggered by aperiodic transmission. If not configured, the size of contiguous partial sensing window in logical slot units is 31. The valid range of this parameter is [0] … 30. We suggest to confirm the valid range of this parameter from 0 to 30 slots. Setting this value to 0 allows the (pre-)configuration to disable continuous based partial sensing for periodic transmissions.

- maxSlotOffsetTRIVScheme1: This parameter indicates the maximum value of logical slot offset with respect to a reference slot that is used for representing the first resource location of each TRIV to indicate the set of resources in Scheme 1. The valid range of this parameter is currently set to [1], …, [256]. We just to change the valid range to be from (0, 1, … 255)* SlotOffsetGranularityTRIVScheme1. Where the SlotOffsetGranularityTRIVScheme1 can be pre-configured to 1, 2, 4, 8, 16 or 32.

Proposal:

On RRC parameters for SL enhancements:

- For contiguousSensingWindowPeriodic, the valid range can be from 0 to 30 logical slots.

- For contiguousSensingWindowAperiodic, confirm the valid range can be from 0 to 30 logical slots.

- For maxSlotOffsetTRIVScheme1, the valid range can be (0, 1, … 255)* SlotOffsetGranularityTRIVScheme1, Where the SlotOffsetGranularityTRIVScheme1 can be pre-configured to 1, 2, 4, 8, 16 or 32

# Conclusions

This contribution presents our views on RRC parameters. The following proposal and observation have been made:

Proposal:

On RRC parameters for SL enhancements:

- For contiguousSensingWindowPeriodic, the valid range can be from 0 to 30 logical slots.

- For contiguousSensingWindowAperiodic, confirm the valid range can be from 0 to 30 logical slots.

- For maxSlotOffsetTRIVScheme1, the valid range can be (0, 1, … 255)* SlotOffsetGranularityTRIVScheme1, Where the SlotOffsetGranularityTRIVScheme1 can be pre-configured to 1, 2, 4, 8, 16 or 32

# References

RP‑193231, “NR Sidelink enhancement,” RAN#86, Stiges, Spain, December, 2019.

RP‑201385, “NR Sidelink enhancement,” RAN#88-e, e-meeting, June, 2020.

RP-202846, “NR Sidelink enhancement,” RAN#90-e, e-meeting, December, 2020.

R1-2108681, “[Post-106-e-Rel17-RRC-11] Summary of email discussion on RRC parameters for Sidelink enhancement”, Moderator (LG Electronics), RAN1#106-e, e-meeting, August 2021.

R1-2110649, “[106bis-e-R17-RRC-Sidelink] Summary of email discussion on Rel-17 RRC parameters for sidelink enhancement”, Moderator (LG Electronics), RAN1#106bis-e, e-meeting, October 2021.

R1-2110573, “Consolidated higher layers parameter list for Rel-17 NR”, Moderator (Ericsson), RAN1#106bis-e, e-meeting, October 2021.

R1-2112650, “[107-e-R17-RRC-Sidelink] Summary of email discussion on Rel-17 RRC parameters for sidelink enhancement”, Moderator (LG Electronics), RAN1#107-e, e-meeting, November 2021.

R1-212976, “Consolidated higher layers parameter list for Rel-17 NR”, Moderator (Ericsson), RAN1#107-e, e-meeting, November 2021.

R1-2200807, “[107bis-e-R17-RRC-Sidelink] Summary of email discussion on Rel-17 RRC parameters for sidelink enhancement”, Moderator (LG Electronics), RAN1#107bis-e, e-meeting, January 2022.

R1-2200702, “Collection of updated higher layers parameter list for Rel-17 NR”, Moderator (Ericsson), RAN1#107bis-e, e-meeting, January 2022.