3GPP TSG RAN WG1 #108-e	R1-2202045

# e-Meeting, February 21st – March 3rd, 2022

Agenda item:	8.16.8

Source:	Samsung

Title:	UE features for NR Coverage Enhancement

Document for:	Discussion and decision

- Introduction

This contribution discusses the remaining issues on UE features for Rel-17 Coverage Enhancement WI based on the latest RAN1 UE features list [1].

- Whether/how to separate/merge FGs

So far, the major discussion point of FG for coverage enhancement was whether to split/merge the corresponding FGs. During the discussion, it was pointed out that RAN2 guidelines [2] had provided to avoid defining functionality that has no RRC configuration but is dependent on capability bits (copied below for the reference). Unless otherwise justified, it is still valid for Rel-17 UE capability discussion:

RAN2 guidelines for UE capability definitions [2] … 5 Avoid defining functionality that has no RRC configuration but is dependent on capability bits. The specification should not be written so that the network determines what configuration it can use for a UE implicitly by the reported UE capabilities. Instead, the gNB should always configure the UE explicitly by DL RRC signalling, respecting the reported capabilities. A problematic case in Rel-15 was the UL/DL MIMO layers, which resulted in a late-stage introduction of explicit MIMO signalling support by RAN2 (maxLayersMIMO-Indication).

In previous meetings, RAN1 has kept debating whether to split the FGs 30-1/1a, 30-2/2a, and 30-3 according to DG and CG.

In case of PUSCH repetition Type A enhancements, RAN1 has agreed the introduction of repK-r17 applicable to Type 1 CG-PUSCH and Type 2 CG-PUSCH while numberOfRepetitions-r17 is not applicable to Type 1 CG-PUSCH repetition Type A. This seems to resolve the raised concern during RAN1#107-e with respect to above RAN2 guidelines.

In this regard, we are fine to keep current FG structure for FG 30-1/1a/2/2a for the sake of progress. Further, we suggest to remove FFSs in FG 30-1a/2a.

Proposal 1: Keep current FG structure for FG 30-1/1a/2/2a and remove FFSs.

For FG 30-3, we still do not see the need to split current FG. It was argued that FG associated with DG and CG were split in Rel-15 (e.g., FG 5-16/5-17). On the contrary, in Rel-16, a single capability (FG 11-5) consists of DG and CG. Moreover, NO SPLIT would be compliant with above RAN2 guidelines [2].

Proposal 2: Keep current FG structure for FG 30-3 and remove FFS.

- Feature Groups for DM-RS bundling

FG 30-4 ([The maximum duration for DM-RS bundling]) has two FFS points:

- FFS dependence on modulation order

- FFS dependence on back-to-back vs. non-back-to-back repetitions

According to recent RAN4 LS [3], there is a possibility that a single value of maximum duration for DM-RS bundling per band is supported for the capability. However, it is not finally decided yet in RAN4. In addition, it seems that the aspects of modulation order and (non) back-to-back repetitions are not the discussion point from RAN4 perspective:

In an earlier LS RAN4 indicated that up to 32 slots was being discussed. RAN4 will further discuss the feasible value(s) for maximum duration and has considered the following: UE reports the single value per band from a set of up to 4 values, and RAN4 does not consider the value more than 32 slots for the capability for maximum duration. Values RAN4 being considered are 5, 8, 16 or 32 slots.

Observation: With respect to the maximum duration for DM-RS bundling, RAN4 discussion is still on-going which includes UE reporting of single value per band according to RAN4 LS [3].

In RAN1#107bis-e, it was raised that FG 30-4g did not capture the relevant RAN1 agreement, i.e., UE capability of restarting DMRS bundling is applied only to dynamic events. RAN1 discussed the similar point for corresponding RRC parameter, PUSCH-Window-Restart, and ended up with the following agreement [4]:

PUSCH-Window-Restart: UE bundles PUSCH DM-RS remaining in a nominal time domain window after event(s) triggered by DCI or MAC-CE that violate power consistency and phase continuity requirements. Note: Events which are triggered by DCI or MAC CE, but regarded as semi-static events, e.g. frequency hopping, UL beam switching for multi-TRP operation, or other if defined, are excluded.

Therefore, we propose to take the same approach for FG 30-4g.

Proposal 3: Update 30-4g as following in alignment with the corresponding RRC parameter description.

| 30-4g | Restart DM-RS bundling after the events that violate power consistency and phase continuity | Support restarting DM-RS bundling after the events triggered by DCI or MAC-CE that violate power consistency and phase continuity Note: Events which are triggered by DCI or MAC CE, but regarded as semi-static events, e.g. frequency hopping, UL beam switching for multi-TRP operation, or other if defined, are excluded. |
| --- | --- | --- |

- Mandatory/optional

Like other FGs for Rel-17 Coverage Enhancement, FG 30-6 should be “Optional with capability signaling”.

Proposal 4: Set “Optional with capability signaling” for FG 30-6.

- Conclusions

We propose the following based on above discussion:

Proposal 1: Keep current FG structure for FG 30-1/1a/2/2a and remove FFSs.

Proposal 2: Keep current FG structure for FG 30-3 and remove FFS.

Proposal 3: Update 30-4g as following in alignment with the corresponding RRC parameter description.

| 30-4g | Restart DM-RS bundling after the events that violate power consistency and phase continuity | Support restarting DM-RS bundling after the events triggered by DCI or MAC-CE that violate power consistency and phase continuity Note: Events which are triggered by DCI or MAC CE, but regarded as semi-static events, e.g. frequency hopping, UL beam switching for multi-TRP operation, or other if defined, are excluded. |
| --- | --- | --- |

Proposal 4: Set “Optional with capability signaling” for FG 30-6.

In addition, we provide the following observation:

Observation: With respect to the maximum duration for DM-RS bundling, RAN4 discussion is still on-going which includes UE reporting of single value per band according to RAN4 LS [3].

References:

- R1-2200780, Updated RAN1 UE features list for Rel-17 NR after RAN1#107bis-e, Moderators (AT&T, NTT DOCOMO, INC.)

- R1-2001513, Guidelines for UE capability definitions, RAN2

- R1-2200908 (R4-2202368), Reply LS on Maximum duration for DMRS bundling, RAN4

- R1-2200699, Consolidated higher layers parameter list for Rel-17 NR, Moderator (Ericsson)

# Annex: Updated RAN1 UE features list for Rel-17 NR after RAN1#107bis-e [1]

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30. NR_cov_enh | 30-1 | Increased maximum number of dynamic grant PUSCH Type A repetitions | K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions. The number of repetitions is indicated in a TDRA list. A row index of the TDRA list is indicated by a DCI. | [5-17] | Yes | N/A | UE does not support more than 16 repetitions for dynamic grant PUSCH. | [Per UE] | No | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-1a | Increased maximum number of Type 2 configurecd grant PUSCH Type A repetitions | K = 1, 2, 3, 4, 7, 8, 12, 16, 20, 24, 28, 32 times repetitions. The number of repetitions is indicated in a TDRA list. A row index of the TDRA list is indicated by a Type 2 configured grant configuration. FFS whether to merge with FG 30-1 FFS whether to have a separate FG for CG (including both Type 1 and Type 2) with repK-r17 | [5-16], [30-1] | Yes | N/A | UE does not support more than 16 repetitions for Type 2configurecd grant PUSCH. | [Per UE] | No | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-2 | Dynamic grant PUSCH Type A repetitions based on available slots | Transmission occasions for K repetitions are determined on the basis of available slots. | [5-17] | Yes | N/A | UE does not support dynamic grant PUSCH repetitions counted on the basis of available slots. | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-2a | Configurecd grant PUSCH Type A repetitions based on available slots | Transmission occasions for K repetitions for configured grant PUSCH are determined on the basis of available slots. FFS whether to merge with FG 30-2 | [5-14 or 5-16], [30-2] | Yes | N/A | UE does not support configured grant PUSCH repetitions counted on the basis of available slots. | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-3 | TB processing over multi-slot PUSCH | Support of TB processing over multi-slot PUSCH for DG and CG in RRC connected mode. FFS whether to split FG 30-3 into at least 2 separate FGs: 1st one for DG, 2nd one for CG | [11-6] | Yes | N/A | UE does not support TB processing over multi-slot PUSCH. | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-3a | Repetition of TB processing over multi-slot PUSCH | Support repetition of TB processing over multi-slot PUSCH in RRC connected mode. | TBD | Yes | N/A |  | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4 | [The maximum duration for DM-RS bundling] | The maximum duration during which UE is able to maintain power consisitency and phase continuity to support DM-RS bundling for PUSCH/PUCCH FFS dependence on modulation order FFS dependence on back-to-back vs. non-back-to-back repetitions |  | Yes | N/A | UE does not support DM-RS bundling for PUSCH/PUCCH | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4a | DM-RS bundling for PUSCH repetition type A | Support DM-RS bundling for PUSCH repetition type A | [30-4], [30-1] or [30-2] | Yes | N/A | UE does not Support DM-RS bundling for PUSCH repetition type A | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4b | DM-RS bundling for PUSCH repetition type B | Support DM-RS bundling for PUSCH repetition type B | [30-4], [11-5] [30-1] | Yes | N/A | UE does not Support DM-RS bundling for PUSCH repetition type B | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4c | DM-RS bundling for TB processing over multi-slot PUSCH | Support DM-RS bundling for TB processing over multi-slot PUSCH | [30-4], [30-3] | Yes | N/A | UE does not Support DM-RS bundling for TB processing over multi-slot PUSCH | [Per UE] | FFS | No | N/A | FFS whether to Add a note in FG 30-4c: “Note: If a UE reports support of FG [30-4a], 30-4c, [30-3] and/or 30-3a, the UE supports DMRS bundling for the repetitions of TBoMS” | Optional with capability signalling |
| 30. NR_cov_enh | 30-4d | DMRS bunding for PUCCH repetitions | Support DM-RS bundling for PUCCH repetitions for PUCCH formats 1/3/4 | [30-4], [4-23] | Yes | N/A | UE does not support DMRS bunding for PUCCH repetitions | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4e | Enhanced inter-slot frequency hopping with inter-slot bundling for PUSCH | Support enhanced inter-slot frequency hopping with inter-slot bundling for PUSCH | [30-4a] or [30-4b] or [30-4c] | Yes | N/A | UE does not support enhanced inter-slot frequency hopping with inter-slot bundling for PUSCH | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4f | Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling | Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling | [30-4d] | Yes | N/A | UE does not support Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4g | Restart DM-RS bundling after the events that violate power consistency and phase continuity | Support restarting DM-RS bundling after the events that violate power consistency and phase continuity | [30-4] | Yes | N/A | UE does not support restarting DM-RS bundling after the events that violate power consistency and phase continuity | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4h | DM-RS bundling for non-back-to-back transmission | Support DM-RS bundling for non-back-to-back transmission for consecutive slots for PUSCH and PUCCH only for corresponding supported back-to-back transmission FGs (30-4a, 30-4b, 30-4c, or 30-4d) | 30-4a, 30-4b, 30-4c, or 30-4d | Yes | N/A | UE does not Support DM-RS bundling for non-back-to-back transmission | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-5 | Slot based dynamic PUCCH repetition indication | Support slot based dynamic PUCCH repetition indication for PUCCH formats 0/1/2/3/4 support slot based dynamic PUCCH repetition for PUCCH formats 0/1/2/3/4 FFS whether to split FG 30-5 into 2 FGs; one for PUCCH formats 0/2 and the other for PUCCH formats 1/3/4 | 4-23 and/or 25-2 | Yes | N/A | UE does not support Dynamic PUCCH repetition indication | [Per UE] | FFS | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-6 | Repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI | Support of repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI |  | Yes | N/A | UE does not support repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI. | [Per UE] | No | No | N/A |  | [Optional with capability signalling] |