3GPP TSG RAN WG1 #108-e			R1-2202053

e-Meeting, Feb. 21st – Mar. 3rd, 2022

Agenda item:	8.16.7

Source: 	MediaTek Inc.

Title: 	On UE features for UE power saving enhancements

Document for:	Discussion and decision

- Introduction

This paper provides our views on Rel-17 UE features in R1-2200781 (Updated RAN1 UE features list for Rel-17 NR after RAN1 #107bis-e) for UE power saving enhancements. Detailed proposals are described in the following sections.

- Views on UE features for idle mode UE power saving enhancements

In RAN1 #107bis-e, RAN1 agrees the following draft UE features for idle mode UE power saving enhancements:

Note: yellow highlight above means FFS

For FG 29-1 “Paging enhancement”, it includes both support of paging early indication and support of UE subgrouping indication. For support of UE subgrouping indication, RAN2 agrees that

- UE’s capability of supporting the UE ID based subgrouping is reported to RAN by AS UE capability signalling while R2 assumes that UE’s capability of supporting the CN controlled subgrouping is reported to CN by NAS signalling.

It can be seen that the subgrouping UE capability would be reported to RAN or CN. Besides, in RAN1 #107b-e, the following conclusion is achieved:

Conclusion

For a UE supporting R17 paging enhancement feature, it is up to UE implementation whether the UE monitors the MO(s) for a PEI.

- If UE decides to not to monitor PEI, it has to monitor UE’s PO as defined in 38.304.

- Note: No specification impact

It means that, if in some scenario (ex. some band) UE is not able to support FG 29-1 “Paging enhancement”, UE can just fallback to monitor its PO as defined in 38.304.

Observation 1: For FG 29-1, according to RAN2 agreement, the subgrouping UE capability would be reported to RAN or CN. Besides, according to RAN1 #107b-e conclusion, if in some scenario (ex. some band) UE is not able to support FG 29-1 “Paging enhancement”, UE can just fallback to monitor its PO as defined in 38.304.

Therefore, we have the following proposal:

Proposal 1: For FG 29-1, considering it would be desired to minimize the signalling overhead of subgrouping reported to RAN or CN, and UE can fallback to monitor its PO as defined in 38.304 if in some scenario (ex. some band) UE is not able to support FG 29-1

- The “Type” of FG29-1 should be “per UE”

For FG 29-2 "TRS resources for idle/inactive UEs", the current contents for “Consequence if the feature is not supported by the UE” is “Lose of power saving gain on AGC, time/frequency tracking in idle/inactive mode”, which may be too detailed about UE implementation.

Observation 2: For FG 29-2 "TRS resources for idle/inactive UEs", the current contents for “Consequence if the feature is not supported by the UE” is “Lose of power saving gain on AGC, time/frequency tracking in idle/inactive mode”, which may be too detailed about UE implementation.

Therefore, we have the following proposal:

Proposal 2: For FG 29-2 "TRS resources for idle/inactive UEs", modify the contents of “Consequence if the feature is not supported by the UE” to be

- “UE can not receive TRS resources for idle/inactive mode”

- Views on UE features for connected mode UE power saving enhancements

In RAN1 #107bis-e, RAN1 agrees the following draft UE features for connected mode UE power saving enhancements

FG 29-3d is defined considering the interaction between search group switching and PDCCH skipping, which may require additional spec definition. Therefore, 29-3b and 29-3c are intended to be used without PDCCH skipping configured. This can be described more clear in the components.

Proposal 3:  For 29-3b and 29-3c, adopt the following sentence to “Components”:

- if PDCCHSkippingDurationList is not configured

# Proposed changes on current RAN1 UE features table

# NR_UE_pow_sav_enh (modified contents in red text)

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 29. NR_UE_pow_sav_enh | 29-1 | Paging enhancement | 1. Support paging early indication 2. Support UE subgroup indication |  |  |  | UE does not support paging enhancement | Per UE | N | N | N | For component 2, it is up to RAN2 whether/how to separate the capability for UE subgroup indication Leave RAN2 to decide whether ‘optional with capability signalling’ or ‘optional without capability signalling’ Leave RAN2 to decide whether Need for the gNB to know if the feature is supported is Yes or No | Optional |
| 29. NR_UE_pow_sav_enh | 29-2 | TRS resources for idle/inactive UEs | TRS occassions for idle/inactive UEs 1. Support reading TRS configuration from SIB 2. Support receving L1 indication for TRS availability |  | N |  | Lose of power saving gain on AGC, time/frequency tracking in idle/inactive mode UE can not receive TRS resources for idle/inactive mode | Per UE | N | N | N |  | Optional without capability signalling |
| 29. NR_UE_pow_sav_enh | 29-3a | PDCCH skipping | Support of up to 2-bit indication of PDCCH skipping by scheduling DCI if SSSG is not configured |  | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |
| 29. NR_UE_pow_sav_enh | 29-3b | 2 search space sets group switching | Support of 1-bit indication of SSSG switching between 2 SSSGs by scheduling DCI, and timer based switching, without PDCCH skippingif PDCCHSkippingDurationList is not configured |  | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |
| 29. NR_UE_pow_sav_enh | 29-3c | 3 search space sets group switching | Support of 2-bit indication of SSSG switching among 3 SSSGs by scheduling DCI and timer based switching, if PDCCHSkippingDurationList is not configured | 29-3b | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |
| 29. NR_UE_pow_sav_enh | 29-3d | 2 search space sets group switching with PDCCH skipping | Support of 2-bit indication of SSSG switching between 2 SSSGs with PDCCH skipping by scheduling DCI and timer based switching | 29-3a, 29-3b | Y |  |  | Per UE | N | N | N |  | Optional with capability signaling |

Note that yellow highlight means FFS and to be discussed further. These parts are provides as placeholders.