3GPP TSG RAN WG1 #108-e													    R1-2202064

e-Meeting, February 21st – March 3rd, 2022

Agenda item:	8.16.11

Title:	UE features for NR sidelink enhancements

Source:	MediaTek Inc.

Document for:	Discussion and Decision

# Introduction

Updated RAN1 UE features for Rel-17 after RAN1#107bis-e are available in [1].

The agreements reached in the last RAN1 meeting are available in the Section 7 of the feature lead summary from RAN1-107bis-e [2].

Based on these, we discuss our views on Rel-17 UE features for sidelink enhancements on power saving aspects and mode-2 enhancements in this contribution.

# Discussion

## UE features for SL power saving

In RAN1-107-e, the following working assumption was made on Rel-17 basic FGs.

Working assumption - FGs 32-x are not basic FGs for sidelink enhancement

We agree with the above working assumption and we prefer to confirm it.

Proposal 1: Confirm the following working assumption from RAN1-107-e.

- FGs 32-x are not basic FGs for sidelink enhancement.

In addition, in RAN1-107bis-e, the following working assumption was made on the handling of Rel-16 basic FGs.

Working assumption - Necessary Rel-16 FGs are handled, via pre-requisites of Rel-17 FGs for sidelink enhancements - Necessary components in Rel-16 FGs are added to Rel-17 FG components for sidelink enhancements if an entire Rel-16 FG cannot be included as pre-requisites - FFS whether/how to address the case where pre-requisites or additional components cannot address the issue for FG 15-x supported by Rel-17 UEs

We agree with the above working assumption and we prefer to confirm it.

Proposal 2: Confirm the following working assumption from RAN1-107bis-e,

- Necessary Rel-16 FGs are handled, via pre-requisites of Rel-17 FGs for sidelink enhancements

  - Necessary components in Rel-16 FGs are added to Rel-17 FG components for sidelink enhancements if an entire Rel-16 FG cannot be included as pre-requisites.

### Potential RX capabilities

In RAN1-107-e, the following agreement was made on Rel-17 FGs related to RX capabilities.

Agreement - For Rel-17 SL Rx capabilities, - Remove FG 32-1 from Rel-17 UE feature list - Note: support of receiving NR sidelink of PSCCH/PSSCH, PSFCH, and S-SSB are reported by FG 15-1 (Receiving NR sidelink), FG 15-11 (PSFCH format 0), and FG 15-4 (Synchronization sources for NR sidelink), respectively - FG 32-2 is kept as “Receiving NR sidelink of PSFCH/S-SSB” as follows 32. NR_SL_enh; 32-2; [Receiving NR sidelink of PSFCH/S-SSB only]; 1) UE can receive NR PSFCH/S-SSB only. FFS whether to split the capabilities for PSFCH and S-SSB receptions as different FGs FFS whether other components will be included; None; [Yes]; [No]; [Per band]; N.A.; N.A.; N.A.; Optional with capability signalling. FFS: For UE supports NR sidelink, UE must indicate this FG is supported.. Note that any contents highlighted in yellow mean FFS and to be discussed further. - FFS: If it exists, UE without NR SL reception supports none of FGs 15-14, 15-19, 15-22, 15-23, 15-24, 15-1, 15-11, 15-4, and 32-2. - FFS number of simultaneous PSFCH receptions and/or transmissions and whether/how to report separately or jointly for inter-UE coordination and HARQ-ACK reporting

In RAN1-107bis-e, the following conclusion was reached on the splitting of PSFCH and S-SSB.

Conclusion - There is no consensus in RAN1 on “FFS whether to split the capabilities for PSFCH and S-SSB receptions as different FGs” - Remove the FFS in FG 32-2, highlight FG 32-2

We have a preference to split as FG 32-2 into two FGs. These PHY channels essentially have different functionalities in possibly different user scenarios. A UE may prefer to have RX capability for PSFCH in a performance-sensitive SL use case wherein reliable packet transmissions are essential while reception of S-SSB is not necessary (e.g., in sidelink use cases for in-coverage scenario). Also, support for S-SSB reception may be an important necessity for UE if the coverage conditions are varying, although high system reliability may not be required. For these reasons, in our view, it is preferable to split the reception of PSFCH and S-SSB. We propose the following.

Proposal 3: FG 32-2 should be split into two FGs as “reception of PSFCH” and “reception of S-SSB”.

### Potential TX capabilities

In RAN1-107-e, the following agreement was made on Rel-17 FGs related to TX capabilities.

Agreement - For Rel-17 SL Tx capabilities, - Remove FG 32-3 from Rel-17 UE feature list - Note: support of transmitting NR sidelink mode 2 with full sensing is reported by FG 15-3 (Transmitting NR sidelink mode 2) - FG 32-4 is kept as “Transmitting NR sidelink mode 2 with partial sensing” as follows 32. NR_SL_enh; 32-4; Transmitting NR sidelink mode 2 with partial sensing; 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 with partial sensing configured by NR Uu or preconfiguration. 2) UE can perform periodic-based partial sensing and resource allocation operation. 3) UE can perform contiguous partial sensing and resource allocation operation. FFS whether any other components should be added; [TBD]; [Yes]; [No]; UE does not support transmission according to the partial sensing and resource allocation; [Per band]; N.A.; N.A.; N.A.; Optional with capability signalling. FFS: For UE supports NR sidelink, UE must indicate this FG is supported.. Note that any contents highlighted in yellow mean FFS and to be discussed further. - Add an FG for “Transmitting NR sidelink mode 2 with random resource selection” as follows 32. NR_SL_enh; 32-4a; Transmitting NR sidelink mode 2 with random resource selection; 1) UE can transmit PSCCH/PSSCH using NR sidelink mode 2 with random resource selection configured by NR Uu or preconfiguration. FFS whether any other components should be added; [TBD]; [Yes]; [No]; UE does not support transmission according to the random resource selection and resource allocation; [Per band]; N.A.; N.A.; N.A.; Optional with capability signalling. FFS: For UE supports NR sidelink, UE must indicate this FG is supported.. Note that any contents highlighted in yellow mean FFS and to be discussed further.

In the previous RAN1-106bis-e meeting, there were discussions whether multiple sensing schemes (i.e., any combination of full sensing, partial sensing, and/or random resource selection) should be bundled as a separate UE capability. Hence an FFS point was noted on such TX capability.

Each of these sensing schemes are intended to provide UE with a level of power saving gain at the expense of decreased reliability Each sensing scheme can essentially be applied in a different use case. If UE intends to support a different use case tailored for a different power saving or reliability target, the UE can indicate capability for more than one of these sensing schemes. However, the benefit of supporting a single capability that indicates a set of different sensing schemes is not clear. Each sensing scheme in Rel-17 is designed essentially to be self-sufficient without any additional pre-requisite of supporting another sensing scheme.

With these views, we propose the following.

Proposal 4: Do not support any TX capabilities as bundled with more than one sensing schemes (e.g., {full sensing, partial sensing, random selection}, {partial sensing, random selection}) as FGs for Rel-17 SL.

## UE features for Mode-2 enhancements

For inter-UE coordination, the following changes are preferred considering the different behaviors and support for UE-A and UE-B. Moreover, it is also related to the reception capability in SL power saving session. Thus it is not necessary for UE to receive the explicit request if the UE can’t receive PSSCH/PSCCH or condition based scheme 1 is applied.

Proposal 5: FG 32-5a is split as following Table below.

| Features | Index | Feature group | Components |
| --- | --- | --- | --- |
| 32. NR_SL_enh | 32-5a-1 | Inter-UE coordination scheme 1 in NR sidelink mode 2 (UE-B w/o transmission of the explicit request for scheme 1) | 1) UE can receive inter-UE coordination information of preferred resource set/non-preferred resource set and use the received information in its own resource (re-)selection in NR sidelink mode 2. |
| 32. NR_SL_enh | 32-5a-2 | Inter-UE coordination scheme 1 in NR sidelink mode 2 (UE-B w/ transmission of the explicit request for scheme 1) | 1) UE can transmit explicit request. 2) UE can receive inter-UE coordination information of preferred resource set/non-preferred resource set and use the received information in its own resource (re-)selection in NR sidelink mode 2. |
| 32. NR_SL_enh | 32-5a-3 | Inter-UE coordination scheme 1 in NR sidelink mode 2 (UE-A w/o reception of the explicit request) | 1) UE can transmit inter-UE coordination information of preferred resource set/non-preferred resource set |
| 32. NR_SL_enh | 32-5a-4 | Inter-UE coordination scheme 1 in NR sidelink mode 2 (UE-A w/ reception of the explicit request) | 1) UE can receive explicit request 2) UE can transmit inter-UE coordination information of preferred resource set/non-preferred resource set. |

# Conclusions

We have the following proposals:

Proposal 1: Confirm the following working assumption from RAN1-107-e.

- FGs 32-x are not basic FGs for sidelink enhancement.

Proposal 2: Confirm the following working assumption from RAN1-107bis-e,

- Necessary Rel-16 FGs are handled, via pre-requisites of Rel-17 FGs for sidelink enhancements

  - Necessary components in Rel-16 FGs are added to Rel-17 FG components for sidelink enhancements if an entire Rel-16 FG cannot be included as pre-requisites.

Proposal 3: FG 32-2 should be split into two FGs as “reception of PSFCH” and “reception of S-SSB”.

Proposal 4: Do not support any TX capabilities as bundled with more than one sensing schemes (e.g., {full sensing, partial sensing, random selection}, {partial sensing, random selection}) as FGs for Rel-17 SL.

Proposal 5: FG 32-5a is split as following Table below.

| Features | Index | Feature group | Components |
| --- | --- | --- | --- |
| 32. NR_SL_enh | 32-5a-1 | Inter-UE coordination scheme 1 in NR sidelink mode 2 (UE-B w/o transmission of the explicit request for scheme 1) | 1) UE can receive inter-UE coordination information of preferred resource set/non-preferred resource set and use the received information in its own resource (re-)selection in NR sidelink mode 2. |
| 32. NR_SL_enh | 32-5a-2 | Inter-UE coordination scheme 1 in NR sidelink mode 2 (UE-B w/ transmission of the explicit request for scheme 1) | 1) UE can transmit explicit request. 2) UE can receive inter-UE coordination information of preferred resource set/non-preferred resource set and use the received information in its own resource (re-)selection in NR sidelink mode 2. |
| 32. NR_SL_enh | 32-5a-3 | Inter-UE coordination scheme 1 in NR sidelink mode 2 (UE-A w/o reception of the explicit request) | 1) UE can transmit inter-UE coordination information of preferred resource set/non-preferred resource set |
| 32. NR_SL_enh | 32-5a-4 | Inter-UE coordination scheme 1 in NR sidelink mode 2 (UE-A w/ reception of the explicit request) | 1) UE can receive explicit request 2) UE can transmit inter-UE coordination information of preferred resource set/non-preferred resource set. |

# References

- R1-2200780, Updated RAN1 UE features list for Rel-17 NR after RAN1 #107bis-e, Moderators (AT&T, NTT DOCOMO, INC.), January 2022.

- Chairman’s notes, RAN1-107bis-e, January 2022.